3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230321

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3050 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC MDT test case 8.1.6.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For TC_8_1_6_1_1_2, SS shall configureTest Loop mode A as per test spec, the current implementation uses Test Loop mode B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated to configure Test Loop mode A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function f_MultipleQoSPreamble_Part1() |
| --- | --- |
| Reason for change | For TC_8_1_6_1_1_2, SS shall configureTest Loop mode A as per test spec, the current implementation uses Test Loop mode B. |
| Summary of change | Updated common function f_MultipleQoSPreamble_Part1 to take parameter to control if test loop mode A or B to use |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted in principle, but passed the test mode as a parameter, rather than a Boolean (please see MCC160 Implementation) |

Before Change

function f_MultipleQoSPreamble_Part1 (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return GSM_MobilityInfo_Type { var NR_SRB_COMMON_IND v_ReceivedAsp; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; f_NR5GC_ProvideSNPNData(); // @sic R5-225300 sic@ // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_UT_SwitchOnUE(UT); f_NR5GC_Registration(p_NR_CellId, TEST_LOOPModeB_ON, Initial_NoSecurity); if (pc_noOf_PDUsSameConnection == 0) { // @sic R5s200751, R5s201358 sic@ // UE will release signalling connection after T3540 // so release connection and allow it to move back to connected @sic R5s220107 sic@ f_NR_RRCRelease(p_NR_CellId); f_NR5GC_RRC_Idle_Extension_Steps1_7 (p_NR_CellId); } ......... ......... }

After Change

function f_MultipleQoSPreamble_Part1 (NR_CellId_Type p_NR_CellId, boolean p_TestLoopModeA := false) runs on NR5GC_PTC return GSM_MobilityInfo_Type { var NR_SRB_COMMON_IND v_ReceivedAsp; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; f_NR5GC_ProvideSNPNData(); // @sic R5-225300 sic@ // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_UT_SwitchOnUE(UT); if( v_TestLoopModeA ) { f_NR5GC_Registration(p_NR_CellId, TEST_LOOPModeA_ON, Initial_NoSecurity); } else { f_NR5GC_Registration(p_NR_CellId, TEST_LOOPModeB_ON, Initial_NoSecurity); } if (pc_noOf_PDUsSameConnection == 0) { // @sic R5s200751, R5s201358 sic@ // UE will release signalling connection after T3540 // so release connection and allow it to move back to connected @sic R5s220107 sic@ f_NR_RRCRelease(p_NR_CellId); f_NR5GC_RRC_Idle_Extension_Steps1_7 (p_NR_CellId); } ......... ......... }

#### MCC160 Implementation:

function f_MultipleQoSPreamble_Part1 (NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TEST_LOOPModeB_ON) runs on NR5GC_PTC return GSM_MobilityInfo_Type

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

f_NR5GC_ProvideSNPNData(); // @sic R5-225300 sic@

// turn on NR cell

f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

// Switch on UE

f_UT_SwitchOnUE(UT);

f_NR5GC_Registration(p_NR_CellId, p_TestMode, Initial_NoSecurity);

if (pc_noOf_PDUsSameConnection == 0) { // @sic R5s200751, R5s201358 sic@

// UE will release signalling connection after T3540

// so release connection and allow it to move back to connected @sic R5s220107 sic@

f_NR_RRCRelease(p_NR_CellId);

f_NR5GC_RRC_Idle_Extension_Steps1_7 (p_NR_CellId);

}

// Receive PDU Establishment

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

return v_GSM_MobilityInfo_Type;

}

## Change 2

| Function name | function f_TC_8_1_6_1_1_2_NR5GC() |
| --- | --- |
| Reason for change | As change 1 |
| Summary of change | Updated to configure Test Loop mode A |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, but passed TEST_LOOPModeA_ON, rather than true (please see MCC160 Implementation) |

Before Change

function f_TC_8_1_6_1_1_2_NR5GC() runs on NR5GC_PTC { //Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB ......... ......... f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1. // and using the message condition UE TEST LOOP MODE A active. 3 DRBs are configured where DRB 1 is defined as default DRB. // Perform Registration and get PDU Establishment Request message v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1); v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId); //Get DRB identities v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB v_FirstDRB := v_DRBList[0]; v_SecondDRB := v_DRBList[1]; v_ThirdDRB := v_DRBList[2]; ......... ......... }

After Change

function f_TC_8_1_6_1_1_2_NR5GC() runs on NR5GC_PTC { //Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB ......... ......... f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1. // and using the message condition UE TEST LOOP MODE A active. 3 DRBs are configured where DRB 1 is defined as default DRB. // Perform Registration and get PDU Establishment Request message v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1, true); v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId); //Get DRB identities v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB v_FirstDRB := v_DRBList[0]; v_SecondDRB := v_DRBList[1]; v_ThirdDRB := v_DRBList[2]; ......... ......... }

#### MCC160 Implementation:

function f_TC_8_1_6_1_1_2_NR5GC() runs on NR5GC_PTC { //Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB ......... ......... f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1. // and using the message condition UE TEST LOOP MODE A active. 3 DRBs are configured where DRB 1 is defined as default DRB. // Perform Registration and get PDU Establishment Request message v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1, TEST_LOOPModeA_ON); v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId); //Get DRB identities v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB v_FirstDRB := v_DRBList[0]; v_SecondDRB := v_DRBList[1]; v_ThirdDRB := v_DRBList[2]; ......... ......... }