3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230371

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4731 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTE CA test cases 8.5.4.2 and 8.5.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UE does not support extendedBands (i.e. band no. > 64), the SupportedBandCombination_v1090 may not be included. The current TTCN reads SupportedBandCombination_v1090 regardless, and causes error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated to read SupportedBandCombination_v1090 if present |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2, 8.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_5_4_2or3_RequestedFrequencyBandList () |
| --- | --- |
| Reason for change | For UE does not support extendedBands (i.e. band no. > 64), the SupportedBandCombination_v1090 may not be included. The current function reads SupportedBandCombination_v1090 regardless, and causes error. |
| Summary of change | Updated to read SupportedBandCombination_v1090 if present |
| TTCN module | ATS_LTE_A_R10_r11\8_5\RRC_Others_CA.ttcn |
| MCC160 Comment | Accepted in principle MCC160 implementation see below. |

Before Change

function fl_TC_8_5_4_2or3_RequestedFrequencyBandList(UE_EUTRA_Capability p_ReceivedCapabilityMsg, TC_8_5_4_2or3_TestVariable_Type p_TestVariable) return RequestedFrequencyBands_r11_Type { var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var RequestedFrequencyBands_r11_Type v_RequestedFrequencyBands; var integer v_FreqBandIndicator; //@sic R5-163085 FrequencyBandIndicator type changed to integer sic@ var SupportedBandCombination_v1090 v_SupportedBandCombination_v1090 := p_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension. nonCriticalExtension.nonCriticalExtension.rf_Parameters_v1090.supportedBandCombination_v1090;//@sic R5s221245 sic@ var integer i; var integer k; v_RequestedFrequencyBands[0] := p_TestVariable.B3; v_RequestedFrequencyBands[1] := p_TestVariable.B1; i := 2; for (k := 0; (k < lengthof(v_SupportedBandListEUTRA)) and (i < 16); k := k+1) { // v_RequestedFrequencyBands shall have a maximum of 16 entries v_FreqBandIndicator := v_SupportedBandListEUTRA[k].bandEUTRA; if ((v_FreqBandIndicator != p_TestVariable.B1) and (v_FreqBandIndicator != p_TestVariable.B3)) { if (v_FreqBandIndicator < 64 ){ // 64 == maxFBI used as placeholder v_RequestedFrequencyBands[i] := v_FreqBandIndicator; } else { v_RequestedFrequencyBands[i] := v_SupportedBandCombination_v1090[i][0].bandEUTRA_v1090; //@sic R5s221245 sic@ } i := i + 1; } } return v_RequestedFrequencyBands; }

After Change

function fl_TC_8_5_4_2or3_RequestedFrequencyBandList(UE_EUTRA_Capability p_ReceivedCapabilityMsg, TC_8_5_4_2or3_TestVariable_Type p_TestVariable) return RequestedFrequencyBands_r11_Type { var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var RequestedFrequencyBands_r11_Type v_RequestedFrequencyBands; var integer v_FreqBandIndicator; //@sic R5-163085 FrequencyBandIndicator type changed to integer sic@ var SupportedBandCombination_v1090 v_SupportedBandCombination_v1090; var integer i; var integer k; v_RequestedFrequencyBands[0] := p_TestVariable.B3; v_RequestedFrequencyBands[1] := p_TestVariable.B1; if(ispresent( p_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v1090.supportedBandCombination_v1090)) { v_SupportedBandCombination_v1090 := p_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v1090.supportedBandCombination_v1090; } i := 2; for (k := 0; (k < lengthof(v_SupportedBandListEUTRA)) and (i < 16); k := k+1) { // v_RequestedFrequencyBands shall have a maximum of 16 entries v_FreqBandIndicator := v_SupportedBandListEUTRA[k].bandEUTRA; if ((v_FreqBandIndicator != p_TestVariable.B1) and (v_FreqBandIndicator != p_TestVariable.B3)) { if (v_FreqBandIndicator < 64 ){ // 64 == maxFBI used as placeholder v_RequestedFrequencyBands[i] := v_FreqBandIndicator; } else { v_RequestedFrequencyBands[i] := v_SupportedBandCombination_v1090[i][0].bandEUTRA_v1090; //@sic R5s221245 sic@ } i := i + 1; } } return v_RequestedFrequencyBands; }

MCC160 Implementation

(Proposed solution is not handling absence of v_SupportedBandCombination_v1090 )

function fl_TC_8_5_4_2or3_RequestedFrequencyBandList(UE_EUTRA_Capability p_ReceivedCapabilityMsg, TC_8_5_4_2or3_TestVariable_Type p_TestVariable) runs on EUTRA_PTC return RequestedFrequencyBands_r11_Type { var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var RequestedFrequencyBands_r11_Type v_RequestedFrequencyBands; var integer v_FreqBandIndicator; //@sic R5-163085 FrequencyBandIndicator type changed to integer sic@ var SupportedBandCombination_v1090 v_SupportedBandCombination_v1090; var integer i; var integer k; v_RequestedFrequencyBands[0] := p_TestVariable.B3; v_RequestedFrequencyBands[1] := p_TestVariable.B1; i := 2; for (k := 0; (k < lengthof(v_SupportedBandListEUTRA)) and (i < 16); k := k+1) { // v_RequestedFrequencyBands shall have a maximum of 16 entries v_FreqBandIndicator := v_SupportedBandListEUTRA[k].bandEUTRA; if ((v_FreqBandIndicator != p_TestVariable.B1) and (v_FreqBandIndicator != p_TestVariable.B3)) { if (v_FreqBandIndicator < 64 ){ // 64 == maxFBI used as placeholder v_RequestedFrequencyBands[i] := v_FreqBandIndicator; } else { //@sic R5s230371 sic@ if(ispresent( p_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v1090.supportedBandCombination_v1090)) { v_SupportedBandCombination_v1090 := p_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v1090.supportedBandCombination_v1090; } v_RequestedFrequencyBands[i] := v_SupportedBandCombination_v1090[i][0].bandEUTRA_v1090; //@sic R5s221245 sic@ } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.5.4.2/3 Step 2/4 - Inconsistent FreqBandIndicator information"); } } i := i + 1; } } return v_RequestedFrequencyBands; }