3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230497

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3118 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC 5GSM test case 10.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 1 the UE may transmit RRCReconfigurationComlpete and PDU SESSION RELEASE COMPLETE at around the same time. Current TTCN implementation will fail if PDU SESSION RELEASE COMPLETE is received at TTCN first. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add interleave block to allow the messages to be received in either order. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_10_1_4_1_Body() |
| --- | --- |
| Reason for change | At step 1 the UE may transmit RRCReconfigurationComlpete and PDU SESSION RELEASE COMPLETE at around the same time. Current TTCN implementation will fail if PDU SESSION RELEASE COMPLETE is received at TTCN first. |
| Summary of change | Add interleave block to allow the messages to be received in either order. |
| TTCN module | GSM_NR5GC.ttcn. |
| MCC160 Comment | Accepted. See MCC160 implementation below |

Before Change:

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToRelease; ………………. //The UE transmits a RRCReconfigurationComplete SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); //The UE transmits a PDU SESSION RELEASE COMPLETE message? SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session Release Complete"); } ……………. //@siclog "Steps 2 - 15 void" siclog@ /*Skipping the code */ }

After Change:

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToRelease; ……………………… //The UE transmits a RRCReconfigurationComplete interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) {} // The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session Release Complete"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1"); //@siclog "Steps 2 - 15 void" siclog@ ……………. }

MCC160 Implementation

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ?

var O1_Type v_PDUSessionIdToRelease;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var template (omit) DNN v_RequestedDNN;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_Count := 0;

var float v_T3580Val := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 16.0);

var charstring v_NSSAI := ""; // @sic R5-213490 sic@

var charstring v_DNN := ""; // @sic R5-213490 sic@

var template (omit)S_NSSAI_Type v_RequestedNSSAI; // @sic R5s210148 sic@

timer t_Wait;

f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN);// @sic R5-217818 sic@

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5-217818 sic@

v_PDUSessionIdToRelease := v_PDUSessionInfoList[f_NR5GC_MobileInfo_GetNoOfSessions()-2].SessionId; // @sic R5-217818 sic@

//v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

// @sic R5-213490 sic@

if (isvalue(v_RequestedNSSAI)){ // compiler warning may be ignored due to check

// @sic R5s201028 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI.lvPart); // @sic R5s221179 Baseline Moving sic@

}

if (isvalue(v_RequestedDNN)){

// @sic R5s210437 sic@

v_DNN := oct2char(valueof(substr((v_RequestedDNN.dnnValue),1,lengthof(v_RequestedDNN.dnnValue)-1)));

}

// @sic R5-206357 sic@

//@siclog "Steps 1" siclog@

// The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X.

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA,  // @sic R5s230497 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause "Reactivation requested"

//@siclog "Steps 2 - 15 void" siclog@

//@siclog "Steps 16" siclog@

//@sic R5-202664 sic@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2).

t_Wait.start (1.0);

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

t_Wait.stop;

}

[]t_Wait.timeout

{

// @sic R5-213490 sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, v_DNN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

}

}

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

//@siclog "Steps 17-25" siclog@

for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) {

//@siclog "Steps 17-19-21-23" siclog@

//The SS waits 16 seconds (T3580).

f_Delay(v_T3580Val);

//@siclog "Steps 18-20-22-24" siclog@

//Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message sent in step 16?

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -,  v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

//@siclog "Step 25" siclog@

//The SS waits 16 seconds (5th expiry of T3580).

f_Delay(v_T3580Val);

//@siclog "Step 26" siclog@

t_Wait.start(20.0);

alt{

[] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu,-, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN))// @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

[]t_Wait.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26");

}

}

} //End of test body of TC_10_1_4_1