3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230533

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3138 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Jun'23 partial baseline upgrade for 5GS TTCN-3 Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2023-07-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to develop some of the Rel-17 5GS test cases in TTCN-3, the current baseline of type definitions shall be upgraded for NR RRC ASN.1 (TS 38.331) and for 5GC NAS (TS 24.501). It is proposed to upgrade to June 2023 release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline upgrade to June 2023 Rel-17. The changes of the type definitions in NR ASN.1 have been taken from TS 38.331-h50. The changes of the type definitions in 5GC NAS have been taken from TS 24.501-hb1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some Rel-17 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 5GS Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Added clause 3.2 to include updates made for restrictions of objects. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	2

1	Baseline Moving – 38.331 ASN.1	2

1.1	NR_RRC_ASN1_Definitions	2

1.1.1	Type change of nonCriticalExtension	2

1.1.2	Name changes	3

1.1.3	Fields added with extension marker (ellipsis)	5

1.1.4	Further ASN.1 changes	7

1.2	NR_SideLink_Preconf	7

1.3	NR_PC5_RRC_Definitions	7

1.3.1	Further ASN.1 changes	7

2	Baseline Moving – 24.501 (NAS)	7

2.1	Modified Message Definitions	7

2.2	Modified Information Elements	7

2.3	New Information Elements	8

3	Other changes	8

3.1	Streamlined code in TC 11.3.7	8

3.2            Updates on object restrictions…………………………………………………………………………………11

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# 1	Baseline Moving – 38.331 ASN.1

## 1.1	NR_RRC_ASN1_Definitions

### 1.1.1	Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| SIB1-v1700-IEs | SIB1-v1740-IEs |
| UE-MRDC-Capability-v1700 | UE-MRDC-Capability-v1730 |
| UE-NR-Capability-v16a0 | UE-NR-Capability-v16c0 |
| UE-NR-Capability-v1700 | UE-NR-Capability-v1740 |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| SRS-PosRRC-InactiveConfig-r17 | Field renamed from inactivePosSRS-RSRP-changeThreshold-r17 to inactivePosSRS-RSRP-ChangeThreshold-r17 | No impact |
| UEAssistanceInformation-v1700-IEs | Field renamed from scg-DeactivationPreference to scg-DeactivationPreference-r17 | Field renamed in cr_UEAssistanceInformation_v1700_IEs |
| ApplicableDisasterInfo-r17 | Field renamed from disasterRoamingFromAnyPLMN-r17 to disasterRelatedIndication-r17 | No impact |
| TCI-UL-State-Id-r17 | Type renamed to UL-StateId-r17 | No impact |
| DeactivatedSCG-Config-r17 | Field renamed from bfd-and-RLM to bfd-and-RLM-r17 | No impact |
| ControlResourceSet | Field renamed from followUnifiedTCIstate-r17 To followUnifiedTCI-State-r17 | Field renamed in cs_38508_ControlResourceSet |
| DL-PPW-PreConfig-r17 | Field renamed from dl-PPW-Periodicity-and-StartSlot-r17 To dl-PPW-PeriodicityAndStartSlot-r17 | No impact |
| DL-PPW-Periodicity-and-StartSlot-r17 | Type renamed to DL-PPW-PeriodicityAndStartSlot-r17 | No impact |
| PDCCH-ConfigCommon | Field renamed from followUnifiedTCIstate-v1720 To followUnifiedTCI-State-v1720 | Field renamed in cs_38508_PDCCH_ConfigCommon |
| PDSCH-Config | Fields renamed respectively from dl-OrJoint-TCIStateList-r17 pdsch-TimeDomainAllocationListForMultiPDSCH-r17 To dl-OrJointTCI-StateList-r17 dummy | Fields renamed in cs_NR_PDSCH_ConfigDef |
| MultiPDSCH-TDRA-List-r17 | Type renamed to Dummy-TDRA-List | No impact |
| MulticastConfig-r17 | Field renamed from type1-Codebook-GenerationMode-r17 To type1CodebookGenerationMode-r17 | No impact |
| PUCCH-ResourceExt-v1610 | Field renamed from formatExt-v1700 to format-v1700 | No impact |
| PUSCH-PathlossReferenceRS-v1710 | Type renamed to DummyPathlossReferenceRS-v1710 | No impact |
| PUSCH-PowerControl-v1610 | Field renamed from pathlossReferenceRSToAddModListExt-v1710 to dummy | No impact |
| RadioLinkMonitoringConfig | Field renamed from beamfailure-r17 to beamFailure-r17 | Field renamed in cs_38508_RadioLinkMonitoringConfigDef cs_RadioLinkMonitoringConfig |
| SI-SchedulingInfo-v1700 | Field renamed from si-RequestConfigRedCap-r17 to dummy | Field renamed in cs_SI_SchedulingInfo_v1700_Def |
| SRS-ResourceSet | Field renamed from followUnifiedTCIstateSRS-r17 to followUnifiedTCI-StateSRS-r17 | Field renamed in cs_38508_SRS_ResourceSet |
| SRS-Resource | Field renamed from srs-TCIState-r17 to srs-TCI-State-r17 | Field renamed in cs_38508_SRS_Resource |
| TCI-UL-State-r17 | Field renamed from tci-UL-State-Id-r17 to tci-UL-StateId-r17 | No impact |
| TCI-UL-State-Id-r17 | Type renamed to TCI-UL-StateId-r17 | No impact |
| CodebookParametersfetype2-r17 | Fields renamed respectively from fetype2Rank1-r17 fetype2Rank2-r17 To fetype2R1-r17 fetype2R2-r17 | No impact |
| CodebookParametersfetype2PerBC-r17 | Fields renamed respectively from fetype2Rank1-r17 fetype2Rank2-r17 To fetype2R1-r17 fetype2R2-r17 | No impact |
| MIMO-ParametersPerBand | Field renamed from maxNumberConfiguredTCIstatesPerCC to maxNumberConfiguredTCI-StatesPerCC | No impact |
| BandNR | Field renamed from pusch-RepetitionCRC-r17 to pusch-RepetitionMsg3-r17 | Field renamed in fl_Check_Rf_Parameters_NRBand_PICS_Supp |
| SL-RemoteUE-ToAddMod-r17 | Field renamed from sl-SRAP-Config-Relay-r17 to sl-SRAP-ConfigRelay-r17 | No impact |
| Phy-ParametersCommon | Field renamed fro, updated-T-DeltaRangeRecption-r17 to updated-T-DeltaRangeReception-r17 | Field renamed in cr_Phy_ParametersCommon |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| SuspendConfig | ncd-SSB-RedCapInitialBWP-SDT-r17 | Field added as omit in cs_SuspendConfigDef |
| SIB3 | channelAccessMode2-r17 | No impact |
| SIB4 | interFreqCarrierFreqList-v1730 | Field added as omit in cs_38508_SIB4_Def |
| BWP-UplinkDedicated | pathlossReferenceRSToAddModList-r17 pathlossReferenceRSToReleaseList-r17 | Fields added as omit in cs_BWP_UplinkDedicated_Def, cs_NR_SRS_Config |
| ConfiguredGrantConfig | harq-ProcID-Offset-v1730 cg-nrofSlots-r17 | Fields added as omit in cs_ConfiguredGrantConfig_Def |
| CSI-ReportConfig | codebookConfig-v1730 | No impact |
| MAC-CellGroupConfig | posMG-Request-r17 | Field added as omit in cs_NR_MAC_CellGroupConfig |
| SSB-ConfigMobility | cca-CellsToAddModList-r17 cca-CellsToRemoveList-r17 | Fields added as omit in cs_SSB_ConfigMobility |
| PDSCH-Config | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 | Field added as omit in cs_NR_PDSCH_ConfigDef |
| PDSCH-TimeDomainResourceAllocation-r16 | repetitionNumber-v1730 | No impact |
| PhysicalCellGroupConfig | intraBandNC-PRACH-simulTx-r17 pdcch-BlindDetection4-r17 | Fields added as omit in cs_NR_PhysicalCellGroupConfigDef |
| ServingCellConfigCommon | ra-ChannelAccess-r17 | Field added as omit in cs_NR_ServingCellConfigCommonDef |
| ServingCellConfigCommonSIB | enhancedMeasurementLEO-r17 ra-ChannelAccess-r17 | Fields added as omit in cs_38508_ServingCellConfigCommonSIB |
| SRS-Resource | repetitionFactor-v1730 srs-DLorJointTCI-State-v1730 | Fields added as omit in cs_38508_SRS_Resource |
| FeatureSets | featureSetsDownlink-v1730 featureSetsDownlinkPerCC-v1730 | Fields added as * in cr_UE_NR_FeatureSets |
| MeasAndMobParametersCommon | eventD1-MeasReportTrigger-r17 independentGapConfig-maxCC-r17 interSatMeas-r17 deriveSSB-IndexFromCellInterNon-NCSG-r17 | Fields added as * in cr_NR_MeasAndMobParametersCommon |
| MIMO-ParametersPerBand | supportRepNumPDSCH-TDRA-DCI-1-2-r17 | No impact |
| Phy-ParametersCommon | dummy1 dummy2 dummy3 dummy4 srs-AdditionalRepetition-r17 pusch-Repetition-CG-SDT-r17 | Added fields as * in cr_Phy_ParametersCommon |
| RF-Parameters | supportedBandCombinationList-v1730 supportedBandCombinationList-UplinkTxSwitch-v1730 supportedBandCombinationListSL-RelayDiscovery-v1730 supportedBandCombinationListSL-NonRelayDiscovery-v1730 supportedBandCombinationList-v1740 supportedBandCombinationList-UplinkTxSwitch-v1740 | Fields added as * in cr_NR_RF_Parameters |
| BandNR | dmrs-BundlingPUSCH-RepTypeA-r17 dmrs-BundlingPUSCH-RepTypeB-r17 dmrs-BundlingPUSCH-multiSlot-r17 dmrs-BundlingPUCCH-Rep-r17 interSlotFreqHopInterSlotBundlingPUSCH-r17 interSlotFreqHopPUCCH-r17 dmrs-BundlingRestart-r17 dmrs-BundlingNonBackToBackTX-r17 maxDynamicSlotRepetitionForSPS-Multicast-r17 nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17 sps-MulticastDCI-Format4-2-r17 sps-MulticastMultiConfig-r17 priorityIndicatorInDCI-Multicast-r17 priorityIndicatorInDCI-SPS-Multicast-r17 twoHARQ-ACK-CodebookForUnicastAndMulticast-r17 multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17 releaseSPS-MulticastWithCS-RNTI-r17 | No impact |
| RF-ParametersMRDC | supportedBandCombinationList-v1730 supportedBandCombinationListNEDC-Only-v1730 supportedBandCombinationList-UplinkTxSwitch-v1730 supportedBandCombinationList-v1740 supportedBandCombinationListNEDC-Only-v1740 supportedBandCombinationList-UplinkTxSwitch-v1740 | Fields added as * in cr_RF_ParametersMRDC |
| UE-CapabilityRequestFilterCommon | fallbackGroupFiveRequest-r17 | No impact |
| SCellConfig | plmn-IdentityInfoList-r17 npn-IdentityInfoList-r17 | Fields added as omit in cs_SCellConfig_Def |
| ServingCellConfig | crossCarrierSchedulingConfigRelease-r17 | Field added as omit in cs_NR_ServingCellConfigDef |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| CA-BandwidthClassNR | Enumerated values added | No impact |

## NR_SideLink_Preconf

No changes

## NR_PC5_RRC_Definitions

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| SL-SIB-ReqInfo-r17 | Enumerated values renamed | No impact |

# 2	Baseline Moving – 24.501 (NAS)

24.501 is upgraded to 24.501-hb1.

## 2.1	Modified Message Definitions

| PDU Definition | Changes Required | Ref: 3GPP 24.501 clause |
| --- | --- | --- |
| Registration Request | New optional fields: Requested T3512 value | 8.2.6 |

## 2.2	Modified Information Elements

| PDU Definition | Changes Required | Ref: 3GPP 24.501 clause |
| --- | --- | --- |
| 5GMM Capability | New fields: Spare, MPSIU, UAS, RCMAN, RCMAP Removed fields:Spare | 9.11.3.1 |
| Service-level-AA Container | New field: Service-level-AA Service Status Indication | 9.11.2.10 |
| NSAG | Modified field: Length of NSAG changed from 2 to 1 octet | 9.11.3.87 |
| Received MBS Container | New field: IPAT Removed field:Spare | 9.11.4.31 |

## 2.3	New Information Elements

| PDU Definition | Ref: 3GPP 24.501 clause |
| --- | --- |
| Service-level-AA Service Status Indication | 9.11.2.18 NOTE: Jun23 version specifes an IEI of 'B'H, but the IEI is defined as 1 octet. IEI implemented as 1 octet. |

# 3	Other changes

## 3.1	Streamlined code in TC 11.3.7

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC

{

var float v_TimerValue := 30.0;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var float v_T390Max := f_NR_SetTimerToleranceMax(nr_Cell1, rrcTimer, 84.0); // @sic R5-213504, R5s210424 sic@

var SubFrameTiming_Type v_TimingOfLocalRelease;

timer t_WaitTime; // @sic R5-213504, R5s210424 sic@

timer t_T390;

//@siclog "Step 1" siclog@

//Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911).

//NOTE 1: This could be done by e.g. MMI or AT command.

f_UT_RequestIMSEmergencyCall (UT);

t_T390.start(v_T390Max); // @sic R5-213504, R5s210424 sic@

//@siclog "Step 2" siclog@

//Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'?

f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, emergency, "Step 2");

//@siclog "Step 3" siclog@

//@sic R5-211410, R5-213504, R5s210424 sic@

//SS modifies NR Cell 1 SIB1 to remove Access barring for AC2 and sends short message. (NOTE 2, NOTE 3)

//NOTE 2: Since barring is alleviated, UE may re-attempt an emergency call, triggered at step 1, automatically after T390 stop/expiry without manual intervention.

//NOTE 3: Steps 3 to 9 are there to ensure that UE under test supports emergency call and is not initiating it at Step 2 due to access barring for AC2.

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit);

f_NR_ModifySysinfo(nr_Cell1);

//@siclog "Step 4" siclog@

//@sic R5-211410 sic@

//Wait for 2.1* modification period to allow the new system information to take effect

f_NR_WaitModificationPeriods(nr_Cell1);

//@siclog "Step 5" siclog@

//@sic R5-211410, R5-213504, R5s210424, R5-224716 sic@

//Start Timer = 84s (NOTE 2, NOTE 4).

//NOTE 2: Since barring is alleviated, UE may re-attempt an emergency call, triggered at step 1, automatically after T390 stop/expiry without manual intervention.

//NOTE 4: T390 timer value is derived from 38.331[12] T390 = (0.7 + 0.6*rand)*uac-BarringTime where the uac-BarringTime value of 64s is provided in 38.523-1 Table 11.3.7.3.3-1 and where the value of rand is chosen to be 1 as to arrive at the maximum value for T390 of 83.2s rounded up to 84s.

//EXCEPTION: Steps 5Aa1-5Ab3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 5Aa1" siclog@

//@sic R5-224716 sic@

//UE transmits an RRCSetupRequest message  with establishmentCause set to 'highPriorityAccess' to establish an emergency call.(NOTE 7)

//UE may perform domain selection and stop T390. If UE move back to the NR Cell 1 during 84s timer running, UE may initiate emergency call before 84s timer expire.

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(highPriorityAccess,?))))

{

f_NR_RRC_SetupRequest_Def(nr_Cell1, highPriorityAccess); //@sic R5s230152 sic@

//@siclog "Step 5Aa2" siclog@

//Stop Timer = 84s

t_T390.stop;

}

//@siclog "Step 5Ab1" siclog@

//@sic R5-224716 sic@

//Timer = 84s expires

[] t_T390.timeout

{

//@siclog "Step 5Ab2" siclog@

//Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall (UT);

//@siclog "Step 5Ab3" siclog@

//UE transmits an RRCSetupRequest message  with establishmentCause set to 'highPriorityAccess' to establish an emergency call.

f_NR_RRC_SetupRequest_Def(nr_Cell1, highPriorityAccess);

}

}

//@siclog "Step 6" siclog@

//@sic R5-211410, R5-213504, R5-224716 sic@

//Void.

//@siclog "Step 7" siclog@

//@sic R5-213504 sic@

//The SS transmits an RRCSetup message.

f_NR_RRCSetup_Def (nr_Cell1);

//@sic R5-213504 sic@

//EXCEPTION: Steps 8a1 to 8b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test.

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

tsc_SHT_IntegrityProtected);

//SERVICE REQUEST

if (ischosen (v_ReceivedMsg.Pdu.Msg.service_Request)){ // @sic R5s210731, R5s221179 Baseline Moving sic@

//@siclog "Step 8a1" siclog@

//The UE transmits an RRCSetupComplete message AND a SERVICE REQUEST message with 'Service type' set to 'high priority access'.

if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0101'B)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Service Request");

//@siclog "Step 8a2" siclog@

//Steps 4-8 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.9.11.

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

v_ReceivedAsp.Signalling.Nas[0].Pdu,

-, -, -, -,

crs_SSC_Mode('A'H, '001'B),

-, -, -, -, -, -,

cr_NG_Request_Type('011'B),//initial emergency request

omit,

omit)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

else { // @sic R5s220934 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

}

//REGISTRATION REQUEST

else if ( ischosen (v_ReceivedMsg.Pdu.Msg.registration_Request

)) { // @sic R5s210731 sic@

//@siclog "Step 8b1" siclog@

//The UE transmits an RRCSetupComplete message AND a REGISTRATION REQUEST message with 'registration type' set to 'emergency'. (Note 6)

//Note 6: UE may try to find service to place the emergency call (step 1) on other RATs since emergency access is barred on NR Cell 1. In this case UE will send a REGISTRATION REQUEST instead of a SERVICE REQUEST.

if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request");

// Store GMM Mobility Info now

f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo);

//@siclog "Step 8b2" siclog@

//@sic R5s210424, R5-214742 sic@

//Steps 5-15 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.5.2.2-2

f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Emergency);

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

v_ReceivedAsp.Signalling.Nas[0].Pdu,

-, -, -, -,

crs_SSC_Mode('A'H, '001'B),

-, -, -, -, -, -,

cr_NG_Request_Type('011'B),//initial emergency request

omit,

omit)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

else { // @sic R5s220934 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message or Registration Request Message Failed");

}

//@siclog "Step 9" siclog@

//@sic R5-211410, R5-213504 sic@

//Void.

//@sic R5-213504 sic@

//EXCEPTION: Steps 10a1 to 10b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test.

//@siclog "Step 10a1" siclog@

//IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM.

if(pc_SwitchOnOff){

f_UT_SwitchOffUE(UT, true);

//@siclog "Step 10a2" siclog@

//SS starts timer1 = 5 sec to see if UE performs optional Step 10a3a1 (Note 5)

t_WaitTime.start(5.0);

//@siclog "Steps 10a3a1-10a3a2" siclog@

//The UE transmits a DEREGISTRATION REQUEST message.

//The SS transmits an RRCRelease message IF step 10a3a1 was performed ELSE SS implicitly releases the RRC connection.

alt {

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{

if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

}

f_NR_RRCRelease(nr_Cell1);

}

[] t_WaitTime.timeout {

v_TimingOfLocalRelease := f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1));

f_Delay(f_SubFrameTimingDuration_CheckShortestDelay(f_NR_GetCurrentTiming(nr_Cell1), v_TimingOfLocalRelease)); //@sic R5s210839, R5s211474, R5s220024 sic@

}

}

}

//@siclog "Step 10b1" siclog@

//ELSE power off UE

else {

f_UT_PowerOffUE (UT);

}

}

## 3.2	Updates on object restrictions

Clause added to include updates made for restrictions of objects.

Variables been assigned to omit instead of value at the beginning of several functions are not needed, reducing the template restriction check. Restriction of functions updated to value accordingly. See example below.

function f_NR_GetServinCellConfig (NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template (value) ServingCellConfig

{ //@sic R5-202561 R5s230533 sic@

var template (value) ServingCellConfig v_ServingCellConfig;

var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated;

var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig := f_NR_CellInfo_GetRadioLinkMonitoringConfig(p_NR_CellId); //@sic R5s190244 sic@

var NR_DownlinkBWP_Type v_NR_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_NR_CellId, tsc_NR_BWP_Id);

var SearchSpace v_SearchSpaceUSS := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_NR_CellId, tsc_NR_SearchSpaceId_USS, tsc_NR_BWP_Id);

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, tsc_NR_BWP_Id);

var template(omit) PDCCH_Config.controlResourceSetToAddModList v_ControlResourceSetToAddModList := f_NR_GetPDCCH_ConfigControlResourceSet(p_NR_CellId);

v_BWP_DownlinkDedicated := cs_38508_BWP_DownlinkDedicated(cs_38508_PDCCH_Config (v_SearchSpaceUSS, -, v_ControlResourceSetToAddModList),

v_NR_DownlinkBWP.Pdsch.ConfigDedicated.R15,

-,

v_RadioLinkMonitoringConfig); //@sic R5s190244 sic@

v_ServingCellConfig := cs_38508_ServingCellConfig(v_BWP_DownlinkDedicated,

0,

cs_38508_UplinkConfig(v_NR_UplinkBWP.Dedicated.R15,

cs_38508_PUSCH_ServingCellConfig),

cs_38508_PDCCH_ServingCellConfig,

cs_38508_PDSCH_ServingCellConfig);

return v_ServingCellConfig;

}

…

function f_NR_GetSpCellConfigDef(NR_CellId_Type p_NR_CellId,

template (omit) ServingCellConfig p_ServingCellConfig := omit,

boolean  p_ENDC_SpecificCell := false

) runs on NR_BASE_PTC return template(value) SpCellConfig

{ //@sic R5-202561 R5-201320 sic@

var template (value) ServingCellConfig v_ServingCellConfig;

var template (omit) ServCellIndex v_ServCellIndex;

var template (omit) ReconfigurationWithSync v_ReconfigurationWithSync;

var template (value) ServingCellConfigCommon v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon(p_NR_CellId);

var template (value) RLF_TimersAndConstants v_RLF_TimersAndConstants := f_NR_CellInfo_GetRLF_TimersAndConstants(p_NR_CellId);

if (f_GetTestcaseAttrib_ENDC(testcasename()) or p_ENDC_SpecificCell ) {

v_ServCellIndex := f_NR_ServCellIndex(p_ENDC_SpecificCell);   // @sic R5-192274 R5s220981 sic@

v_ReconfigurationWithSync := cs_38508_ReconfigurationWithSync(v_ServingCellConfigCommon, f_NR_CellInfo_GetRNTI(p_NR_CellId));

//spCellConfigDedicated included for EN-DC

v_ServingCellConfig := f_NR_GetServinCellConfig(p_NR_CellId);

} else {

v_ServCellIndex := omit;

v_ReconfigurationWithSync := omit;

//spCellConfigDedicated included for condition SRB1 => value to be used in RRCSetup function

v_ServingCellConfig := p_ServingCellConfig;

}

return cs_38508_SpCellConfig(v_ServCellIndex, v_ReconfigurationWithSync, v_ServingCellConfig, v_RLF_TimersAndConstants);

}