3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230535

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4748 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NB-IoT NTN testcase 22.3.2.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add verified NB-IOT NTN testcase 22.3.2.7a to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 22.3.2.7a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of NB-IoT NTN test case 22.3.2.7a which is part of the NBIOT test suite in iwd-TTCN3-B2022-09_D23wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	22_3_2_7a_NBIOT

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	R&S® CMW 500 Protocol Conformance Test platform

UE used:	MediaTek MTK 6825, Qualcomm 9205S

Verification Status:	PASS

# Corrections required for test case 22.3.2.7a

## f_TC_22_3_2_7a_NBIOT

| Function name | f_TC_22_3_2_7a_NBIOT |
| --- | --- |
| Reason for change | After switch on, the function f_UT_AutomaticPLMN_Select_WithSwitchOnOff calls switch off procedure, but there is no switch on procedure to follow up. |
| Summary of change | Removed the function and replaced it with f_UT_AutomaticPLMN_Select to avoid the additional switch off call |
| TTCN module | NBIOT_RLC_NTN |
| MCC160 Comment | Accepted |

Before change

function f_TC_22_3_2_7a_NBIOT() runs on NBIOT_PTC { /* NB-IoT / NTN / AM RLC / Receiver status triggers / extended t-Reordering configured */ var RLC_SS_State v_RLC_Rec; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoList; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var T_PollRetransmit_NB_r13 v_T_PollRetransmit_NB_r13; var RLC_Config_NB_v1430.t_Reordering_r14 v_T_Reordering_r14; var RLC_Config_NB_v1700.t_ReorderingExt_r17.setup v_T_Reordering_r17; var UL_AM_RLC_NB_r13.maxRetxThreshold_r13 v_MaxRetxThreshold_r13; var template (value) SRB_ToAddModList_NB_r13 v_SRB_ToAddModList_NB_r13; var integer v_HARQ_ProcessNumber := 0; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); //The HARQ process 0 is used for DL and UL transmissions as default. f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CcchDcchDtchDL_Config_REQ(nbiot_Cell1, cs_TimingInfo_Now, -, cs_NB_DciDlInfo_Auto(v_HARQ_ProcessNumber))); //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); //Initialise acc. Table 22.3.2.7a.3.3-1 v_T_PollRetransmit_NB_r13 := ms4000; v_RLC_Rec.t_PollRetransmit_r13 := 4.0; // t_PollRetransmit_r13 set to 4000 ms v-_MaxRetxThreshold_r13 := t2; v_RLC_Rec.maxRetxThreshold_r13 := 2; // maxRetxThreshold_r13 set to 2 (=t2) v_RLC_Rec.enableStatusReportSN_Gap_r13 := true; // enableStatusReportSN-Gap-r13 set to true v_T_Reordering_r14 := ms1600_v1310; v_RLC_Rec.t_Reordering_r14 := 1.6; // t_Reordering_r14 set to 1600 ms v_T_Reordering_r17 := ms3200; v_RLC_Rec.t_Reordering_r17 :=3.2; //t_Reordering_r17 set to 3200 ms //UE RLC settings are as initialized v_SRB_ToAddModList_NB_r13 := {cs_SRB_ToAddMod_NB_RLC_Config (cs_UL_AM_RLC_NB_r13 (v_T_PollRetransmit_NB_r13, v_MaxRetxThreshold_r13), cs_DL_AM_RLC_NB_r13_GapReport, cs_RLC_Config_NB_v1430(v_T_Reordering_r14))}; v_SRB_ToAddModList_NB_r13[0].rlc_Config_v1700.t_ReorderingExt_r17.setup := v_T_Reordering_r17; //acc to Table 22.3.2.7a.3.3-1 // RRCConnectionSetup-NB with twoHARQ-ProcessesConfig-r14 configured v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, v_SRB_ToAddModList_NB_r13, -, -, true_); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnSetup); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); //The UE is in state 2B-NB according to TS 36.508 [18] clause 8.1.5.2B with test loop mode G closed // Switch on UE f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE, STATE2A_NB_TESTLOOP_ModeG);

After change

function f_TC_22_3_2_7a_NBIOT() runs on NBIOT_PTC { /* NB-IoT / NTN / AM RLC / Receiver status triggers / extended t-Reordering configured */ var RLC_SS_State v_RLC_Rec; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoList; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var T_PollRetransmit_NB_r13 v_T_PollRetransmit_NB_r13; var RLC_Config_NB_v1430.t_Reordering_r14 v_T_Reordering_r14; var RLC_Config_NB_v1700.t_ReorderingExt_r17.setup v_T_Reordering_r17; var UL_AM_RLC_NB_r13.maxRetxThreshold_r13 v_MaxRetxThreshold_r13; var template (value) SRB_ToAddModList_NB_r13 v_SRB_ToAddModList_NB_r13; var integer v_HARQ_ProcessNumber := 0; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); //The HARQ process 0 is used for DL and UL transmissions as default. f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CcchDcchDtchDL_Config_REQ(nbiot_Cell1, cs_TimingInfo_Now, -, cs_NB_DciDlInfo_Auto(v_HARQ_ProcessNumber))); //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); //Initialise acc. Table 22.3.2.7a.3.3-1 v_T_PollRetransmit_NB_r13 := ms4000; v_RLC_Rec.t_PollRetransmit_r13 := 4.0; // t_PollRetransmit_r13 set to 4000 ms v_MaxRetxThreshold_r13 := t2; v_RLC_Rec.maxRetxThreshold_r13 := 2; // maxRetxThreshold_r13 set to 2 (=t2) v_RLC_Rec.enableStatusReportSN_Gap_r13 := true; // enableStatusReportSN-Gap-r13 set to true v_T_Reordering_r14 := ms1600_v1310; v_RLC_Rec.t_Reordering_r14 := 1.6; // t_Reordering_r14 set to 1600 ms v_T_Reordering_r17 := ms3200; v_RLC_Rec.t_Reordering_r17 :=3.2; //t_Reordering_r17 set to 3200 ms //UE RLC settings are as initialized v_SRB_ToAddModList_NB_r13 := {cs_SRB_ToAddMod_NB_RLC_Config (cs_UL_AM_RLC_NB_r13 (v_T_PollRetransmit_NB_r13, v_MaxRetxThreshold_r13), cs_DL_AM_RLC_NB_r13_GapReport, cs_RLC_Config_NB_v1430(v_T_Reordering_r14))}; v_SRB_ToAddModList_NB_r13[0].rlc_Config_v1700.t_ReorderingExt_r17.setup := v_T_Reordering_r17; //acc to Table 22.3.2.7a.3.3-1 // RRCConnectionSetup-NB with twoHARQ-ProcessesConfig-r14 configured v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, v_SRB_ToAddModList_NB_r13, -, -, true_); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnSetup); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode // REMOVED f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); f_UT_AutomaticPLMN_Select (UT); //(similar to other test cases //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE, STATE2A_NB_TESTLOOP_ModeG);

## fl_TC_22_3_2_7a_TestBody

| Function name | fl_TC_22_3_2_7a_TestBody |
| --- | --- |
| Reason for change | - In order to transmit the RLC PDUs the SDUs are not generated and results in runtime error - When the 3 RLC PDUs are transmitted, the SS cannot honour the current activation time specified as the next search space cycle is 64msec, and this does not consider the Koffset into account, since the Koffset is 550msec, updated the timing to use 10* search space (640msec) as p_Offset - After Step 6, the DL data cannot be sent on the next PDCCH cycle due to Koffset, therefore updated to use 10* speech space (640msec) as p_Offset - When checking for the timer duration , the test case needs to take into account the Koffset value - Since the test procedure triggers out of order and UE generated a status report with NACK PDU, the current prose does not transmit the NACK PDU, and this results in test case failure at postamble, In TTCN, in order to perform postamble procedure the RLC count is updated to the SS and SRB3 is moved from test mode to normal mode). SS RLC is now aware of the NACK PDU and results in TC failure. In order to avoid this additional steps are added for UE to loopback the received data is necessary. |
| Summary of change | - Added f_NBIOT_Generate_RLC_SDUs - v_TimingInfo1 and v_TimingInfo2 is updated to use p_Offset value as 11 and 21 and step 2 grant will be triggered in 31 - Step 8 data is started with p_Offset as 10 and grant is sent with p_Offset as 11 - Introduced new function to take into account the Koffset for max tolerance - Introduced Steps 10-17 to transmit the NACK PDU and receive all loopback data before proceeding into postamble Changes proposed here requires an update to 36.523-3 |
| TTCN module | NBIOT_RLC_NTN |
| MCC160 Comment | Accepted in principle. Merged with test body of TC 22.3.2.7 and changes handled: 1.Accepted 2.Accepted but aligned to the prose: step 2 grant will be triggered in 41 (4th “NPDCCH period” after step 1) 3.Accepted 4.Accepted in principle. See change 4 below. 5.Accepted TF160 Implementation below |

Before change

function fl_TC_22_3_2_7a_TestBody(inout RLC_SS_State p_RLC_Rec) runs on NBIOT_PTC { var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; //The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. //Note: In steps 1 to 8 the size of each RLC UL SDU #1 - #3 is PRBS of 288 bits (36 octets). The size of the RLC SDUs used in downlink is L. //@siclog "Step 1" siclog@ //The SS transmits 3 AMD PDUs with SN=0, 1, 2. The SS sets the P field of all the AMD PDUs to 0. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_TimingInfo1 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 2.0)); v_TimingInfo2 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 3.0)); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU1); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo1, tsc_P_NoPoll, tsc_RLC_SDU2); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo2, tsc_P_NoPoll, tsc_RLC_SDU3); //@siclog "Step 2" siclog@ //In the search space of the 4th NPDCCH period after the first transmission at step 1 //the SS schedules 3 consecutive UL grants with a time spacing of 3 NPDCCH cycles of size 328 bits. v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 5.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); // UL Grant of 328 bits f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(v_DciUlInfo, 3, 3)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE transmits RLC UL SDU#1. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU1); //@siclog "Step 4" siclog@ //The UE transmits RLC UL SDU#2. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU2); //@siclog "Step 5" siclog@ //The UE transmits RLC UL SDU#3. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_Poll, tsc_RLC_SDU3); //@siclog "Step 6" siclog@ //The SS transmits a STATUS PDU f_NB_TxSTATUS_PDU (p_RLC_Rec); //@siclog "Step 7" siclog@ //The SS starts the UL periodic grant transmission of size 40 bits. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (0,3); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 8" siclog@ //The SS transmits 1 AMD PDUs with SN=4 and P=0. //Record time TA. v_SubFrameTimingTa := f_NBIOT_IncrementSearchSpace(v_TimingStart, 2.0); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTimingTa); p_RLC_Rec.AM_VTS := v_VTS_Offset + 4; f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU5); //@siclog "Step 9" siclog@ //Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? //Record time TB // Check 2: (TB - TA) = t-Reordering? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; v_NACK_List := cr_NACK_List(v_VTS_Offset + 3, tsc_E2_NoSO_Follows, omit); f_NB_RxSTATUS_PDU_NACK(p_RLC_Rec, v_NACK_List, v_Padding_Any); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 1"); v_SubFrameTimingTb := p_RLC_Rec.TimeStampLastReceivedPDU; v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTa, v_SubFrameTimingTb); if ( f_NBIOT_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r17 * 1000.0) ) ) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } }

After change

import from EUTRA_NB_CommonL2 all; … … … function fl_TC_22_3_2_7a_TestBody(inout RLC_SS_State p_RLC_Rec) runs on NBIOT_PTC { var integer v_RLC_DL_SDU_Length; var integer v_RLC_UL_SDU_Length := 36; var integer v_L; var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; //The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. //Note: In steps 1 to 8 the size of each RLC UL SDU #1 - #3 is PRBS of 288 bits (36 octets). The size of the RLC SDUs used in downlink is L. // generate SDUs with PRBS size 36 octets v_RLC_DL_SDU_Length := f_NBIOT_Generate_RLC_SDUs (p_RLC_Rec, 6, v_RLC_UL_SDU_Length, 0); v_L := v_RLC_DL_SDU_Length - v_RLC_UL_SDU_Length; // RLC DL SDU length - RLC UL SDU length, Length difference is the NAS and RRC overhead //@siclog "Step 1" siclog@ //The SS transmits 3 AMD PDUs with SN=0, 1, 2. The SS sets the P field of all the AMD PDUs to 0. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_TimingInfo1 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 11.0)); v_TimingInfo2 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 21.0)); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU1); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo1, tsc_P_NoPoll, tsc_RLC_SDU2); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo2, tsc_P_NoPoll, tsc_RLC_SDU3); //@siclog "Step 2" siclog@ //In the search space of the 4th NPDCCH period after the first transmission at step 1 //the SS schedules 3 consecutive UL grants with a time spacing of 3 NPDCCH cycles of size 328 bits. v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 31.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); // UL Grant of 328 bits f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(v_DciUlInfo, 3, 3)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE transmits RLC UL SDU#1. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU1); //@siclog "Step 4" siclog@ //The UE transmits RLC UL SDU#2. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU2); //@siclog "Step 5" siclog@ //The UE transmits RLC UL SDU#3. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_Poll, tsc_RLC_SDU3); //@siclog "Step 6" siclog@ //The SS transmits a STATUS PDU f_NB_TxSTATUS_PDU (p_RLC_Rec); //@siclog "Step 7" siclog@ //The SS starts the UL periodic grant transmission of size 40 bits. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 11.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (0,3); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 8" siclog@ //The SS transmits 1 AMD PDUs with SN=4 and P=0. //Record time TA. v_SubFrameTimingTa := f_NBIOT_IncrementSearchSpace(v_TimingStart, 10.0); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTimingTa); p_RLC_Rec.AM_VTS := v_VTS_Offset + 4; f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU5); //@siclog "Step 9" siclog@ //Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? //Record time TB // Check 2: (TB - TA) = t-Reordering? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; v_NACK_List := cr_NACK_List(v_VTS_Offset + 3, tsc_E2_NoSO_Follows, omit); f_NB_RxSTATUS_PDU_NACK(p_RLC_Rec, v_NACK_List, v_Padding_Any); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 1"); v_SubFrameTimingTb := p_RLC_Rec.TimeStampLastReceivedPDU; v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTa, v_SubFrameTimingTb); if ( f_NBIOT_NTN_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r17 * 1000.0) ) ) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; //@siclog "Step 10" siclog@ //SS stops to allocate any uplink grant. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_TimingInfo1 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 11.0)); v_TimingInfo2 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 21.0)); //@siclog "Step 11" siclog@ p_RLC_Rec.AM_VTS := v_VTS_Offset + 3; f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_Poll, tsc_RLC_SDU4); //@siclog "Step 12" siclog@ v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (0,3); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo1); //@siclog "Step 14" siclog@ // //Executed before step 13 to allow proper scheduling of UL grants ! v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); // UL Grant of 328 bits f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(v_DciUlInfo, 2)), v_TimingInfo2); //@siclog "Step 13" siclog@ //Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN = 5? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; f_NBNTN_RxSTATUS_PDU_ACK(p_RLC_Rec); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 15" siclog@ //The UE transmits RLC UL SDU#4. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU4); //@siclog "Step 16" siclog@ //The UE transmits RLC UL SDU#5. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_Poll, tsc_RLC_SDU5); //@siclog "Step 17" siclog@ //The SS transmits a STATUS PDU f_NB_TxSTATUS_PDU (p_RLC_Rec); p_RLC_Rec.AM_VTS := v_VTS_Offset + 5 } }

## f_NBNTN_RxSTATUS_PDU_ACK

| Function name | f_NBNTN_RxSTATUS_PDU_ACK |
| --- | --- |
| Reason for change | New function introduced to handle RxSTATUS_PDU_ACK, see change 2.2.5 |
| Summary of change | Introduced new function |
| TTCN module | NBIOT_RLC_NTN |
| MCC160 Comment | Rejected, identical function f_NB_RxSTATUS_PDU_ACK can be used |

New function

function f_NBNTN_RxSTATUS_PDU_ACK(RLC_SS_State p_RLC_Rec) runs on NBIOT_PTC { L2DATA.receive(car_NB_SRB1bis_RLCStatusInd(nbiot_Cell1, cr_RLC_AM_StatusPDU_ACK_SN(p_RLC_Rec.AM_VTA))); }

## f_NBIOT_NTN_SubFrameTimingCheckDuration

| Function name | f_NBIOT_NTN_SubFrameTimingCheckDuration |
| --- | --- |
| Reason for change | New function introduced to handle SubframeTiming check and taking into account Koffset (see 2.2.4 change) |
| Summary of change | Introduced new function f_NBIOT_NTN_SubFrameTimingCheckDuration |
| TTCN module | NBIOT_RLC_NTN |
| MCC160 Comment | Accepted in principle Modifications to the proposal: -replace Koffset by propogation (NTN delay is targeted) -propagation to be considered may vary (e.g. 1 or 2 * NTN delay) Does the same as function f_NBIOT_SubFrameTimingCheckDuration with propagation set to 0. |

New function

function f_NBIOT_NTN_SubFrameTimingCheckDuration(ProtocolTimer_Type p_ProtocolTimer, integer p_Duration, integer p_ExpectedDuration, integer p_Koffset := 550) return boolean { var float v_ToleranceFloat := fl_NBIOT_TimerTolerance(p_ProtocolTimer, int2float(p_ExpectedDuration)/1000.0); var integer v_ToleranceInt := float2int(v_ToleranceFloat * 1000.0); var integer v_Min := p_ExpectedDuration - v_ToleranceInt; var integer v_Max := p_ExpectedDuration + v_ToleranceInt+ 2*p_Koffset; return ((v_Min <= p_Duration) and (p_Duration <= v_Max)); }

Modified function

function f_NBIOT_NTN_SubFrameTimingCheckDuration(ProtocolTimer_Type p_ProtocolTimer, integer p_Duration, integer p_ExpectedDuration, integer p_Propogation := 0) return boolean { var float v_ToleranceFloat := fl_NBIOT_TimerTolerance(p_ProtocolTimer, int2float(p_ExpectedDuration)/1000.0); var integer v_ToleranceInt := float2int(v_ToleranceFloat * 1000.0); var float v_PropogationFloat := fl_NBIOT_TimerTolerance(p_ProtocolTimer, int2float(p_Propogation)/1000.0); var integer v_PropogationInt := float2int(v_PropogationFloat * 1000.0); var integer v_Min := p_ExpectedDuration - v_ToleranceInt; var integer v_Max := p_ExpectedDuration + v_ToleranceInt+ v_PropogationInt; return ((v_Min <= p_Duration) and (p_Duration <= v_Max)); }

MCC160 Implementation

Test body using common function

function fl_TC_22_3_2_7a_TestBody(inout RLC_SS_State p_RLC_Rec) runs on NBIOT_PTC { fl_TC_22_3_2_7_and7a_TestBody(p_RLC_Rec, true); }

Common function

function fl_TC_22_3_2_7_and7a_TestBody(inout RLC_SS_State p_RLC_Rec, boolean p_NTN := false) runs on NBIOT_PTC { var integer v_RLC_DL_SDU_Length; var integer v_RLC_UL_SDU_Length := 36; var integer v_L; var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (value) TimingInfo_Type v_TimingInfo3; var template (value) TimingInfo_Type v_TimingInfo4; var template (value) TimingInfo_Type v_TimingInfo5; var template (value) TimingInfo_Type v_TimingInfo6; var template (value) TimingInfo_Type v_TimingInfo7; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (present) RLC_Status_Padding_Type v_Padding_3bits_Any := '???'B; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var RLC_DataField_Type v_RLC_Data1; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var boolean v_NTN := p_NTN; var IntegerList_Type v_SearchSpaceIncr; var IntegerList_Type v_SearchSpaceIncr_NTN; var integer v_NTN_delay := 0; var integer v_SearchSpaceInc := 0; var float v_Watch_T_Reordering := f_NBIOT_SetTimerToleranceMax(l2Timer, p_RLC_Rec.t_Reordering_r14); timer t_Watch_T_Reordering; // The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. // Note: In steps 1 to 25 the size of each RLC UL SDU #1 - #6 is PRBS of 288 bits (36 octets). The size of the RLC SDUs used in downlink is L. // establish RLC SDUs (DL and UL) and obtain the length of the DL RLC SDU // generate SDUs for SDU#1-6 with PRBS size 36 octets v_RLC_DL_SDU_Length := f_NBIOT_Generate_RLC_SDUs (p_RLC_Rec, 6, v_RLC_UL_SDU_Length, 0); // @sic R5s230535 sic@ v_L := v_RLC_DL_SDU_Length - v_RLC_UL_SDU_Length; // RLC DL SDU length - RLC UL SDU length, Length difference is the NAS and RRC overhead // values of Search Space Increment for use in chosen TC --- for GSO only if (v_NTN) { v_NTN_delay := f_NBIOT_NTN_Get_t_NTNdelay(nbiot_Cell1); v_SearchSpaceInc := ((v_NTN_delay / 64) + 2); // NPDCCH cycle is 64 ms } // values of Search Space Increments in TC 22_3_2_7 in the order of use v_SearchSpaceIncr := {1, // step 1 2, // step 1 3, // step 1 5, // step 2 1, // step 7 2, // step 8 6, // step 11 9, // step 12 10}; // step 14 // values of Search Space Increments in TC 22_3_2_7a in the order of use (steps 7 and 8 differing !) // as per @sic R5s230535 sic@ resp @sic R5-235470 sic@ v_SearchSpaceIncr_NTN := {1, // step 1 1 + v_SearchSpaceInc, // step 1 1 + 2*v_SearchSpaceInc, // step 1 1 + 4*v_SearchSpaceInc, // step 2 aliged to the prose not the TTCN CR ! v_SearchSpaceInc,// step 7 1 + v_SearchSpaceInc,// step 8 1, // step 11 1 + v_SearchSpaceInc, // step 12 1 + 2*v_SearchSpaceInc}; // step 14 if (v_NTN) {v_SearchSpaceIncr := v_SearchSpaceIncr_NTN;}; //@siclog "Step 1" siclog@ //The SS transmits 3 AMD PDUs with SN=0, 1, 2. The SS sets the P field of all the AMD PDUs to 0. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[0]))); v_TimingInfo1 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[1]))); v_TimingInfo2 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[2]))); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU1); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo1, tsc_P_NoPoll, tsc_RLC_SDU2); f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo2, tsc_P_NoPoll, tsc_RLC_SDU3); //@siclog "Step 2" siclog@ //In the search space of the 4th NPDCCH period after the first transmission at step 1 //the SS schedules 3 consecutive UL grants with a time spacing of 3 NPDCCH cycles of size 328 bits. v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[3]))); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); // UL Grant of 328 bits f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(v_DciUlInfo, 3, 3)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE transmits RLC UL SDU#1. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU1); //@siclog "Step 4" siclog@ //The UE transmits RLC UL SDU#2. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU2); //@siclog "Step 5" siclog@ //The UE transmits RLC UL SDU#3. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_Poll, tsc_RLC_SDU3); //@siclog "Step 6" siclog@ //The SS transmits a STATUS PDU f_NB_TxSTATUS_PDU (p_RLC_Rec); // !!! steps 7 and 8 interchanged - to be revisited once prose fixed ! if (v_NTN) {// 22.3.2.7a //@siclog "Step 7" siclog@ //The SS transmits 1 AMD PDUs with SN=4 and P=0. //Record time TA. // v_SubFrameTimingTa to be set. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_SubFrameTimingTa := f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[5])); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTimingTa); p_RLC_Rec.AM_VTS := v_VTS_Offset + 4; f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU5); //@siclog "Step 8" siclog@ //The SS starts the UL periodic grant transmission of size 40 bits. v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[4]))); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (0,3); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(v_DciUlInfo)), v_TimingInfo); } else {// 22.3.2.7 //@siclog "Step 7" siclog@ //The SS starts the UL periodic grant transmission of size 40 bits. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[4]))); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (0,3); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 8" siclog@ //The SS transmits 1 AMD PDUs with SN=4 and P=0. //Record time TA. // v_SubFrameTimingTa to be set. v_SubFrameTimingTa := f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[5])); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTimingTa); p_RLC_Rec.AM_VTS := v_VTS_Offset + 4; f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_NoPoll, tsc_RLC_SDU5); } //@siclog "Step 9" siclog@ //Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? //Record time TB // Check 2: (TB - TA) = t-Reordering? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; v_NACK_List := cr_NACK_List(v_VTS_Offset + 3, tsc_E2_NoSO_Follows, omit); f_NB_RxSTATUS_PDU_NACK(p_RLC_Rec, v_NACK_List, v_Padding_Any); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 1"); v_SubFrameTimingTb := p_RLC_Rec.TimeStampLastReceivedPDU; v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTa, v_SubFrameTimingTb); if (v_NTN) { //22.3.2.7a if ( f_NBIOT_NTN_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r17 * 1000.0), 2*v_NTN_delay ))// @sic R5s230535 sic@ { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } else { //22.3.2.7 if ( f_NBIOT_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r14 * 1000.0))) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } //@siclog "Step 10" siclog@ //SS stops to allocate any uplink grant. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[6]))); v_TimingInfo1 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[7]))); v_TimingInfo2 := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_SearchSpaceIncr[8]))); //@siclog "Step 11" siclog@ //In the search space of the 6th NPDCCH period after step 10 the SS transmits 1 AMD PDU with SN=3. The SS sets the P field to 1. p_RLC_Rec.AM_VTS := v_VTS_Offset + 3; f_NB_TxAMD_SDU(p_RLC_Rec, v_TimingInfo, tsc_P_Poll, tsc_RLC_SDU4); //@siclog "Step 12" siclog@ //In the search space of the 3rd NPDCCH period after the transmission at step 11 the SS schedules 1 UL grant of size 40 bits. v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (0,3); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo1); //@siclog "Step 14" siclog@ should become step 15 ?? //In the search space of the next NPDCCH period after the transmission at step 12 the SS schedules 2 UL grants of size 328 bits. //Executed before step 13 to allow proper scheduling of UL grants ! v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); // UL Grant of 328 bits f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(v_DciUlInfo, 2)), v_TimingInfo2); //@siclog "Step 13" siclog@ //Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN = 5? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; f_NB_RxSTATUS_PDU_ACK(p_RLC_Rec); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 15" siclog@ //The UE transmits RLC UL SDU#4. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_NoPoll, tsc_RLC_SDU4); //@siclog "Step 16" siclog@ //The UE transmits RLC UL SDU#5. f_NB_RxAMD_SDU(p_RLC_Rec, tsc_P_Poll, tsc_RLC_SDU5); //@siclog "Step 17" siclog@ //The SS transmits a STATUS PDU f_NB_TxSTATUS_PDU (p_RLC_Rec); p_RLC_Rec.AM_VTS := v_VTS_Offset + 5; // moved upwards to adjust VTS for the postamble if (v_NTN == false) {// steps of 22.3.2.7 only … }; }

TC 22.3.2.7 was modified to use the common function above.

# Branches executed in test case 22.3.2.7a

The test case implementation was executed with Integrity and ciphering enabled (eea1 and eia1) using NTN bands 255 and 256.

# Execution Log Files

## Qualcomm 9205S

The Qualcomm 9205S device passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

Test Case Execution log file: 
tc_22_3_2_7a_QC.log 
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

## MediaTek 6825

The MediaTek 6825 device passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

Test Case Execution log file: 
tc_22_3_2_7a_MTK.log 
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

n the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230536: Supporting information for addition of NB-IOT NTN testcase 22.3.2.7a. |
| --- | --- |