3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230677

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0932 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS eCall over IMS testcase 11.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 11.6 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	4

4	Execution Log Files	5

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.6 which is part of the IMS/NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.6

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes of R5s230480_MCC160Comments.docx are also required and implemented.

## Change 1

| Function name | fl_IMS_ECall_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The test cases of 34.229-5 11.x just state “UE contains either ISIM and USIM applications or only USIM application with eCall subscription on UICC”. But since the preamble condition states “UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS”, it follows the USIM profile to be used should be “ecall capable” rather than “eCall only” For this reason the Sim configuration defined in 38.508-1 6.4.1-23 should be used rather than 6.4.1.24, |
| Summary of change | Changed the USIM prompt to use configuration 23. |
| TTCN module | IMS_NR5GC_eCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_IMS_ECall_NR5GC_IMS1 () runs on IMS_PTC { f_IMS_ECallOverIMS_IMS1("TS 38.508-1 Table 6.4.1-24: USIM Configuration 24"); }

After Change:

function fl_IMS_ECall_NR5GC_IMS1 () runs on IMS_PTC { f_IMS_ECallOverIMS_IMS1("TS 38.508-1 Table 6.4.1-23: USIM Configuration 23"); }

## Change 2

| Function name | f_IMS_ECallOverIMS_IMS1() |
| --- | --- |
| Reason for change | Following the MCC comments for R5s230480, the call to f_IMS_CC_Postamble() is moved from IMS1 component to IMS2, which means the function f_IMS_ECallOverIMS_IMS1() will exit and the IMS1 component will terminate before the emergency call is even established. This means any REGISTER sent by the UE for de-registration at switch off during postamble cannot be routed to the IMS1 component, and hence the SS will not reply with 200 OK |
| Summary of change | Called the function f_IMS_OptionalDeregistrationOrC24Reinvite() with parameter p_WaitForIpcanRelease as false so that, the function will not wait for a final trigger from IPCAN and exit after sending the 200 OK for REGISTER. |
| TTCN module | IMS_CC_TestcaseImplementation_eCall.ttcn |
| MCC160 Comment | Alternative solution implemented: release the emergency call in IMS2, then call the postamble in IMS1. Also implemented in 10.1, 10.2, 10.6, 10.9, 10.10, 10.12, 10.13, 10.14 and other 11.* test cases |

Before Change:

function f_IMS_ECallOverIMS_IMS1(charstring p_USIM_ToInsert) runs on IMS_PTC { // @sic R5-221598 re-used common code with 4G and 5G sic@ /* IMS implementation for PDN1 - IMS normal registration */ f_UT_USIM_Insert(MMI, p_USIM_ToInsert); f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

After Change:

function f_IMS_ECallOverIMS_IMS1(charstring p_USIM_ToInsert) runs on IMS_PTC { timer t_Dummy; // @sic R5-221598 re-used common code with 4G and 5G sic@ /* IMS implementation for PDN1 - IMS normal registration */ f_UT_USIM_Insert(MMI, p_USIM_ToInsert); f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_OptionalDeregistrationOrC24Reinvite(IPCAN, t_Dummy, false); }

#### MCC160 Implementation:

function f_IMS_ECallOverIMS_IMS1(charstring p_USIM_ToInsert) runs on IMS_PTC

{

// @sic R5-221598 re-used common code with 4G and 5G sic@

/* IMS implementation for PDN1 -  IMS normal registration */

f_UT_USIM_Insert(MMI, p_USIM_ToInsert);

f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED);

f_IMS_IMS_SendCoOrdMsg(OtherPDN);

f_IMS_IMS_WaitForTrigger(OtherPDN);

f_IMS_CC_Postamble();

}

function f_IMS_ECallOverIMS_IMS2(ECall_Type p_CallType,

ECall_MSD_Update_Type p_MSDUpdate,

template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request) runs on IMS_PTC

{ // @sic R5-221598 re-used common code with 4G and 5G sic@

/* IMS implementation for PDN2 -  IMS normal registration */

var SipUrl v_ContactUrl;

f_IMS_PTC_Init(PDN_2);

f_IMS_IMS_WaitForTrigger(OtherPDN);

f_IMS_TestBody_Set(true);

// Trigger eCall initiation, PDN and DRB establishment according to TS 36.508/4.5A.26.3-1

// @siclog "Steps 1" siclog@

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_EmergencyCall_NormalService);

f_UT_InitiateECall(MMI, p_CallType);

//@siclog "Steps 2-5 (21.*)" siclog@

//@siclog "Steps 2-11 (11.*)" siclog@

f_IMS_EmergencyRegistration();

// @siclog "Step 6-8 (21.1, 21.2)" siclog@

// @siclog "Step 6-12 (21.4, 21.5, 21.6)" siclog@

// @siclog "Step 12-18 (11.4, 11.5, 11.6)" siclog@

if (f_IMS_PTC_RanTypeIsNR5GC()){    // @sic R5s230480 sic@

f_ECall_CallSetup_A_23 (p_CallType);

} else {

f_ECall_CallSetup_AnnexC47(p_CallType);

}

if (p_MSDUpdate == msdUpdateRequired) {

f_ECall_MSDUpdate_AnnexC47(p_EmergencyCallData_Control_Step4);

}

// @siclog "Step 9-12 (21.1, 21.2)" siclog@

// @siclog "Step 13-14 (21.4, 21.5, 21.6)" siclog@

// @siclog "Step 19-20 (11.4, 11.5, 11.6)" siclog@

// @sic R5-221598 removed option for MO release as this is never used sic@

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog);

f_IMS_CallReleaseMT(v_ContactUrl);

f_IMS_CC_ ReleaseConnection (IPCAN_EmergencyCall_NormalService);     // @sic R5s230480 sic@

f_IMS_TestBody_Set(false);

f_IMS_IMS_SendCoOrdMsg(OtherPDN);

}

function f_TC_10_14_NR5GC_IMS1() runs on IMS_PTC

{ /* In parallel emergency and non-emergency registrations / 5GS */

var template (value) Reginfo_Type v_XmlMessage;

var SipUrl v_ContactUrlUE;

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var REGISTER_Request v_RegisterRequest;

var boolean v_WaitForPublish := true;

f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_NULL);

f_IMS_CC_StartSignalling(IPCAN_InitialRegistration);

v_IMS_REGISTER_REQ := f_IMS_Register();

f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, v_WaitForPublish);

f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration);

f_IMS_IMS_SendCoOrdMsg(OtherPDN);

f_IMS_IMS_WaitForTrigger(OtherPDN);

//@siclog "Steps 15" siclog@

v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register;

v_XmlMessage := fl_TC_10_14_XmlMessageForNotify_Step15 (v_RegisterRequest);

v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(registration); // @sic R5s221189 sic@

fl_TC_10_14_Steps15_Notify (v_ContactUrlUE, v_XmlMessage);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @sic R5-227488 sic@

f_IMS_IMS_SendCoOrdMsg(OtherPDN);

f_IMS_IMS_WaitForTrigger(OtherPDN);

if (px_UE_CS_PS_UsageSetting_Tested == VOICE_CENTRIC){  // @sic R5-233446 sic@

f_IMS_CC_Postamble (IPCAN_Deregistration);

}

else {

f_IMS_CC_Postamble();

}

}

function f_TC_10_14_NR5GC_IMS2() runs on IMS_PTC

{ /* IMS implementation for PDN2 - Emergency call */

var SipUrl v_ContactUrlUE;

f_IMS_PTC_Init(PDN_2);

// Test Case Body

f_IMS_TestBody_Set(true);

f_IMS_IMS_WaitForTrigger(OtherPDN);

//@siclog "Steps 1 - 14" siclog@

f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService);

f_IMS_EmergencyRegistration();

f_IMS_EmergencyCallSetup_NR5GC_A_6();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14");

f_IMS_IMS_SendCoOrdMsg(OtherPDN); // Send sync message to other IMS PTC (for steps 16/17)

f_IMS_IMS_WaitForTrigger(OtherPDN); // Wait for sync message from other IMS PTC

v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(registration); // @sic R5s221189 sic@

//@siclog "Steps 17" siclog@

//Void // @sic R5-227488 sic@

//@siclog "Steps 18" siclog@

f_Delay(5.0); // @sic R5-227488 sic@

//@siclog "Steps 19 - 20" siclog@

f_IMS_CallReleaseMT(v_ContactUrlUE);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20");

f_IMS_CC_ReleaseConnection(IPCAN_EmergencyCall_NormalService);

f_IMS_TestBody_Set(false);

f_IMS_IMS_SendCoOrdMsg(OtherPDN);

}

# Branches executed

The test case was executed on NR band n78 using ciphering algorithm nea1 and integrity algorithm nia1

# Execution Log Files

## Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

The Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_6_LOG.html

- PICS/PIXIT parameter file:

TC_11_6_PIXIT.xml

# References

| [1] | R5s230678: Supporting information for addition of IMS 5GS eCall over IMS testcase 11.6 |
| --- | --- |