3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230734

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0936 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With current implementation, the criteria to expect a MIME message in INVITE message body is that the test case must be an emergency call (with geolocation) or an eCall. This is valid but we can find an exception with User Initiated USSI test cases wich also require MIME with an xml message body. This can be solved adding an exception to current implementation to expect also a MIME message when a User Initiated USSI test case is run. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a function to support the check of MIME contents when a User Initiated USSI test case is run. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.40.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	New function f_IsMimeWithXMLRequired	3

2.2	Correction to function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## New function f_IsMimeWithXMLRequired

| Function name | f_IsMimeWithXMLRequired |
| --- | --- |
| Reason for change | At the moment criteria to check to expect a MIME message in INVITE message body, test case to be run must be an emergency (with geolocaiton) or an eCall test case. This is valid but we can find an exception with USSD test cases wich also require MIME with an xml message body. |
| Summary of change | Added a function to support the check of MIME contents when USSD test cases are run. |
| TTCN module | \Common\IMS\IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | not needed (see MCC160 implementation below) |

New Function:

function f_IsMimeWithXMLRequired(template (present) MessageHeader p_MessageHeader_Invite) return boolean { var boolean v_MimeWithXML := false; if (ispresent (p_MessageHeader_Invite.recvInfo.infoPackageList)) { v_MimeWithXML := valueof (p_MessageHeader_Invite.recvInfo.infoPackageList[0].infoPackageName) =="g.3gpp.ussd" ; } else if( ispresent(p_MessageHeader_Invite.geolocation) ) { v_MimeWithXML := true; }; return v_MimeWithXML; }

## Correction to function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional

| Function name | f_IMS_INVITE_ReceiveRequest_A_2_1_Optional |
| --- | --- |
| Reason for change | At the moment criteria to check to expect a MIME message in INVITE message body, test case to be run must be an emergency (with geolocaiton) or an eCall test case. This is valid but we can find an exception with USSD test cases wich also require MIME with an xml message body. |
| Summary of change | Added a function to support the check of MIME contents when USSD test cases are run. |
| TTCN module | \Common\IMS\IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted in principle |

Before Change:

<<SKIPPED CODE>> function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { /* p_OtherCalleeUri .. if present to be used as CalleeUri; if omit default CalleeUri is used acc. to PIXITs and context */ /* @sic R5-133276, R5-133301 Cleanup of functions for MO Call Setup sic@ */ /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5-140799: additional parameter p_MessageHeader to deal with Reason header sic@ */ /* @sic R5s140370 change 7: pc_IMS_SRVCCAlert default value for p_A12, p_A12 renamed to p_SrvccAlertingSupported sic@ */ /* @sic R5s150180: p_A10 removed sic@ */ /* @sic R5s150177 change 9, R5s150204: p_OtherCalleeUri changed to "template SipUrl p_OtherCalleeUri"; NOTE: p_OtherCalleeUri shall either omit or "template (present)" (but not e.g. "*") sic@ */ /* @sic R5s150846: p_A8 := false per default sic@ */ /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ /* @sic R5s170305: p_A9 := pc_IMS_SessionIdSupported per default sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5s190882: p_AdditionalOptionTagsForSupported removed sic@ */ /* @sic R5s220485: p_A8 removed sic@ */ var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_MimeWithXML := ispresent(p_MessageHeader_Invite.geolocation);/* @sic R5s230480 sic@ NOTE: there is no Geolocation without PIDF body part and no PIDF body part without Geolocation and the Geolcation addresses the PIDF body part within the MIME body */ var boolean v_MimeWithBinaryData := ispresent(p_MessageHeader_Invite.callInfo); /* @sic R5s230480 sic@ NOTE: there is no Call-Info without MSD and no MSD without Call-Info and Call-Info addresses the MSD body part within the MIME body */ var template (present) MessageBody v_MessageBody := f_SIP_MessageBodyWithSDP_RX(v_MimeWithXML, v_MimeWithBinaryData); /* @sic R5w140112 sic@ @sic R5s210009: v_UseMIME instead of p_A8 sic@ @sic R5s230480: v_MimeWithXML, v_MimeWithBinaryData instead of v_UseMIME sic@ */ var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); // @sic R5s190882 sic@ if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction); /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ } return v_IMS_DATA_REQ; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { /* p_OtherCalleeUri .. if present to be used as CalleeUri; if omit default CalleeUri is used acc. to PIXITs and context */ /* @sic R5-133276, R5-133301 Cleanup of functions for MO Call Setup sic@ */ /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5-140799: additional parameter p_MessageHeader to deal with Reason header sic@ */ /* @sic R5s140370 change 7: pc_IMS_SRVCCAlert default value for p_A12, p_A12 renamed to p_SrvccAlertingSupported sic@ */ /* @sic R5s150180: p_A10 removed sic@ */ /* @sic R5s150177 change 9, R5s150204: p_OtherCalleeUri changed to "template SipUrl p_OtherCalleeUri"; NOTE: p_OtherCalleeUri shall either omit or "template (present)" (but not e.g. "*") sic@ */ /* @sic R5s150846: p_A8 := false per default sic@ */ /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ /* @sic R5s170305: p_A9 := pc_IMS_SessionIdSupported per default sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5s190882: p_AdditionalOptionTagsForSupported removed sic@ */ /* @sic R5s220485: p_A8 removed sic@ */ var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_MimeWithXML := f_IsMimeWithXMLRequired(p_MessageHeader_Invite); /* @sic R5s230480 sic@ NOTE: there is no Geolocation without PIDF body part and no PIDF body part without Geolocation and the Geolcation addresses the PIDF body part within the MIME body */ var boolean v_MimeWithBinaryData := ispresent(p_MessageHeader_Invite.callInfo); /* @sic R5s230480 sic@ NOTE: there is no Call-Info without MSD and no MSD without Call-Info and Call-Info addresses the MSD body part within the MIME body */ var template (present) MessageBody v_MessageBody := f_SIP_MessageBodyWithSDP_RX(v_MimeWithXML, v_MimeWithBinaryData); /* @sic R5w140112 sic@ @sic R5s210009: v_UseMIME instead of p_A8 sic@ @sic R5s230480: v_MimeWithXML, v_MimeWithBinaryData instead of v_UseMIME sic@ */ var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); // @sic R5s190882 sic@ if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction); /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ } return v_IMS_DATA_REQ; } <<SKIPPED CODE>>

MCC160 Implementation

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_MimeWithPIDF := ispresent(p_MessageHeader_Invite.geolocation); var boolean v_MimeWithUSSD := match(valueof(cs_RecvInfo_Def("g.3gpp.ussd")), p_MessageHeader_Invite.recvInfo); var boolean v_MimeWithXML := v_MimeWithPIDF or v_MimeWithUSSD; var boolean v_MimeWithBinaryData := ispresent(p_MessageHeader_Invite.callInfo); var template (present) MessageBody v_MessageBody := f_SIP_MessageBodyWithSDP_RX(v_MimeWithXML, v_MimeWithBinaryData); var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction); } return v_IMS_DATA_REQ; }