3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230791

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3251 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC logged MDT test cases 8.1.6.1.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the logged MDT test cases, the other contents of UE-MeasurementsAvailable-r16 apart from logMeasAvailable-r16 are not supposed to be checked. These other fields may be reported by a conformant UE if it has stored information such as a previous RLF. Therefore current TTCN implementation for affected TCs should be modified to use * instead of omit. The same change has already been accepted for 8.1.6.1.2.3 in R5s230553[1]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For logged MDT test cases, change all calls of cr_UE_MeasurementsAvailable_r16 to use default value instead of omit except for the first input argument (p_LogMeasAvailable_r16) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.1.NR5GC, 8.1.6.1.2.2.NR5GC, 8.1.6.1.2.4.NR5GC, 8.1.6.1.2.5.NR5GC, 8.1.6.1.2.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

2.3	Change 3	11

2.4	Change 4	15

2.5	Change 5	19

2.6	Change 6	25

3	References	26

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_6_1_2_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | For the logged MDT test cases, the other contents of UE-MeasurementsAvailable-r16 apart from logMeasAvailable-r16 are not supposed to be checked. These other fields may be reported by a conformant UE if it has stored information such as a previous RLF. Therefore current TTCN implementation for affected TCs should be modified to use * instead of omit. The same change has already been accepted for 8.1.6.1.2.3 in R5s230553[1]. |
| Summary of change | Change cr_UE_MeasurementsAvailable_r16 to use default value instead of omit except for the first input argument (p_LogMeasAvailable_r16) |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted. Renamed cr_UE_MeasurementsAvailable_r16_LogMeas to cr_UE_MeasurementsAvailable_r16_LOG and used it instead of proposed cr_UE_MeasurementsAvailable_r16(true_) |

Before Change

function f_TC_8_1_6_1_2_1_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); //@sic R5s220121 sic@ var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); //@sic R5s220121 sic@ var integer i; var integer v_FieldCnt; var LogMeasInfo_r16 v_LogMeasInfo_r16; var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var ARFCN_ValueNR v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(nr_Cell11); //cell 11 ARFCN //@sic R5s220121 sic@ var PhysCellId v_PhysCellIdNeighbour:= f_NR_CellInfo_GetPhysicalCellId(nr_Cell11); //cell 11 CellId //@sic R5s220121 sic@ //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfiguration message SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ // Wait 5s to allow UE to activate logging. f_Delay (5.0); //@siclog "Step 4" siclog@ //Set SS/PBCH EPRE levels acc. to row "T1" f_NR_SetCellPower (nr_Cell1, -85, -91); //@sic R5-220329 R5-222116 sic@ f_NR_SetCellPower (nr_Cell11, -78, -82); //@sic R5-220329 R5-222116 sic@ //@siclog "Step 5" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11. f_NR_RRC_SetupRequest_Def(nr_Cell11); //@siclog "Step 6" siclog@ // The SS transmit an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell11); //@siclog "Step 7" siclog@ // Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_,omit,omit,omit,omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.6.1.2.1 Step 7"); //@siclog "Steps 8-10" siclog@ // Steps 4 to 6a1 of generic test procedure in TS 38.508-1[4] subclause 4.9.5.2.2-1 are performed on Cell 11. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11-18" siclog@ // Steps 1 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_,omit,omit,omit,omit)))); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); //@siclog "Step 19" siclog@ // The SS transmits a UEInformationRequest message on Cell 11. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, true_))); // siclog "Step 20" siclog@ // Check: Does the UE transmit a UEInformationResponse message with the IEs absoluteTimeStamp, traceReference, traceRecordingSessionRef and tce-Id set to same value as received in the LoggedMeasurementConfiguration message in step 1; and a LogMeasInfoList with at least two entries with serving cell idle mode measurements and where in at least one of the entries the IE measResultListNR include a neighbouring cell measurement of Cell 11; and where the relativeTimeStamp is increased between the subsequent LogMeasInfoList entries by at least the value of configured loggingInterval in the received LoggedMeasurementConfiguration message in step 1. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, // received on SRB2 //@sic R5s220121 sic@ cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: Not enough LogMeasurementInfo entries") } for (i:=0; i<v_FieldCnt; i:=i+1){ v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def(-, -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), //@sic R5s220121 sic@ cr_MeasResultServCell_r16(v_MeasQuantityResults), {cr_MeasResultLogging2NR_r16(v_ARFCN_Neighbour, {cr_MeasResultLoggingNR_r16(v_PhysCellIdNeighbour)})}, //@sic R5s220121 sic@, omit))) { if (i == v_FieldCnt - 1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: None of the LogMeasurementInfo matched the template"); } } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); break; } } }//end of f_TC_8_1_6_1_2_1_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_2_1_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); //@sic R5s220121 sic@ var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); //@sic R5s220121 sic@ var integer i; var integer v_FieldCnt; var LogMeasInfo_r16 v_LogMeasInfo_r16; var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var ARFCN_ValueNR v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(nr_Cell11); //cell 11 ARFCN //@sic R5s220121 sic@ var PhysCellId v_PhysCellIdNeighbour:= f_NR_CellInfo_GetPhysicalCellId(nr_Cell11); //cell 11 CellId //@sic R5s220121 sic@ //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfiguration message SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ // Wait 5s to allow UE to activate logging. f_Delay (5.0); //@siclog "Step 4" siclog@ //Set SS/PBCH EPRE levels acc. to row "T1" f_NR_SetCellPower (nr_Cell1, -85, -91); //@sic R5-220329 R5-222116 sic@ f_NR_SetCellPower (nr_Cell11, -78, -82); //@sic R5-220329 R5-222116 sic@ //@siclog "Step 5" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11. f_NR_RRC_SetupRequest_Def(nr_Cell11); //@siclog "Step 6" siclog@ // The SS transmit an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell11); //@siclog "Step 7" siclog@ // Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.6.1.2.1 Step 7"); //@siclog "Steps 8-10" siclog@ // Steps 4 to 6a1 of generic test procedure in TS 38.508-1[4] subclause 4.9.5.2.2-1 are performed on Cell 11. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11-18" siclog@ // Steps 1 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_)))); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); //@siclog "Step 19" siclog@ // The SS transmits a UEInformationRequest message on Cell 11. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, true_))); // siclog "Step 20" siclog@ // Check: Does the UE transmit a UEInformationResponse message with the IEs absoluteTimeStamp, traceReference, traceRecordingSessionRef and tce-Id set to same value as received in the LoggedMeasurementConfiguration message in step 1; and a LogMeasInfoList with at least two entries with serving cell idle mode measurements and where in at least one of the entries the IE measResultListNR include a neighbouring cell measurement of Cell 11; and where the relativeTimeStamp is increased between the subsequent LogMeasInfoList entries by at least the value of configured loggingInterval in the received LoggedMeasurementConfiguration message in step 1. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, // received on SRB2 //@sic R5s220121 sic@ cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: Not enough LogMeasurementInfo entries") } for (i:=0; i<v_FieldCnt; i:=i+1){ v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def(-, -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), //@sic R5s220121 sic@ cr_MeasResultServCell_r16(v_MeasQuantityResults), {cr_MeasResultLogging2NR_r16(v_ARFCN_Neighbour, {cr_MeasResultLoggingNR_r16(v_PhysCellIdNeighbour)})}, //@sic R5s220121 sic@, omit))) { if (i == v_FieldCnt - 1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: None of the LogMeasurementInfo matched the template"); } } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); break; } } }//end of f_TC_8_1_6_1_2_1_NR5GC_TestBody

## Change 2

| Function name | function f_TC_8_1_6_1_2_4_NR5GC_TestBody |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#101 |

Before Change

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300, R5-220331, R5-222118, R5-223496 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var PLMN_Identity v_PlmnID_NRCell1; var PLMN_Identity v_PlmnID_NRCell11; var TrackingAreaCode v_TAC_NRCell11; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity_NRCell11; var PhysCellId v_PhysCellId_NRCell1; var ARFCN_ValueNR v_CarrierFreq_NRCell1; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -78, -82), cs_NR_CellPower(nr_Cell11, -85, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, -85, -91), cs_NR_CellPower(nr_Cell11, -78, -82)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to 'periodical' and LoggingInterval is set to 'ms10240', to configure the UE to perform logging of measurement results while in RRC_IDLE v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 5s and the LoggingInterval will not be expired f_Delay(5.0); //@siclog "Steps 4" siclog@ //The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16_NologMeasAvailable) ifpresent); // @sic R5s220072, R5s220812 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 8-10" siclog@ //Steps 4 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 12-14" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); //@siclog "Steps 15" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); *skipped code*

After Change

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300, R5-220331, R5-222118, R5-223496 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var PLMN_Identity v_PlmnID_NRCell1; var PLMN_Identity v_PlmnID_NRCell11; var TrackingAreaCode v_TAC_NRCell11; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity_NRCell11; var PhysCellId v_PhysCellId_NRCell1; var ARFCN_ValueNR v_CarrierFreq_NRCell1; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -78, -82), cs_NR_CellPower(nr_Cell11, -85, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, -85, -91), cs_NR_CellPower(nr_Cell11, -78, -82)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to 'periodical' and LoggingInterval is set to 'ms10240', to configure the UE to perform logging of measurement results while in RRC_IDLE v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 5s and the LoggingInterval will not be expired f_Delay(5.0); //@siclog "Steps 4" siclog@ //The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16_NologMeasAvailable) ifpresent); // @sic R5s220072, R5s220812 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 8-10" siclog@ //Steps 4 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 12-14" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); //@siclog "Steps 15" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); *skipped code*

## Change 3

| Function name | function f_TC_8_1_6_1_2_5_NR5GC_TestBody |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#101 |

Before Change

function f_TC_8_1_6_1_2_5_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var TrackingAreaCode v_TAC; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; var RSRP_Range v_Thres; var DeltaValues_Type v_Deltas; v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Thres := 69; // @sic R5-216301 sic@ } else { v_Thres := 65 + v_Deltas.DeltaNRf1; // @sic R5-232393 sic@ } //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to eventL1, to configure the UE to perform logging of measurement results while in RRC_IDLE v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_EventL1(v_Thres)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16_NologMeasAvailable) ifpresent); // @sic R5s220073, R5-223496, R5s220812 sic@ v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 12" siclog@ //The SS transmits a UEInformationRequest message with logMeasReportReq present SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 13" siclog@ //Check: Does the UE transmit a UEInformationResponse message on SRB1 including logMeasReport SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, omit, omit, omit, omit, omit))); //@siclog "Steps 14" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 15" siclog@ //The SS changes NR Cell 1 levels according to the row "T1" in table 8.1.6.1.2.5.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -98, tsc_NR_NonSuitableCellSSS_EPRE_FR2); // @sic R5-232393 sic@ //@siclog "Steps 16" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 17-19" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Steps 20" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit)))); // @sic R5-223496 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); *skipped code*

After Change

function f_TC_8_1_6_1_2_5_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var TrackingAreaCode v_TAC; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; var RSRP_Range v_Thres; var DeltaValues_Type v_Deltas; v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Thres := 69; // @sic R5-216301 sic@ } else { v_Thres := 65 + v_Deltas.DeltaNRf1; // @sic R5-232393 sic@ } //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to eventL1, to configure the UE to perform logging of measurement results while in RRC_IDLE v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_EventL1(v_Thres)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16_NologMeasAvailable) ifpresent); // @sic R5s220073, R5-223496, R5s220812 sic@ v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 12" siclog@ //The SS transmits a UEInformationRequest message with logMeasReportReq present SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 13" siclog@ //Check: Does the UE transmit a UEInformationResponse message on SRB1 including logMeasReport SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, omit, omit, omit, omit, omit))); //@siclog "Steps 14" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 15" siclog@ //The SS changes NR Cell 1 levels according to the row "T1" in table 8.1.6.1.2.5.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -98, tsc_NR_NonSuitableCellSSS_EPRE_FR2); // @sic R5-232393 sic@ //@siclog "Steps 16" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 17-19" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Steps 20" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_)))); // @sic R5-223496 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); *skipped code*

## Change 4

| Function name | function f_TC_8_1_6_1_2_6_NR5GC_TestBody |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#101 |

Before Change

function f_TC_8_1_6_1_2_6_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var TrackingAreaCode v_TAC; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_RelativeTimeStamp1; var integer v_RelativeTimeStamp2; var integer v_FieldCnt; var integer i; //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage, to configure the UE to perform logging of measurement results v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_OutOfCoverage))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,(cr_UE_MeasurementsAvailable_r16_NologMeasAvailable)ifpresent) ifpresent); // @sic R5s220094, R5-223496, R5s220812, R5-234635 sic@ v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 12" siclog@ //The SS transmits a UEInformationRequest message with logMeasReportReq present SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 13" siclog@ //Check: Does the UE transmit a UEInformationResponse message on SRB1 including logMeasReport SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, omit, omit, omit, omit, omit))); //@siclog "Steps 14" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 15" siclog@ //The SS changes NR Cell 1 level according to the row "T1" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @sic R5-216302 sic@ //@siclog "Steps 16" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 17" siclog@ //The SS changes NR Cell 1 level according to the row "T2" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -85, -82); // @sic R5-216302 sic@ //@siclog "Steps 18" siclog@ //Wait 10s to allow UE to go to camp normaly state and activate logging f_Delay(10.0); //@siclog "Steps 19" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Steps 20" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit))));// @sic R5-223496 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); *skipped code*

After Change

function f_TC_8_1_6_1_2_6_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var TrackingAreaCode v_TAC; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_RelativeTimeStamp1; var integer v_RelativeTimeStamp2; var integer v_FieldCnt; var integer i; //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage, to configure the UE to perform logging of measurement results v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_OutOfCoverage))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,(cr_UE_MeasurementsAvailable_r16_NologMeasAvailable)ifpresent) ifpresent); // @sic R5s220094, R5-223496, R5s220812, R5-234635 sic@ v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 12" siclog@ //The SS transmits a UEInformationRequest message with logMeasReportReq present SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 13" siclog@ //Check: Does the UE transmit a UEInformationResponse message on SRB1 including logMeasReport SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, omit, omit, omit, omit, omit))); //@siclog "Steps 14" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 15" siclog@ //The SS changes NR Cell 1 level according to the row "T1" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @sic R5-216302 sic@ //@siclog "Steps 16" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 17" siclog@ //The SS changes NR Cell 1 level according to the row "T2" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -85, -82); // @sic R5-216302 sic@ //@siclog "Steps 18" siclog@ //Wait 10s to allow UE to go to camp normaly state and activate logging f_Delay(10.0); //@siclog "Steps 19" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Steps 20" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_))));// @sic R5-223496 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); *skipped code*

## Change 5

| Function name | function f_TC_8_1_6_1_2_10_NR5GC_TestBody |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted. See change 1. |

Before Change

function f_TC_8_1_6_1_2_10_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-221566 R5s220370 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN (nr_Cell11); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var CellIdentity v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var TrackingAreaCode v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var boolean v_EntryFound_NR_Cell_1 := false; var boolean v_EntryFound_NR_Cell_11 := false; var integer i:=0; var float v_WaitToLogging := 10.0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell3, -85, -82) }; var template (value) NR_CellPowerList_Type v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell11, -85, -82), cs_NR_CellPower (nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ // The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1 SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ // Wait 10 seconds for UE performing the logging at regular time intervals f_Delay(v_WaitToLogging); //@siclog "Step 4" siclog@ // The SS changes NR Cell 1 and NR Cell 3 level according to the row "T1" in Table 8.1.6.1.2.10.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); // Table 8.1.6.1.2.10.3.2-1 //@siclog "Step 5-10" siclog@ // Steps 1 to 6a1 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 3. f_NR5GC_MobilityRegistration (nr_Cell3); //@siclog "Step 11" siclog@ // Wait 10 seconds for UE not to perform the logging at regular time intervals. f_Delay(v_WaitToLogging); //@siclog "Step 12" siclog@ // TThe SS transmits a Paging message. f_NR_Paging_Def(nr_Cell3, RRC_IDLE); //@siclog "Step 13" siclog@ // The UE transmits an RRCRequest message. //@siclog "Step 14" siclog@ // The SS transmits an RRCSetup message. //@siclog "Step 15" siclog@ // Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell3,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit) ifpresent) ifpresent ); //@sic R5s225389 R5-234636 sic@ //@siclog "Step 15A-15D" siclog@ // Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell3, v_ReceivedMsg); //@siclog "Step 16" siclog@ // The SS transmits a RRCRelease message to release RRC connection and move to RRC_IDLE on NR Cell 3. f_NR_RRCRelease(nr_Cell3); //@siclog "Step 17" siclog@ // The SS changes NR Cell 1, NR Cell 11 and NR Cell 3 level according to the row "T2" in Table 8.1.6.1.2.10.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 18-23" siclog@ // Steps 1 to 6a1 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 11. f_NR5GC_MobilityRegistration (nr_Cell11); //@siclog "Step 24" siclog@ // Wait 10 seconds for UE performing the logging at regular time intervals f_Delay(v_WaitToLogging); // @siclog "Step 25" siclog@ // The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); // @siclog "Step 26" siclog@ // The UE transmits an RRCRequest message. // @siclog "Step 27" siclog@ // The SS transmits an RRCSetup message. // @siclog "Step 28" siclog@ // Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable set as true? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_,omit,omit,omit,omit))); // receive RRC Connection Setup Complete as per Table 8.1.6.1.2.10.3.3-2 // @siclog "Step 29-31" siclog@ // Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); // @siclog "Step 32" siclog@ // The SS send a UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_NR_UEInformationRequest_LOG)); // Logged MDT // @siclog "Step 33" siclog@ // Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for NR Cell 1 and NR Cell 11 and without serving cell measurements for NR Cell 3? v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell11,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33: Not enough LogMeasurementInfo entries") } while ((i < v_FieldCnt) and (not v_EntryFound_NR_Cell_1 or not v_EntryFound_NR_Cell_11)) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def(-,-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { v_EntryFound_NR_Cell_1 := true; } else if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def(-, -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell11, v_CellIdentity_NRCell11, v_TAC_NRCell11), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { v_EntryFound_NR_Cell_11 := true; } i:=i+1; } if (v_EntryFound_NR_Cell_1 and v_EntryFound_NR_Cell_11) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 33"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33: condition not fulfilled for the LogMeasurementInfo entries"); } }//end of f_TC_8_1_6_1_2_10_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_2_10_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-221566 R5s220370 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN (nr_Cell11); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var CellIdentity v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var TrackingAreaCode v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var boolean v_EntryFound_NR_Cell_1 := false; var boolean v_EntryFound_NR_Cell_11 := false; var integer i:=0; var float v_WaitToLogging := 10.0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell3, -85, -82) }; var template (value) NR_CellPowerList_Type v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell11, -85, -82), cs_NR_CellPower (nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ // The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1 SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ // Wait 10 seconds for UE performing the logging at regular time intervals f_Delay(v_WaitToLogging); //@siclog "Step 4" siclog@ // The SS changes NR Cell 1 and NR Cell 3 level according to the row "T1" in Table 8.1.6.1.2.10.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); // Table 8.1.6.1.2.10.3.2-1 //@siclog "Step 5-10" siclog@ // Steps 1 to 6a1 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 3. f_NR5GC_MobilityRegistration (nr_Cell3); //@siclog "Step 11" siclog@ // Wait 10 seconds for UE not to perform the logging at regular time intervals. f_Delay(v_WaitToLogging); //@siclog "Step 12" siclog@ // TThe SS transmits a Paging message. f_NR_Paging_Def(nr_Cell3, RRC_IDLE); //@siclog "Step 13" siclog@ // The UE transmits an RRCRequest message. //@siclog "Step 14" siclog@ // The SS transmits an RRCSetup message. //@siclog "Step 15" siclog@ // Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell3,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit) ifpresent) ifpresent ); //@sic R5s225389 R5-234636 sic@ //@siclog "Step 15A-15D" siclog@ // Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell3, v_ReceivedMsg); //@siclog "Step 16" siclog@ // The SS transmits a RRCRelease message to release RRC connection and move to RRC_IDLE on NR Cell 3. f_NR_RRCRelease(nr_Cell3); //@siclog "Step 17" siclog@ // The SS changes NR Cell 1, NR Cell 11 and NR Cell 3 level according to the row "T2" in Table 8.1.6.1.2.10.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 18-23" siclog@ // Steps 1 to 6a1 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 11. f_NR5GC_MobilityRegistration (nr_Cell11); //@siclog "Step 24" siclog@ // Wait 10 seconds for UE performing the logging at regular time intervals f_Delay(v_WaitToLogging); // @siclog "Step 25" siclog@ // The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); // @siclog "Step 26" siclog@ // The UE transmits an RRCRequest message. // @siclog "Step 27" siclog@ // The SS transmits an RRCSetup message. // @siclog "Step 28" siclog@ // Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable set as true? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_))); // receive RRC Connection Setup Complete as per Table 8.1.6.1.2.10.3.3-2 // @siclog "Step 29-31" siclog@ // Steps 5 to 8 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); // @siclog "Step 32" siclog@ // The SS send a UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_NR_UEInformationRequest_LOG)); // Logged MDT // @siclog "Step 33" siclog@ // Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for NR Cell 1 and NR Cell 11 and without serving cell measurements for NR Cell 3? v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell11,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33: Not enough LogMeasurementInfo entries") } while ((i < v_FieldCnt) and (not v_EntryFound_NR_Cell_1 or not v_EntryFound_NR_Cell_11)) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def(-,-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { v_EntryFound_NR_Cell_1 := true; } else if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def(-, -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell11, v_CellIdentity_NRCell11, v_TAC_NRCell11), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { v_EntryFound_NR_Cell_11 := true; } i:=i+1; } if (v_EntryFound_NR_Cell_1 and v_EntryFound_NR_Cell_11) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 33"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33: condition not fulfilled for the LogMeasurementInfo entries"); } }//end of f_TC_8_1_6_1_2_10_NR5GC_TestBody

## Change 6

| Function name | Template cr_RRCResumeComplete_ue_MeasurementsAvailable_r16 |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted. See change 1. |

Before Change

template(present) UL_DCCH_Message cr_RRCResumeComplete_ue_MeasurementsAvailable_r16 ( RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def ) := { /* @status APPROVED (NR5GC) */ message_ := { c1 := { rrcResumeComplete := { rrc_TransactionIdentifier := p_RRC_TI, criticalExtensions := { rrcResumeComplete := { dedicatedNAS_Message := *, selectedPLMN_Identity := *, uplinkTxDirectCurrentList := *, lateNonCriticalExtension := *, nonCriticalExtension := { idleMeasAvailable_r16 := *, measResultIdleEUTRA_r16 := *, measResultIdleNR_r16 := *, scg_Response_r16 := *, ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(true_,omit,omit,omit,omit), /*{ logMeasAvailable_r16 := true_, logMeasAvailableBT_r16 := omit, logMeasAvailableWLAN_r16 := omit, connEstFailInfoAvailable_r16 := omit, rlf_InfoAvailable_r16 := omit, successHO_InfoAvailable_r17 := *, // @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ sigLogMeasConfigAvailable_r17 := * // @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ },*/ mobilityHistoryAvail_r16 := *, mobilityState_r16 := *, needForGapsInfoNR_r16 := *, nonCriticalExtension := * } } } } } } };

After Change

template(present) UL_DCCH_Message cr_RRCResumeComplete_ue_MeasurementsAvailable_r16 ( RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def ) := { /* @status APPROVED (NR5GC) */ message_ := { c1 := { rrcResumeComplete := { rrc_TransactionIdentifier := p_RRC_TI, criticalExtensions := { rrcResumeComplete := { dedicatedNAS_Message := *, selectedPLMN_Identity := *, uplinkTxDirectCurrentList := *, lateNonCriticalExtension := *, nonCriticalExtension := { idleMeasAvailable_r16 := *, measResultIdleEUTRA_r16 := *, measResultIdleNR_r16 := *, scg_Response_r16 := *, ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(true_), /*{ logMeasAvailable_r16 := true_, logMeasAvailableBT_r16 := omit, logMeasAvailableWLAN_r16 := omit, connEstFailInfoAvailable_r16 := omit, rlf_InfoAvailable_r16 := omit, successHO_InfoAvailable_r17 := *, // @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ sigLogMeasConfigAvailable_r17 := * // @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ },*/ mobilityHistoryAvail_r16 := *, mobilityState_r16 := *, needForGapsInfoNR_r16 := *, nonCriticalExtension := * } } } } } } };

# References

| [1] | R5s230553: Correction for NR5GC MDT test case 8.1.6.1.2.3 |
| --- | --- |