3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240685

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4868 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE RACS testcase 8.5.5.1 in TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add LTE test case 8.5.5.1 to the IWD_24wk37 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.5.5.1(TDD) required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant device will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1(TDD) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR R5-247536 is raised for this in RAN5#105. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

1.2	Change 1	3

2	Corrections required	5

3	Branches executed	5

4	Execution Log Files	5

4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE	5

4.2	Samsung Exynos 2400	5

4.3	Samsung Shannon 5400ERD(S5E9915 ERD)	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.5.5.1(TDD) which is part of the LTE test suite in iwd-TTCN3-B2024-06_D24wk37 TTCN-3.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Parikshit Bhise
	parikshit.bhise@rohde-schwarz.com

Mohit Kanchan
	mohit.kamchan@keysight.com

## Verification Test Summary

Test Case: 	8.5.5.1(TDD)

ATS Version:	iwd-TTCN3-B2024-06_D24wk37

System Simulator used:	R&S CMW500 platform, Keysight S8704A Protocol Conformance Toolset solution

UE used:	The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE ,Samsung Exynos 2400, Samsung Shannon 5400 ERD (S5E9915 ERD) UE

Verification Status:	PASS

## Change 1

| Function name | f_TC_8_5_5_1_EUTRA() |
| --- | --- |
| Reason for change | - At step 5, SS is allocating UL grants periodically from 18 (TDD) subframes after successful completion of step 4. This results in UL grant starts from Subframe = 2 . As per the 36.211 Table 4.2-2: Uplink-downlink configurations, it is for uplink. So to have the grant scheduled properly, it is proposed to start the Periodic grant 20 subframes after the completion of step 4 , so that TTCN can configure starting subframe as 4 which is downlink subframe. - Current TTCN allocation use MCS and NPRB as 20, 4 which is equivalent to 220 bytes which are not sufficient to handle the additional UL grant on top of 16 UL grant to handle the RLC, MAC header and BSR. The 16 times periodic UL grant is 501 bytes which is equivalent to 16* 498 =7968 bytes (501 bytes is 498 bytes of data after reducing 3 bytes of RLC+MAC header). This leaves residual 17th segment requiring 220 bytes (8188 - 7968 = 220) of data. On top of this BSR also needs to be added. So it is proposed to change the MCS and NPRB UL grant allocation to 225 bytes |
| Summary of change | - It is proposed to start the Periodic grant 20 subframes after the completion of step 4 , so that TTCN can configure starting subframe as 4 which is DL subframe. - Changed the MCS(18) and NPRB(5) for UL grant allocation == 225 bytes |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment | Accepted. Implementation as per R5-247692. |

Before change

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC

{ /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */

<<SKIPPED CODE >>

//@siclog "Step 5" siclog@

// The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms

v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.

}

else {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms //WA#WI=1323866

v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.

<<SKIPPED CODE >>

//@sic R5s240175 sic@

v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,16*v_GrantPeriod);

f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 20, 4); // Imcs  & Nprb => Acc. 36.523-3, 1416 bits TBS

<<SKIPPED CODE >>

}

After change

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC

{ /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */

<<SKIPPED CODE >>

//@siclog "Step 5" siclog@

// The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms

v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.

}

else {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 20); // 20 subframes = 20 ms

v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.

}

<<SKIPPED CODE >>

//@sic R5s240175 sic@

v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,16*v_GrantPeriod);

f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 18, 5); // Imcs  & Nprb => Acc. 36.523-3, 1416 bits TBS

<<SKIPPED CODE >>

}

# Corrections required

# Branches executed

The test case was executed on Eutra band 38 using ciphering algorithm eea1 and integrity algorithm eia1 on R&S CMW 500 platform, and on Eutra Band 41 using ciphering algorithm eea1 and integrity algorithm eia1 on Keysight S8704A Protocol Conformance Toolset solution.

# Execution Log Files

## Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE passed this test case on R&S CMW500 platform in Band 38 with eea1 and eia1 algorithms.

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE passed this test case on Keysight S8704A Protocol Conformance Toolset solution in Band 41 with eea1 and eia1 algorithms.

The documentation below is enclosed as evidence of the successful test case run [1]:

R&S log :

Test case execution log file:

TC_8_5_5_1_Qualcomm_TDD.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)
In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented, and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

Keysight log :

TC_8_5_5_1_PIXIT.xml

TC_8_5_5_1_LOG.html

## Samsung Exynos 2400

The Samsung Exynos 2400 UE passed this test case on R&S CMW500 platform in Band 38 with eea1 and eia1 algorithms. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_5_5_1_TDD.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)
In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented, and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

## Samsung Shannon 5400ERD(S5E9915 ERD)

The Samsung Shannon 5400 ERD (S5E9915 ERD) UE passed this test case on Keysight S8704A Protocol Conformance Toolset solution in Band 41 with eea1 and eia1 algorithms. The documentation below is enclosed as evidence of the successful test case run [1]:

TC_8_5_5_1_PIXIT.xml

TC_8_5_5_1_LOG.html

# References

| [1] | R5s240683: Supporting information for addition of LTE RACS testcase 8.5.5.1 in TDD |
| --- | --- |