3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250057

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4883 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcases 22.3.1.3 and 22.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For NB-IoT NTN, Koffset needs to be taken into account to ensure the RLC STATUS PDU is received by UE and then SS transmits MAC PDU.This change was covered in R5s240706 for Steps 7A/8 of test case 22.3.1.4 but is required in several places for test case 22.3.1.3 and 22.3.1.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0) After several steps in both the test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.3 and 22.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the f_TC_22_3_1_3_NBIOT	2

2.2	Change to the f_TC_22_3_1_4_NBIOT	11

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_3_NBIOT

| Function name | f_TC_22_3_1_3_NBIOT |
| --- | --- |
| Reason for change | For NB-IoT NTN, Koffset needs to be taken into account to ensure the RLC STATUS PDU is received by UE and then SS transmits MAC PDU.This change was covered in R5s240706 for Steps 7A/8 of test case 22.3.1.4 but is required in several places for test case 22.3.1.3 and 22.3.1.4 |
| Summary of change | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0) - After Step 6 - After Step 12 - After Step 16 - After Step 20A - After Step 23 |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment |  |

Before Change:

function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC { … … … //@siclog "Step 6" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ //@siclog "Step 7" siclog@ f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 8, 10" siclog@ v_TransRetransmissionList := { cs_NB_UL_TransRetransmission_NewTransmission, // step 8 cs_NB_UL_TransRetransmission_ReTransmission(1) // step 10 }; v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); /* @sic R5-181306: Iru := 2, Imcs := 5 => TBS=224 sic@ @sic R5-182584: Iru := 2, Imcs := 4 => TBS=208 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); // MAC PDU as expected for step 9 and 11 v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //@siclog "Step 9" siclog@ v_IncrementVRR := 0; /* AM_VRR shall not be incremented for this step as there is an adaptive HARQ restransmission at step 11 */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX, v_IncrementVRR); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Step 11" siclog@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 11"); //@siclog "Step 12" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 13" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSduList := { crs_PDCP_SDU_15B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList, tsc_Poll); //@siclog "Step 14" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 15" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_RlcAmdPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_RlcSduList); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 16" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 17" siclog@ // Void //@siclog "Step 18" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 19" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 20" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 20A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ //@siclog "Step 21" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_11B; // RLC SDU of 11 bytes results in AM RLC PDU of 13 bytes v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 22" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-2415646, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 23" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); // @sic R5s170236 change 8: noBSR sic@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Step 23A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ //@siclog "Step 24" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 25" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 26" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := (f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, twoBytePadding, anyBSR, v_MAC_SDUList_RX), f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX)); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 26"); //@siclog "Step 27" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); }

After Change:

| function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC { … … … //@siclog "Step 6" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //WA#WI=1381782 Delay shall be after Step 7A due to immediate activation time if(f_NBIOT_MobileInfo_IsNTN()) { f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ //@siclog "Step 7" siclog@ f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 8, 10" siclog@ v_TransRetransmissionList := { cs_NB_UL_TransRetransmission_NewTransmission, // step 8 cs_NB_UL_TransRetransmission_ReTransmission(1) // step 10 }; v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); /* @sic R5-181306: Iru := 2, Imcs := 5 => TBS=224 sic@ @sic R5-182584: Iru := 2, Imcs := 4 => TBS=208 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); // MAC PDU as expected for step 9 and 11 v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //@siclog "Step 9" siclog@ v_IncrementVRR := 0; /* AM_VRR shall not be incremented for this step as there is an adaptive HARQ restransmission at step 11 */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX, v_IncrementVRR); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Step 11" siclog@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 11"); //@siclog "Step 12" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 13" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSduList := { crs_PDCP_SDU_15B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList, tsc_Poll); //@siclog "Step 14" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 15" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_RlcAmdPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_RlcSduList); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 16" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 17" siclog@ // Void //@siclog "Step 18" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 19" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 20" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 20A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 21" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_11B; // RLC SDU of 11 bytes results in AM RLC PDU of 13 bytes v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 22" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-2415646, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 23" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); // @sic R5s170236 change 8: noBSR sic@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Step 23A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 24" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 25" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 26" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := (f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, twoBytePadding, anyBSR, v_MAC_SDUList_RX), f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX)); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 26"); //@siclog "Step 27" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } |  |
| --- | --- |

## Change to the f_TC_22_3_1_4_NBIOT

| Function name | f_TC_22_3_1_4_NBIOT () |
| --- | --- |
| Reason for change | For NB-IoT NTN, Koffset needs to be taken into account to ensure the RLC STATUS PDU is received by UE and then SS transmits MAC PDU.This change was covered in R5s240706 for Steps 7A/8 of test case 22.3.1.4 but is required in several places for test case 22.3.1.3 and 22.3.1.4 |
| Summary of change | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0) - After Step 10C - After Step 14 - After Step 18 - After Step 21A |
| TTCN module |  |
| MCC160 Comment |  |

Before Change:

function fl_TC_22_3_1_4_RetxBSR_Timer_NpdcchPeriods(RetxBSR_Timer_NB_r13 p_RetxBSR_Timer) return integer { … … … //@siclog "Step 10C" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 11" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_10B, crs_PDCP_SDU_10B, crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 12" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170821 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 9, v_TimingInfo); /* UL grant of 296 bits (ITBS=9, IRU=1) -> 37 bytes */ //@siclog "Step 13" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 3 bytes for 2 LIs, 3 * 10 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); /* 1 byte padding, 1 byte SDU sub-header, 35 bytes MAC SDU */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 15" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170238 change 2.5: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_8B, crs_PDCP_SDU_7B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 16" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-172835/R5s170238 change 2.5: three NPDCCH cycles to give the UE time for processing the data sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 8, v_TimingInfo); /* UL grant of 256 bits (ITBS=8, IRU=1) @sic R5s170238 change 2.6: ITBS=8 sic@ */ //@siclog "Step 17" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 2 bytes for 1 LI, 8 + 7 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, zeroBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 19" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5s170821 sic@ */ v_RlcSduList := { crs_PDCP_SDU_12B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 20" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170785: scheduled grant rather than "now" sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 0, 9, v_TimingInfo); /* UL grant of 136 bits (ITBS=9, IRU=0) */ //@siclog "Step 21" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, zeroBSR, v_MAC_SDUList_RX); /* @sic R5s170238 change 2.8 - MMC160 implementation: zeroBSR instead of anyBSR sic@ */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@siclog "Step 21A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172835/R5s170238 change 2.9 sic@ */ //@siclog "Step 21B": leave test mode and release RRC connection siclog@ f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, disable); f_NBIOT_L2_CPMode_RestoreSRBs(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start); f_NBIOT_RRC_ConnectionRelease(v_CellId); /* @sic R5-176882, R5s170821: RRC Connecntion Release sic@ */ //@siclog "Step 21C" siclog@ /* 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed: Get UE back to connected mode but with changed BSR timer values */ v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer_Step21C; v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Step21C; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_SS_ConfigRachProcedureMsg4(v_CellId, v_RrcConnSetup); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(v_CellId); /* @sic R5-176882, R5s170821: 'Generic Test Procedure acc. to 8.1.5A.2.3 (check Idle state) sic@ */ // return to test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(v_CellId, TransparentMode_RLC_MAC); /* @sic R5s180148 sic@ */ //@siclog "Step 22" siclog@ v_RlcSduList := { crs_PDCP_SDU_14B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, -, v_RlcSduList); //@siclog "Step 23 - 24" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); v_BSR_Index_Min := 1; fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); for (i := 0; i < 2; i := i + 1) { //@siclog "Step 25" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); //@sic R5s240593 sic@ v_BSR_Index_Min := 1; v_NoOfNpdcchPeriods := fl_TC_22_3_1_4_PeriodicBSR_Timer_NpdcchPeriods(v_PeriodicBSR_Timer_Step21C) + v_NpdcchBundleOffset; // @sic R5s240266, R5-241564 sic@ "+1" (Npdcch period bundle) to be sure that PeriodicBSR-Timer has been expired v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, int2float(v_NoOfNpdcchPeriods)); //@siclog "Step 26 - 27" siclog@ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 27 - " & int2str(i)); } f_NBIOT_TestBody_Set(false); //------------------------------------ // give default grant to the UE to allow transmission of pending data f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170238 change 2.11: single grant sic@ */ v_MAC_PDU_RX := ?; // any MAC PDU containing pending data f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); }

After Change:

| function fl_TC_22_3_1_4_RetxBSR_Timer_NpdcchPeriods(RetxBSR_Timer_NB_r13 p_RetxBSR_Timer) return integer { … … … //@siclog "Step 10C" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-176882, R5s170821 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 11" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_10B, crs_PDCP_SDU_10B, crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 12" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170821 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 9, v_TimingInfo); /* UL grant of 296 bits (ITBS=9, IRU=1) -> 37 bytes */ //@siclog "Step 13" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 3 bytes for 2 LIs, 3 * 10 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); /* 1 byte padding, 1 byte SDU sub-header, 35 bytes MAC SDU */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 15" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170238 change 2.5: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_8B, crs_PDCP_SDU_7B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 16" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-172835/R5s170238 change 2.5: three NPDCCH cycles to give the UE time for processing the data sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 8, v_TimingInfo); /* UL grant of 256 bits (ITBS=8, IRU=1) @sic R5s170238 change 2.6: ITBS=8 sic@ */ //@siclog "Step 17" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 2 bytes for 1 LI, 8 + 7 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, zeroBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 19" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5s170821 sic@ */ v_RlcSduList := { crs_PDCP_SDU_12B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 20" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170785: scheduled grant rather than "now" sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 0, 9, v_TimingInfo); /* UL grant of 136 bits (ITBS=9, IRU=0) */ //@siclog "Step 21" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, zeroBSR, v_MAC_SDUList_RX); /* @sic R5s170238 change 2.8 - MMC160 implementation: zeroBSR instead of anyBSR sic@ */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@siclog "Step 21A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172835/R5s170238 change 2.9 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 21B": leave test mode and release RRC connection siclog@ f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, disable); f_NBIOT_L2_CPMode_RestoreSRBs(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start); f_NBIOT_RRC_ConnectionRelease(v_CellId); /* @sic R5-176882, R5s170821: RRC Connecntion Release sic@ */ //@siclog "Step 21C" siclog@ /* 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed: Get UE back to connected mode but with changed BSR timer values */ v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer_Step21C; v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Step21C; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_SS_ConfigRachProcedureMsg4(v_CellId, v_RrcConnSetup); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(v_CellId); /* @sic R5-176882, R5s170821: 'Generic Test Procedure acc. to 8.1.5A.2.3 (check Idle state) sic@ */ // return to test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(v_CellId, TransparentMode_RLC_MAC); /* @sic R5s180148 sic@ */ //@siclog "Step 22" siclog@ v_RlcSduList := { crs_PDCP_SDU_14B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, -, v_RlcSduList); //@siclog "Step 23 - 24" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); v_BSR_Index_Min := 1; fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); for (i := 0; i < 2; i := i + 1) { //@siclog "Step 25" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); //@sic R5s240593 sic@ v_BSR_Index_Min := 1; v_NoOfNpdcchPeriods := fl_TC_22_3_1_4_PeriodicBSR_Timer_NpdcchPeriods(v_PeriodicBSR_Timer_Step21C) + v_NpdcchBundleOffset; // @sic R5s240266, R5-241564 sic@ "+1" (Npdcch period bundle) to be sure that PeriodicBSR-Timer has been expired v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, int2float(v_NoOfNpdcchPeriods)); //@siclog "Step 26 - 27" siclog@ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 27 - " & int2str(i)); } f_NBIOT_TestBody_Set(false); //------------------------------------ // give default grant to the UE to allow transmission of pending data f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170238 change 2.11: single grant sic@ */ v_MAC_PDU_RX := ?; // any MAC PDU containing pending data f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } |  |
| --- | --- |