3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250081

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4872 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.1.4.24b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the currentTTCN implementation, at Step 2 the timing calculation to configure UL grant is not handling the incrementing of HSFN in case of a SFN wrap around. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN to handle HSFN handling for Step 2. Please see below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.24b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_4_24b_EUTRA |
| --- | --- |
| Reason for change | In the currentTTCN implementation, at Step 2 the timing calculation to configure UL grant is not handling the incrementing of HSFN in case of a SFN wrap around. |
| Summary of change | Updated the TTCN to handle HSFN handling for Step 2. |
| TTCN module | MAC_eMTC_eDRX.ttcn |

Before Change:

<<SKIPPED CODE>> v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, 200); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing), cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRLCPDU)))); // Pre config SS for UL Grant of 120 bits //Note the function is not used here to guarantee sub frame 9 //schedule 59 sub frames in future; //Subframe 9 means for both FDD and TDD, UE send ul data in sub frame 3. v_Timing.SFN.Number := (v_Timing.SFN.Number + 5) mod 1024; v_Timing.Subframe.Number := 9; //@siclog "Step 2, 4, 6 & 8" siclog@ f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing), 4, /* Itbs = 4, */ 2, /* Nprb = 2 Table 7.1.7.2.1-1 of 36.213: Imcs = 4 -> Itbs = 4 MCS index of table 8.6.1-1 of 36.213 */ v_UL_TransRetransmissionList1, -, i); //Harq Process i // Timing is not checked in TTCN and SS will take care of the timing. <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, 200); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing), cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRLCPDU)))); // Pre config SS for UL Grant of 120 bits //Note the function is not used here to guarantee sub frame 9 //schedule 59 sub frames in future; //Subframe 9 means for both FDD and TDD, UE send ul data in sub frame 3. v_Timing.SFN.Number := (v_Timing.SFN.Number + 5) mod 1024; v_Timing.HSFN.Number := (v_Timing.HSFN.Number + (v_Timing.SFN.Number + 5)/1024) mod 1024; v_Timing.Subframe.Number := 9; //@siclog "Step 2, 4, 6 & 8" siclog@ f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing), 4, /* Itbs = 4, */ 2, /* Nprb = 2 Table 7.1.7.2.1-1 of 36.213: Imcs = 4 -> Itbs = 4 MCS index of table 8.6.1-1 of 36.213 */ v_UL_TransRetransmissionList1, -, i); //Harq Process i // Timing is not checked in TTCN and SS will take care of the timing. <<SKIPPED CODE>>