3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250090

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3637 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_IRAT_MobilityRegistration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of this function, when function f_Check_NG_RegistrationReqMsg is called, it expects the IE EPS NAS Message Container to be present (set as ?). As per 24.501 cl 5.5.1.3.2 (Rel-17 and above) : Quote … If the UE operating in the single-registration mode performs inter-system change from S1 mode to N1 mode, the UE: a) shall include the UE status IE with the EMM registration status set to "UE is in EMM-REGISTERED state" in the REGISTRATION REQUEST message; NOTE 7: Inclusion of the UE status IE with this setting corresponds to the indication that the UE is "moving from EPC" as specified in 3GPP TS 23.502 [9], subclause 4.11.1.3.3 and 4.11.2.3. NOTE 8: The value of the 5GMM registration status included by the UE in the UE status IE is not used by the AMF. b) may include the PDU session status IE in the REGISTRATION REQUEST message indicating the status of the PDU session(s) mapped during the inter-system change from S1 mode to N1 mode from the PDN connection(s) for which the EPS indicated that interworking to 5GS is supported, if any (see subclause 6.1.4.1); c) shall include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE in the REGISTRATION REQUEST message if the registration procedure is initiated in 5GMM-IDLE mode and the UE has received an "interworking without N26 interface not supported" indication from the network; c1) may include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE in the REGISTRATION REQUEST message if the registration procedure is initiated in 5GMM-IDLE mode and the UE has received an "interworking without N26 interface supported" indication from the network; and …. Unquote The current TTCN will fail a Rel-17 (or above) device that does not include the IE EPS NAS Message Container. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the TTCN function to accommodate the above behaviour. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.11.NR5GC, and possibly other testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | function f_NR5GC_IRAT_MobilityRegistration |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, when function f_Check_NG_RegistrationReqMsg is called, it expects the IE EPS NAS Message Container to be present (set as ?). As per 24.501 cl 5.5.1.3.2 (Rel-17 and above) : Quote … If the UE operating in the single-registration mode performs inter-system change from S1 mode to N1 mode, the UE: a) shall include the UE status IE with the EMM registration status set to "UE is in EMM-REGISTERED state" in the REGISTRATION REQUEST message; NOTE 7: Inclusion of the UE status IE with this setting corresponds to the indication that the UE is "moving from EPC" as specified in 3GPP TS 23.502 [9], subclause 4.11.1.3.3 and 4.11.2.3. NOTE 8: The value of the 5GMM registration status included by the UE in the UE status IE is not used by the AMF. b) may include the PDU session status IE in the REGISTRATION REQUEST message indicating the status of the PDU session(s) mapped during the inter-system change from S1 mode to N1 mode from the PDN connection(s) for which the EPS indicated that interworking to 5GS is supported, if any (see subclause 6.1.4.1); c) shall include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE in the REGISTRATION REQUEST message if the registration procedure is initiated in 5GMM-IDLE mode and the UE has received an "interworking without N26 interface not supported" indication from the network; c1) may include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE in the REGISTRATION REQUEST message if the registration procedure is initiated in 5GMM-IDLE mode and the UE has received an "interworking without N26 interface supported" indication from the network; and …. Unquote The current TTCN will fail a Rel-17 (or above) device that does not include the IE EPS NAS Message Container. This needs to be addressed. |
| Summary of change | Updated the TTCN implementation of the function to accommodate the above behaviour. |
| TTCN module | NR5GC_IRAT_CommonFunctions.ttcn |

Before Change:

function f_NR5GC_IRAT_MobilityRegistration (NR_CellId_Type p_NR_CellId, NG_NAS_SecurityParams_Type p_SecurityParams, // @sic R5s201060 sic@ template (omit) NAS_KeySetIdentifier p_NonCurrentKSI := omit, template (omit) NG_MobileIdentity p_GUTIFromPreamble, template NG_TrackingAreaId p_TAI := *, template (omit) NG_MobileIdentity p_GUTIToSend := omit, template (omit) NG_TrackingAreaIdList p_TAIList := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, boolean p_Registered_Previously_5GCN := true, template (value) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, template (omit) UERadioCapId p_UERadioCapIdToSend := omit, template PDU_SessionStatus p_PDU_SessionStatus := cr_PDU_SessionStatusAny, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { //@sic R5-198854: added p_Registered_Previously_5GCN sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NasCount_Type v_NasCount; var template (present) B1_Type v_Tsc := '0'B; //@sic R5-198854 sic@ var template (present) NAS_KsiValue v_KeySetId := p_SecurityParams.KSIamf; //@sic R5-198854, R5s200798 sic@ var TypeOfRegistration_Type v_RegType := Initial_Secure; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@ var AccessStratumRelease v_AccessStratumRelease;//@sic R5-237320 sic@ if ( not p_Registered_Previously_5GCN) { //@sic R5-198854 sic@ v_KeySetId := '111'B; v_Tsc := '?'B;//('1'B, '0'B); v_RegType := Initial_NoSecurity; } // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); // @sic R5s201060 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit), // @sic R5s201060 sic@ v_KeySetId, v_Tsc, p_NonCurrentKSI, cdr_NG_GMM_Cap_S1, -,-, p_TAI, cdr_NR5GC_UENetworkCap('17'O), -, p_PDU_SessionStatus, // @sic R5-231417 sic@ -, cr_UEStatus_S1, p_GUTIFromPreamble, -, -, -, ?, -, -, -, -, -, -, -, omit)) { // @sic R5-206290 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease();//@sic R5-237320 sic@ if ((p_IwkN26 == SUPPORTED) and ((v_AccessStratumRelease != rel15) and (v_AccessStratumRelease != rel16))){ //@sic R5-237320, R5s240678 sic@ // Send an empty message f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData('00'O, "TAUReq")); } else { // Send TAU Req for checking in EUTRA f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq")); } // @sic R5-198854 sic@ //Steps 4-9: 5GMM Authentication and 5GMM security v_NasCount :=f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), v_RegType); f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //Steps 8-9 RRC security activation f_NR_RRC_ActivateSecurity (p_NR_CellId, v_NasCount); f_NR5GC_MobilityRegistration_Steps4_6(p_NR_CellId, v_GMM_MobilityInfo, p_GUTIToSend, p_TAIList, p_ReleaseConnection, p_NwkFeatureSupport, -, -, -, p_UERadioCapIdToSend); // @sic R5-222047 sic@ }

After Change:

function f_NR5GC_IRAT_MobilityRegistration (NR_CellId_Type p_NR_CellId, NG_NAS_SecurityParams_Type p_SecurityParams, // @sic R5s201060 sic@ template (omit) NAS_KeySetIdentifier p_NonCurrentKSI := omit, template (omit) NG_MobileIdentity p_GUTIFromPreamble, template NG_TrackingAreaId p_TAI := *, template (omit) NG_MobileIdentity p_GUTIToSend := omit, template (omit) NG_TrackingAreaIdList p_TAIList := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, boolean p_Registered_Previously_5GCN := true, template (value) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, template (omit) UERadioCapId p_UERadioCapIdToSend := omit, template PDU_SessionStatus p_PDU_SessionStatus := cr_PDU_SessionStatusAny, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { //@sic R5-198854: added p_Registered_Previously_5GCN sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NasCount_Type v_NasCount; var template (present) B1_Type v_Tsc := '0'B; //@sic R5-198854 sic@ var template (present) NAS_KsiValue v_KeySetId := p_SecurityParams.KSIamf; //@sic R5-198854, R5s200798 sic@ var TypeOfRegistration_Type v_RegType := Initial_Secure; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@ var AccessStratumRelease v_AccessStratumRelease;//@sic R5-237320 sic@ var template EPS_MessageContainer v_EPS_MessageContainer := ?; //WA#WI1395819 if ( not p_Registered_Previously_5GCN) { //@sic R5-198854 sic@ v_KeySetId := '111'B; v_Tsc := '?'B;//('1'B, '0'B); v_RegType := Initial_NoSecurity; } v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease(); //WA#WI1395819 MOVED here //For a device that is Rel-17 and above its not mandatory to include this IE //This is inline with 25.401 cl 5.5.1.3.2 if ((p_IwkN26 == SUPPORTED) and (v_AccessStratumRelease != rel15) and (v_AccessStratumRelease != rel16)) { //WA#WI1395819 v_EPS_MessageContainer := *; } // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); // @sic R5s201060 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit), // @sic R5s201060 sic@ v_KeySetId, v_Tsc, p_NonCurrentKSI, cdr_NG_GMM_Cap_S1, -,-, p_TAI, cdr_NR5GC_UENetworkCap('17'O), -, p_PDU_SessionStatus, // @sic R5-231417 sic@ -, cr_UEStatus_S1, p_GUTIFromPreamble, -, -, -, v_EPS_MessageContainer, //WA#WI1395819 -, -, -, -, -, -, -, omit)) { // @sic R5-206290 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } if ((p_IwkN26 == SUPPORTED) and ((v_AccessStratumRelease != rel15) and (v_AccessStratumRelease != rel16))){ //@sic R5-237320, R5s240678 sic@ // Send an empty message f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData('00'O, "TAUReq")); } else { // Send TAU Req for checking in EUTRA f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq")); } // @sic R5-198854 sic@ //Steps 4-9: 5GMM Authentication and 5GMM security v_NasCount :=f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), v_RegType); f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //Steps 8-9 RRC security activation f_NR_RRC_ActivateSecurity (p_NR_CellId, v_NasCount); f_NR5GC_MobilityRegistration_Steps4_6(p_NR_CellId, v_GMM_MobilityInfo, p_GUTIToSend, p_TAIList, p_ReleaseConnection, p_NwkFeatureSupport, -, -, -, p_UERadioCapIdToSend); // @sic R5-222047 sic@ }