3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250092

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3639 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_PDUSessionEstablishment_InnerLoop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of this function, for the case when v_GSM_MobilityInfo.PDUType is IMS_DNN or Emergency_PDN, the variable v_MediaType is assigned IPCAN_EmergencyCall. This works ok if there is just IMS PDU Session, but as soon as the Emergency PDU Session is established, the EPS Bearer ID that is used in IMS PDU Session is reused in Emergency PDU Session. This results in the UEs trying to transmit a PDU Session Modification REQ for the IMS PDU Session, which causes testcase failure. Also, in the current TTCN implementation if v_PDUType is set to Emergency_PDN then following returns an error : v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the TTCN function to address the above behaviour. Basically the variable v_MediaType is assigned IPCAN_EmergencyCall only when v_PDUType == Emergency_PDN. Also updated the TTCN to address the error caused when v_PDUType is set to Emergency_PDN, and following is called : v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All testcases that have emergency call. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_NR5GC_PDUSessionEstablishment_InnerLoop |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, for the case when v_GSM_MobilityInfo.PDUType is IMS_DNN or Emergency_PDN, the variable v_MediaType is assigned IPCAN_EmergencyCall. This works ok if there is just IMS PDU Session, but as soon as the Emergency PDU Session is established, the EPS Bearer ID that is used in IMS PDU Session is reused in Emergency PDU Session. This results in the UEs trying to transmit a PDU Session Modification REQ for the IMS PDU Session, which causes testcase failure. Also, in the current TTCN implementation if v_PDUType is set to Emergency_PDN then following returns an error : v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); This needs to be addressed. |
| Summary of change | Update the TTCN function to address the above behaviour. Basically the variable v_MediaType is assigned IPCAN_EmergencyCall only when v_PDUType == Emergency_PDN. Also updated the TTCN to address the error caused when v_PDUType is set to Emergency_PDN, and following is called : v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |

Before Change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, // set to required value if a specific QoSFlow must be built (with interworkingWithEPS parameters) template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // Only included when called from S1ToN1 TAU boolean p_OmitDNN := false // @sic R5-223987 sic@ - Only set to true if the DNN is to be omitted from Accept message ) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@ var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@ var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SSConfig; var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var SdapConfig_Type v_SdapConfig; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) SDAP_Config v_SDAP_Config; var PDN_Index_Type v_PDN_Index; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@ var template (omit) DNN v_DNN; var PDU_PDN_DNN_Type v_PDUType := None; // @sic R5s210148 sic@ var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@ var boolean v_ExistingPDN := false; // @sic R5s210149, R5s221126 sic@ var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit; var template (omit) EthernetHeaderCompressConfig v_EthernetHeaderCompressConfig := omit; var integer i; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, p_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId, p_PTI, p_IntegrityProtMaxDataRate, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_ExpectedS_NSSAI, p_ExpectedDNN, p_AdditionalInfo)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } if (not isvalue(p_DNN) and not p_OmitDNN ) { // @sic R5s210148, R5s221229 sic@ // Use the details received in the Request msg v_PDUType := v_GSM_MobilityInfo.PDUType; // @sic R5-205939, R5s240708 sic@ if (v_PDUType == None) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN"); } else { v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); // @sic R5s240641 sic@ } } else { // need to overwrite DNN received in Request msg @sic R5s221229 sic@ if (isvalue(p_DNN)) { v_DNN := p_DNN; v_GSM_MobilityInfo.DNN := valueof(p_DNN); } if (p_OmitDNN) { // @sic R5-223987 sic@ v_DNN := omit; v_GSM_MobilityInfo.DNN := omit; } // If using a different DNN, need to update stored PDU Type too v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN); if (v_PDUType == None) { // if this DNN isn't known, just use default v_PDUType := Internet_DNN; } v_GSM_MobilityInfo.PDUType := v_PDUType; } if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@ // Ethernet PDU session if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address"); } if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) { v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header"); } } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case(IMS_DNN, Emergency_PDN) { // @sic R5-206296, R5s210518 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486, R5s240641 sic@ } case (URLLC_DNN) { v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@ } case else { v_SDAP_Config := cs_SDAP_Config; } } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); //It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]); v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; // @sic R5s210480 sic@ // @sic R5-204400 sic@ if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_MediaType)); } } if (isvalue(p_PDU_SessionType)) { // @sic R5-227473 sic@ if (match ('101'B, p_PDU_SessionType.typeValue)) { v_ExistingPDN := true; // just set the flag to avoid updating the routing table } } // If this is a default DRB, enable IP handling for this DRB if (valueof(v_SDAP_Config.defaultDRB)) { if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@ // The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) { if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) { // Data from EUTRA should be used, rather than created anew v_ExistingPDN := f_NR_ChangeDrbMappingForExistingPDNs(p_NR_CellId, valueof(p_PduInfoFromLTE[i]), v_QosFlowId.QFI); // @sic R5s221126 sic@ v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduInfoFromLTE[i].EPS_Bearer); // Overwrite with stored value break; } } } if (not v_ExistingPDN) { // completely new PDU session @sic R5s210149, R5s221126 sic@ f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@ /* @sic R5-217449, R5s220617 sic@ */ } } if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@ //Perform PDU Session Authentication if required v_SecurityParams := f_NR5GC_Security_Get(); if (p_SRB == SRB1_2) { // @sic R5s201412 sic@ v_SRB := tsc_NR_RbId_SRB2; } f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@ } if (f_NR5GC_MobileInfo_GetNumOfIPAddressesToAllocate() != 1) { // @sic R5-217830 sic@ // According to NOTE 4 in PIXIT definition, this is currently only used for the 2nd PCSCF address of PDN_1 v_ExtraPDN_Address := f_PDN_AddressInfo_Get(PDN_1); v_ExtraPDN_Address.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; v_ExtraPDN_Address.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; } // Send Accept // Created common function for sending Accept message @sic R5-241606 sic@ f_SendPDUSessionAcceptOverReconfig (p_NR_CellId, v_GSM_MobilityInfo, v_DNN, // @sic R5-205939 sic@ v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, p_SSC_Mode, p_Session_AMBR, p_PDU_SessionType, p_Cause, p_RQTimer, p_S_NSSAI, -, p_EAP, p_QoS_Rules, p_QoSFlowDescr, omit, p_BackOff, p_BuildQosFlow, -, -, -, -, -, -, v_EthernetHeaderCompressConfig, // @sic R5-227473 sic@ v_ExtraPDN_Address); //@sic R5-213400 sic@ return v_PDUType; // @sic R5-206296 sic@ }

After Change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, // set to required value if a specific QoSFlow must be built (with interworkingWithEPS parameters) template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // Only included when called from S1ToN1 TAU boolean p_OmitDNN := false // @sic R5-223987 sic@ - Only set to true if the DNN is to be omitted from Accept message ) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@ var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@ var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SSConfig; var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var SdapConfig_Type v_SdapConfig; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) SDAP_Config v_SDAP_Config; var PDN_Index_Type v_PDN_Index; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@ var template (omit) DNN v_DNN; var PDU_PDN_DNN_Type v_PDUType := None; // @sic R5s210148 sic@ var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@ var boolean v_ExistingPDN := false; // @sic R5s210149, R5s221126 sic@ var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit; var template (omit) EthernetHeaderCompressConfig v_EthernetHeaderCompressConfig := omit; var integer i; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, p_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId, p_PTI, p_IntegrityProtMaxDataRate, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_ExpectedS_NSSAI, p_ExpectedDNN, p_AdditionalInfo)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } if (not isvalue(p_DNN) and not p_OmitDNN ) { // @sic R5s210148, R5s221229 sic@ // Use the details received in the Request msg v_PDUType := v_GSM_MobilityInfo.PDUType; // @sic R5-205939, R5s240708 sic@ if (v_PDUType == None) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN"); } else { if (v_PDUType == Emergency_PDN) { v_MediaType := IPCAN_EmergencyCall; //WA#WI1396015 v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(IMS_DNN))); } else { v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); // @sic R5s240641 sic@ } } } else { // need to overwrite DNN received in Request msg @sic R5s221229 sic@ if (isvalue(p_DNN)) { v_DNN := p_DNN; v_GSM_MobilityInfo.DNN := valueof(p_DNN); } if (p_OmitDNN) { // @sic R5-223987 sic@ v_DNN := omit; v_GSM_MobilityInfo.DNN := omit; } // If using a different DNN, need to update stored PDU Type too if (v_GSM_MobilityInfo.PDUType == Emergency_PDN) { //WA#WI1396015 v_MediaType := IPCAN_EmergencyCall; } else { v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN); if (v_PDUType == None) { // if this DNN isn't known, just use default v_PDUType := Internet_DNN; } v_GSM_MobilityInfo.PDUType := v_PDUType; } } if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@ // Ethernet PDU session if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address"); } if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) { v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header"); } } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case(IMS_DNN, Emergency_PDN) { // @sic R5-206296, R5s210518 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); //WA#WI1396015 MOVED UP and assigned based on v_PDUType v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486, R5s240641 sic@ } case (URLLC_DNN) { v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@ } case else { v_SDAP_Config := cs_SDAP_Config; } } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); //It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]); v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; // @sic R5s210480 sic@ // @sic R5-204400 sic@ if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_MediaType)); } } if (isvalue(p_PDU_SessionType)) { // @sic R5-227473 sic@ if (match ('101'B, p_PDU_SessionType.typeValue)) { v_ExistingPDN := true; // just set the flag to avoid updating the routing table } } // If this is a default DRB, enable IP handling for this DRB if (valueof(v_SDAP_Config.defaultDRB)) { if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@ // The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) { if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) { // Data from EUTRA should be used, rather than created anew v_ExistingPDN := f_NR_ChangeDrbMappingForExistingPDNs(p_NR_CellId, valueof(p_PduInfoFromLTE[i]), v_QosFlowId.QFI); // @sic R5s221126 sic@ v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduInfoFromLTE[i].EPS_Bearer); // Overwrite with stored value break; } } } if (not v_ExistingPDN) { // completely new PDU session @sic R5s210149, R5s221126 sic@ f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@ /* @sic R5-217449, R5s220617 sic@ */ } } if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@ //Perform PDU Session Authentication if required v_SecurityParams := f_NR5GC_Security_Get(); if (p_SRB == SRB1_2) { // @sic R5s201412 sic@ v_SRB := tsc_NR_RbId_SRB2; } f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@ } if (f_NR5GC_MobileInfo_GetNumOfIPAddressesToAllocate() != 1) { // @sic R5-217830 sic@ // According to NOTE 4 in PIXIT definition, this is currently only used for the 2nd PCSCF address of PDN_1 v_ExtraPDN_Address := f_PDN_AddressInfo_Get(PDN_1); v_ExtraPDN_Address.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; v_ExtraPDN_Address.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; } // Send Accept // Created common function for sending Accept message @sic R5-241606 sic@ f_SendPDUSessionAcceptOverReconfig (p_NR_CellId, v_GSM_MobilityInfo, v_DNN, // @sic R5-205939 sic@ v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, p_SSC_Mode, p_Session_AMBR, p_PDU_SessionType, p_Cause, p_RQTimer, p_S_NSSAI, -, p_EAP, p_QoS_Rules, p_QoSFlowDescr, omit, p_BackOff, p_BuildQosFlow, -, -, -, -, -, -, v_EthernetHeaderCompressConfig, // @sic R5-227473 sic@ v_ExtraPDN_Address); //@sic R5-213400 sic@ return v_PDUType; // @sic R5-206296 sic@ }