3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250133

# Online,  , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-5 | CR | 0008 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 6.1.1.16 to the MCX ATS with MCX-EUTRA model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To re-verify MCPTT TC 6.1.1.16 to the MCX ATS on MCX-EUTRA test model |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX-EUTRA test model. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase will not be added to MCX ATS with MCX-EUTRA model |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.16.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 2	4

3	Branches executed	7

4	Execution Log Files	7

4.1	Alea - a Leonardo Company, MCXPTT Platform v. 4.2.0 - Android Client v 1.12.0	7

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.1.16.MCPTT which is part of the MCX_EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	6.1.1.16.MCPTT

ATS Version:	iwd-TTCN3-B2024-06_D24wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Alea - a Leonardo Company, MCXPTT Platform v. 4.2.0 - Android Client v 1.12.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MT() |
| --- | --- |
| Reason for change | This function is defined for MT but the functionality is for MO (with Paging) |
| Summary of change | Remove paging |
| TTCN module | MCX_SIP_Registration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MT(EUTRA_CellId_Type p_CellId, template (present) EstablishmentCause p_EstablishmentCause := cr_EstablishmentCause_mt_Access, DRB_IdentityList_Type p_DedicatedDrbList := {}) runs on EUTRA_PTC { var DRB_IdentityList_Type v_DefaultDrbList := f_MCX_EUTRA_IPCAN_DefaultDrbList(); var GutiParameters_Type v_GutiParameters := f_EUTRA_CellInfo_GetGuti(p_CellId); f_EUTRA_UE_Page(p_CellId, cs_NasPaging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI))); f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment(p_CellId, p_EstablishmentCause, v_DefaultDrbList, p_DedicatedDrbList); }

After Change:

function f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MT(EUTRA_CellId_Type p_CellId, template (present) EstablishmentCause p_EstablishmentCause := cr_EstablishmentCause_mt_Access, DRB_IdentityList_Type p_DedicatedDrbList := {}) runs on EUTRA_PTC { var DRB_IdentityList_Type v_DefaultDrbList := f_MCX_EUTRA_IPCAN_DefaultDrbList(); // REMOVED PAGING f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment(p_CellId, p_EstablishmentCause, v_DefaultDrbList, p_DedicatedDrbList); }

## Change 2

| Function name | f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MO() |
| --- | --- |
| Reason for change | This function is defined for MO but the functionality is for MT (without paging). |
| Summary of change | Adding paging |
| TTCN module | IPCAN_MCX_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MO(EUTRA_CellId_Type p_CellId, template (present) EstablishmentCause p_EstablishmentCause := cr_EstablishmentCause_MoData, DRB_IdentityList_Type p_DedicatedDrbList := {}) runs on EUTRA_PTC { var DRB_IdentityList_Type v_DefaultDrbList := f_MCX_EUTRA_IPCAN_DefaultDrbList(); f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment(p_CellId, p_EstablishmentCause, v_DefaultDrbList, p_DedicatedDrbList); }

After Change:

function f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MO(EUTRA_CellId_Type p_CellId, template (present) EstablishmentCause p_EstablishmentCause := cr_EstablishmentCause_MoData, DRB_IdentityList_Type p_DedicatedDrbList := {}) runs on EUTRA_PTC { var DRB_IdentityList_Type v_DefaultDrbList := f_MCX_EUTRA_IPCAN_DefaultDrbList(); var GutiParameters_Type v_GutiParameters := f_EUTRA_CellInfo_GetGuti(p_CellId); f_EUTRA_UE_Page(p_CellId, cs_NasPaging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI))); f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment(p_CellId, p_EstablishmentCause, v_DefaultDrbList, p_DedicatedDrbList); }

## Change 3

| Function name | f_TC_6_1_1_16_MCPTT_MCX_IMS() |
| --- | --- |
| Reason for change | In this function, the second SIP-MESSAGE (step 4) is sent for Emergency cancellation. However the Message-Header is not constructed again. Hence the the call-id & seq.no is sent same as previous message header (ie for step 1). |
| Summary of change | The SIP-Message header is constructed again at step 4 so that the Message will be unique always. |
| TTCN module | MCPTT_GroupCall_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_1_1_16_MCPTT_MCX_IMS() runs on MCX_IMS_PTC { // On-network / Emergency Alert / Client Terminated (CT) var template (value) MessageHeader v_MessageHeader; var template (value) MCPTT_Info_Type v_McpttInfo; var template (value) MCPTT_LocationInfo_Type v_LocationInfo; var integer v_Longitude := f_MCX_LocationInfo_Degrees2Longitude(px_MCX_CoordinateLongitude_Client_B); var integer v_Latitude := f_MCX_LocationInfo_Degrees2Latitude(px_MCX_CoordinateLatitude_Client_B); var boolean v_ACCEPT_CONTACT_WITH_MEDIA_FEATURE_TAG := true; f_MCX_Preamble_IMS(MCPTT, IPCAN_SignallingOnly); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_MCX_IPCAN_StartSignalling(IPCAN_MT_IMS_Signalling); v_McpttInfo := f_MCX_CallControl_MCPTTInfo_TX(); /* Table 6.1.1.16.3.3-1B: MCPTT-Info in SIP MESSAGE @sic R5-242353: no GROUP_CALL => no session-type sic@ */ v_McpttInfo.mcptt_Params.mcptt_calling_group_id := f_MCPTT_Content_URI(px_MCPTT_Group_A_ID); /* Table 6.1.1.16.3.3-1B: @sic R5-242353 sic@ */ v_McpttInfo.mcptt_Params.alert_ind := cs_MCPTT_Content_True; /* Table 6.1.1.16.3.3-1B: MCPTT-Info */ v_McpttInfo.mcptt_Params.mc_org := px_MCX_DomainName_Organization_A; /* Table 6.1.1.16.3.3-1B: @sic R5-242353 sic@ */ v_LocationInfo := cs_MCPTT_LocationInfo_Report(v_Longitude, v_Latitude); /* Table 6.1.1.16.3.3-1A: Location-Info in SIP MESSAGE */ v_MessageHeader := f_MCX_MessageRequest_MessageHeaderTX(MCPTT, v_ACCEPT_CONTACT_WITH_MEDIA_FEATURE_TAG); /* @sic R5-242353: condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG sic@ */ f_MCPTT_SendMESSAGEwithMIME_Receive200OK(v_MessageHeader, v_McpttInfo, -, -, v_LocationInfo); f_MCX_IPCAN_ReleaseConnection(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 1"); // @siclog "Step 2" Void siclog@ if (pc_MCX_DisplayInfoEmergencyAlert) { // @sic R5-231942: pc_MCX_DisplayInfoEmergencyAlert sic@ // @siclog "Step 3a1" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (MCPTT client) notify the user of the emergency alert?", "Step 3a1"); } // @siclog "Step 4" siclog@ f_MCX_IPCAN_StartSignalling(IPCAN_MT_IMS_Signalling); v_McpttInfo.mcptt_Params.alert_ind := cs_MCPTT_Content_False; /* Table 6.1.1.16.3.3-3A: MCPTT-Info in SIP MESSAGE */ v_McpttInfo.mcptt_Params.mc_org := omit; /* Table 6.1.1.16.3.3-3A @sic R5-242353 sic@ */ f_MCPTT_SendMESSAGEwithMIME_Receive200OK(v_MessageHeader, v_McpttInfo); f_MCX_IPCAN_ReleaseConnection(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" Void siclog@ if (pc_MCX_DisplayInfoEmergencyAlert) { // @sic R5-231942: pc_MCX_DisplayInfoEmergencyAlert sic@ // @siclog "Step 6a1" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (MCPTT client) notify the user of the emergency alert cancellation?", "Step 6a1"); } f_IMS_TestBody_Set(false); f_MCX_Postamble_IMS(); }

After Change:

function f_TC_6_1_1_16_MCPTT_MCX_IMS() runs on MCX_IMS_PTC { // On-network / Emergency Alert / Client Terminated (CT) var template (value) MessageHeader v_MessageHeader; var template (value) MCPTT_Info_Type v_McpttInfo; var template (value) MCPTT_LocationInfo_Type v_LocationInfo; var integer v_Longitude := f_MCX_LocationInfo_Degrees2Longitude(px_MCX_CoordinateLongitude_Client_B); var integer v_Latitude := f_MCX_LocationInfo_Degrees2Latitude(px_MCX_CoordinateLatitude_Client_B); var boolean v_ACCEPT_CONTACT_WITH_MEDIA_FEATURE_TAG := true; f_MCX_Preamble_IMS(MCPTT, IPCAN_SignallingOnly); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_MCX_IPCAN_StartSignalling(IPCAN_MT_IMS_Signalling); v_McpttInfo := f_MCX_CallControl_MCPTTInfo_TX(); /* Table 6.1.1.16.3.3-1B: MCPTT-Info in SIP MESSAGE @sic R5-242353: no GROUP_CALL => no session-type sic@ */ v_McpttInfo.mcptt_Params.mcptt_calling_group_id := f_MCPTT_Content_URI(px_MCPTT_Group_A_ID); /* Table 6.1.1.16.3.3-1B: @sic R5-242353 sic@ */ v_McpttInfo.mcptt_Params.alert_ind := cs_MCPTT_Content_True; /* Table 6.1.1.16.3.3-1B: MCPTT-Info */ v_McpttInfo.mcptt_Params.mc_org := px_MCX_DomainName_Organization_A; /* Table 6.1.1.16.3.3-1B: @sic R5-242353 sic@ */ v_LocationInfo := cs_MCPTT_LocationInfo_Report(v_Longitude, v_Latitude); /* Table 6.1.1.16.3.3-1A: Location-Info in SIP MESSAGE */ v_MessageHeader := f_MCX_MessageRequest_MessageHeaderTX(MCPTT, v_ACCEPT_CONTACT_WITH_MEDIA_FEATURE_TAG); /* @sic R5-242353: condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG sic@ */ f_MCPTT_SendMESSAGEwithMIME_Receive200OK(v_MessageHeader, v_McpttInfo, -, -, v_LocationInfo); f_MCX_IPCAN_ReleaseConnection(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 1"); // @siclog "Step 2" Void siclog@ if (pc_MCX_DisplayInfoEmergencyAlert) { // @sic R5-231942: pc_MCX_DisplayInfoEmergencyAlert sic@ // @siclog "Step 3a1" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (MCPTT client) notify the user of the emergency alert?", "Step 3a1"); } // @siclog "Step 4" siclog@ f_MCX_IPCAN_StartSignalling(IPCAN_MT_IMS_Signalling); v_McpttInfo.mcptt_Params.alert_ind := cs_MCPTT_Content_False; /* Table 6.1.1.16.3.3-3A: MCPTT-Info in SIP MESSAGE */ v_McpttInfo.mcptt_Params.mc_org := omit; /* Table 6.1.1.16.3.3-3A @sic R5-242353 sic@ */ v_MessageHeader := f_MCX_MessageRequest_MessageHeaderTX(MCPTT, v_ACCEPT_CONTACT_WITH_MEDIA_FEATURE_TAG); /* @sic R5-242353: condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG sic@ */ f_MCPTT_SendMESSAGEwithMIME_Receive200OK(v_MessageHeader, v_McpttInfo); f_MCX_IPCAN_ReleaseConnection(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" Void siclog@ if (pc_MCX_DisplayInfoEmergencyAlert) { // @sic R5-231942: pc_MCX_DisplayInfoEmergencyAlert sic@ // @siclog "Step 6a1" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (MCPTT client) notify the user of the emergency alert cancellation?", "Step 6a1"); } f_IMS_TestBody_Set(false); f_MCX_Postamble_IMS(); }

# Branches executed

The test case was executed with

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE FDD Band 5

- Ciphering algorithm eea1 and Integrity algorithm eia1

# Execution Log Files

## Alea - a Leonardo Company, MCXPTT Platform v. 4.2.0 - Android Client v 1.12.0

The Alea - a Leonardo Company, MCXPTT Platform v. 4.2.0 - Android Client v 1.12.0 passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_1_16_MCPTT_LOG.txt

- PICS/PIXIT parameter file:

TC_6_1_1_16_MCPTT_PIXIT.xml

# References

| [1] | R5s250061 : Supporting information for Addition of MCPTT test case 6.1.1.16 to the MCX ATS with MCX-EUTRA model |
| --- | --- |