3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250138

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3658 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cr_UE_NR_Capability_v1640_IEs_RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 25wk11 Implementation , In line with https://www.3gpp.org/ftp/Meetings_3GPP_SYNC/RAN5/Docs/R5-250329.zip Check is introduced for below IEs as part of appLayerMeasParameters-r17 appLayerMeasParameters-r17 SEQUENCE { Not checked qoe-Streaming-MeasReport-r17 Not checked qoe-MTSI-MeasReport-r17 Checked based on pc_qoe_MTSI_MeasReport_r17 qoe-VR-MeasReport-r17 Not checked ran-VisibleQoE-Streaming-MeasReport-r17 Not checked ran-VisibleQoE-VR-MeasReport-r17 Not checked ul-MeasurementReportAppLayer-Seg-r17 Not checked } As per the core specification 38.331 , the value can be optional for the UE appLayerMeasParameters-r17 AppLayerMeasParameters-r17 OPTIONAL, Hence it is proposed to add ifpresent condition while checking appLayerMeasParameters-r17 SEQUENCE |  |  |  |  |  |  |  |  |
| Summary of change: |  | it is proposed to add ifpresent condition while checking appLayerMeasParameters-r17 SEQUENCE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | cr_UE_NR_Capability_v1640_IEs_RedCap |
| --- | --- |
| Reason for change | In 25wk11 Implementation , In line with https://www.3gpp.org/ftp/Meetings_3GPP_SYNC/RAN5/Docs/R5-250329.zip Check is introduced for below IEs as part of appLayerMeasParameters-r17 appLayerMeasParameters-r17 SEQUENCE { Not checked qoe-Streaming-MeasReport-r17 Not checked qoe-MTSI-MeasReport-r17 Checked based on pc_qoe_MTSI_MeasReport_r17 qoe-VR-MeasReport-r17 Not checked ran-VisibleQoE-Streaming-MeasReport-r17 Not checked ran-VisibleQoE-VR-MeasReport-r17 Not checked ul-MeasurementReportAppLayer-Seg-r17 Not checked } As per the core specification 38.331 , the value can be optional for the UE appLayerMeasParameters-r17 AppLayerMeasParameters-r17 OPTIONAL, Hence it is proposed to add ifpresent condition while checking appLayerMeasParameters-r17 SEQUENCE |
| Summary of change | it is proposed to add ifpresent condition while checking appLayerMeasParameters-r17 SEQUENCE |
| TTCN module | ttcn\develop\Common\NR\NR_CapabilityFunctions.ttcn |

Before Change:

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs_RedCap (template UE_NR_Capability_v1740 p_UE_NR_Capability_v1740 := *) := // @sic R5-231410 R5s211246 R5-225384 R5s221155 R5s221179 R5-227571 R5-227572 R5-227573 R5s230456 R5-233835 R5-240518 R5s240531 sic@ { /* @status APPROVED (ENDC, NR5GC) */ redirectAtResumeByNAS_r16 := *, phy_ParametersSharedSpectrumChAccess_r16 := cr_V1640_phy_ParametersSharedSpectrumChAccess_r16 ifpresent, nonCriticalExtension := { // UE-NR-Capability-v1650 mpsPriorityIndication_r16 := f_Derive_NR_mpsPriorityIndication_r16() ifpresent, highSpeedParameters_v1650 := *, nonCriticalExtension := { // UE-NR-Capability-v1690 ul_RRC_Segmentation_r16 := f_Derive_ul_RRC_Segmentation_r16(), nonCriticalExtension := { // UE-NR-Capability-v1700 inactiveStatePO_Determination_r17 := *, highSpeedParameters_v1700 := *, powSav_Parameters_v1700 := *, mac_Parameters_v1700 := { mac_ParametersFR2_2_r17 := { // MAC-ParametersFR2-2-r17 directMCG_SCellActivation_r17 := *, // @sic R5-232320 sic@ directMCG_SCellActivationResume_r17 := *, // @sic R5-232320 sic@ directSCG_SCellActivation_r17 := *, directSCG_SCellActivationResume_r17 := *, drx_Adaptation_r17 := * } ifpresent } ifpresent, ims_Parameters_v1700 := *, measAndMobParameters_v1700 := ?, //MeasAndMobParameters_v1700 non optional appLayerMeasParameters_r17 := { qoe_Streaming_MeasReport_r17 := *, qoe_MTSI_MeasReport_r17 := f_Derive_qoe_MTSI_MeasReport_r17(), // pc_qoe_MTSI_MeasReport_r17 - @sic R5-250329 sic@ qoe_VR_MeasReport_r17 := *, ran_VisibleQoE_Streaming_MeasReport_r17 := *, ran_VisibleQoE_VR_MeasReport_r17 := *, ul_MeasurementReportAppLayer_Seg_r17 := *, qoe_IdleInactiveMeasReport_r18 := *, qoe_NRDC_MeasReport_r18 := *, qoe_AdditionalMemoryMeasReport_r18 := *, qoe_PriorityBasedDiscarding_r18 := *, srb5_r18 := * }, redCapParameters_r17 := { supportOfRedCap_r17 := f_Derive_supportOfRedCap_r17() ifpresent, // pc_supportOfRedCap_r17 - only present in NR5GC and RedCap supportOf16DRB_RedCap_r17 := f_Derive_supportOf16DRB_RedCap_r17() ifpresent // pc_16DRB_RedCap_r17 - only present in NR5GC and RedCap } ifpresent, ra_SDT_r17 := f_Derive_ra_SDT_r17() ifpresent, // pc_ra_SDT_r17 - only present in NR5GC srb_SDT_r17 := f_Derive_srb_SDT_r17() ifpresent, // pc_srb_SDT_r17 - only present in NR5GC gNB_SideRTT_BasedPDC_r17 := *, bh_RLF_DetectionRecovery_Indication_r17 := *, nrdc_Parameters_v1700 := *, bap_Parameters_v1700 := *, musim_GapPreference_r17 := f_Derive_musim_GapPreference_r17() ifpresent, // @sic R5-236307 sic@ - pc_musim_GapPreference_r17 musimLeaveConnected_r17 := f_Derive_musimLeaveConnected_r17() ifpresent, // @sic R5-236307 sic@ - pc_musimLeaveConnected_r17 mbs_Parameters_r17 := ?, //MBS_Parameters_r17 non optional nonTerrestrialNetwork_r17 := f_Derive_nonTerrestrialNetwork_r17() ifpresent, // @sic R5-245527 sic@ - pc_nonTerrestrialNetwork_r17 ntn_ScenarioSupport_r17 := *, sliceInfoforCellReselection_r17 := f_Derive_sliceInfoforCellReselection_r17() ifpresent, // @sic R5-232202 sic@ ue_RadioPagingInfo_r17 := { pei_SubgroupingSupportBandList_r17 := * // @sic R5-233835 sic@ } ifpresent, ul_GapFR2_Pattern_r17 := *, ntn_Parameters_r17 := *, nonCriticalExtension := p_UE_NR_Capability_v1740 // UE_NR_Capability_v1740 } ifpresent // UE_NR_Capability_v1700 IEs } ifpresent // UE_NR_Capability_v1690 IEs } ifpresent // UE_NR_Capability_v1650 IEs } ifpresent; // UE_NR_Capability_v1640 IEs

After Change:

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs_RedCap (template UE_NR_Capability_v1740 p_UE_NR_Capability_v1740 := *) := // @sic R5-231410 R5s211246 R5-225384 R5s221155 R5s221179 R5-227571 R5-227572 R5-227573 R5s230456 R5-233835 R5-240518 R5s240531 sic@ { /* @status APPROVED (ENDC, NR5GC) */ redirectAtResumeByNAS_r16 := *, phy_ParametersSharedSpectrumChAccess_r16 := cr_V1640_phy_ParametersSharedSpectrumChAccess_r16 ifpresent, nonCriticalExtension := { // UE-NR-Capability-v1650 mpsPriorityIndication_r16 := f_Derive_NR_mpsPriorityIndication_r16() ifpresent, highSpeedParameters_v1650 := *, nonCriticalExtension := { // UE-NR-Capability-v1690 ul_RRC_Segmentation_r16 := f_Derive_ul_RRC_Segmentation_r16(), nonCriticalExtension := { // UE-NR-Capability-v1700 inactiveStatePO_Determination_r17 := *, highSpeedParameters_v1700 := *, powSav_Parameters_v1700 := *, mac_Parameters_v1700 := { mac_ParametersFR2_2_r17 := { // MAC-ParametersFR2-2-r17 directMCG_SCellActivation_r17 := *, // @sic R5-232320 sic@ directMCG_SCellActivationResume_r17 := *, // @sic R5-232320 sic@ directSCG_SCellActivation_r17 := *, directSCG_SCellActivationResume_r17 := *, drx_Adaptation_r17 := * } ifpresent } ifpresent, ims_Parameters_v1700 := *, measAndMobParameters_v1700 := ?, //MeasAndMobParameters_v1700 non optional appLayerMeasParameters_r17 := { qoe_Streaming_MeasReport_r17 := *, qoe_MTSI_MeasReport_r17 := f_Derive_qoe_MTSI_MeasReport_r17(), // pc_qoe_MTSI_MeasReport_r17 - @sic R5-250329 sic@ qoe_VR_MeasReport_r17 := *, ran_VisibleQoE_Streaming_MeasReport_r17 := *, ran_VisibleQoE_VR_MeasReport_r17 := *, ul_MeasurementReportAppLayer_Seg_r17 := *, qoe_IdleInactiveMeasReport_r18 := *, qoe_NRDC_MeasReport_r18 := *, qoe_AdditionalMemoryMeasReport_r18 := *, qoe_PriorityBasedDiscarding_r18 := *, srb5_r18 := * }ifpresent, //WA#WI=1398373 redCapParameters_r17 := { supportOfRedCap_r17 := f_Derive_supportOfRedCap_r17() ifpresent, // pc_supportOfRedCap_r17 - only present in NR5GC and RedCap supportOf16DRB_RedCap_r17 := f_Derive_supportOf16DRB_RedCap_r17() ifpresent // pc_16DRB_RedCap_r17 - only present in NR5GC and RedCap } ifpresent, ra_SDT_r17 := f_Derive_ra_SDT_r17() ifpresent, // pc_ra_SDT_r17 - only present in NR5GC srb_SDT_r17 := f_Derive_srb_SDT_r17() ifpresent, // pc_srb_SDT_r17 - only present in NR5GC gNB_SideRTT_BasedPDC_r17 := *, bh_RLF_DetectionRecovery_Indication_r17 := *, nrdc_Parameters_v1700 := *, bap_Parameters_v1700 := *, musim_GapPreference_r17 := f_Derive_musim_GapPreference_r17() ifpresent, // @sic R5-236307 sic@ - pc_musim_GapPreference_r17 musimLeaveConnected_r17 := f_Derive_musimLeaveConnected_r17() ifpresent, // @sic R5-236307 sic@ - pc_musimLeaveConnected_r17 mbs_Parameters_r17 := ?, //MBS_Parameters_r17 non optional nonTerrestrialNetwork_r17 := f_Derive_nonTerrestrialNetwork_r17() ifpresent, // @sic R5-245527 sic@ - pc_nonTerrestrialNetwork_r17 ntn_ScenarioSupport_r17 := *, sliceInfoforCellReselection_r17 := f_Derive_sliceInfoforCellReselection_r17() ifpresent, // @sic R5-232202 sic@ ue_RadioPagingInfo_r17 := { pei_SubgroupingSupportBandList_r17 := * // @sic R5-233835 sic@ } ifpresent, ul_GapFR2_Pattern_r17 := *, ntn_Parameters_r17 := *, nonCriticalExtension := p_UE_NR_Capability_v1740 // UE_NR_Capability_v1740 } ifpresent // UE_NR_Capability_v1700 IEs } ifpresent // UE_NR_Capability_v1690 IEs } ifpresent // UE_NR_Capability_v1650 IEs } ifpresent; // UE_NR_Capability_v1640 IEs