3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250139

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3626 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 8.1.5.2.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest, TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 8.1.5.2.3 to the IWD_24wk50 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.5.2.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to the testcase TC_8_1_5_2_3_NR5GC ()	3

2.2	Correction to the Function fl_TC_8_1_5_2_3_TestBody()	5

2.3	Correction to the Function f_NR_ModifySysinfo ()	21

3	Branches executed	27

4	Execution Log Files	27

4.1	MediaTek MT6815	27

4.2	Hisilicon Balong 5000	27

5	References	28

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.2.3 which is part of the NR5GC test suite in iwd- iwd-TTCN3-B2024-06_D24wk50

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.5.2.3.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk50

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UEs used:	MediaTek MT6815 ,  Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Correction to the testcase TC_8_1_5_2_3_NR5GC ()

| Testcase Function name | TC_8_1_5_2_3_NR5GC() |
| --- | --- |
| Reason for change | In the function “TC_8_1_5_2_3_NR5GC”, Guard Timer needs to be updated as 10800sec, as we are waiting for HSFN Wrap Around |
| Summary of change | In the function “TC_8_1_5_2_3_NR5GC”, Guard Timer is updated as 10800, as we are waiting for HSFN Wrap Around |
| TTCN module | develop\NR5GC\NR5GC_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_8_1_5_2_3_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // eDRX / IDLE / Paging for notification of BCCH modification var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_8_1_5_2_3_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_8_1_5_2_3_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // eDRX / IDLE / Paging for notification of BCCH modification var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(10800); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_8_1_5_2_3_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Correction to the Function fl_TC_8_1_5_2_3_TestBody()

| Function name | fl_TC_8_1_5_2_3_TestBody |
| --- | --- |
| Reason for change | In the function fl_TC_8_1_5_2_3_TestBody - v_NegotiatedExtdDRXParams value is not in line with the current prose Table 8.1.5.2.3.3.3-3: REGISTRATION ACCEPT (Step 14, Table 8.1.5.2.3.3.2-1) - at step 16-17 of the test case , to perform //Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] incorrect function call f_NR5GC_RRC_Idle_Extension (nr_Cell1); is used , this needs to be corrected - At step 19 , SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification-eDRX. In order to make sure that P-RNTI Transmission happens in line with eDRX cycle , p_EDRXValue_Index needs to be passed - At Step 21 , when sending the paging using the function f_NR5GC_UE_Page_eDRX, incorrect values are used for p_PTWLength_Index and p_EDRXValue_Index , this needs to be corrected - Before steps 24-29 , UL grant configuration needs to be provided as current TTCN is not making use of default function for handling setup request - At step 30 , when handling f_NR_UE_DeRegisterOnSwitchOff , currently SRB1 is provided . this needs to be corrected to use the default value of SRB2 - v_NegotiatedExtdDRXParams is not in line as per Table 8.1.5.2.3.3.3-4: REGISTRATION ACCEPT (Step 44, Table 8.1.5.2.3.3.2-1) at step 45 , and this needs correction - at step 46-47 of the test case , to perform //Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] incorrect function call f_NR5GC_RRC_Idle_Extension (nr_Cell1); is used , this needs to be corrected - at step 49, as per the prose “SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.” , current TTCN sends tsc_NR_ShortMessage_SystemInfoModificationAndEDRX, this needs to be corrected as systemInfoModification - at step 51 , when sending the paging using the function f_NR5GC_UE_Page_eDRX, incorrect values are used for p_PTWLength_Index and p_EDRXValue_Index , this needs to be corrected - At step 53 , UL grant configuration needs to be provided as current TTCN is not making use of default function for handling setup request |
| Summary of change | In the function fl_TC_8_1_5_2_3_TestBody - v_NegotiatedExtdDRXParams value is updated to be in line with the current prose Table 8.1.5.2.3.3.3-3: REGISTRATION ACCEPT (Step 14, Table 8.1.5.2.3.3.2-1) - at step 16-17 of the test case , to perform //Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] incorrect function call f_NR5GC_RRC_Idle_Extension (nr_Cell1); is used , this needs to be replaced with the function call f_NR5GC_RRC_Idle_Steps16_20 - At step 19 , SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification-eDRX. In order to make sure that P-RNTI Transmission happens in line with eDRX cycle , p_EDRXValue_Index is passed - At Step 21 , when sending the paging using the function f_NR5GC_UE_Page_eDRX, incorrect values are used for p_PTWLength_Index and p_EDRXValue_Index , this is corrected - Before steps 24-29 , UL grant configuration is provided . - At step 30 , when handling f_NR_UE_DeRegisterOnSwitchOff , currently SRB1 is provided . this is corrected to use the default value of SRB2 - v_NegotiatedExtdDRXParams is made to be in line as per Table 8.1.5.2.3.3.3-4: REGISTRATION ACCEPT (Step 44, Table 8.1.5.2.3.3.2-1) at step 45 - at step 46-47 of the test case , to perform //Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] incorrect function call f_NR5GC_RRC_Idle_Extension (nr_Cell1); is used , this needs to be replaced with the function call f_NR5GC_RRC_Idle_Steps16_20 - at step 49, as per the prose “SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.” , current TTCN sends tsc_NR_ShortMessage_SystemInfoModificationAndEDRX, this is corrected as systemInfoModification - at step 51 , when sending the paging using the function f_NR5GC_UE_Page_eDRX, incorrect values are used for p_PTWLength_Index and p_EDRXValue_Index , this is corrected - At step 53 , UL grant configuration is provided |
| TTCN module | develop\NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_5_2_3_TestBody() runs on NR5GC_PTC { var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0001'B, '00000001'B); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1; var integer v_PrachConfigurationIndex; var NR_PhysicalParameters_Type v_PhysicalParameters; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var NG_5G_S_TMSI v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); var template (present) TimingInfo_Type v_TimingInfo; var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; var float v_WaitForNext_eDRX_Acquisition; var SubFrameTiming_Type v_CurrentTime; var integer v_HSFN_Gap; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 Table 4.5.2.2-2 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. //The UE transmit a REGISTRATION COMPLETE message. f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-17" siclog@ //Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Extension (nr_Cell1); // @sic R5-237333 sic@ //@siclog "Steps 18" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 19" siclog@ //The SIB1 is updated according to Table 8.1.5.2.3.3.3-2 and SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_PrachConfigurationIndex := 91; //FR1 TDD - As per 38.211 Table 6.3.3.2-3 //nSFN mod 2 = 1=> any odd SFN Subframe 8/9 v_TimingInfo := ( cr_TimingInfo(?, (8)), cr_TimingInfo(?, (9)) ); } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { v_PrachConfigurationIndex := 120; //Half Duplex FR1 FDD - As per 38.211 Table 6.3.3.2-2 v_TimingInfo := (cr_TimingInfo(?, (4))); } else //FR1 FDD { v_PrachConfigurationIndex := 119; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 8 = 1=> any SFN Subframe 2/6/9 v_TimingInfo := ( cr_TimingInfo(?, (2)), cr_TimingInfo(?, (6)), cr_TimingInfo(?, (9)) ); } } else { // FR2 v_PrachConfigurationIndex := 6; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 4 // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); // Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //Modify sysinfo and send short msg indication. f_NR_ModifySysinfo(nr_Cell1, -, -, -, tsc_NR_ShortMessage_SystemInfoModificationAndEDRX); //@siclog "Steps 20" siclog@ //Wait till the beginning of next eDRX acquisition time (next H-SFN for which H-SFN mod 1024 =0). v_CurrentTime := f_NR_GetCurrentTiming(nr_Cell1); v_HSFN_Gap := 1024 - (v_CurrentTime.HSFN.Number mod 1024); // calculate HSFNs left to reach HSFN mod 1024 v_WaitForNext_eDRX_Acquisition := (int2float(v_HSFN_Gap) * 10.24) - (int2float(v_CurrentTime.SFN.Number + 1) * 0.01); // reduce by SFNs already elapsed + 1 f_Delay(v_WaitForNext_eDRX_Acquisition); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), 3, 5); //@siclog "Steps 22-23" siclog@ //Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 19? //The UE transmits an RRCSetupRequest. //SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PrachConfigurationIndex)) f_NR_RacingCond_AwaitSysIndBeforeSrbMsg(nr_Cell1, car_NR_PRACH_Preamble_IND (nr_Cell1, v_TimingInfo, ?), v_SrbIndByRef, v_SysIndByRef); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); f_NR_RacingCond_MatchRrcMsg(v_SrbIndByRef, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Steps 24-29" siclog@ //Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed. f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 30" siclog@ // Switch off UE in RRC CONNECTED as described in TS 38.508-1 subclause 4.9.6.3 f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1, STATE_CONNECTED_3A, v_SRB); //@siclog "Step 31" siclog@ // Switch on UE. f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Steps 32-45" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 Table 4.5.2.2-2 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. //The UE transmit a REGISTRATION COMPLETE message. f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 46-47" siclog@ //Steps 19a1-19Aa2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. //IF connected without release is not present THEN, the SS transmits an RRCRelease message. f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); //@siclog "Steps 48" siclog@ //IF connected without release is not present THEN, the SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 49" siclog@ //The SIB1 is updated according to Table 8.1.5.2.3.3.3-1 and SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { v_PrachConfigurationIndex := 157; //Half Duplex FR1 FDD - As per 38.211 Table 6.3.3.2-2 v_TimingInfo := (cr_TimingInfo(?, (4))); } else //FR1 { v_PrachConfigurationIndex := 160; if (f_NR_CellInfo_GetIsFDD(nr_Cell2)) { //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 1 = 0=> any SFN Subframe 1 v_TimingInfo := ( cr_TimingInfo(?, (1))); } else{ //FR1 TDD - As per 38.211 Table 6.3.3.2-3 //nSFN mod 1 = 0=> any odd SFN Subframe 9 v_TimingInfo := ( cr_TimingInfo(?, (9))); } } } else { // FR2 v_PrachConfigurationIndex := 149; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); // Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //Modify sysinfo and send short msg indication f_NR_ModifySysinfo(nr_Cell1, -, -, -, tsc_NR_ShortMessage_SystemInfoModificationAndEDRX); //@siclog "Steps 50" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 51" siclog@ //The SS transmits a Paging message within a paging occasion. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), 3, 5); //@siclog "Steps 52-53" siclog@ //Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 49? //The UE transmits an RRCSetupRequest. f_NR_RacingCond_AwaitSysIndBeforeSrbMsg(nr_Cell1, car_NR_PRACH_Preamble_IND (nr_Cell1, v_TimingInfo, ?), v_SrbIndByRef, v_SysIndByRef); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 52"); f_NR_RacingCond_MatchRrcMsg(v_SrbIndByRef, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 53" siclog@ //The UE transmits an RRCSetupRequest. f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); //@siclog "Steps 54-59" siclog@ //Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed. f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); }

After Change:

function fl_TC_8_1_5_2_3_TestBody() runs on NR5GC_PTC { var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0100'B, '00000001'B); //WA#WI=1330545 var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1; var integer v_PrachConfigurationIndex; var NR_PhysicalParameters_Type v_PhysicalParameters; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var NG_5G_S_TMSI v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); var template (present) TimingInfo_Type v_TimingInfo; var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; var float v_WaitForNext_eDRX_Acquisition; var SubFrameTiming_Type v_CurrentTime; var integer v_HSFN_Gap; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 Table 4.5.2.2-2 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. //The UE transmit a REGISTRATION COMPLETE message. f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-17" siclog@ //Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. //f_NR5GC_RRC_Idle_Extension (nr_Cell1); // @sic R5-237333 sic@ f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); //WA#WI=1330545 //@siclog "Steps 18" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 19" siclog@ //The SIB1 is updated according to Table 8.1.5.2.3.3.3-2 and SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_PrachConfigurationIndex := 91; //FR1 TDD - As per 38.211 Table 6.3.3.2-3 //nSFN mod 2 = 1=> any odd SFN Subframe 8/9 v_TimingInfo := ( cr_TimingInfo(?, (8)), cr_TimingInfo(?, (9)) ); } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { v_PrachConfigurationIndex := 120; //Half Duplex FR1 FDD - As per 38.211 Table 6.3.3.2-2 v_TimingInfo := (cr_TimingInfo(?, (4))); } else //FR1 FDD { v_PrachConfigurationIndex := 119; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 8 = 1=> any SFN Subframe 2/6/9 v_TimingInfo := ( cr_TimingInfo(?, (2)), cr_TimingInfo(?, (6)), cr_TimingInfo(?, (9)) ); } } else { // FR2 v_PrachConfigurationIndex := 6; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 4 // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); // Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //Modify sysinfo and send short msg indication. f_NR_ModifySysinfo(nr_Cell1, -, -, -, tsc_NR_ShortMessage_SystemInfoModificationAndEDRX, 4); //WA#WI=1330545 //@siclog "Steps 20" siclog@ //Wait till the beginning of next eDRX acquisition time (next H-SFN for which H-SFN mod 1024 =0). v_CurrentTime := f_NR_GetCurrentTiming(nr_Cell1); v_HSFN_Gap := 1024 - (v_CurrentTime.HSFN.Number mod 1024); // calculate HSFNs left to reach HSFN mod 1024 v_WaitForNext_eDRX_Acquisition := (int2float(v_HSFN_Gap) * 10.24) - (int2float(v_CurrentTime.SFN.Number + 1) * 0.01); // reduce by SFNs already elapsed + 1 f_Delay(v_WaitForNext_eDRX_Acquisition); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), 1, 4); //WA#WI=1330545 //@siclog "Steps 22-23" siclog@ //Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 19? //The UE transmits an RRCSetupRequest. //SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PrachConfigurationIndex)) f_NR_RacingCond_AwaitSysIndBeforeSrbMsg(nr_Cell1, car_NR_PRACH_Preamble_IND (nr_Cell1, v_TimingInfo, ?), v_SrbIndByRef, v_SysIndByRef); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); f_NR_RacingCond_MatchRrcMsg(v_SrbIndByRef, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_ULGrantConfiguration_Start(nr_Cell1); //WA#WI=1330545 //@siclog "Steps 24-29" siclog@ //Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed. f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 30" siclog@ // Switch off UE in RRC CONNECTED as described in TS 38.508-1 subclause 4.9.6.3 f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1, STATE_CONNECTED_3A/*, v_SRB*/); //WA#WI=1330545 //@siclog "Step 31" siclog@ // Switch on UE. f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Steps 32-45" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 Table 4.5.2.2-2 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. //The UE transmit a REGISTRATION COMPLETE message. v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0000'B, '00000011'B); //WA#WI=1330545 f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 46-47" siclog@ //Steps 19a1-19Aa2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. //IF connected without release is not present THEN, the SS transmits an RRCRelease message. //f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); //WA#WI=1330545 f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); //WA#WI=1330545 //@siclog "Steps 48" siclog@ //IF connected without release is not present THEN, the SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 49" siclog@ //The SIB1 is updated according to Table 8.1.5.2.3.3.3-1 and SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { v_PrachConfigurationIndex := 157; //Half Duplex FR1 FDD - As per 38.211 Table 6.3.3.2-2 v_TimingInfo := (cr_TimingInfo(?, (4))); } else //FR1 { v_PrachConfigurationIndex := 160; if (f_NR_CellInfo_GetIsFDD(nr_Cell2)) { //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 1 = 0=> any SFN Subframe 1 v_TimingInfo := ( cr_TimingInfo(?, (1))); } else{ //FR1 TDD - As per 38.211 Table 6.3.3.2-3 //nSFN mod 1 = 0=> any odd SFN Subframe 9 v_TimingInfo := ( cr_TimingInfo(?, (9))); } } } else { // FR2 v_PrachConfigurationIndex := 149; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); // Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //Modify sysinfo and send short msg indication f_NR_ModifySysinfo(nr_Cell1, -, -, -); //WA#WI=1330545 //@siclog "Steps 50" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 51" siclog@ //The SS transmits a Paging message within a paging occasion. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), 3, 0); //WA#WI=1330545 //@siclog "Steps 52-53" siclog@ //Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 49? //The UE transmits an RRCSetupRequest. f_NR_RacingCond_AwaitSysIndBeforeSrbMsg(nr_Cell1, car_NR_PRACH_Preamble_IND (nr_Cell1, v_TimingInfo, ?), v_SrbIndByRef, v_SysIndByRef); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 52"); f_NR_RacingCond_MatchRrcMsg(v_SrbIndByRef, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 53" siclog@ //The UE transmits an RRCSetupRequest. //f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); //WA#WI=1330545 f_NR_ULGrantConfiguration_Start(nr_Cell1); //WA#WI=1330545 //@siclog "Steps 54-59" siclog@ //Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed. f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); }

## Correction to the Function f_NR_ModifySysinfo ()

| Function name | f_NR_ModifySysinfo |
| --- | --- |
| Reason for change | Current TTCN implementation of this function does not take care of the case when eDrx cycle is greater than 1024 radio frames Please refer to TS38.304 1. HSFN calculation is in "7.4 Paging in extended DRX": The PH for CN is the H-SFN satisfying the following equations: H-SFN mod TeDRX_CN= (UE_ID_H mod TeDRX_CN), where - UE_ID_H: 13 most significant bits of the Hashed ID. - TeDRX_CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_CN = 2, …, 1024 Hyper-frames) configured by upper layers. [HSFN Calculation] UE id = (0x41C2345678) % 4096 = 1656. TeDRX_CN = 4096/1024 = 4 HSFNs (1 HSFN=4096rf) UE_ID_H = 5138 H-SFN mod 4 = UE_ID_H mod TeDRX_CN => H-SFN mod 4 = 5138 mod 4 = 2 => H-SFN of PH = 2 + 4*n = {2, 6, 10, 14, 18, ... , 666, 670, ... ,(2 + 4*n), ... ,1022} Current TTCN function f_NR_ModifySysinfo for sending tsc_NR_ShortMessage_SystemInfoModificationAndEDRX , is scheduling the short message at the current HSFN cycle . but in order to match the HSFN as per above snippet , TTCN needs to make sure , the current HSFN value mod 4 ==2 . Hence specific implementation is added to handle p_EDRXValue_Index > 2 cases |
| Summary of change | Implementation is added to handle p_EDRXValue_Index > 2 cases where eDrx cycle is greater than 1024 radio frames |
| TTCN module | develop\Common\NR5GC\NR5GC_SysinfoPaging.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_ModifySysinfo( NR_CellId_Type p_NR_CellId, boolean p_ShortMessageFlag := true, NR_RRC_STATE_Type p_UE_State := RRC_IDLE, boolean p_BroadcastAllSI := false, B8_Type p_ShortMessages := tsc_NR_ShortMessage_SystemInfoModification ) runs on NR5GC_PTC return SubFrameTiming_Type { //Note: in INACTIVE state UE may have a different DRX cycle - that is not needed so far //@sic R5-235343: added p_ShortMessages sic@ var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_TimeOfShortMessage; var SubFrameTiming_Type v_TimeOfSysinfoModification; var integer v_Timing_SFN; var SubFrameTiming_Type v_StartNextModPeriod; var integer v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod (p_NR_CellId); var integer v_ProcessingDelay := tsc_NR_SfnDelay; //For IDLE and INACTIVE state var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId,p_BroadcastAllSI); //@sic R5s200703 sic@ var integer v_StartNextModPeriod_SFN; //When UE is in connected mode, short message will be sent during a complete modification period //Add a longer processing delay to current SFN if (p_UE_State == RRC_CONNECTED and not(pc_ETWS_NR or pc_CMAS_NR)) {v_ProcessingDelay := tsc_NR_SfnDelayConnected} //Get the current SFN and subframe v_Timing := f_NR_GetCurrentTiming(p_NR_CellId); //Add processing delay to current SFN v_Timing_SFN := v_Timing.SFN.Number + v_ProcessingDelay; // Calculate the SFN of the next Modification Period //@sic R5-211417 R5s210404 sic@ v_StartNextModPeriod_SFN := v_Timing_SFN + (v_ModificationPeriodFrames -(v_Timing_SFN mod v_ModificationPeriodFrames)); v_StartNextModPeriod.HSFN.Number := (v_Timing.HSFN.Number + (v_StartNextModPeriod_SFN/1024) ) mod 1024; v_StartNextModPeriod.SFN.Number := v_StartNextModPeriod_SFN mod 1024; //Set Subframe and slot to 0, i.e. start of the modification period v_StartNextModPeriod.Subframe.Number := 0; v_StartNextModPeriod.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); //@sic R5s221246 sic@ v_StartNextModPeriod := f_NR_SubFrameTiming_SetSlot(v_StartNextModPeriod, 0, v_SCS); if (not p_ShortMessageFlag) { //Short Msg Indication not to be sent v_TimeOfSysinfoModification := v_StartNextModPeriod; } else { //@sic R5-211417 sic@ v_TimeOfShortMessage := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; v_TimeOfSysinfoModification := v_StartNextModPeriod; f_NR_ShortMessageIndication(p_NR_CellId, v_TimeOfShortMessage, p_UE_State, p_ShortMessages); } f_NR_SS_ConfigureSysinfo (p_NR_CellId, cs_TimingInfo_SFNOnly(v_TimeOfSysinfoModification.SFN.Number), v_NR_BcchToPdschConfig, p_BroadcastAllSI); //@sic R5s220880 sic@ //Wait for the Short Message to be broadcasted @sic R5s210404 sic@ if (p_ShortMessageFlag){ // wait until the short message is to be sent f_Delay(int2float(f_SubFrameTiming_Duration(v_Timing, v_TimeOfShortMessage) - 100 )/ 1000.0); } return v_TimeOfSysinfoModification; }

After Change:

function f_NR_ModifySysinfo( NR_CellId_Type p_NR_CellId, boolean p_ShortMessageFlag := true, NR_RRC_STATE_Type p_UE_State := RRC_IDLE, boolean p_BroadcastAllSI := false, B8_Type p_ShortMessages := tsc_NR_ShortMessage_SystemInfoModification, integer p_EDRXValue_Index := 0 ) runs on NR5GC_PTC return SubFrameTiming_Type { //Note: in INACTIVE state UE may have a different DRX cycle - that is not needed so far //@sic R5-235343: added p_ShortMessages sic@ var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_TimeOfShortMessage; var SubFrameTiming_Type v_TimeOfSysinfoModification; var integer v_Timing_SFN; var SubFrameTiming_Type v_StartNextModPeriod; var integer v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod (p_NR_CellId); var integer v_ProcessingDelay := tsc_NR_SfnDelay; //For IDLE and INACTIVE state var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId,p_BroadcastAllSI); //@sic R5s200703 sic@ var integer v_StartNextModPeriod_SFN; var integer v_H_UE_ID; //WA#WI=1330545 var NG_5G_S_TMSI v_NG_5G_S_TMSI; //WA#WI=1330545 var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); //WA#WI=1330545 v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); // Extract the bits b31, b30..., b0 of 5G-S-TMSI //WA#WI=1330545 v_H_UE_ID := bit2int(substr((fx_CalculateFCS32(substr(v_NG_5G_S_TMSI,16,32))),0, 13)); // UE_ID_H: 13 most significant bits of the Hashed ID. //WA#WI=1330545 var integer v_HSFN_Gap := 0; var float v_HSFN_Gap_delay; //When UE is in connected mode, short message will be sent during a complete modification period //Add a longer processing delay to current SFN if (p_UE_State == RRC_CONNECTED and not(pc_ETWS_NR or pc_CMAS_NR)) {v_ProcessingDelay := tsc_NR_SfnDelayConnected} //Get the current SFN and subframe v_Timing := f_NR_GetCurrentTiming(p_NR_CellId); // We can not choose the current HSFN Value for short message to be transmitted // TTCN should ensure current HSFN calculation is as per TS38.304 "7.4 Paging in extended DRX": // // Calculate next Paging Occasion // TS 38.304 clause 7.4: If the UE is configured with an extended DRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE configured with eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. if(p_EDRXValue_Index > 2)//WA#WI=1330545 { if (v_Timing.HSFN.Number mod p_EDRXValue_Index != v_H_UE_ID mod p_EDRXValue_Index) { // Find the HSFN Gap to reach the HSFN which matches the condition v_HSFN_Gap := (v_H_UE_ID mod p_EDRXValue_Index) - (v_Timing.HSFN.Number mod p_EDRXValue_Index); if(v_HSFN_Gap < 0) { v_HSFN_Gap := v_HSFN_Gap + p_EDRXValue_Index; } v_HSFN_Gap_delay := (int2float(v_HSFN_Gap) * 10.24) - (int2float(v_Timing.SFN.Number + 1) * 0.01); f_Delay(v_HSFN_Gap_delay); // Re-fetch the current timing to apply for short Message Transmission v_Timing := f_NR_GetCurrentTiming(nr_Cell1); } }//WA#WI=1330545 //Add processing delay to current SFN v_Timing_SFN := v_Timing.SFN.Number + v_ProcessingDelay; // Calculate the SFN of the next Modification Period //@sic R5-211417 R5s210404 sic@ v_StartNextModPeriod_SFN := v_Timing_SFN + (v_ModificationPeriodFrames -(v_Timing_SFN mod v_ModificationPeriodFrames)); v_StartNextModPeriod.HSFN.Number := (v_Timing.HSFN.Number + (v_StartNextModPeriod_SFN/1024) ) mod 1024; v_StartNextModPeriod.SFN.Number := v_StartNextModPeriod_SFN mod 1024; //Set Subframe and slot to 0, i.e. start of the modification period v_StartNextModPeriod.Subframe.Number := 0; v_StartNextModPeriod.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); //@sic R5s221246 sic@ v_StartNextModPeriod := f_NR_SubFrameTiming_SetSlot(v_StartNextModPeriod, 0, v_SCS); if (not p_ShortMessageFlag) { //Short Msg Indication not to be sent v_TimeOfSysinfoModification := v_StartNextModPeriod; } else { //@sic R5-211417 sic@ v_TimeOfShortMessage := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; v_TimeOfSysinfoModification := v_StartNextModPeriod; f_NR_ShortMessageIndication(p_NR_CellId, v_TimeOfShortMessage, p_UE_State, p_ShortMessages); } f_NR_SS_ConfigureSysinfo (p_NR_CellId, cs_TimingInfo_SFNOnly(v_TimeOfSysinfoModification.SFN.Number), v_NR_BcchToPdschConfig, p_BroadcastAllSI); //@sic R5s220880 sic@ //Wait for the Short Message to be broadcasted @sic R5s210404 sic@ if (p_ShortMessageFlag){ // wait until the short message is to be sent f_Delay(int2float(f_SubFrameTiming_Duration(v_Timing, v_TimeOfShortMessage) - 100 )/ 1000.0); } return v_TimeOfSysinfoModification; }

# Branches executed

## This NR TC was executed in

- MediaTek MT6815 with FR1 Band n5 , NEA2 ciphering and NIA2 integrity algorithms

- HiSilicon Balong 5000 with FR1 Band n78 , NEA1 ciphering and NIA1 integrity algorithms .

# Execution Log Files

## MediaTek MT6815

The MediaTek MT6815 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_5_2_3_NR5GC_MTK
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## Hisilicon Balong 5000

The Hisilicon Balong 5000 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_5_2_3_NR5GC_HiSi
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250069: Supporting information for addition of NR5GC testcase 8.1.5.2.3 in FR1 |
| --- | --- |