3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250150

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3638 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.12.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A TTCN change was missed out to be highlighted in TTCN CR R5s250070 in that as per TS 38.523-3, section 5.2.2.4.2, Figure 5.2.2.4.2-1: “The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field. When multi-cell scheduling by a single DCI is configured, the MacBearerRouting field in the common part of the data ASP provides the id of the cell hosting the physical layer, which the SS shall use to send the DCI; the MacBearerRouting field in the U_Plane part of the data ASP indicates the NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell” Here, the RLC entity is connected to PCell. Hence, at Steps 5 and 8, the initiation of the transmission of the Data on the DRB should be on the PCell and it then gets routed to the SCell which hosts the physical layer to send the PDCCH DCI |  |  |  |  |  |  |  |  |
| Summary of change: |  | The first parameter to the template cas_NR_DRB_COMMON_REQ_MAC_PDUList is modified to PCell at Steps 5 and 8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_4_NR_Steps3_9 |
| --- | --- |
| Reason for change | A TTCN change was missed out to be highlighted in TTCN CR R5s250070 in that as per TS 38.523-3, section 5.2.2.4.2, Figure 5.2.2.4.2-1: “The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field. When multi-cell scheduling by a single DCI is configured, the MacBearerRouting field in the common part of the data ASP provides the id of the cell hosting the physical layer, which the SS shall use to send the DCI; the MacBearerRouting field in the U_Plane part of the data ASP indicates the NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell” Here, the RLC entity is connected to PCell. Hence, at Steps 5 and 8, the initiation of the transmission of the Data on the DRB should be on the PCell and it then gets routed to the SCell which hosts the physical layer to send the PDCCH DCI |
| Summary of change | The first parameter to the template cas_NR_DRB_COMMON_REQ_MAC_PDUList is modified to PCell at Steps 5 and 8 |
| TTCN module | MAC_PowerSaving_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC { <<SKIPPED CODE>> //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. //f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-225297, R5s250070 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); <<SKIPPED CODE>> //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); //f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-250070 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC { <<SKIPPED CODE>> //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. //f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-225297, R5s250070 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_PCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //WA# <<SKIPPED CODE>> //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); //f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-250070 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_PCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //WA# <<SKIPPED CODE>>