3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250154

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3659 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC eCall over IMS testcase 8.1.4.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During handover from Cell1 to Cell3 an eCall is currently on-going, implementation applies the change of routing table after all the signalling has happened. Issue is that RTP packets might be received before this changes are completed failing the test case with unexpected data received in wrong Cell. This can be solved with an altstep to handle the RTP packets send by the UE while transitioning from Cell1 to Cell3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a defautl altstep to handle RTP packets send by the UE during Handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Online, , 13th Dec 2024 - 31st Dec 2025	1

1	Overview	2

2	Corrections required:	2

2.1	Correction to function fl_TC_8_1_4_1_10_TestBody	2

2.2	New altstep a_Handle_RTP_Packets()	4

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required:

  - Correction to function fl_TC_8_1_4_1_10_TestBody

| Function name | fl_TC_8_1_4_1_10_TestBody |
| --- | --- |
| Reason for change | During handover from Cell1 to Cell3 an eCall is currently on-going, implementation applies the change of routing table after all the signalling has happened. Issue is that RTP packets might be received before this changes are completed failing the test case with unexpected data received in wrong Cell. |
| Summary of change | Added a defautl altstep to handle RTP packets send by the UE during Handover transition. |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_1_10_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR2; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR2; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR2; var integer v_Rsrp; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var UE_NR_DeltaValues_Type v_UE_NR_DeltaValues; <<SKIPPED CODE>> //@siclog "Steps 5-6" siclog@ //The SS transmits an RRCReconfiguration message including reconfigurationWithSync with rach-ConfigDedicated, keySetChangeIndicator set to true and including nas-Container IE to order the UE to perform inter-frequency handover to NR Cell 3 //Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 3? v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); f_NR5GC_508RRC_IntraNR_HO_InterCell(nr_Cell1, nr_Cell3, -, -, // RBConfig_KeyChange := true cs_NR_MeasConfigRemove(cs_MeasObjectToRemoveList_2Entry(tsc_NR_MeasObjectId1, tsc_NR_MeasObjectId2), {tsc_NR_MeasId1}), //@sic R5-204237 sic@ -, -, cs_MasterKeyUpdate(true, 0, f_BuildIntraN1TransparentContainer ()), //@sic R5-202641 sic@ cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(nr_Cell3)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IP_ChangeNrCell(IP, nr_Cell3); // @sic R5s230685 sic@ //@siclog "Step 7" siclog@ //Release eCall over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8. f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //This step is done in the IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); }// End of fl_TC_8_1_4_1_10_TestBody

After Change:

function fl_TC_8_1_4_1_10_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR2; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR2; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR2; var integer v_Rsrp; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var UE_NR_DeltaValues_Type v_UE_NR_DeltaValues; var default v_Default; <<SKIPPED CODE>> //@siclog "Steps 5-6" siclog@ //The SS transmits an RRCReconfiguration message including reconfigurationWithSync with rach-ConfigDedicated, keySetChangeIndicator set to true and including nas-Container IE to order the UE to perform inter-frequency handover to NR Cell 3 //Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 3? v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); v_Default := activate(a_Handle_RTP_Packets());//Handle RTP packets during Handover f_NR5GC_508RRC_IntraNR_HO_InterCell(nr_Cell1, nr_Cell3, -, -, // RBConfig_KeyChange := true cs_NR_MeasConfigRemove(cs_MeasObjectToRemoveList_2Entry(tsc_NR_MeasObjectId1, tsc_NR_MeasObjectId2), {tsc_NR_MeasId1}), //@sic R5-204237 sic@ -, -, cs_MasterKeyUpdate(true, 0, f_BuildIntraN1TransparentContainer ()), //@sic R5-202641 sic@ cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(nr_Cell3)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IP_ChangeNrCell(IP, nr_Cell3); // @sic R5s230685 sic@ deactivate(v_Default); //@siclog "Step 7" siclog@ //Release eCall over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8. f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //This step is done in the IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); }// End of fl_TC_8_1_4_1_10_TestBody

  - New altstep a_Handle_RTP_Packets()

| Function name | a_Handle_RTP_Packets |
| --- | --- |
| Reason for change | During handover from Cell1 to Cell3 an eCall is currently on-going, implementation applies the change of routing table after all the signalling has happened. Issue is that RTP packets might be received before this changes are completed failing the test case with unexpected data received in wrong Cell. |
| Summary of change | Added a defautl altstep to handle RTP packets send by the UE during Handover. |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment |  |

New altstep:

altstep a_Handle_RTP_Packets() runs on NR_BASE_PTC { [] DRB.receive (cr_NR_DRB_COMMON_IND_Any) {repeat;} }