3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250155

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3660 | rev | - | Current version: | 18.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC SOR testcase 6.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1 Timer p_SorcmValue should be adjusted for the 10% tolerance, SS may receive DEREGISTRATION REQUEST message before Tsor-cm timer expires. 2 The iel is not correct if pass omit to p_CriterionValue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Called function f_NR_SetTimerToleranceMin on timer p_SorcmValue. 2 Get iel according to the value of p_CriterionValue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact: Ting Gao
gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Function name | f_NR5GC_TransportSorCmCheckfunction |
| --- | --- |
| Reason for change | Timer p_SorcmValue should be adjusted for the 10% tolerance, SS may receive DEREGISTRATION REQUEST message before Tsor-cm timer expires. |
| Summary of change | Called function f_NR_SetTimerToleranceMin on timer p_SorcmValue. |
| TTCN module | SOR_CMCI_NR5GC.ttcn |

Before Change:

function f_NR5GC_TransportSorCmCheckfunction(NR_CellId_Type p_NR_CellId, NR_CellId_Type p_NR_TargetCellId, template (value) NG_NAS_DL_Pdu_Type p_PduToSend, template (present) PayloadContainerType p_PayloadContainerType:=cr_PayloadContainerType(tsc_PayloadContainerSORMsg),//LYLY template (omit) charstring p_VerdictMsg := "Step 26 PASS", template (omit) charstring p_SecondVerdictMsg := "Step 28 PASS", float p_TmaxValue:=660.0, float p_SorcmValue:=60.0) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Tmax; timer t_Sorcm; //@siclog "Step n+1" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,p_PduToSend.Msg))); //@siclog "Step n+2" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start(p_SorcmValue); //@siclog "Step n+3" siclog@ //The SS starts timer of tmax =(6 minutes + cell selection time) //@siclog "Step n+4" siclog@ //The UE transmits an ULInformationTransfer message … }

After Change:

function f_NR5GC_TransportSorCmCheckfunction(NR_CellId_Type p_NR_CellId, NR_CellId_Type p_NR_TargetCellId, template (value) NG_NAS_DL_Pdu_Type p_PduToSend, template (present) PayloadContainerType p_PayloadContainerType:=cr_PayloadContainerType(tsc_PayloadContainerSORMsg),//LYLY template (omit) charstring p_VerdictMsg := "Step 26 PASS", template (omit) charstring p_SecondVerdictMsg := "Step 28 PASS", float p_TmaxValue:=660.0, float p_SorcmValue:=60.0) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Tmax; timer t_Sorcm; //@siclog "Step n+1" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,p_PduToSend.Msg))); //@siclog "Step n+2" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start(f_NR_SetTimerToleranceMin(p_NR_CellId, nonProtocolTimer, p_SorcmValue)); //@siclog "Step n+3" siclog@ //The SS starts timer of tmax =(6 minutes + cell selection time) //@siclog "Step n+4" siclog@ //The UE transmits an ULInformationTransfer message … }

  - Change 2

| Template name | cs_SOR_CMCIRule |
| --- | --- |
| Reason for change | The iel is not correct if pass omit to p_CriterionValue |
| Summary of change | Get iel according to the value of p_CriterionValue. |
| TTCN module | NG_NAS_Templates.ttcn |

Before Change:

template (value) SOR_CMCIRule cs_SOR_CMCIRule (O1_Type p_TsorcmTimer, O1_Type p_CriterionType, template (omit) octetstring p_CriterionValue, octetstring p_Len := '00'O):= // @sic R5s240580 sic@ { /* @status APPROVED (NR5GC) */ /* 24.501 cl. 9.11.3.51 */ iel := p_Len, tSORcmTimer := p_TsorcmTimer, criterionType := p_CriterionType, criterionValue := p_CriterionValue };

After Change:

template (value) SOR_CMCIRule cs_SOR_CMCIRule (O1_Type p_TsorcmTimer, O1_Type p_CriterionType, template (omit) octetstring p_CriterionValue):= { /* @status APPROVED (NR5GC) */ /* 24.501 cl. 9.11.3.51 */ iel := f_get_CMCIRule_length(p_CriterionValue), tSORcmTimer := p_TsorcmTimer, criterionType := p_CriterionType, criterionValue := p_CriterionValue };

  - Change 3

| Function name | f_get_CMCIRule_length |
| --- | --- |
| Reason for change | For change 2. |
| Summary of change | Add a new function |
| TTCN module | NG_NAS_Common.ttcn |

New function:

function f_get_CMCIRule_length(template (omit) octetstring p_CriterionValue) return Type6Length_Type { if(ispresent(p_CriterionValue)) { return int2oct(lengthof (p_CriterionValue) + 2, 2); } else { return int2oct(2, 2); } }