3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250158

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3662 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In current TTCN, DRB ID (tsc_LCID_SCG_Bearer=5) is being assigned on the data transmitted (v_SDU_SubPDUListDL) DL DRB request and preamble step assigns different DRB ID 4 with ims DNN and DRB ID 5 with internet DNN. Hence when UE is set with IMS DNN, DL DRB request assign with wrong DRB ID 5 which lead test failure with No SDUs on UL DRB indications. - Currently at step19, TTCN is using template which is used for NoCRNTI based grants (SuppressPdcchForC_RNTI=true) or can say TTCN assigning SPS based DL transmission, And in this case DL SPS was never configured, So SS will not transmit these DL DRB packet at step19 and UE will never feedback them at step20 which leads this test as non-conformant pass. To correct this behaviour SS should assing CRNTI based transmission at step19 so UE can feedback them at step20 if UL configuredgrant type1 is not disabled by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added variable tsc_LCID_SCG_Bearer_IMS =4 for the IMS DNN and applied condition according to DNN (for internet or IMS) to select correct value for DRB ID. - Updated template for DL transmission at step19 cas_NR_DRB_COMMON_REQ_MAC_PDUList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_2_NR_TestBody1 |
| --- | --- |
| Reason for change | In current TTCN, DRB ID (tsc_LCID_SCG_Bearer=5) is being assigned on the data transmitted (v_SDU_SubPDUListDL) DL DRB request and preamble step assigns different DRB ID 4 with ims DNN and DRB ID 5 with internet DNN. Hence when UE is set with IMS DNN, DL DRB request assign with wrong DRB ID 5 which lead test failure with No SDUs on UL DRB indications. |
| Summary of change | Added variable tsc_LCID_SCG_Bearer_IMS =4 for the IMS DNN and applied condition according to DNN (for internet or IMS) to select correct value for DRB ID. |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_2_NR_TestBody1(DRB_Identity p_NR_DRB_Id ) runs on NR_BASE_PTC { //@sic R5s230287 sic@ const integer const_Cfg_Period :=40; // period in milli seconds const integer const_Cfg_TOffset := 9; // In slots var SubFrameTiming_Type v_TimingDL; //@sic R5s230287 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL ; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var UInt_Type v_SlotsPerSubframe := f_NR_CellInfo_GetSCS_SlotsNumberPerSubframe(v_SCS); var UInt_Type v_SlotsPerCfg_Period := const_Cfg_Period *v_SlotsPerSubframe; var integer v_SlotsT1; timer t_Watchdog := 11.0; // @siclog "Step 3" siclog@ //@sic R5s240269 sic@ v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_96B,1))); //@sic R5s240432 sic@ v_EncodedRlcPdu2 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(1, crs_NR_PDCP_SDU_96B1,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingDL.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); //@sic R5s230287 r5s240432 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; // @siclog "Step 4" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL1))-> value v_NR_DRB_COMMON_IND; v_TimingUL_1 := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SlotsT1 := f_NR_SubFrameTiming_NumSlots(nr_Cell1, v_TimingUL_1); if (v_SlotsT1 mod v_SlotsPerCfg_Period == const_Cfg_TOffset) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 5 Timing // @siclog "Step 5" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; }

After Change:

function f_TC_7_1_1_6_2_NR_TestBody1(DRB_Identity p_NR_DRB_Id ) runs on NR_BASE_PTC { //@sic R5s230287 sic@ const integer const_Cfg_Period :=40; // period in milli seconds const integer const_Cfg_TOffset := 9; // In slots var SubFrameTiming_Type v_TimingDL; //@sic R5s230287 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL ; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var UInt_Type v_SlotsPerSubframe := f_NR_CellInfo_GetSCS_SlotsNumberPerSubframe(v_SCS); var UInt_Type v_SlotsPerCfg_Period := const_Cfg_Period *v_SlotsPerSubframe; var integer v_SlotsT1; timer t_Watchdog := 11.0; var B6_Type LCID_SCG_Bearer; var B6_Type tsc_LCID_SCG_Bearer_IMS := int2bit(4, 6); if (p_NR_DRB_Id ==1){ LCID_SCG_Bearer := tsc_LCID_SCG_Bearer_IMS; } else { LCID_SCG_Bearer := tsc_LCID_SCG_Bearer; } // @siclog "Step 3" siclog@ //@sic R5s240269 sic@ v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_96B,1))); //@sic R5s240432 sic@ v_EncodedRlcPdu2 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(1, crs_NR_PDCP_SDU_96B1,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu2)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingDL.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); //@sic R5s230287 r5s240432 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; // @siclog "Step 4" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL1))-> value v_NR_DRB_COMMON_IND; v_TimingUL_1 := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SlotsT1 := f_NR_SubFrameTiming_NumSlots(nr_Cell1, v_TimingUL_1); if (v_SlotsT1 mod v_SlotsPerCfg_Period == const_Cfg_TOffset) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 5 Timing // @siclog "Step 5" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; }

## Change 2

| Function name | f_TC_7_1_1_6_2_NR_TestBody2 |
| --- | --- |
| Reason for change | Similar reason mentioned on change 1 |
| Summary of change | Similar change mentioned on change1 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_2_NR_TestBody2(DRB_Identity p_NR_DRB_Id, NR_ResourceAllocation_Type p_NR_ResourceAllocation_CSRNTI ) runs on NR_BASE_PTC {//@sic R5s230287 sic@ -------skip code------ timer t_Watchdog := 11.0; // Initialise v_SubFrameInfo, v_SlotTimingInfo for UL grant on PDCCH K2 =4 slots ahead; // @siclog "Step 7A" siclog@ // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =18 byte =144 bits // 6 MAC Sub PDU's 6*144 = 864. 888 is next allowed downlink TBS in all tables, 24 bit =3 byte padding in DL. //@sic R5-227518 sic@ v_EncodedRlcPdu3 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_96B2,1))); v_EncodedRlcPdu4 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_96B3,1))); v_EncodedRlcPdu5 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_96B4,1))); v_EncodedRlcPdu6 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_96B5,1))); v_EncodedRlcPdu7 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_96B6,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu4), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu5), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu6), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu7)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; //Prepare expected UL MAC PDU's //Step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 10 & 12 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 13 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 15 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; ---------skip code---- }

After Change:

function f_TC_7_1_1_6_2_NR_TestBody2(DRB_Identity p_NR_DRB_Id, NR_ResourceAllocation_Type p_NR_ResourceAllocation_CSRNTI ) runs on NR_BASE_PTC {//@sic R5s230287 sic@ -------skip code------ timer t_Watchdog := 11.0; var B6_Type LCID_SCG_Bearer; var B6_Type tsc_LCID_SCG_Bearer_IMS := int2bit(4, 6); if (p_NR_DRB_Id ==1){ LCID_SCG_Bearer := tsc_LCID_SCG_Bearer_IMS; } else { LCID_SCG_Bearer := tsc_LCID_SCG_Bearer; } // Initialise v_SubFrameInfo, v_SlotTimingInfo for UL grant on PDCCH K2 =4 slots ahead; // @siclog "Step 7A" siclog@ // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =18 byte =144 bits // 6 MAC Sub PDU's 6*144 = 864. 888 is next allowed downlink TBS in all tables, 24 bit =3 byte padding in DL. //@sic R5-227518 sic@ v_EncodedRlcPdu3 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_96B2,1))); v_EncodedRlcPdu4 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_96B3,1))); v_EncodedRlcPdu5 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_96B4,1))); v_EncodedRlcPdu6 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_96B5,1))); v_EncodedRlcPdu7 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_96B6,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu3), cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu4), cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu5), cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu6), cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu7)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; //Prepare expected UL MAC PDU's //Step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 10 & 12 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 13 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 15 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; ---------skip code---- }

## Change 3

| Function name | f_TC_7_1_1_6_2_NR_TestBody3 |
| --- | --- |
| Reason for change | - Similar reason mentioned on change 1 - Currently at step19, TTCN is using template which is used for NoCRNTI based grants (SuppressPdcchForC_RNTI=true) or can say TTCN assigning SPS based DL transmission, And, in this case DL SPS was never configured, So SS will not transmit these DL DRB packet at step19 and UE will never feedback them at step20 which leads this test as non-conformant pass. To correct this behaviour SS should assing CRNTI based transmission at step19 so UE can feedback them at step20 if UL configuredgrant type1 is not disabled by the UE. |
| Summary of change | - Similar Change mentioned on change 1. - Updated template for DL transmission at step19 cas_NR_DRB_COMMON_REQ_MAC_PDUList. |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_2_NR_TestBody3(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_MAC_SDU_Type v_EncodedRlcPdu8; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(*, *, *) }; timer t_Watchdog := 0.2; v_EncodedRlcPdu8 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_96B7,1))); v_SDU_SubPDUListDL := {cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu8)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); ----skip code---- };

After Change:

function f_TC_7_1_1_6_2_NR_TestBody3(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_MAC_SDU_Type v_EncodedRlcPdu8; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(*, *, *) }; timer t_Watchdog := 0.2; var B6_Type LCID_SCG_Bearer; var B6_Type tsc_LCID_SCG_Bearer_IMS := int2bit(4, 6); if (p_NR_DRB_Id ==1){ LCID_SCG_Bearer := tsc_LCID_SCG_Bearer_IMS; } else { LCID_SCG_Bearer := tsc_LCID_SCG_Bearer; } v_EncodedRlcPdu8 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_96B7,1))); v_SDU_SubPDUListDL := {cs_NR_MAC_SDU_SubPDU_LI8(LCID_SCG_Bearer, v_EncodedRlcPdu8)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; -----skip code--- };