3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250173

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3667 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.4.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The T2 power levels as they are currenty defined in 38.523-1 Table 8.1.4.1.6.3.2-2 set identical level for both nr_cell1 and nr_cell2. Since the cells are intra-frequency neighbours, this creates interferences that especially in FR2 OTA scenario can disrupt the RRC procedure expected to happen in steps 4-6. Prose CR associated with this change will be raised during RAN5#107 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Decreased power level of neigbour cell nr_cell2 in instance T2 in Table 8.1.4.1.6.3.2-2 from -91dBm to -100dBm. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case due to interferences. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_1_6_TestBody |
| --- | --- |
| Reason for change | The T2 power levels as they are currenty defined in 38.523-1 Table 8.1.4.1.6.3.2-2 set identical level for both nr_cell1 and nr_cell2. Since the cells are intra-frequency neighbours, this creates interferences that especially in FR2 OTA scenario can disrupt the RRC procedure expected to happen in steps 4-6. Prose CR associated with this change will be raised during RAN5#107 meeting |
| Summary of change | Decreased power level of neigbour cell nr_cell2 in instance T2 in Table 8.1.4.1.6.3.2-2 from -91dBm to -100dBm. |
| TTCN module | RRC_IntraNR_Handover_NR5GC.ttcn |

Before Change:

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ShortMAC_I v_ShortMAC_I; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NextHopChainingCount v_NextHopChainingCount; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1) }; //As per Table 8.1.4.1.6.3.2-1 var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2), //@sic R5-237355 sic@ cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1 @sic R5s220824 R5-224130 sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@sic R5s201221 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.6.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); t_T304Min.start(v_T304Min); //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.6.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell1); //UE re-establish SRB1 - UE releases SRB2 and DRB //In SS: re-establish SRB1 and SRB2 - Release DRBs //In SS: It helps with clearing the RB buffer handling reconfiguration retx //@sic R5s201221 R5s210197 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); // Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(nr_Cell1, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, true); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell1, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), //@sic R5s2211094 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); //@siclog "Step 4" siclog@

After Change:

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ShortMAC_I v_ShortMAC_I; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NextHopChainingCount v_NextHopChainingCount; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1) }; //As per Table 8.1.4.1.6.3.2-1 var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2), //@sic R5-237355 sic@ cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1 @sic R5s220824 R5-224130 sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@sic R5s201221 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.6.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); t_T304Min.start(v_T304Min); //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.6.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell1); //UE re-establish SRB1 - UE releases SRB2 and DRB //In SS: re-establish SRB1 and SRB2 - Release DRBs //In SS: It helps with clearing the RB buffer handling reconfiguration retx //@sic R5s201221 R5s210197 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); // Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(nr_Cell1, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, true); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell1, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), //@sic R5s2211094 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); //@siclog "Step 4" siclog@