3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250176

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3669 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC UAC test case 11.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per Table 11. 3.6.3.3-3B, the service type in the SERVICE REQUEST message in the preamble should be“0101” but TTCN sets it as “0000” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the service type as “0101” instead of “0000” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_11_3_6_NR5GC() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | As per Table 11. 3.6.3.3-3B, the service type in the SERVICE REQUEST message in the preamble should be“0101” but TTCN sets it as “0000” |
| Summary of change | Corrected the service type as “0101” instead of “0000” |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_3_6_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MCS indicator / HPLMN/0%/100% accessibility AC7 / RRC_INACTIVE var NR_CellId_Type v_NR_CellId := nr_Cell1; // @sic R5s210669 sic@ var integer v_DRBId; //@sic R5-244664 sic@ var template (omit) NR_DRBInfo_Type v_DRBInfo; //@sic R5-244664 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@sic R5-244664 sic@ f_NR5GC_Init(NR_1); /***************** Skipping the code *************************************/ //The UE is in state 1N-A on NR Cell 1(serving cell) with at least one PDU session for Internet active according to TS 38.508-1 [4] Table 4.4A.2-1 and using the message condition UE TEST LOOP MODE B prepared according to TS 38.508-1 [4] f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeB_ON, -, -, -, '0101'B); // @sic R5-216194, R5s211714, R5s240122 sic@ //@sic R5-244664 sic@ //Check if internet PDU session established in preamble v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0000'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (nr_Cell1, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs(nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))}); f_NR5GC_PDUSessionEstablishment (nr_Cell1, 1); f_NR_RRCRelease(nr_Cell1); } f_NR_TestBody_Set(true); /***************** Skipping the code *************************************/ }

After Change:

function f_TC_11_3_6_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MCS indicator / HPLMN/0%/100% accessibility AC7 / RRC_INACTIVE var NR_CellId_Type v_NR_CellId := nr_Cell1; // @sic R5s210669 sic@ var integer v_DRBId; //@sic R5-244664 sic@ var template (omit) NR_DRBInfo_Type v_DRBInfo; //@sic R5-244664 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@sic R5-244664 sic@ f_NR5GC_Init(NR_1); /***************** Skipping the code *************************************/ //The UE is in state 1N-A on NR Cell 1(serving cell) with at least one PDU session for Internet active according to TS 38.508-1 [4] Table 4.4A.2-1 and using the message condition UE TEST LOOP MODE B prepared according to TS 38.508-1 [4] f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeB_ON, -, -, -, '0101'B); // @sic R5-216194, R5s211714, R5s240122 sic@ //@sic R5-244664 sic@ //Check if internet PDU session established in preamble v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0101'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (nr_Cell1, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs(nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))}); f_NR5GC_PDUSessionEstablishment (nr_Cell1, 1); f_NR_RRCRelease(nr_Cell1); } f_NR_TestBody_Set(true); /***************** Skipping the code *************************************/ }