3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250189

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3678 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 10.1.3.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 10.1.3.3 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 10.1.3.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes 3 – 9 of R5s250098 are required in addition to those mentioned below. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 10.1.3.3	4

2.1	Change 1	4

2.2	Change 2	11

2.3	Change 3	14

2.4	Change 4	27

3	Branches executed	31

4	Execution Log Files	31

4.1	Qualcomm	31

5	References	31

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 10.1.3.3 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Contact:	Kalahasti Narendra

Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	10.1.3.3.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform, Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# Corrections required to test case 10.1.3.3

## Change 1

| Function name | fl_TC_10_1_3_3_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation : - The variable v_PDN_Address2 is initialized based on PDN_5. This would also mean that in the testcase body, PDN_5 would then need to be started. The impact of this is that a totally new set of IP addresses will be deployed, which is unnecessary. - Function f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); is called after the PDU Session Release procedure. After this call, it is also needed to close the TCP connection on the IMS PTC. This is missing - The current TTCN implementation for build up of v_QosFlowID changes the QFI from 25. This is not specified in Prose, and this will also result in change of SDAP configuration which is not needed for any of the TPs. This change of QFI is not needed. Also, the TTCN call to start PDN_5 after this is not needed, because this will be handled later in the testcase when function f_NR5GC_PDUSessionEstablishment_InnerLoop is called at Step 3. - At Step 2, a timer of 1s is started as per current TTCN implementation. Expectation is that the UE should be able to transmit the PDU Session Establishment REQ within 1s. Some UEs take slightly longer than 1s for this to happen. In such a scenario, the SS triggers the PDU Session Establishment via an AT command, and by the time this has been issued to the UE, the UE has already sent a PDU Session Establishment REQ. So this results in the UE sending a “second” PDU Establishment REQ, which is then unhandled and causes TC failure. To avoid this, it is proposed that this timer be extended to 5s instead. However, there are some UEs that are very fast, and transmit the PDU Session Establishment REQ immediately after transmitting PDU Session Release complete. The current TTCN fails such UEs as well, and this needs to be handled. - In the current TTCN, the function f_NR5GC_PDUSessionEstablishment_InnerLoop is called to implement Step 3. However, in this function f_NR5GC_PDUSessionEstablishment_InnerLoop f_SendPDUSessionAcceptOverReconfig is called, where OMIT is sent as a parameter for IE ExtdProtocolConfigOptions. This would mean that the new P-CSCF addresses wont be sent. This needs to be addressed. |
| Summary of change | - Instead of using PDN_5 to initialize v_PDN_Address2, it is proposed to reuse the PDN_1 addresses, and only update the v_PDN_Address2. PCSCF_IPAddressIPv4 and v_PDN_Address2. PCSCF_IPAddressIPv6 but assigning them values from px_IPv4_Address_Other and px_IPv6_Address_Other respectively. These PICS are already used in IMS TCs 6.2 and 6.3. - Sent a coordination message to IMS PTC to tell the IMS PTC to close the TCP connection. And then waited for a coordination message from IMS PTC to get a confirm that TCP connection is closed. - Removed the TTCN that changed the QFI from 52 while build up of v_QosFlowId, and also removed the call to function f_IP_Handling_StartPDN that starts PDN_5 - Increased the wait timer at Step 2 from 1s 8s. A Prose CR will be raised for this.Also declared a default variable v_MyDefaultVar and activated it to capture any PDU Session Establishment REQ for a “fast” UE. Deactivated this default handler just before the start of timer at Step 2. Please see below. - It is proposed to pass another parameter p_CSCFRestoration (type boolean) to function f_NR5GC_PDUSessionEstablishment_InnerLoop, and based on this the parameter for IE ExtdProtocolConfigOptions is passed to f_SendPDUSessionAcceptOverReconfigfunction |
| TTCN module | GSM_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_10_1_3_3_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var O1_Type v_PDUSessionIdToRelease; var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); var PDN_AddressInfo_Type v_PDN_Address3 := f_PDN_AddressInfo_Get(PDN_6); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfoList; var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var octetstring v_Contents; var integer i; var integer v_Count:=0; var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) NAS_ExtdProtocolConfigOptions_Type v_ExtdPCO; var template (omit)S_NSSAI_Type v_RequestedNSSAI:= omit; var template (omit) DNN v_RequestedDNN:= omit; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO_IE := omit; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS; timer t_Wait; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN); v_GSM_MobilityInfo := valueof(v_GSM_MobilityInfo_IMS); v_PDUSessionIdToRelease := v_GSM_MobilityInfo.SessionId; v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); //@siclog "Steps 1" siclog@ //The generic test procedure in TS 38.508-1[4] clause 4.9.21 for PDU Session Release is performed with PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release IMS PDU session. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause Reactivation Request // Remove PDN_1 from the routing table then add PDN_5 f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); // Build up QoSFlowId for PDN_5 v_QosFlowId := cs_IP_QosFlowId(oct2int(v_PDUSessionIdToRelease), 5); f_IP_Handling_StartPDN(IP, PDN_5, cs_DrbInfo_NR(v_NGC_NASCellA, v_QosFlowId)); for (i := 0; i < lengthof(v_GSM_MobilityInfo.PCO.pco); i := i +1) { if (v_GSM_MobilityInfo.PCO.pco[i].protocolID == '0001'O) { // IPv6 v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count] := cs_ProtocolContainer('0001'O, v_Contents); v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('0001'O, v_Contents); v_Count := v_Count + 2; } else if(v_GSM_MobilityInfo.PCO.pco[i].protocolID == '000C'O) { // IPv4 v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count] := cs_ProtocolContainer('000C'O, v_Contents); v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('000C'O, v_Contents); v_Count := v_Count + 2; } } // Convert into a sending template in TLV format v_ExtdPCO_IE := f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ExtdPCO); //@siclog "Steps 2" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 1). --> 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST 1 P t_Wait.start (1.0); alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { f_UT_RequestPDUSession(UT, v_NoOfSessions +1); SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } //@siclog "Steps 3" siclog@ //The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message containing new P-CSCF addresses. f_NR5GC_PDUSessionEstablishment_InnerLoop(v_NGC_NASCellA, v_ReceivedAsp, -, omit, v_RequestedNSSAI, v_RequestedDNN, -, -, -, -, -, -, -, -, -, -, -, -, -, -,-,v_ExtdPCO_IE); //@siclog "Steps 4-11" siclog@ //Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? //(Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registration done"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 4-11"); //@siclog "Step 12" siclog@ //SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA); }

After Change:

function fl_TC_10_1_3_3_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var O1_Type v_PDUSessionIdToRelease; //WA#WI1382276 WA#10_1_3_3 COMMENTED OUT var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_1); //WA#WI1382276 WA#10_1_3_3 var PDN_AddressInfo_Type v_PDN_Address3 := f_PDN_AddressInfo_Get(PDN_6); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfoList; var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var octetstring v_Contents; var integer i; var integer v_Count:=0; var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); var default v_MyDefaultVar := null; //WA#WI1382276 WA#10_1_3_3 var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) NAS_ExtdProtocolConfigOptions_Type v_ExtdPCO; var template (omit)S_NSSAI_Type v_RequestedNSSAI:= omit; var template (omit) DNN v_RequestedDNN:= omit; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO_IE := omit; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS; timer t_Wait; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN); v_GSM_MobilityInfo := valueof(v_GSM_MobilityInfo_IMS); v_PDUSessionIdToRelease := v_GSM_MobilityInfo.SessionId; v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); //@siclog "Steps 1" siclog@ //The generic test procedure in TS 38.508-1[4] clause 4.9.21 for PDU Session Release is performed with PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release IMS PDU session. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause Reactivation Request v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(v_NGC_NASCellA)); //WA#WI1382276 WA#10_1_3_3 // Remove PDN_1 from the routing table then add PDN_5 f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //WA#WI1382276 WA#10_1_3_3 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Closed TCP connection"); //WA#WI1382276 WA#10_1_3_3 // Build up QoSFlowId for PDN_5 //WA#WI1382276 WA#10_1_3_3 COMMENTED OUT v_QosFlowId := cs_IP_QosFlowId(oct2int(v_PDUSessionIdToRelease), 5); //WA#WI1382276 WA#10_1_3_3 COMMENTED OUT f_IP_Handling_StartPDN(IP, PDN_5, cs_DrbInfo_NR(v_NGC_NASCellA, v_QosFlowId)); v_PDN_Address2.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; //WA#WI1382276 WA#10_1_3_3 v_PDN_Address2.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; //WA#WI1382276 WA#10_1_3_3 for (i := 0; i < lengthof(v_GSM_MobilityInfo.PCO.pco); i := i +1) { if (v_GSM_MobilityInfo.PCO.pco[i].protocolID == '0001'O) { // IPv6 v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count] := cs_ProtocolContainer('0001'O, v_Contents); v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('0001'O, v_Contents); v_Count := v_Count + 2; } else if(v_GSM_MobilityInfo.PCO.pco[i].protocolID == '000C'O) { // IPv4 v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count] := cs_ProtocolContainer('000C'O, v_Contents); v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('000C'O, v_Contents); v_Count := v_Count + 2; } } // Convert into a sending template in TLV format v_ExtdPCO_IE := f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ExtdPCO); deactivate(v_MyDefaultVar); //WA#WI1382276 WA#10_1_3_3 //@siclog "Steps 2" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 1). --> 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST 1 P t_Wait.start (8.0); //WA#WI1382276 WA#10_1_3_3 1-->8 alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { f_UT_RequestPDUSession(UT, v_NoOfSessions +1); SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } //@siclog "Steps 3" siclog@ //The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message containing new P-CSCF addresses. f_NR5GC_PDUSessionEstablishment_InnerLoop(v_NGC_NASCellA, v_ReceivedAsp, -, omit, v_RequestedNSSAI, v_RequestedDNN, -, -, -, -, -, -, -, -, -, -, -, -, -, -,-,v_ExtdPCO_IE, -,-,-,-,-, -,-,-,-,-, -,true); //WA#WI1382276 WA#10_1_3_3 //@siclog "Steps 4-11" siclog@ //Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? //(Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registration done"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 4-11"); //@siclog "Step 12" siclog@ //SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA); }

## Change 2

| Function name | f_TC_10_1_3_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In the current TTCN implementation : - The variable v_PDN_Address2 is initialized based on PDN_5. This would also mean that in the testcase body, PDN_5 would then need to be started. The impact of this is that a totally new set of IP addresses will be deployed, which is unnecessary. - There is no call to initialize PDN_1 on IMS PTC. This needs to be implemented. Also after the initial IMS registration, on the NR PTC the PDN_1 is stopped and after that coordination message is sent by NR PTC for closing of TCP connection. This also needs to be accommodated. Please see Change 1 above, point 2. - After initial IMS registration, the IMS PTC needs to be reset for the nex IMS registration (Steps 4-11). This needs to be implemented. - The current TTCN implementation calls function f_IMS_Register_NonGIBA to handle the IMS re-registration after P-CSCF restoration. It is proposed to call function f_IMS_Register instead, which handles the GIBA and nonGIBA cases both. |
| Summary of change | - Instead of using PDN_5 to initialize v_PDN_Address2, it is proposed to reuse the PDN_1 addresses, and only update the v_PDN_Address2. PCSCF_IPAddressIPv4 and v_PDN_Address2. PCSCF_IPAddressIPv6 but assigning them values from px_IPv4_Address_Other and px_IPv6_Address_Other respectively. These PICS are already used in IMS TCs 6.2 and 6.3. - Waited for a coordination message from NR PTC to close TCP connection. Then called the funciton f_IMS_TCP_CloseHandling, and then called function f_IMS_IPCAN_SendCoOrdMsgto inform NR PTC that TCP connection is closed. - Called function f_IMS_GlobalRelease(); Please see below. - Instead of calling function f_IMS_Register_NonGIBA, called function f_IMS_Register. |
| TTCN module | GSM_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; var IP_AddrInfo_Type v_NW_Address; var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); //v_IMS_DATA_REQ := f_IMS_Register(p_IpcanType, p_IMS_DATA_REQ, p_SigCompIsActive, p_RegisterExpiration); // Now check that that the NW address of the REGISTER's RoutingInfo contains the newly allocated P-CSCF address v_NW_Address := v_IMS_DATA_REQ.RoutingInfo.NW_Address; /* at least one address check must succeed ... */ if ((ischosen(v_NW_Address.V4) and not (f_IPAddresses_Compare(v_NW_Address.V4.Addr, v_PDN_Address2.PCSCF_IPAddressIPv4))) or (ischosen(v_NW_Address.V6) and not (f_IPAddresses_Compare(v_NW_Address.V6.Addr, v_PDN_Address2.PCSCF_IPAddressIPv6)))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "P-CSCF address check failed"); } f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true); // Inform NR: IMS registration done f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registration done")); }

After Change:

import from IMS_Procedures_Common all; //WA#10_1_2_3 WA#WI1371153 import from Parameters all; //WA#10_1_2_3 WA#WI1371153 . . . function f_TC_10_1_3_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session release / reactivation requested / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; var IP_AddrInfo_Type v_NW_Address; //WA#WI1382276 WA#10_1_3_3 COMMENTED OUT var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_1); //WA#WI1382276 WA#10_1_3_3 f_IMS_PTC_Init(PDN_1); //WA#WI1382276 WA#10_1_3_3 f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //WA#WI1382276 WA#10_1_3_3 f_IMS_TCP_CloseHandling(waitAndClose); //WA#WI1382276 WA#10_1_3_3 f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Closed TCP connection")); //WA#WI1382276 WA#10_1_3_3 f_IMS_GlobalRelease();//WA#WI1382276 WA#10_1_3_3 //RouteBody fix //WA#WI1382276 WA#10_1_3_3 COMMENTED OUT v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); v_IMS_DATA_REQ := f_IMS_Register(); //WA#WI1382276 WA#10_1_3_3 v_PDN_Address2.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; //WA#WI1382276 WA#10_1_3_3 v_PDN_Address2.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; //WA#WI1382276 WA#10_1_3_3 // Now check that that the NW address of the REGISTER's RoutingInfo contains the newly allocated P-CSCF address v_NW_Address := v_IMS_DATA_REQ.RoutingInfo.NW_Address; /* at least one address check must succeed ... */ if ((ischosen(v_NW_Address.V4) and not (f_IPAddresses_Compare(v_NW_Address.V4.Addr, v_PDN_Address2.PCSCF_IPAddressIPv4))) or (ischosen(v_NW_Address.V6) and not (f_IPAddresses_Compare(v_NW_Address.V6.Addr, v_PDN_Address2.PCSCF_IPAddressIPv6)))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "P-CSCF address check failed"); } f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true); // Inform NR: IMS registration done f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registration done")); }

## Change 3

| Function name | f_NR5GC_PDUSessionEstablishment_InnerLoop |
| --- | --- |
| Reason for change | - Please refer to Change 1, point 5 - Please refer to Change 1, point 3 |
| Summary of change | - Updated the implementation of function f_NR5GC_PDUSessionEstablishment_InnerLoop and passed a parameter p_CSCFRestoration to it. Set this value to False by default. Also created a new variable v_ExtdProtocolConfigOptions, and assigned v_ExtdProtocolConfigOptions := p_ExtdPCO; based on parameter p_CSCFRestoration. As a result of this, replaced p_ExtdPCO with – in the call to funciton f_Check_NG_PDUSessionEstablishmentReq. Lastly, instead of passing OMIT for the IE ExtdProtocolConfigOptions in function f_SendPDUSessionAcceptOverReconfig, passed v_ExtdProtocolConfigOptions instead. Please see below. - Before calling the function f_NR5GC_PDUSessionEstablishment_InnerLoop, in the function fl_TC_10_1_3_3_Body, the PDN_1 is stopped. In the function f_NR5GC_PDUSessionEstablishment_InnerLoop, PDN_1 is then started again. This would mean that the new IP addresses wont be deployed. Hence it is proposed to initialize PDN_1a based on flag p_CSCFRestoration = TRUE. Please see R5s250098 Changes 3-9 for details regarding PDN_1a. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, // set to required value if a specific QoSFlow must be built (with interworkingWithEPS parameters) template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // Only included when called from S1ToN1 TAU boolean p_OmitDNN := false // @sic R5-223987 sic@ - Only set to true if the DNN is to be omitted from Accept message ) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@ var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@ var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SSConfig; var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var SdapConfig_Type v_SdapConfig; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) SDAP_Config v_SDAP_Config; var PDN_Index_Type v_PDN_Index; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@ var template (omit) DNN v_DNN; var PDU_PDN_DNN_Type v_PDUType := None; // @sic R5s210148 sic@ var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@ var boolean v_ExistingPDN := false; // @sic R5s210149, R5s221126 sic@ var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit; var template (omit) EthernetHeaderCompressConfig v_EthernetHeaderCompressConfig := omit; var integer i; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, p_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId, p_PTI, p_IntegrityProtMaxDataRate, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_ExpectedS_NSSAI, p_ExpectedDNN, p_AdditionalInfo)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } if (not isvalue(p_DNN) and not p_OmitDNN ) { // @sic R5s210148, R5s221229 sic@ // Use the details received in the Request msg v_PDUType := v_GSM_MobilityInfo.PDUType; // @sic R5-205939, R5s240708 sic@ if (v_PDUType == None) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN"); } else { v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); // @sic R5s240641 sic@ } } else { // need to overwrite DNN received in Request msg @sic R5s221229 sic@ if (isvalue(p_DNN)) { v_DNN := p_DNN; v_GSM_MobilityInfo.DNN := valueof(p_DNN); } if (p_OmitDNN) { // @sic R5-223987 sic@ v_DNN := omit; v_GSM_MobilityInfo.DNN := omit; } // If using a different DNN, need to update stored PDU Type too v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN); if (v_PDUType == None) { // if this DNN isn't known, just use default v_PDUType := Internet_DNN; } v_GSM_MobilityInfo.PDUType := v_PDUType; } if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@ // Ethernet PDU session if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address"); } if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) { v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header"); } } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case(IMS_DNN) { // @sic R5-206296, R5s210518 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); } case(Emergency_PDN) { // @sic R5-206296, R5s210518, R5s50092 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486, R5s240641 sic@ } case (URLLC_DNN) { v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@ } case else { v_SDAP_Config := cs_SDAP_Config; } } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); //It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]); v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; // @sic R5s210480 sic@ // @sic R5-204400 sic@ if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_MediaType)); } } if (isvalue(p_PDU_SessionType)) { // @sic R5-227473 sic@ if (match ('101'B, p_PDU_SessionType.typeValue)) { v_ExistingPDN := true; // just set the flag to avoid updating the routing table } } // If this is a default DRB, enable IP handling for this DRB if (valueof(v_SDAP_Config.defaultDRB)) { if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@ // The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) { if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) { // Data from EUTRA should be used, rather than created anew v_ExistingPDN := f_NR_ChangeDrbMappingForExistingPDNs(p_NR_CellId, valueof(p_PduInfoFromLTE[i]), v_QosFlowId.QFI); // @sic R5s221126 sic@ v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduInfoFromLTE[i].EPS_Bearer); // Overwrite with stored value break; } } } if (not v_ExistingPDN) { // completely new PDU session @sic R5s210149, R5s221126 sic@ f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@ /* @sic R5-217449, R5s220617 sic@ */ } } if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@ //Perform PDU Session Authentication if required v_SecurityParams := f_NR5GC_Security_Get(); if (p_SRB == SRB1_2) { // @sic R5s201412 sic@ v_SRB := tsc_NR_RbId_SRB2; } f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@ } if (f_NR5GC_MobileInfo_GetNumOfIPAddressesToAllocate() != 1) { // @sic R5-217830 sic@ // According to NOTE 4 in PIXIT definition, this is currently only used for the 2nd PCSCF address of PDN_1 v_ExtraPDN_Address := f_PDN_AddressInfo_Get(PDN_1); v_ExtraPDN_Address.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; v_ExtraPDN_Address.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; } // Send Accept // Created common function for sending Accept message @sic R5-241606 sic@ f_SendPDUSessionAcceptOverReconfig (p_NR_CellId, v_GSM_MobilityInfo, v_DNN, // @sic R5-205939 sic@ v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, p_SSC_Mode, p_Session_AMBR, p_PDU_SessionType, p_Cause, p_RQTimer, p_S_NSSAI, -, p_EAP, p_QoS_Rules, p_QoSFlowDescr, omit, p_BackOff, p_BuildQosFlow, -, -, -, -, -, -, v_EthernetHeaderCompressConfig, // @sic R5-227473 sic@ v_ExtraPDN_Address); //@sic R5-213400 sic@ return v_PDUType; // @sic R5-206296 sic@ }

After Change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, // set to required value if a specific QoSFlow must be built (with interworkingWithEPS parameters) template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // Only included when called from S1ToN1 TAU boolean p_OmitDNN := false, // @sic R5-223987 sic@ - Only set to true if the DNN is to be omitted from Accept message boolean p_CSCFRestoration := false //WA#WI1382276 WA#10_1_3_3 ) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@ var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@ var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SSConfig; var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var SdapConfig_Type v_SdapConfig; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) SDAP_Config v_SDAP_Config; var PDN_Index_Type v_PDN_Index; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@ var template (omit) DNN v_DNN; var PDU_PDN_DNN_Type v_PDUType := None; // @sic R5s210148 sic@ var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@ var boolean v_ExistingPDN := false; // @sic R5s210149, R5s221126 sic@ var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit; var template (omit) EthernetHeaderCompressConfig v_EthernetHeaderCompressConfig := omit; var template (omit) ExtdProtocolConfigOptions v_ExtdProtocolConfigOptions := omit; //WA#WI1382276 WA#10_1_3_3 var integer i; if (ispresent(p_ExtdPCO)) { //WA#WI1382276 WA#10_1_3_3 v_ExtdProtocolConfigOptions := p_ExtdPCO; } if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, p_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId, p_PTI, p_IntegrityProtMaxDataRate, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_AlwaysOnPDUSessionReq, p_PDUReq, -, //WA#WI1382276 WA#10_1_3_3 p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_ExpectedS_NSSAI, p_ExpectedDNN, p_AdditionalInfo)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } if (not isvalue(p_DNN) and not p_OmitDNN ) { // @sic R5s210148, R5s221229 sic@ // Use the details received in the Request msg v_PDUType := v_GSM_MobilityInfo.PDUType; // @sic R5-205939, R5s240708 sic@ if (v_PDUType == None) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN"); } else { if (v_PDUType == Emergency_PDN) { v_MediaType := IPCAN_EmergencyCall; //WA#WI1396015 v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(IMS_DNN))); } else { v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); // @sic R5s240641 sic@ } } } else { // need to overwrite DNN received in Request msg @sic R5s221229 sic@ if (isvalue(p_DNN)) { v_DNN := p_DNN; v_GSM_MobilityInfo.DNN := valueof(p_DNN); } if (p_OmitDNN) { // @sic R5-223987 sic@ v_DNN := omit; v_GSM_MobilityInfo.DNN := omit; } // If using a different DNN, need to update stored PDU Type too if (v_GSM_MobilityInfo.PDUType == Emergency_PDN) { //WA#WI1396015 v_MediaType := IPCAN_EmergencyCall; } else { v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN); if (v_PDUType == None) { // if this DNN isn't known, just use default v_PDUType := Internet_DNN; } v_GSM_MobilityInfo.PDUType := v_PDUType; } } if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@ // Ethernet PDU session if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address"); } if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) { v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header"); } } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case(IMS_DNN, Emergency_PDN) { // @sic R5-206296, R5s210518 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); //WA#WI1396015 MOVED UP and assigned based on v_PDUType v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486, R5s240641 sic@ } case (URLLC_DNN) { v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@ } case else { v_SDAP_Config := cs_SDAP_Config; } } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); //It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]); v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo); if (p_CSCFRestoration) { //WA#WI1382276 WA#10_1_3_3 v_PDN_Index := PDN_1a; } v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; // @sic R5s210480 sic@ // @sic R5-204400 sic@ if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@); if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_MediaType)); } } if (isvalue(p_PDU_SessionType)) { // @sic R5-227473 sic@ if (match ('101'B, p_PDU_SessionType.typeValue)) { v_ExistingPDN := true; // just set the flag to avoid updating the routing table } } // If this is a default DRB, enable IP handling for this DRB if (valueof(v_SDAP_Config.defaultDRB)) { if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@ // The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) { if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) { // Data from EUTRA should be used, rather than created anew v_ExistingPDN := f_NR_ChangeDrbMappingForExistingPDNs(p_NR_CellId, valueof(p_PduInfoFromLTE[i]), v_QosFlowId.QFI); // @sic R5s221126 sic@ v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduInfoFromLTE[i].EPS_Bearer); // Overwrite with stored value break; } } } if (not v_ExistingPDN) { // completely new PDU session @sic R5s210149, R5s221126 sic@ f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@ /* @sic R5-217449, R5s220617 sic@ */ } } if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@ //Perform PDU Session Authentication if required v_SecurityParams := f_NR5GC_Security_Get(); if (p_SRB == SRB1_2) { // @sic R5s201412 sic@ v_SRB := tsc_NR_RbId_SRB2; } f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@ } if (f_NR5GC_MobileInfo_GetNumOfIPAddressesToAllocate() != 1) { // @sic R5-217830 sic@ // According to NOTE 4 in PIXIT definition, this is currently only used for the 2nd PCSCF address of PDN_1 v_ExtraPDN_Address := f_PDN_AddressInfo_Get(PDN_1); v_ExtraPDN_Address.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; v_ExtraPDN_Address.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; } // Send Accept // Created common function for sending Accept message @sic R5-241606 sic@ f_SendPDUSessionAcceptOverReconfig (p_NR_CellId, v_GSM_MobilityInfo, v_DNN, // @sic R5-205939 sic@ v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, p_SSC_Mode, p_Session_AMBR, p_PDU_SessionType, p_Cause, p_RQTimer, p_S_NSSAI, -, p_EAP, p_QoS_Rules, p_QoSFlowDescr, v_ExtdProtocolConfigOptions, //WA#WI1382276 WA#10_1_3_3 omit, p_BackOff, p_BuildQosFlow, -, -, -, -, -, -, v_EthernetHeaderCompressConfig, // @sic R5-227473 sic@ v_ExtraPDN_Address); //@sic R5-213400 sic@ return v_PDUType; // @sic R5-206296 sic@ }

## Change 4

| Function name | f_GetNextEPSBearerId |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function in case v_NoOfIds > 1, then the TTCN allocates an EPS Bearer ID that is already allocated. This causes unexpected UE behaviour. For example, while running 10.1.3.3.NR5GC in MultiPDU mode 2 PDU Sessions are established in preamble : 1 for IMS, and 1 for Internet. When IMS PDU Session is released with reactivation requested, the UE tries to establish the IMS PDU Session again. In this case, the parameter v_ExistingIds contains {5,11}, where EPS Bearer 11 is assigned for Internet PDU Session. But while sending the PDU Session Establishment Accept for the re-established IMS PDU Session, the EPS Bearer ID 11 is allocated again. This is incorrect because that is already assigned to Internet PDU Session established earlier. This causes enexpected E behaviour, and fails the testcase 10.1.3.3 (In MultiPDU mode). |
| Summary of change | Updated the TTCN implementation to check if the Bearer ID is already configured, and based on that the next available EPS Bearer ID is allocated. Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetNextEPSBearerId (GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo, template (omit) IMS_TestConfiguration_Type p_MediaType := omit) runs on NR5GC_PTC return template (omit) integer { var template (omit) integer v_BearerId := omit; var IntegerList_Type v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); var integer v_NoOfIds := lengthof(v_ExistingIds); var integer i; // Re-written to hardcode values to allow for possible repeated media calls // - this assumes not more than 1 type of media call at any one time, and not more than 6 'other' PDN types @sic R5s211486 sic@ if (isvalue(p_GMM_MobilityInfo.GMMCap)) { if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { if (isvalue(p_MediaType)) { // @sic R5s211486 sic@ // check to see if we can hardcode the dedicated bearer first select (valueof(p_MediaType)) { case (IPCAN_SpeechCall) { v_BearerId := 7; } case (IPCAN_VideoCall) { v_BearerId := 8; } case (IPCAN_EmergencyCall) { v_BearerId := 9; } } // now allocate id according to default bearer type } else if (p_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-205939 sic@ v_BearerId := 5; for (i:= 0; i < v_NoOfIds; i :=i+1) { if (match (v_ExistingIds[i], 5)) { // IMS Bearer id = 5 v_BearerId := v_NoOfIds + (10-1); // allocate the next one, from 10 onwards @sic R5s211486 sic@ break; } } } else if (p_GSM_MobilityInfo.PDUType == Emergency_PDN) { // @sic R5s211486 sic@ v_BearerId := 6; } else { // Not IMS or emergency v_BearerId := v_NoOfIds + 10; // @sic R5s211486 sic@ } // Add the new id to the end of the list and store it v_ExistingIds[v_NoOfIds] := valueof(v_BearerId); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } } return v_BearerId; }

After Change:

function f_GetNextEPSBearerId (GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo, template (omit) IMS_TestConfiguration_Type p_MediaType := omit) runs on NR5GC_PTC return template (omit) integer { var template (omit) integer v_BearerId := omit; var IntegerList_Type v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); var integer v_NoOfIds := lengthof(v_ExistingIds); var integer i; // Re-written to hardcode values to allow for possible repeated media calls // - this assumes not more than 1 type of media call at any one time, and not more than 6 'other' PDN types @sic R5s211486 sic@ if (isvalue(p_GMM_MobilityInfo.GMMCap)) { if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { if (isvalue(p_MediaType)) { // @sic R5s211486 sic@ // check to see if we can hardcode the dedicated bearer first select (valueof(p_MediaType)) { case (IPCAN_SpeechCall) { v_BearerId := 7; } case (IPCAN_VideoCall) { v_BearerId := 8; } case (IPCAN_EmergencyCall) { v_BearerId := 9; } } // now allocate id according to default bearer type } else if (p_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-205939 sic@ v_BearerId := 5; for (i:= 0; i < v_NoOfIds; i :=i+1) { if (match (v_ExistingIds[i], 5)) { // IMS Bearer id = 5 v_BearerId := v_NoOfIds + (10-1); // allocate the next one, from 10 onwards @sic R5s211486 sic@ break; } } /*This additional loop is needed to ensure that in case v_NoOfIds > 1, then the TTCN does not allocate an EPS Bearer ID that is already allocated. For example, in case 2 PDU Sessions are established in preamble : 1 for IMS, and 1 for Internet and if IMS PDU Session is released with reactivation requested In such a scenario the IMS PDU session that is established again should not get an EPS Bearer allocated that is already assigned to Internet PDU Session established earlier. */ for (i:= 0; i < v_NoOfIds; i :=i+1) { //WA#WI1382276 WA#10_1_3_3 if (match (v_ExistingIds[i], v_BearerId)) { v_BearerId := v_NoOfIds + 10; // allocate the next one break; } } } else if (p_GSM_MobilityInfo.PDUType == Emergency_PDN) { // @sic R5s211486 sic@ v_BearerId := 6; } else { // Not IMS or emergency v_BearerId := v_NoOfIds + 10; // @sic R5s211486 sic@ } // Add the new id to the end of the list and store it v_ExistingIds[v_NoOfIds] := valueof(v_BearerId); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } } return v_BearerId; }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n2” using NIA1 Integrity and NEA3 ciphering algorithms on R&S 5G Conformance Test Platform.

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms on Anritsu Protocol Conformance Test System ME7834NR.

# Execution Log Files

## Qualcomm

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE  passed this test case on R&S® 5G Protocol Conformance Test platform and Anritsu Protocol Conformance Test System ME7834NR . The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_10_1_3_3_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

Anritsu :
tc_10_1_3_3_Logs.xml

tc_10_1_3_3_PIXIT.xml

# References

| [1] | R5s250190: Supporting information for addition of NR5GC testcase 10.1.3.3 in FR1 |
| --- | --- |