3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250196

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3681 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Before Step 14, the reception of the UL HARQ is enabled. However, this causes issues when the RLC ACK for the RRC Reconfiguration complete at Steps 12 – 13 is transmitted to the UE to which the UE responds with an HARQ ACK. This, then, gets sent to TTCN where it is absorbed by the default handler causing the TC to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the enabling of the reception of UL HARQ after the delay timer of 0.4s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_1_12_3_NR_Steps14_19 |
| --- | --- |
| Reason for change | Before Step 14, the reception of the UL HARQ is enabled. However, this causes issues when the RLC ACK for the RRC Reconfiguration complete at Steps 12 – 13 is transmitted to the UE to which the UE responds with an HARQ ACK. This, then, gets sent to TTCN where it is absorbed by the default handler causing the TC to fail. |
| Summary of change | Move the enabling of the reception of UL HARQ after the delay timer of 0.4s |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var integer v_SlotsInSubframe; var integer v_DRC_onDurationTimer; var PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var float v_PS_Offset_ms; timer t_Watchdog := 5.0; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@sic R5230001 sic@ //@siclog Step 14 siclog@ //Wait 400ms to ensure UE is out DRX active time. f_Delay(0.4); //@sic R5s230309, R5-233467 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var integer v_SlotsInSubframe; var integer v_DRC_onDurationTimer; var PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var float v_PS_Offset_ms; timer t_Watchdog := 5.0; //@siclog Step 14 siclog@ //Wait 400ms to ensure UE is out DRX active time. f_Delay(0.4); //@sic R5s230309, R5-233467 sic@ //WA#MOVED f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@sic R5230001 sic@ <<SKIPPED CODE>>