3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250238

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0973 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_ECall_USIM_Insert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Function f_ECall_USIM_Insert, will be called at the very start of the test case to configure USIM profile for eCall test cases execution. Current implementation will check through Global variable if test case is an LTE or NR one, problem is this check is done before global variables have been initialised causing a runtime error. This check can be done based on testcasename, using an already implemented function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called function to check if test case is LTE or 5G based on test case name. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.x and 11.x.NR5GC test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Rename of function fl_NR_TestcaseIs5GTestcase	3

2.2	Correction to fl_EUTRA_GetPlmnInfoList	4

2.3	Correction to f_ECall_USIM_Insert	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Rename of function fl_NR_TestcaseIs5GTestcase

| Function name | fl_NR_TestcaseIs5GTestcase |
| --- | --- |
| Reason for change | Function fl_NR_TestcaseIs5GTestcase will return a false or true depending if a test case is a LTE or a 5G test case based on test case name. Currently, this function, is only defined to be a local auxiliar function. Proposal will be to rename and move it to a common module so can be used for original purpose, but in addition, to reutilise code and help resolve existing issue in function f_ECall_USIM_Insert. |
| Summary of change | Renamed and moved function to common module to be used by f_ECall_USIM_Insert |
| TTCN module | EUTRA_SysInfo_Templates |
| MCC160 Comment |  |

Before Change:

function fl_NR_TestcaseIs5GTestcase() return boolean { var charstring v_TestcaseName := testcasename(); var template charstring v_TestcasePattern5G := pattern "*_NR5GC"; if (match(v_TestcaseName, v_TestcasePattern5G) or f_GetTestcaseAttrib_ENDC(v_TestcaseName)){ return true; } else { return false; } }

After Change:

function f_NR_TestcaseIs5GTestcase() return boolean { var charstring v_TestcaseName := testcasename(); var template charstring v_TestcasePattern5G := pattern "*_NR5GC"; if (match(v_TestcaseName, v_TestcasePattern5G) or f_GetTestcaseAttrib_ENDC(v_TestcaseName)){ return true; } else { return false; } }

## Correction to fl_EUTRA_GetPlmnInfoList

| Function name | fl_EUTRA_GetPlmnInfoList |
| --- | --- |
| Reason for change | Due to changes in proposed in 2.1, function call needs to be update. |
| Summary of change | Updated function called fl_NR_TestcaseIs5GTestcase to f_NR_TestcaseIs5GTestcase |
| TTCN module | EUTRA_SysInfo_Templates |
| MCC160 Comment |  |

Before Change:

function fl_EUTRA_GetPlmnInfoList() return template (omit) PLMN_InfoList_r15 { /* @sic R5-198995 sic@ */ var template (omit) PLMN_InfoList_r15 v_PLMN_InfoList := omit; if (fl_NR_TestcaseIs5GTestcase()) { v_PLMN_InfoList := cs_PLMN_InfoList_Def; } return v_PLMN_InfoList; }

After Change:

function fl_EUTRA_GetPlmnInfoList() return template (omit) PLMN_InfoList_r15 { /* @sic R5-198995 sic@ */ var template (omit) PLMN_InfoList_r15 v_PLMN_InfoList := omit; if (f_NR_TestcaseIs5GTestcase()) { v_PLMN_InfoList := cs_PLMN_InfoList_Def; } return v_PLMN_InfoList; }

## Correction to f_ECall_USIM_Insert

| Function name | f_ECall_USIM_Insert |
| --- | --- |
| Reason for change | Function f_ECall_USIM_Insert, will be called at the very start of the test case to configure USIM profile for eCall test cases execution. Current implementation will check through Global variable if test case is an LTE or NR one, problem is this check is done before global variables have been initialised causing a runtime error. This check can be done based on testcasename, using an already implemented function. |
| Summary of change | Called function to check tif test case is LTE or 5G based on test case name. |
| TTCN module | IMS_CC_TestcaseImplementation_eCall |
| MCC160 Comment |  |

Before Change:

function f_ECall_USIM_Insert(IMS_ECall_UEType p_UEType) runs on IMS_PTC { var charstring v_USIM_ToInsert; if (f_IMS_PTC_RanTypeIsNR5GC()){ select (p_UEType) { case (eCallCapableUE) { v_USIM_ToInsert := "38.508-1 Table 6.4.1-23 for eCall Capable UE"; } case (eCallOnlyUE) { v_USIM_ToInsert := "38.508-1 Table 6.4.1-24 for eCall-only UE"; } } } else { select (p_UEType) { case (eCallCapableUE) { v_USIM_ToInsert := "36.508 Table 4.9.3.5-1 for eCall Capable UE"; } case (eCallOnlyUE) { v_USIM_ToInsert := "36.508 Table 4.9.3.5-2 for eCall-only UE"; } } } f_UT_USIM_Insert(MMI, v_USIM_ToInsert); }

After Change:

function f_ECall_USIM_Insert(IMS_ECall_UEType p_UEType) runs on IMS_PTC { var charstring v_USIM_ToInsert; if (f_NR_TestcaseIs5GTestcase()){ select (p_UEType) { case (eCallCapableUE) { v_USIM_ToInsert := "38.508-1 Table 6.4.1-23 for eCall Capable UE"; } case (eCallOnlyUE) { v_USIM_ToInsert := "38.508-1 Table 6.4.1-24 for eCall-only UE"; } } } else { select (p_UEType) { case (eCallCapableUE) { v_USIM_ToInsert := "36.508 Table 4.9.3.5-1 for eCall Capable UE"; } case (eCallOnlyUE) { v_USIM_ToInsert := "36.508 Table 4.9.3.5-2 for eCall-only UE"; } } } f_UT_USIM_Insert(MMI, v_USIM_ToInsert); }