3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250242

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0974 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of f_ECall_CallSetup_Steps2_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the test case prose at establishment of a test eCall condition A6 shall not be applied for the 200 OK for INVITE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Parameter p_EmergencyType added to f_ECall_CallSetup_Steps2_3 so that NORMAL_CALL can be used for test eCall. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation, conformant may fail due to unexpected route set in the SIP ACK. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS_EUTRA: 21.24, 21.26 IMS_NR5GC: 11.9, 11.14 LTE_A_PRO: 11.3.2 NR5GC_IRAT: 11.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

| Function name | f_ECall_CallSetup_Steps2_3 |
| --- | --- |
| Reason for change | Call type needs to be handed over as parameter so that test eCall can be distinguished from other eCalls |
| TTCN module | IMS_eCall_Functions |

function f_ECall_CallSetup_Steps2_3(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, boolean p_AckRvd := true, IMS_CallType_Type p_EmergencyType := REGULAR_EMERGENCY) runs on IMS_PTC { // Split Invite into a separate function @sic R5-227567 sic@ var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SipUrl v_CalleeContactSipUri; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (value) AcceptBody_List v_AcceptBodyList; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) SDP_Message v_SDP_Message_Step2; var template (value) MessageBody v_MessageBody; var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; var charstring v_MSDContentIdRef; var charstring v_BoundaryStep2 := "boundary1"; var charstring v_ContentId := "psap@3gpp.org"; var charstring v_CidUrl_Step2 := "cid:" & v_ContentId; /* RFC 8147, RFC 2392 clause 2: cid-url = "cid" ":" content-id @sic R5s210849 change 2.1 sic@ */ v_InviteRequest := p_InviteRequestWithSdp.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(p_InviteRequestWithSdp.RoutingInfo); v_AudioCodecInfo := f_IMS_ECall_CheckAndGetAudioCodec(p_InviteRequestWithSdp.SdpOffer); // @sic R5-223392 sic@ v_MSDContentIdRef := f_IMS_MIME_MessageBody_GetContentIdAddrSpec(v_InviteRequest.messageBody); // Step 2. SS sends 200 OK with SDP contents v_SDP_Message_Step2 := f_IMS_BuildSDP_EmergencyCallAnswer_TX(v_AudioCodecInfo.Fmt, v_AudioCodecInfo.Codec); v_MessageHeader_InviteResponse_200 := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeMIME(v_BoundaryStep2), -, -, p_EmergencyType); v_AcceptBodyList := { cs_AcceptBody(c_sdpApplication), cs_AcceptBody(tsc_MimeType_Pidf), cs_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), cs_AcceptBody(tsc_MimeType_EmergencyCallDataMSD) }; v_MessageHeader_InviteResponse_200.accept := cs_Accept(v_AcceptBodyList); v_MessageHeader_InviteResponse_200.callInfo := cs_CallInfo(v_CidUrl_Step2, tsc_ECall_InfoPackageNameControl); v_MessageHeader_InviteResponse_200.recvInfo := cs_RecvInfo_Def(tsc_ECall_InfoPackageNameMSD); v_MessageBody := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep2, v_ContentId, cs_EmergencyCallData_Control_Ack(p_AckRvd, v_MSDContentIdRef),// @sic R5-251343, R5-251344, R5-251345 sic@ v_SDP_Message_Step2); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_InviteResponse_200, v_MessageBody))); // Step 3. UE sends ACK v_CalleeContactSipUri := f_MessageHeader_GetContactSipUrl(v_MessageHeader_InviteResponse_200); v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200, p_EmergencyType); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeContactSipUri, v_MessageHeader_Ack))); }

| Function name | f_IMS_ECallOnly_TestCall |
| --- | --- |
| Reason for change | In test eCall condition A6 shall not be applied for the 200 OK for INVITE. |
| TTCN module | IMS_eCall_Functions |

function f_IMS_ECallOnly_TestCall(ECall_MSD_Update_Type p_MSDUpdate := msdUpdateRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var SipUrl v_ContactUrl; var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var template (value) MessageHeader v_MessageHeader_Notify; var boolean v_WaitForSubscribe := true; var boolean v_WaitForInvite := true; var boolean v_WaitFor200OK := false; var default v_PublishHandler; v_IMS_REGISTER_REQ := f_IMS_Register(); // REGISTER, 401 Unauthorized, REGISTER, 200 OK for REGISTER v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); v_PublishHandler := activate(a_IMS_PublishHandling()); while (v_WaitForSubscribe or v_WaitForInvite or v_WaitFor200OK) { alt { [v_WaitForSubscribe] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // SUBSCRIBE, 200 OK for SUBSCRIBE v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); /* @sic R5s250153: v_MessageHeader_Notify shall not be assigned before SUBSCRIBE has been received sic@ */ IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), // NOTIFY cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_WaitForSubscribe := false; v_WaitFor200OK := true; } [v_WaitForInvite] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite(TEST, A_2_1_A4); // INVITE f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_WaitForInvite := false; } [v_WaitFor200OK] a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol) { // 200 OK for NOTIFY v_WaitFor200OK := false; } } } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // => ACTIVATE DEDICATED EPS BEARER f_IMS_IPCAN_WaitForTrigger(IPCAN); // ... bearer activated f_ECall_CallSetup_Steps2_3(v_InviteRequestWithSdp, -, NORMAL_CALL); if (p_MSDUpdate == msdUpdateRequired) { f_ECall_MSDUpdate(TEST); } v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); deactivate(v_PublishHandler); return v_InviteRequestWithSdp; }

| Function name | f_IMS_ECallCapable_TestCall_IMS1 |
| --- | --- |
| Reason for change | In test eCall condition A6 shall not be applied for the 200 OK for INVITE. |
| TTCN module | IMS_CC_TestcaseImplementation_eCall |

function f_IMS_ECallCapable_TestCall_IMS1(boolean p_AckRvd := false, boolean p_PSAPCallback := false) runs on IMS_PTC { var SipUrl v_ContactUrl; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_ECall_USIM_Insert(eCallCapableUE); f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_SpeechCall); // UE is triggered to initiate test eCall // @siclog "Step 1" siclog@ f_UT_InitiateECall(MMI, TEST); //@siclog "Steps 2-4 (21.24)" siclog@ //@siclog "Steps 2-13 (11.9.NR5GC)" siclog@ //@siclog "Step 14 (11.9.NR5GC)" siclog@ //@siclog "Step 15 (11.9.NR5GC)" siclog@ //@siclog "Step 16 (11.9.NR5GC)" siclog@ v_InviteRequestWithSdp := f_ECall_InitialInvite(TEST, A_2_1_A4); f_IMS_CC_TriggerDedicatedBearerActivation (IPCAN_MO_SpeechCall); f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, p_AckRvd, NORMAL_CALL); // @siclog "Step 5 (21.24)" siclog@ // @siclog "Step 17 (11.9.NR5GC)" siclog@ f_Delay(5.0); // @siclog "Step 6-9 (21.24)" siclog@ // @siclog "Step 18 (11.9.NR5GC)" siclog@ v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_CC_ReleaseConnection(IPCAN_MO_SpeechCall); if(p_PSAPCallback){ f_IMS_ECall_PSAPCallback(TEST, v_ContactUrl); f_IMS_CC_ReleaseConnection(IPCAN_MT_SpeechCall); } f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(); }