3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250260

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3711 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 7.1.1.10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 7.1.1.10.4 to the IWD_25wk24 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.10.4 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.10.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 7.1.1.10.4	4

2.1	Change 1	4

2.2	Change 2	11

3	Branches executed	13

4	Execution Log Files	13

4.1	Unisoc UMS9632	13

5	References	13

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.1.10.4 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.1.10.4.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

# Corrections required to test case 7.1.1.10.4

## Change 1

| Function name | f_TC_7_1_1_10_4_NR5GC() |
| --- | --- |
| Reason for change | - At step 28, 1s is too long to check RRCSetupRequest message with no Timing Advance Report MAC, it also has impact on RRC Setup procedure. - Because there is no default function to deal with the UE sends Timing Advance Report message in whole procedure, so disable System indication after checking at step4, to avoid impacts on normal message received.. - Enable system Indication of Timing Advance Report before checking at step 28. - The current TTCN implementation causes type mismatch problem by configure v_DL_CCCH_Message to the v_DL_DCCH_Message. - According to TC spec. from step24 to step 28, , no SRB2 is established, So in the postamle, TTCN expectes UE send Deregistration with SRB1. - At step 24 the period of BCCH modification for NTN GSO is about 500s, So the current Guard Timer (300s) is not suitable. |
| Summary of change | - Reduce the value of t_WaitTime from 1.0 to 0.1. - Disable System indication of Timing Advance Report. - At step 25 enable System indication of Timing Advance Report. - At step 29, invoke f_NR_38508_RRCSetup () directly. - In the postamble procedure, expect Deregistration with SRB1. - Extended the GuardTimer from 300s to 1000s. |
| TTCN module | MAC_NTN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_10_4_NR5GC() runs on NR5GC_PTC { // NTN / UE specific TA report / UE specific Koffset var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_Received_NR_SysInd; var template (value) DL_DCCH_Message v_DL_DCCH_Message; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var SubFrameTiming_Type v_Timing; var integer v_DifferentialKoffset; var template (value) FreqBandInformation v_FirstBand; var octetstring v_EncodeCapRequest; timer t_WaitTime := 1.0; //a general delay to check if TA Report MAC CE is received //Init NTN Cell parameters f_NR5GC_Init(NR_29); /*@sic R5-252350 sic@*/ //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, enabled); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); //@siclog "Step 1" siclog@ //Power on the UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 3" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 2. //Preconfigured at SS //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with Timing Advance Report MAC CE included? interleave { [] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND {} [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) -> value v_Received_NR_SysInd {} } if(v_SRB_COMMON_IND.Common.TimingInfo == v_Received_NR_SysInd.Common.TimingInfo) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@sic Steps 5-22 sic@ //Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); //@sic Step 23 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1, STATE_CONNECTED_3A); f_NR5GC_MobileInfo_Init(); //@sic Step 24 sic@ //SS broadcasts SIB19 with ta-Report-r17 not present. //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@sic Step 25 sic@ //Power on the UE. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 26" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 27" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 26. //Preconfigured at SS //@sic Step 28 sic@ //The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. alt { [] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)) { t_WaitTime.start; alt { [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28, no TA Report MAC CE is expected"); } [] t_WaitTime.timeout {}; } } [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28, no TA Report MAC CE is expected"); } } f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic Step 29 sic@ //The SS transmits an RRCSetup message configuring offsetThresholdTA. //Table 7.1.1.10.4.3.3-5: v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tar_Config_r17 := {setup := cs_TAR_Config_r17(ms15, enabled)}; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_DL_DCCH_Message := f_NR_38508_RRCSetup(nr_Cell1); SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, v_DL_DCCH_Message)); //@sic Step 30 sic@ //Check: Does the UE transmit an RRCSetupComplete message with Timing Advance Report MAC CE included? interleave { [] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),?)) -> value v_SRB_COMMON_IND {} [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) -> value v_Received_NR_SysInd {} } if(v_SRB_COMMON_IND.Common.TimingInfo == v_Received_NR_SysInd.Common.TimingInfo) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 30"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 30"); } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(disable)); //@sic Step 31-45 sic@ //Steps 5 to 19a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. v_DedicatedNAS_Message := v_SRB_COMMON_IND.Signalling.Nas[0]; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_DedicatedNAS_Message.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_DedicatedNAS_Message, v_PLMN, TESTMode_OFF); if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@sic Step 46 sic@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 20 ms. v_DifferentialKoffset := 20; v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_MAC_DifferentialKoffset_CE(int2bit(v_DifferentialKoffset,6)))); //Reconfigure SS with a new TA v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 2); // add 2ms f_NR5GC_SS_NTN_ApplyDifferentialKoffset(nr_Cell1,v_DifferentialKoffset, cs_TimingInfo_NR(v_Timing)) ; //@sic Step 47 sic@ //The SS transmits a UECapabilityEnquiry message v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)); v_EncodeCapRequest := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({v_FirstBand}))); v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequest); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, f_NR_CellInfo_GetNTN_UE_gNB_RTT(nr_Cell1)); // add RTT delay to allow UE to apply DifferentialKoffset SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_DL_DCCH_Message)); //@sic Step 48 sic@ //Check: Does the UE transmit a UECapabilityInformation message after applying differential Koffset of 20ms received at step 46? //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 48"); f_NR_TestBody_Set(false); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }; }

After Change:

function f_TC_7_1_1_10_4_NR5GC() runs on NR5GC_PTC { // NTN / UE specific TA report / UE specific Koffset var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_Received_NR_SysInd; var template (value) DL_DCCH_Message v_DL_DCCH_Message; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var SubFrameTiming_Type v_Timing; var integer v_DifferentialKoffset; var template (value) FreqBandInformation v_FirstBand; var octetstring v_EncodeCapRequest; timer t_WaitTime := 0.1; //WI=1309760//a general delay to check if TA Report MAC CE is received //Init NTN Cell parameters f_NR5GC_Init(NR_29); /*@sic R5-252350 sic@*/ //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, enabled); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); //@siclog "Step 1" siclog@ //Power on the UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 3" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 2. //Preconfigured at SS //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with Timing Advance Report MAC CE included? interleave { [] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND {} [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) -> value v_Received_NR_SysInd {} } if(v_SRB_COMMON_IND.Common.TimingInfo == v_Received_NR_SysInd.Common.TimingInfo) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(disable)); //WI=1309760 } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5-22" siclog@ //Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); //@siclog "Step 23" siclog@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. f_NR_UE_DeRegisterOnSwitchOff(nr_Cell1, STATE_CONNECTED_3A); f_NR5GC_MobileInfo_Init(); //@siclog "Step 24" siclog@ //SS broadcasts SIB19 with ta-Report-r17 not present. //Table 7.1.1.10.4.3.3-2: NTN-Config f_NR_CellInfo_SetSIB19_NTN_ta_Report_r17(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 25" siclog@ //Power on the UE. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(enable)); //WI=1309760 f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 26" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 27" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 26. //Preconfigured at SS //@siclog "Step 28" siclog@ //The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. alt { [] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)) { t_WaitTime.start; alt { [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28, no TA Report MAC CE is expected"); } [] t_WaitTime.timeout {}; } } [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28, no TA Report MAC CE is expected"); } } f_NR_ULGrantConfiguration_Start(nr_Cell1); //@siclog "Step 29" siclog@ //The SS transmits an RRCSetup message configuring offsetThresholdTA. //Table 7.1.1.10.4.3.3-5: v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tar_Config_r17 := {setup := cs_TAR_Config_r17(ms15, enabled)}; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, f_NR_38508_RRCSetup(nr_Cell1))); //WI=1309760 //@siclog "Step 30" siclog@ //Check: Does the UE transmit an RRCSetupComplete message with Timing Advance Report MAC CE included? interleave { [] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),?)) -> value v_SRB_COMMON_IND {} [] SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_MAC(cr_NR_MAC_TimingAdvanceReport_CE),?)) -> value v_Received_NR_SysInd {} } if(v_SRB_COMMON_IND.Common.TimingInfo == v_Received_NR_SysInd.Common.TimingInfo) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 30"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 30"); } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_UL_TA_Report(disable)); //@siclog "Step 31-45" siclog@ //Steps 5 to 19a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. v_DedicatedNAS_Message := v_SRB_COMMON_IND.Signalling.Nas[0]; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_DedicatedNAS_Message.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_DedicatedNAS_Message, v_PLMN, TESTMode_OFF); if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@siclog "Step 46" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 20 ms. v_DifferentialKoffset := 20; v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_MAC_DifferentialKoffset_CE(int2bit(v_DifferentialKoffset,6)))); //Reconfigure SS with a new TA v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 2); // add 2ms f_NR5GC_SS_NTN_ApplyDifferentialKoffset(nr_Cell1,v_DifferentialKoffset, cs_TimingInfo_NR(v_Timing)) ; //@siclog "Step 47" siclog@ //The SS transmits a UECapabilityEnquiry message v_FirstBand := cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)); v_EncodeCapRequest := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({v_FirstBand}))); v_DL_DCCH_Message := cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequest); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, f_NR_CellInfo_GetNTN_UE_gNB_RTT(nr_Cell1)); // add RTT delay to allow UE to apply DifferentialKoffset SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_DL_DCCH_Message)); //@siclog "Step 48" siclog@ //Check: Does the UE transmit a UECapabilityInformation message after applying differential Koffset of 20ms received at step 46? //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 48"); f_NR_TestBody_Set(false); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A,tsc_NR_RbId_SRB1); //WI=1309760 }; }

## Change 2

| Function name | TC_7_1_1_10_4_NR5GC() |
| --- | --- |
| Reason for change | At step 24 the period of BCCH modification for NTN GSO is about 500s, So the current Guard Timer (300s) is not suitable. |
| Summary of change | Extended the GuardTimer from 300s to 1000s. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_7_1_1_10_4_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // NR NTN / UE specific TA report / UE specific Koffset var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_7_1_1_10_4_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_7_1_1_10_4_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // NR NTN / UE specific TA report / UE specific Koffset var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1000); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_7_1_1_10_4_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA1 Integrity and NEA1 ciphering algorithms in GSO scenario on R&S 5G Conformance Test platform.

# Execution Log Files

## Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_7_1_1_10_4_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250261: Supporting information for addition of NR-NTN testcase 7.1.1.10.4 |
| --- | --- |