3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250265

# Online,  , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-5 | CR | 0016 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 5.1 to the MCX ATS with MCX-NR5GC model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2025-07-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add MCPTT TC 5.1 to the MCX ATS on MCX-NR5GC test model |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX-NR5GC test model. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase will not be added to MCX ATS with MCX-NR5GC model |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	7

2.5	Change 5	7

2.6	Change 6	10

2.7	Change 7	13

2.8	Change 8	13

3	Branches executed	15

4	Execution Log Files	15

4.1	Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1	15

5	References	16

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 5.1.MCPTT which is part of the MCX_NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	5.1.MCPTT

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1 on COTS 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | (NEW) f_MCX_IPCAN_NR5GC_PdnIndex() |
| --- | --- |
| Reason for change | The current implementation of f_MCX_IPCAN_PdnIndex has mcxPDN mapped to PDN_1. which works in EUTRA. While for NR5GC, the PDN1 is by default mapped to IMS, which result in mismatching in drb / PDU session in common functions. It is suggested to map mcx on PDN2 and ims on PDN1 for NR5GC |
| Summary of change | Create new function f_MCX_IPCAN_NR5GC_PdnIndex for NR5GC |
| TTCN module | IPCAN_MCX_Common4G5G.ttcn |
| MCC160 Comment |  |

New:

function f_MCX_IPCAN_NR5GC_PdnIndex(MCX_Registration_PDN_Type p_PDN) return PDN_Index_Type { var PDN_Index_Type v_PdnIndex; select (p_PDN) { /* IMS PDN_1, MCX PDN_2 sic@ */ case (imsPDN) { v_PdnIndex := PDN_1; } case (mcxPDN) { v_PdnIndex := PDN_2; } case (internetPDN) { v_PdnIndex := PDN_3; } case else { FatalError(__FILE__, __LINE__, "invalid PDN"); } } return v_PdnIndex; }

## Change 2

| Function name | f_MTC_MCX_InitAndConnect_IP_IMS_HTTP |
| --- | --- |
| Reason for change | when routing SIP messages etc from IPCAN to MCX/IMS PTC, PDN index is used (see f_PDN_PdnIndex2Integer, common in many ATS suites) to determined which port to use, with PDN_1(0) and PDN_2 (1) mapped to tsc_Index_IMS1 (0) and tsc_Index_IMS2 (1) As for change 1, when we swap the mcx/ims to PDN1/2 mapping, we also need to update the PCT accordinly |
| Summary of change | Updated f_MTC_MCX_InitAndConnect_IP_IMS_HTTP to map mcx and ims to on tsc_Index_IMS1 (0) and tsc_Index_IMS2 (1) for NR5GC |
| TTCN module | MTC_Component_MCX_Common.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MTC_MCX_InitAndConnect_IP_IMS_HTTP(SYSTEM_MCX_BASE p_System, MCX_IMS_PTC p_MCX, MCX_HTTP_PTC p_HTTP, IMS_PTC p_IMS := null, boolean p_UseDNS := false) runs on MTC_MCX { var IP_PTC v_IP; var MCX_HTTP_PTC v_HTTP; var boolean v_HttpHandlerRequired := true; var boolean v_UdpHandlerRequired := true; var boolean v_TcpHandlerRequired := true; f_MTC_BASE_Map(p_System); v_IP := f_IP_PTC_CreateAndMap(p_System); vc_Components.MCX := p_MCX; vc_Components.IP := v_IP; if (p_HTTP == null) { v_HTTP := MCX_HTTP_PTC.create("MCX_HTTP") alive; } else { v_HTTP := p_HTTP; } map(p_MCX:SRTP, p_System:SRTP); /* FloorCtrl port of type SRTP_PORT attached to IMS-PTC, see MCX_IMS_Component.ttcn */ connect(p_MCX:IMS_CTRL, v_IP:IMS_CTRL[tsc_Index_IMS1]); connect(p_MCX:IMS_Server, v_IP:IMS_Server[tsc_Index_IMS1]); connect(p_MCX:IMS_Client, v_IP:IMS_Client[tsc_Index_IMS1]); connect(p_MCX:MEDIA_CTRL, v_IP:UDPHANDLER); connect(p_MCX:MSRP, v_IP:TCPHANDLER); connect(v_HTTP:HTTPCTRL, v_IP:HTTPCTRL); connect(v_HTTP:HTTPDATA, v_IP:HTTPDATA); connect(v_HTTP:IMS, p_MCX:HTTP); // Coordination ports connect(mtc:PTC_Ut[tsc_MTC_PortIndex_IMS1], p_MCX:MMI); connect(mtc:PTC_Ut[tsc_MTC_PortIndex_HTTP], v_HTTP:MMI); if (p_IMS != null) { connect(p_IMS:IMS_CTRL, v_IP:IMS_CTRL[tsc_Index_IMS2]); connect(p_IMS:IMS_Server, v_IP:IMS_Server[tsc_Index_IMS2]); connect(p_IMS:IMS_Client, v_IP:IMS_Client[tsc_Index_IMS2]); // connect(mtc:PTC_Ut[tsc_MTC_PortIndex_IMS2], p_IMS:MMI); There shall be no need for any upper tester at the IMS PTC // connect(p_IMS:OtherPDN, p_MCX:OtherPDN); There shall be no need for communication between MCX and IMS } v_IP.start(f_IP_PTC_MainLoop(p_UseDNS, v_HttpHandlerRequired, v_UdpHandlerRequired, v_TcpHandlerRequired)); if (p_HTTP == null) { v_HTTP.start(f_MCX_HTTP_DefaultHandler()); } }

After Change:

function f_MTC_MCX_InitAndConnect_IP_IMS_HTTP(SYSTEM_MCX_BASE p_System, MCX_IMS_PTC p_MCX, MCX_HTTP_PTC p_HTTP, IMS_PTC p_IMS := null, boolean p_UseDNS := false, boolean p_IsNR5GC := false) runs on MTC_MCX { var IP_PTC v_IP; var MCX_HTTP_PTC v_HTTP; var boolean v_HttpHandlerRequired := true; var boolean v_UdpHandlerRequired := true; var boolean v_TcpHandlerRequired := true; f_MTC_BASE_Map(p_System); v_IP := f_IP_PTC_CreateAndMap(p_System); vc_Components.MCX := p_MCX; vc_Components.IP := v_IP; if (p_HTTP == null) { v_HTTP := MCX_HTTP_PTC.create("MCX_HTTP") alive; } else { v_HTTP := p_HTTP; } map(p_MCX:SRTP, p_System:SRTP); /* FloorCtrl port of type SRTP_PORT attached to IMS-PTC, see MCX_IMS_Component.ttcn */ if (p_IsNR5GC) { connect(p_MCX:IMS_CTRL, v_IP:IMS_CTRL[tsc_Index_IMS2]); connect(p_MCX:IMS_Server, v_IP:IMS_Server[tsc_Index_IMS2]); connect(p_MCX:IMS_Client, v_IP:IMS_Client[tsc_Index_IMS2]); } else { connect(p_MCX:IMS_CTRL, v_IP:IMS_CTRL[tsc_Index_IMS1]); connect(p_MCX:IMS_Server, v_IP:IMS_Server[tsc_Index_IMS1]); connect(p_MCX:IMS_Client, v_IP:IMS_Client[tsc_Index_IMS1]); } connect(p_MCX:MEDIA_CTRL, v_IP:UDPHANDLER); connect(p_MCX:MSRP, v_IP:TCPHANDLER); connect(v_HTTP:HTTPCTRL, v_IP:HTTPCTRL); connect(v_HTTP:HTTPDATA, v_IP:HTTPDATA); connect(v_HTTP:IMS, p_MCX:HTTP); // Coordination ports if (p_IsNR5GC) { connect(mtc:PTC_Ut[tsc_MTC_PortIndex_IMS2], p_MCX:MMI); } else { connect(mtc:PTC_Ut[tsc_MTC_PortIndex_IMS1], p_MCX:MMI); } connect(mtc:PTC_Ut[tsc_MTC_PortIndex_HTTP], v_HTTP:MMI); if (p_IMS != null) { if (p_IsNR5GC) { connect(p_IMS:IMS_CTRL, v_IP:IMS_CTRL[tsc_Index_IMS1]); connect(p_IMS:IMS_Server, v_IP:IMS_Server[tsc_Index_IMS1]); connect(p_IMS:IMS_Client, v_IP:IMS_Client[tsc_Index_IMS1]); } else { connect(p_IMS:IMS_CTRL, v_IP:IMS_CTRL[tsc_Index_IMS2]); connect(p_IMS:IMS_Server, v_IP:IMS_Server[tsc_Index_IMS2]); connect(p_IMS:IMS_Client, v_IP:IMS_Client[tsc_Index_IMS2]); } // connect(mtc:PTC_Ut[tsc_MTC_PortIndex_IMS2], p_IMS:MMI); There shall be no need for any upper tester at the IMS PTC // connect(p_IMS:OtherPDN, p_MCX:OtherPDN); There shall be no need for communication between MCX and IMS } v_IP.start(f_IP_PTC_MainLoop(p_UseDNS, v_HttpHandlerRequired, v_UdpHandlerRequired, v_TcpHandlerRequired)); if (p_HTTP == null) { v_HTTP.start(f_MCX_HTTP_DefaultHandler()); } }

## Change 3

| Function name | f_MTC_MCX_CreateMapAndConnectPTCs_NR5GC |
| --- | --- |
| Reason for change | As for change 2 |
| Summary of change | Updated call to f_MTC_MCX_InitAndConnect_IP_IMS_HTTP with parameter indicates NR5GC |
| TTCN module | MTC_Component_MCX_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MTC_MCX_CreateMapAndConnectPTCs_NR5GC(SYSTEM_MCX p_System, MCX_IMS_PTC p_MCX, MCX_HTTP_PTC p_HTTP) runs on MTC_MCX { var NR5GC_PTC v_NR5GC; var NASEMU_NRNG_PTC v_NASEMU_PTC; var IMS_PTC v_IMS; v_IMS := f_IMS_PTC_Create("IMS2"); v_IMS.start(f_IMS_DefaultHandler(PDN_2)); /* @sic R5s240511 - additional changes: PDN_2 sic@ */ f_MTC_MCX_InitAndConnect_IP_IMS_HTTP(p_System, p_MCX, p_HTTP, v_IMS); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_NR_BASE_PTC_Map(p_System, v_NR5GC); // map NR5GC system ports v_NASEMU_PTC := f_NasEmu5G_CreateConnectAndMap_NRNG(p_System, v_NR5GC); // create NasEmu, map system ports and connect with NR5GC PTC v_NASEMU_PTC.start(f_NASEMU_MainLoop_NRNG()); // start NASEMU connect(mtc:PTC_Ut[tsc_MTC_PortIndex_MCX_IPCAN], v_NR5GC:UT); connect(v_NR5GC:IP, vc_Components.IP:IPCAN_CTRL); connect(p_MCX:IPCAN, v_NR5GC:IMS[tsc_Index_IMS1]); connect(v_IMS:IPCAN, v_NR5GC:IMS[tsc_Index_IMS2]); f_UT_InitialPowerOffUE(Ut); v_NR5GC.start(f_MCX_NR5GC_IPCAN_MainLoop()); }

After Change:

function f_MTC_MCX_CreateMapAndConnectPTCs_NR5GC(SYSTEM_MCX p_System, MCX_IMS_PTC p_MCX, MCX_HTTP_PTC p_HTTP) runs on MTC_MCX { var NR5GC_PTC v_NR5GC; var NASEMU_NRNG_PTC v_NASEMU_PTC; var IMS_PTC v_IMS; v_IMS := f_IMS_PTC_Create("IMS2"); v_IMS.start(f_IMS_DefaultHandler(PDN_2)); /* @sic R5s240511 - additional changes: PDN_2 sic@ */ f_MTC_MCX_InitAndConnect_IP_IMS_HTTP(p_System, p_MCX, p_HTTP, v_IMS, -, true); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_NR_BASE_PTC_Map(p_System, v_NR5GC); // map NR5GC system ports v_NASEMU_PTC := f_NasEmu5G_CreateConnectAndMap_NRNG(p_System, v_NR5GC); // create NasEmu, map system ports and connect with NR5GC PTC v_NASEMU_PTC.start(f_NASEMU_MainLoop_NRNG()); // start NASEMU connect(mtc:PTC_Ut[tsc_MTC_PortIndex_MCX_IPCAN], v_NR5GC:UT); connect(v_NR5GC:IP, vc_Components.IP:IPCAN_CTRL); connect(p_MCX:IPCAN, v_NR5GC:IMS[tsc_Index_IMS1]); connect(v_IMS:IPCAN, v_NR5GC:IMS[tsc_Index_IMS2]); f_UT_InitialPowerOffUE(Ut); v_NR5GC.start(f_MCX_NR5GC_IPCAN_MainLoop()); }

## Change 4

| Function name | f_MCX_NR5GC_IPCAN_EndProcedure |
| --- | --- |
| Reason for change | As for change 1 |
| Summary of change | Updated call calls to f_MCX_IPCAN_PdnIndex to f_MCX_IPCAN_NR5GC_PdnIndex |
| TTCN module | IPCAN_MCX_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_NR5GC_IPCAN_EndProcedure(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_CellId, IMS_TestProcedure_Type p_ProcedureType) runs on NR5GC_PTC return boolean { var PDN_Index_Type v_PdnIndex := f_MCX_IPCAN_PdnIndex(mcxPDN); var boolean v_ReleaseRRC := true; ...

After Change:

function f_MCX_NR5GC_IPCAN_EndProcedure(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_CellId, IMS_TestProcedure_Type p_ProcedureType) runs on NR5GC_PTC return boolean { var PDN_Index_Type v_PdnIndex := f_MCX_IPCAN_NR5GC_PdnIndex(mcxPDN); var boolean v_ReleaseRRC := true; ...

## Change 5

| Function name | f_MCX_NR5GC_IPCAN_AddPduSession |
| --- | --- |
| Reason for change | 1. As for change 1 2. call to f_SendPDUSessionAcceptOverReconfig shall has p_InterworkWithEPS as false |
| Summary of change | 1. Updated call to f_MTC_MCX_InitAndConnect_IP_IMS_HTTP with parameter indicates NR5GC 2. updated call to f_SendPDUSessionAcceptOverReconfig with p_InterworkWithEPS as false |
| TTCN module | MTC_Component_MCX_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_NR5GC_IPCAN_AddPduSession(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_CellId, NR_RadioBearerId_Type p_SRB) runs on NR5GC_PTC return PDU_PDN_DNN_Type { var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SSConfig; var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var template (value) DNN v_DNN; var template (omit) QoS_Rules v_QoS_Rules := omit; var template (omit) QoSFlowDescr v_QoSFlowDescr := omit; var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) SDAP_Config v_SDAP_Config; var template (value) IP_DrbInfo_Type v_DrbInfo; var SRBs_AlreadyConfigured v_SRBs_AlreadyConfigured := SRB1; var PDU_PDN_DNN_Type v_PDUType; var SdapConfig_Type v_SdapConfig; var PDN_Index_Type v_PdnIndex; var integer v_QFI; if (p_SRB == tsc_NR_RbId_SRB2) { v_SRBs_AlreadyConfigured := SRB1_2; } // 37.579-1 Table 5.4.2.3.3-3: Step 1 v_GSM_MobilityInfo := fl_MCX_NR5GC_IPCAN_ReceiveAndCheckPDUSessionEstablishmentReq(p_CellId, p_SRB); v_PDUType := v_GSM_MobilityInfo.PDUType; select (v_PDUType) { case (Internet_DNN) { v_QFI := 1; // 38.508-1 Table 4.8.2.1-1: Reference QoS rule #1 v_PdnIndex := f_MCX_IPCAN_PdnIndex(internetPDN); } case (IMS_DNN) { v_QFI := 2; // 38.508-1 Table 4.8.2.1-2: Reference QoS rule #2 v_PdnIndex := f_MCX_IPCAN_PdnIndex(imsPDN); } case (MCX_DNN) { v_QFI := tsc_MCX_NR5GC_QFI_Signalling; v_QoS_Rules := fl_MCX_NR5GC_IPCAN_QoS_Rules(v_QFI); // 38.508-1 Table 4.8.2.1-10: Reference QoS rule #10 (MCX signalling) v_QoSFlowDescr := fl_MCX_NR5GC_IPCAN_QoSFlowDescr(v_QFI); // 38.508-1 Table 4.8.2.3-9: Reference QoS flow #9 (MCX signalling) v_PdnIndex := f_MCX_IPCAN_PdnIndex(mcxPDN); } case else { f_SetVerdictInconc(__FILE__, __LINE__, "Invalid PDU Type"); } } v_GSM_MobilityInfo.PdnIndex := v_PdnIndex; v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); v_SDAP_Config := cs_SDAP_Config(-, -, {v_QFI}); f_BuildRRCReconfigParamsNewDRBs(v_RadioBearerConfig, // out parameter v_CellGroupConfig, // out parameter v_SSConfig, // out parameter v_SDAP_ConfigurationList, // out parameter p_CellId, {v_GSM_MobilityInfo}, v_SRBs_AlreadyConfigured, v_SDAP_Config); // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); // It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_QFI); v_DrbInfo := cs_DrbInfo_NR(p_CellId, v_QosFlowId); f_IP_Handling_StartPDN(IP, v_PdnIndex, v_DrbInfo); if (v_PDUType == MCX_DNN) { f_IMS_IPCAN_SendCoOrdMsg(p_Port, cms_IPCAN_IMS_DrbInfo(v_DrbInfo)); // start media control handler and HTTP servers (controlled by IMS PTC) f_IMS_IPCAN_WaitForTrigger(p_Port); // Wait until HTTP servers are started } // 37.579-1 Table 5.4.2.3.3-3: Step 2 - 3 f_SendPDUSessionAcceptOverReconfig(p_CellId, v_GSM_MobilityInfo, v_DNN, v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, -, -, -, -, -, -, -, -, v_QoS_Rules, v_QoSFlowDescr); return v_PDUType; }

After Change:

function f_MCX_NR5GC_IPCAN_AddPduSession(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_CellId, NR_RadioBearerId_Type p_SRB) runs on NR5GC_PTC return PDU_PDN_DNN_Type { var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SSConfig; var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var template (value) DNN v_DNN; var template (omit) QoS_Rules v_QoS_Rules := omit; var template (omit) QoSFlowDescr v_QoSFlowDescr := omit; var template (value) QosFlow_Identification_Type v_QosFlowId; var template (value) SDAP_Config v_SDAP_Config; var template (value) IP_DrbInfo_Type v_DrbInfo; var SRBs_AlreadyConfigured v_SRBs_AlreadyConfigured := SRB1; var PDU_PDN_DNN_Type v_PDUType; var SdapConfig_Type v_SdapConfig; var PDN_Index_Type v_PdnIndex; var integer v_QFI; if (p_SRB == tsc_NR_RbId_SRB2) { v_SRBs_AlreadyConfigured := SRB1_2; } // 37.579-1 Table 5.4.2.3.3-3: Step 1 v_GSM_MobilityInfo := fl_MCX_NR5GC_IPCAN_ReceiveAndCheckPDUSessionEstablishmentReq(p_CellId, p_SRB); v_PDUType := v_GSM_MobilityInfo.PDUType; select (v_PDUType) { case (Internet_DNN) { v_QFI := 1; // 38.508-1 Table 4.8.2.1-1: Reference QoS rule #1 v_PdnIndex := f_MCX_IPCAN_NR5GC_PdnIndex(internetPDN); } case (IMS_DNN) { v_QFI := 2; // 38.508-1 Table 4.8.2.1-2: Reference QoS rule #2 v_PdnIndex := f_MCX_IPCAN_NR5GC_PdnIndex(imsPDN); } case (MCX_DNN) { v_QFI := tsc_MCX_NR5GC_QFI_Signalling; v_QoS_Rules := fl_MCX_NR5GC_IPCAN_QoS_Rules(v_QFI); // 38.508-1 Table 4.8.2.1-10: Reference QoS rule #10 (MCX signalling) v_QoSFlowDescr := fl_MCX_NR5GC_IPCAN_QoSFlowDescr(v_QFI); // 38.508-1 Table 4.8.2.3-9: Reference QoS flow #9 (MCX signalling) v_PdnIndex := f_MCX_IPCAN_NR5GC_PdnIndex(mcxPDN); } case else { f_SetVerdictInconc(__FILE__, __LINE__, "Invalid PDU Type"); } } v_GSM_MobilityInfo.PdnIndex := v_PdnIndex; v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType))); v_SDAP_Config := cs_SDAP_Config(-, -, {v_QFI}); f_BuildRRCReconfigParamsNewDRBs(v_RadioBearerConfig, // out parameter v_CellGroupConfig, // out parameter v_SSConfig, // out parameter v_SDAP_ConfigurationList, // out parameter p_CellId, {v_GSM_MobilityInfo}, v_SRBs_AlreadyConfigured, v_SDAP_Config); // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); // It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_QFI); v_DrbInfo := cs_DrbInfo_NR(p_CellId, v_QosFlowId); f_IP_Handling_StartPDN(IP, v_PdnIndex, v_DrbInfo); if (v_PDUType == MCX_DNN) { f_IMS_IPCAN_SendCoOrdMsg(p_Port, cms_IPCAN_IMS_DrbInfo(v_DrbInfo)); // start media control handler and HTTP servers (controlled by IMS PTC) f_IMS_IPCAN_WaitForTrigger(p_Port); // Wait until HTTP servers are started } // 37.579-1 Table 5.4.2.3.3-3: Step 2 - 3 f_SendPDUSessionAcceptOverReconfig(p_CellId, v_GSM_MobilityInfo, v_DNN, v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, -, -, -, -, -, -, -, -, v_QoS_Rules, v_QoSFlowDescr, -,-,-, false); return v_PDUType; }

## Change 6

| Function name | f_MCX_NR5GC_IPCAN_InitialRegistration |
| --- | --- |
| Reason for change | 1, runtime error on code that compose the v_ExpectedPduSessionList 2, before active a_NR5GC_AdditionalPDUSessionEstablishmentReq for PDU SESSION ESTABLISHMENT REQ, we need to call f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq to reset the queue. 3, after active a_NR5GC_AdditionalPDUSessionEstablishmentReq, we need to wait PDU SESSION ESTABLISHMENT REQ is added into the queue for processing 4, The UE may immediately send both PDU SESSION Establishment REQ for ims and mcx, before TTCN handle the first one and establish SRB2. |
| Summary of change | 1, Updated the code for composing v_ExpectedPduSessionList 2, added call to f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq before active a_NR5GC_AdditionalPDUSessionEstablishmentReq 3, added a delay after active a_NR5GC_AdditionalPDUSessionEstablishmentReq 4, Removed changing to SRB2 on 2nd PDU SESSION ESTABLISHMENT REQ |
| TTCN module | IPCAN_MCX_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_NR5GC_IPCAN_InitialRegistration(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_CellId) runs on NR5GC_PTC { var boolean v_InitialRegistrationIMS; var boolean v_InitialRegistrationMCX; var boolean v_InitialRegistrationInternet; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling; var PDU_PDN_DNN_List_Type v_ExpectedPduSessionList := {}; var PDU_PDN_DNN_List_Type v_InitialPduSessionList := {}; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var PDU_PDN_DNN_Type v_PDUType; var integer i; var default v_MyDefaultVar; for (i := 0; i < lengthof(tsc_MCX_InitialRegistrationPdnList); i := i + 1) { select (tsc_MCX_InitialRegistrationPdnList[i]) { case (mcxPDN) { v_ExpectedPduSessionList[i] := MCX_DNN; } case (imsPDN) { v_ExpectedPduSessionList[i] := IMS_DNN; } case (internetPDN) { v_ExpectedPduSessionList[i] := Internet_DNN; } case else { break; } } } f_NR_SetCellPower(p_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_UT_SwitchOnUE(UT); // 37.579-1 Table 5.4.2.3.3-1: Steps 1 - 14 (38.508-1 Table 4.5.2.2-2 steps 2 - 15) f_NR5GC_Registration(p_CellId, -, Initial_NoSecurity); /* parameters of f_NR5GC_Registration * - p_CellId * - p_TestMode TESTMode_OFF per default => no test mode => no steps 9a1/9a2 * - p_RegType Initial_NoSecurity => no security context yet * - p_EPLMNs omit per default * - p_NwkFeatureSupport cs_NG_NetworkFeatureSupport_Def per default * => IMS over 3GPP, Emergency services supported, Interworking without N26 interface not supported * - p_IwkN26 NOT_SUPPORTED per default * => Interworking without N26 interface not supported, but N26 interface is supported * => when the UE supports EUTRA (S1 mode supported) then there is a second SECURITY MODE COMMAND procedure * (38.508-1 Table 4.5.2.2-2 steps 9Aa1/9Aa2) even though this is not needed for MCX (no IRAT scenarios) */ // 37.579-1 Table 5.4.2.3.3-1: Step 15 (0, 1, 2 or 3 times) v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_CellId)); for (i := 0; i < lengthof(v_ExpectedPduSessionList); i := i + 1) { v_PDUType := f_MCX_NR5GC_IPCAN_AddPduSession(p_Port, p_CellId, v_SRB); v_InitialPduSessionList := v_InitialPduSessionList & { v_PDUType }; v_SRB := tsc_NR_RbId_SRB2; } deactivate(v_MyDefaultVar);

After Change:

import from MCX_CommonIPCAN all; function f_MCX_NR5GC_IPCAN_InitialRegistration(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_CellId) runs on NR5GC_PTC { var boolean v_InitialRegistrationIMS; var boolean v_InitialRegistrationMCX; var boolean v_InitialRegistrationInternet; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling; var PDU_PDN_DNN_List_Type v_ExpectedPduSessionList := {}; var PDU_PDN_DNN_List_Type v_InitialPduSessionList := {}; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var PDU_PDN_DNN_Type v_PDUType; var integer i; var default v_MyDefaultVar; var MCX_Registration_PDN_Type v_PdnType; for (i := 0; i < lengthof(tsc_MCX_InitialRegistrationPdnList); i := i + 1) { v_PdnType := tsc_MCX_InitialRegistrationPdnList[i]; select (v_PdnType) { case (mcxPDN) { v_ExpectedPduSessionList := v_ExpectedPduSessionList & {MCX_DNN}; } case (imsPDN) { v_ExpectedPduSessionList := v_ExpectedPduSessionList & {IMS_DNN}; } case (internetPDN) { v_ExpectedPduSessionList := v_ExpectedPduSessionList & {Internet_DNN}; } case else { break; } } } f_NR_SetCellPower(p_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_UT_SwitchOnUE(UT); // 37.579-1 Table 5.4.2.3.3-1: Steps 1 - 14 (38.508-1 Table 4.5.2.2-2 steps 2 - 15) f_NR5GC_Registration(p_CellId, -, Initial_NoSecurity); /* parameters of f_NR5GC_Registration * - p_CellId * - p_TestMode TESTMode_OFF per default => no test mode => no steps 9a1/9a2 * - p_RegType Initial_NoSecurity => no security context yet * - p_EPLMNs omit per default * - p_NwkFeatureSupport cs_NG_NetworkFeatureSupport_Def per default * => IMS over 3GPP, Emergency services supported, Interworking without N26 interface not supported * - p_IwkN26 NOT_SUPPORTED per default * => Interworking without N26 interface not supported, but N26 interface is supported * => when the UE supports EUTRA (S1 mode supported) then there is a second SECURITY MODE COMMAND procedure * (38.508-1 Table 4.5.2.2-2 steps 9Aa1/9Aa2) even though this is not needed for MCX (no IRAT scenarios) */ // 37.579-1 Table 5.4.2.3.3-1: Step 15 (0, 1, 2 or 3 times) f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_CellId)); f_Delay(0.5); for (i := 0; i < lengthof(v_ExpectedPduSessionList); i := i + 1) { v_PDUType := f_MCX_NR5GC_IPCAN_AddPduSession(p_Port, p_CellId, v_SRB); v_InitialPduSessionList := v_InitialPduSessionList & { v_PDUType }; //v_SRB := tsc_NR_RbId_SRB2; } deactivate(v_MyDefaultVar);

## Change 7

| Function name | f_GetMultiPdnIndex |
| --- | --- |
| Reason for change | MCX_DNN is not covered |
| Summary of change | Added MCX_DNN, also align with change 1 |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetMultiPdnIndex(PDU_PDN_DNN_Type p_DNNType) return PDN_Index_Type { // @sic R5-221467 sic@ var PDN_Index_Type v_PDN := PDN_3; select (p_DNNType) { case (IMS_DNN){ v_PDN := PDN_1; } case (Emergency_PDN){ v_PDN := PDN_2; } case (URLLC_DNN, UAS_USS_DNN){ // @sic R5-245495 sic@ v_PDN := PDN_4; } case (MIOT_DNN, UAS_C2_DNN){ // @sic R5-245495 sic@ v_PDN := PDN_5; } case (V2X_DNN){ v_PDN := PDN_6; } } return v_PDN; }

After Change:

function f_GetMultiPdnIndex(PDU_PDN_DNN_Type p_DNNType) return PDN_Index_Type { // @sic R5-221467 sic@ var PDN_Index_Type v_PDN := PDN_3; select (p_DNNType) { case (IMS_DNN){ v_PDN := PDN_1; } case (Emergency_PDN, MCX_DNN){ v_PDN := PDN_2; } case (URLLC_DNN, UAS_USS_DNN){ // @sic R5-245495 sic@ v_PDN := PDN_4; } case (MIOT_DNN, UAS_C2_DNN){ // @sic R5-245495 sic@ v_PDN := PDN_5; } case (V2X_DNN){ v_PDN := PDN_6; } } return v_PDN; }

## Change 8

| Function name | f_GetDNNStringFromPICS |
| --- | --- |
| Reason for change | MCX_DNN is not covered |
| Summary of change | Added MCX_DNN |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetDNNStringFromPICS(PDU_PDN_DNN_Type p_PDUType) return charstring { var charstring v_DNN; select (p_PDUType) { case (Internet_DNN) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } case (Emergency_PDN) { // @sic R5s250092 sic@ v_DNN := "sos"; } case (URLLC_DNN) { v_DNN := pc_APN_ID_URLLC; } case (MIOT_DNN) { v_DNN := pc_APN_ID_MIOT; } case (V2X_DNN) { v_DNN := pc_APN_ID_V2X; } case (Ethernet_DNN) { // @sic R5-227472 sic@ v_DNN := pc_APN_ID_Ethernet; } /* case (MBS_DNN) { // @sic R5-245495 sic@ v_DNN := pc_APN_ID_MBS; }*/ case (UAS_USS_DNN) { // @sic R5-246404 sic@ v_DNN := pc_APN_ID_USS; } case (UAS_C2_DNN) { // @sic R5-246404 sic@ v_DNN := pc_APN_ID_C2; } // @sic R5s220670 sic@ case else { FatalError(__FILE__, __LINE__, "Unexpected PICS setting in f_GetDNNStringFromPICS"); } } return v_DNN; }

After Change:

function f_GetDNNStringFromPICS(PDU_PDN_DNN_Type p_PDUType) return charstring { var charstring v_DNN; select (p_PDUType) { case (Internet_DNN) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } case (MCX_DNN) { v_DNN := pc_APN_ID_MCX; } case (Emergency_PDN) { // @sic R5s250092 sic@ v_DNN := "sos"; } case (URLLC_DNN) { v_DNN := pc_APN_ID_URLLC; } case (MIOT_DNN) { v_DNN := pc_APN_ID_MIOT; } case (V2X_DNN) { v_DNN := pc_APN_ID_V2X; } case (Ethernet_DNN) { // @sic R5-227472 sic@ v_DNN := pc_APN_ID_Ethernet; } /* case (MBS_DNN) { // @sic R5-245495 sic@ v_DNN := pc_APN_ID_MBS; }*/ case (UAS_USS_DNN) { // @sic R5-246404 sic@ v_DNN := pc_APN_ID_USS; } case (UAS_C2_DNN) { // @sic R5-246404 sic@ v_DNN := pc_APN_ID_C2; } // @sic R5s220670 sic@ case else { FatalError(__FILE__, __LINE__, "Unexpected PICS setting in f_GetDNNStringFromPICS"); } } return v_DNN; }

# Branches executed

The test case was executed with

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE FDD Band 5

- Ciphering algorithm eea1 and Integrity algorithm eia1

# Execution Log Files

## Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1

The Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1 passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_5_1_MCPTT_LOG.txt

- PICS/PIXIT parameter file:

TC_5_1_MCPTT_PIXIT.xml

# References

| [1] | R5s250266 : Supporting information for Addition of MCPTT test case 5.1 to the MCX ATS with MCX-NR5GC model |
| --- | --- |