3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250290

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4920 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE DRX testcase 7.1.6.2 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the definition of drx-Config from the TS 36.331, E-UTRAN should configure drx-Config-r13 IE if the UE supports CE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to configure drx-Config-r13 IE if the UE supports CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.1a and 7.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_6_2_EUTRA |
| --- | --- |
| Reason for change | According to the definition of drx-Config from the TS 36.331, E-UTRAN should configure drx-Config-r13 IE if the UE supports CE. Draft proseCR would be raised during the RAN5#108 meeting |
| Summary of change | It is proposed to configure drx-Config-r13 IE if the UE supports CE. |
| TTCN module | MAC_716.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC { // SKIPPED CODE // Reconfigure RRC Connection: Step 0A and 0B f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset); //@sic R5s110698 sic@ f_Delay(0.66); // SKIPPED CODE }

After Change

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC { // SKIPPED CODE // Reconfigure RRC Connection: Step 0A and 0B if(pc_ue_CategoryDL_M1){ f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset, cs_DRX_Config_r13_716X)); //@sic R5s110698 sic@ } else { f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset); //@sic R5s110698 sic@ } f_Delay(0.66); // SKIPPED CODE }

## Change 2

| Template name | cs_DRX_Config_r13_716X |
| --- | --- |
| Reason for change | New template similar to cs_DRX_Config_r13_7161a is required to configure drx-Config-r13 IE. |
| Summary of change | It is proposed to duplicate the template cs_DRX_Config_r13_7161a from MAC_eMTC_eDRX.ttcn with a new name cs_DRX_Config_r13_716X under EUTRA_L2_CommonTemplates.ttcn so that it is commonly used for both 7.1.6.1a & 7.1.6.2 testcases. |
| TTCN module | EUTRA_L2_CommonTemplates.ttcn |
| MCC160 Comment |  |

New template

template (value) DRX_Config_r13_Type cs_DRX_Config_r13_716X:= { /* @status APPROVED (LTE_A_PRO) */ setup := { onDurationTimer_v1310 := psf300, drx_RetransmissionTimer_v1310 := psf40, drx_ULRetransmissionTimer_r13 := psf320 } };

## Change 3

| Function name | f_TC_7_1_6_1a_EUTRA |
| --- | --- |
| Reason for change | Due to change 2 template cs_DRX_Config_r13_7161a will be redundant and it is proposed to remove the template and replace it with new template cs_DRX_Config_r13_716X. |
| Summary of change | Remove template definition of cs_DRX_Config_r13_7161a and replace template call of cs_DRX_Config_r13_7161a with cs_DRX_Config_r13_716X at Step0A |
| TTCN module | MAC_eMTC_eDRX.ttcn |
| MCC160 Comment |  |

Before Change

template (value) DRX_Config_r13_Type cs_DRX_Config_r13_7161a:= { /* @status APPROVED (LTE_A_PRO) */ setup := { onDurationTimer_v1310 := psf300, drx_RetransmissionTimer_v1310 := psf40, drx_ULRetransmissionTimer_r13 := psf320 } }; // SKIPPED SECTION function f_TC_7_1_6_1a_EUTRA() runs on EUTRA_PTC { // SKIPPED CODE // Reconfigure RRC Connection: Step 0A and 0B f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset, cs_DRX_Config_r13_7161a)); f_Delay(0.66); // SKIPPED CODE }

After Change

// REMOVED // SKIPPED SECTION function f_TC_7_1_6_1a_EUTRA() runs on EUTRA_PTC { // SKIPPED CODE // Reconfigure RRC Connection: Step 0A and 0B f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset, cs_DRX_Config_r13_716X)); f_Delay(0.66); // SKIPPED CODE }