3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250291

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4921 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NBIOT_PreconfigureNTN_UE (for NSGO testing) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implmentation MMI command to configure NTN position is sent out (without CNF_Required) and then DisableNGSO_SatellitePrediction (with CNF_required) is sent. The UE would start looking for a cell to camp on upon receiving the NTN position. This may result in a situation where the UE sends RRC Connection Request before MMI CNF for DisableNGSO_SatellitePrediction is being received. This causes testcase failure and needs to be addressed. In order to avoid this it is proposed to swap the MMI commands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the code to DisableNGSO_SatellitePrediction and then send configure NTN position. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NB-NTN test cases when tested with NGSO configuration |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Overview	3

2	Corrections required to NB NTN test cases	4

2.1	Change 1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required to NB NTN test cases

## Change 1

| Function name | f_NBIOT_PreconfigureNTN_UE |
| --- | --- |
| Reason for change | In the current TTCN implmentation MMI command to configure NTN position is sent out (without CNF_Required) and then DisableNGSO_SatellitePrediction (with CNF_required) is sent. The UE would start looking for a cell to camp on upon receiving the NTN position. This may result in a situation where the UE sends RRC Connection Request before MMI CNF for DisableNGSO_SatellitePrediction is being received. This causes testcase failure and needs to be addressed. In order to avoid this it is proposed to swap the MMI commands |
| Summary of change | Modified the code to DisableNGSO_SatellitePrediction and then send configure NTN position; |
| TTCN module | NBIOT_ConfigurationSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_PreconfigureNTN_UE() runs on NBIOT_PTC{ f_UT_ConfigureNTNPosition(UT, -, NO_CNF_REQUIRED); // @sic R5s240281 sic@ if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_NGSO){ f_UT_DisableNGSO_SatellitePrediction(UT, DISABLE); /*@sic R5-253187 sic@*/ } }

After Change:

function f_NBIOT_PreconfigureNTN_UE() runs on NBIOT_PTC{ if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_NGSO){ f_UT_DisableNGSO_SatellitePrediction(UT, DISABLE); /*@sic R5-253187 sic@*/ } f_UT_ConfigureNTNPosition(UT, -, NO_CNF_REQUIRED); // @sic R5s240281 sic@ }