3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250319

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3696 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to processing delay testcase 8.1.5.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to changes from R5s250008 the function f_NR_RRC_InactiveDef (which is called prior the steps 14aX) sets default values for RACHprocedureConfig which are however different than those required by the 8.1.5.8.1 for the RRC resume procedure. It is especially the PDSCH-to-HARQ_feedback timing indicator that is wrong and that may cause disruption in the otherwise carefully timed transmissions during the RRCresume. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The f_NR_RRC_InactiveDef is newly called with the RACHprocedure configuration that was populated prior the step 1 (also introduced by R5s240284). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_8_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | Due to changes from R5s250008 the function f_NR_RRC_InactiveDef (which is called prior the steps 14aX) sets default values for RACHprocedureConfig which are however different than those required by the 8.1.5.8.1 for the RRC resume procedure. It is especially the PDSCH-to-HARQ_feedback timing indicator that is wrong and that may cause disruption in the otherwise carefully timed transmissions during the RRCresume |
| Summary of change | The f_NR_RRC_InactiveDef is newly called with the RACHprocedure configuration that was populated prior the step 1 (also introduced by R5s240284). |
| TTCN module | RRC_Others_NR5GC.ttcn |

Before Change:

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { … //@siclog "Step 14a1-14a7" siclog@ if ( pc_inactiveState ){ //@siclog "Step 14a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. //TTCN CR: The RACH params will get rewritten to default values, so the change done by the R5s240284 would get lost again f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 14a2" siclog@

After Change:

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { … //@siclog "Step 14a1-14a7" siclog@ if ( pc_inactiveState ){ //@siclog "Step 14a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1,-,-,-,-,-,v_NR_RachProcedureConfig); //@siclog "Step 14a2" siclog@