3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250330

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3727 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RLC test case 7.1.2.3.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The expiry of t-Reassembly timer started in step 1 will trigger STATUS PDU from the UE. This starts SR. The RRC reconfiguration is scheduled 300 ms in advance. sr-transMax is currently set to n32 so that the UE does not reach sr-transMax and then RACHes before reconfiguring.However the test case processing time in TTCN before scheduling the RRC reconfiguration can take more than 20 ms which can lead to the expiry of sr-transmax and thereby triggering the RACH procedure causing intermittent test case failures Note: R5-254163 will be submitted to RAN5#108 meeting for this change |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the sr-transMax value to n64 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 5096 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_7_1_2_3_11_NR5GC() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | The expiry of t-Reassembly timer started in step 1 will trigger STATUS PDU from the UE. This starts SR. The RRC reconfiguration is scheduled 300 ms in advance. sr-transMax is currently set to n32 so that the UE does not reach sr-transMax and then RACHes before reconfiguring.However the test case processing time before scheduling the RRC reconfiguration can take more than 20 ms which can lead to the expiry of sr-transmax and thereby triggering the RACH procedure causing intermittent test case failures Note: R5-254163 will be submitted to RAN5#108 meeting for this change |
| Summary of change | Changed the sr-transMax value to n64 |
| TTCN module | RLC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_2_3_11_NR5GC() runs on NR5GC_PTC { // AM RLC / RLC re-establishment procedure var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var DRB_Identity v_DRBId; var SN_FieldLengthAM v_SN_FieldLengthAM := size18; f_NR5GC_Init(NR_1); //Set sr-transmax to 32 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n32; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); // @sic R5s191200_r2 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); /***************** Skipping the code *************************************/ }

After Change:

function f_TC_7_1_2_3_11_NR5GC() runs on NR5GC_PTC { // AM RLC / RLC re-establishment procedure var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var DRB_Identity v_DRBId; var SN_FieldLengthAM v_SN_FieldLengthAM := size18; f_NR5GC_Init(NR_1); //Set sr-transmax to 32 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64;//KS#25xxxxx f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); // @sic R5s191200_r2 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); /***************** Skipping the code *************************************/ }