3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250338

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3735 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 12, the function f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); is called. This function starts a timer of 8s within which a PDU Session Establishment REQ is expected, and if the UE does not transmit the PDU Session Establishment REQ within 8s the testacse fails. This is not in-line with TS 38.523-1 Table 10.1.8.1.3.2-1 Step 12, which expects the PDU Session Establishment REQ within 60s. This needs to be addressed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation of Step 12 in TTCN. Please see below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not conformant with Prose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_8_1_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 12, the function f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); is called. This function starts a timer of 8s within which a PDU Session Establishment REQ is expected, and if the UE does not transmit the PDU Session Establishment REQ within 8s the testacse fails. This is not in-line with TS 38.523-1 Table 10.1.8.1.3.2-1 Step 12, which expects the PDU Session Establishment REQ within 60s. This needs to be addressed |
| Summary of change | Updated the implementation of Step 12 in TTCN. Please see below |
| TTCN module | GSM_NR5GC.ttcn |
| MCCTF160 comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 11" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1, after 1 minute since end of Step 10. f_Delay(60.0); //Wait for 1 minute f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //Now trigger a PDU session WITH S-NSSAI used in Step 1 // @sic R5s230356 sic@ // @sic R5-242773 sic@ //@siclog "Step 12 - 13" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); // @sic R5s230217 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12 - 13"); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 11" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1, after 1 minute since end of Step 10. f_Delay(60.0); //Wait for 1 minute f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //Now trigger a PDU session WITH S-NSSAI used in Step 1 // @sic R5s230356 sic@ // @sic R5-242773 sic@ //@siclog "Step 12 - 13" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, 60.0); // @sic R5s230217 sic@ /*WA#WI1443578 WA#10_1_8_1*/ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12 - 13"); <<SKIPPED CODE>>

## Change 2

| Function name | f_NR5GC_PDUSessionEstablishment |
| --- | --- |
| Reason for change | As a result of change 1, the function f_NR5GC_PDUSessionEstablishment needs to be updated. |
| Summary of change | A new argument p_Timer with default set to 8.0s is added for this function. Please see below |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCCTF160 comment |  |

Before Change:

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B), // @sic R5-206290 sic@ template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_TriggerWithThisAPN := omit, boolean p_FailWhenK_LessN := true, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@ boolean p_WaitIfIMS := true, // @sic R5s210515 sic@ integer p_EstReqAlreadyInDefault := 0, boolean p_OmitDNN := false) runs on NR5GC_PTC // @sic R5-223987 sic@ { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540 if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ } <<SKIPPED CODE>>

After Change:

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B), // @sic R5-206290 sic@ template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_TriggerWithThisAPN := omit, boolean p_FailWhenK_LessN := true, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@ boolean p_WaitIfIMS := true, // @sic R5s210515 sic@ integer p_EstReqAlreadyInDefault := 0, boolean p_OmitDNN := false, float p_Timer := 8.0/*WA#WI1443578 WA#10_1_8_1*/) runs on NR5GC_PTC // @sic R5-223987 sic@ { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; timer t_Wait := p_Timer; // @sic R5-206288 sic@ 8s is less than T3540 /*WA#WI1443578 WA#10_1_8_1*/ if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ } <<SKIPPED CODE>>