3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250341

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3737 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.4.1a in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 11.4.1a to the IWD_25wk24 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.4.1a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes 2.1 and 2.2 from R5s250197 are needed in addition to the changes mentioned below. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function fl_TC_11_4_1a_TestBody	3

2.2	Correction to function f_TC_11_4_1a_NR5GC_IMS2	6

3 Branches executed	7

4	Execution Log Files	7

4.1	MediaTek MT6986D UE	7

5	References	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.4.1a.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6986D

Verification Status:	      PASS

# Corrections required

## Correction to function fl_TC_11_4_1a_TestBody

| Function name | fl_TC_11_4_1a_TestBody |
| --- | --- |
| Reason for change | At Step 2-21a1, security is not yet established because of preamble in 0-A. hence, Registration type should be Initial_NoSecurity rather than Initial_Secure. At Step 24, the current TTCN implementation calls f_NR5GC_EmergencyCallReleaseMT(nr_Cell1,-, Keep_PDUSession); As per the conformance requirements from 24.501 cl 5.6.1.2 : [TS 24.501, subclause 5.6.1.2] When an emergency PDU session is successfully established over 3GPP access after the UE received the Negotiated PEIPS assistance information IE during the last registration procedure, the UE and the AMF shall not use PEIPS assistance information until: - the successful completion of the PDU session release procedure of the emergency PDU; - the UE receives PEIPS assistance information during a registration procedure with PDU session status IE or upon successful completion of a service request procedure, if the UE or the network locally releases the emergency PDU session; - the successful completion of handover of emergency PDU session to non-3GPP access; or - the successful transfer of the emergency PDU session in 5GS to the EPS or ePDG connected to EPC. The conformance requirement clearly states PDU session release is needed. Without this, the UE shall not use the PEIPS assistance information. A Prose CR will be raised for this. |
| Summary of change | At step 2-21a1, change TypeOfRegistration_Type from Initial_Secure to Initial_NoSecurity. To completely release emergency PDU session,call f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); instead of f_NR5GC_EmergencyCallReleaseMT(nr_Cell1,-, Keep_PDUSession); |
| TTCN module | Emergency_NR5GC.ttcn |

Before Change:

function fl_TC_11_4_1a_TestBody() runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); var NAS_PlmnId v_PLMN:= f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2 - 21a1" siclog@ //The UE performs steps 1-20a1 of the RRC_IDLE procedure specified in TS 38.508-1 [4], table 4.5.2.2-2. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NRPSSI)) { //@sic R5-241653 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_Secure, NOT_SUPPORTED); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('33'O, '01'O,{{'000'B,'00001'B}})); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20 (nr_Cell1, TESTMode_OFF); //@siclog "Step 22" siclog@ //Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. (NOTE 1) f_UT_RequestIMSEmergencyCall (UT, "144"); //@siclog "Step 23" siclog@ //The UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11. f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); //@siclog "Step 24" siclog@ //The Test procedure for IMS MT Emergency call release as specified in TS 34.508-1 [4], subclause 4.9.12B takes place with "keep emergency PDU session" f_NR5GC_EmergencyCallReleaseMT(nr_Cell1,-, Keep_PDUSession); … //@siclog "Step 30" siclog@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); }

After Change:

function fl_TC_11_4_1a_TestBody() runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); var NAS_PlmnId v_PLMN:= f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2 - 21a1" siclog@ //The UE performs steps 1-20a1 of the RRC_IDLE procedure specified in TS 38.508-1 [4], table 4.5.2.2-2. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NRPSSI)) { //@sic R5-241653 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity, NOT_SUPPORTED); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('33'O, '01'O,{{'000'B,'00001'B}})); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20 (nr_Cell1, TESTMode_OFF); //@siclog "Step 22" siclog@ //Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. (NOTE 1) f_UT_RequestIMSEmergencyCall (UT, "144"); //@siclog "Step 23" siclog@ //The UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11. f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); //@siclog "Step 24" siclog@ //The Test procedure for IMS MT Emergency call release as specified in TS 34.508-1 [4], subclause 4.9.12B takes place with "keep emergency PDU session" f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); … //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); }

## Correction to function f_TC_11_4_1a_NR5GC_IMS2

| Function name | f_TC_11_4_1a_NR5GC_IMS2 |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_IMS_EmergencyCallAndRelease f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_EmergencyCallReleaseMO is called. This enforces the IMS emergency call release as per 38.508 cl 4.9.12A. However, the Prose clearly indicates that the 38.508 cl 4.9.12B is to be followed. |
| Summary of change | Re-worked the TTCN for function f_TC_11_4_1a_NR5GC_IMS2 to accommodate the 38.508 cl 4.9.12B. |
| TTCN module | Emergency_NR5GC_IMS.ttcn |

Before Change:

function f_TC_11_4_1a_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndRelease(); }

After Change:

function f_TC_11_4_1a_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyRegistration(-, -, IPCAN_E); // @sic R5-212384 sic@ f_IMS_EmergencyCallSetup_NR5GC_A_6(A_2_1_A7); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Emergency Call Established")); // @sic R5-225362 sic@ // step 24 initiate MT call release v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_EmergencyCallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); }

# 3 Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n5” with NEA2 and NIA2 ciphering and integrity algorithms.

# Execution Log Files

## MediaTek MT6986D UE

The MediaTek MT6986D UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_11_4_1a_MediaTek.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250342: Supporting information for addition of NR5GC testcase 11.4.1a in FR1 |
| --- | --- |