3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250346

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3739 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 11.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Similarly to R5s240440 there is a need to address the problem where during RRCresume procedure the RLC-ACK/ResumeComplete may be delivered via RACH. In this case the SS needs to be configured for CRNTI based RACH procedure, which is currently missing for TC 11.3.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added CRNTI based RACH procedure SS configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the case due to SS not properly configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_3_TestBody |
| --- | --- |
| Reason for change | Similarly to R5s240440 there is a need to address the problem where during RRCresume procedure the RLC-ACK/ResumeComplete may be delivered via RACH. In this case the SS needs to be configured for CRNTI based RACH procedure, which is currently missing for TC 11.3.3. |
| Summary of change | Added CRNTI based RACH procedure SS configuration. |
| TTCN module | UAC_NR5GC.ttcn |

Before Change:

function fl_TC_11_3_3_TestBody() runs on NR5GC_PTC { var float v_TimerValue1 := 60.0; timer t_WaitTime; //@siclog "Step 1" siclog@ //Wait for 5 minute to make the timer T380 expire and trigger the RNA update procedure in UE. f_Delay(f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(300))); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message with resumeCause value set to rna-Update on NR Cell 1 within 60s? t_WaitTime.start(v_TimerValue1); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 11.3.3 Step 2"); } [] t_WaitTime.timeout{} } //@siclog "Step 3" siclog@

After Change:

function fl_TC_11_3_3_TestBody() runs on NR5GC_PTC { var float v_TimerValue1 := 60.0; timer t_WaitTime; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(nr_Cell1); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def)); //@siclog "Step 1" siclog@ //Wait for 5 minute to make the timer T380 expire and trigger the RNA update procedure in UE. f_Delay(f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(300))); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message with resumeCause value set to rna-Update on NR Cell 1 within 60s? t_WaitTime.start(v_TimerValue1); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 11.3.3 Step 2"); } [] t_WaitTime.timeout{} } //@siclog "Step 3" siclog@