3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250359

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4935 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.13 (NGSO mode) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For NB-IoT NTN, in NGSO mode, the Koffset = 5 or 8. At step19 “The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms.”, which will cause the Koffset value as “zero” and cause the failure of UL sync from UE to send the message at step 21 |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 19 for NGSO mode, the differential Koffset should be corrected to not value “5”, but as value “1” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change to the f_TC_22_3_1_2_NBIOT	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_2_NBIOT

| Function name | f_TC_22_3_1_13_NBIOT () |
| --- | --- |
| Reason for change | - For NB-IoT NTN, in NGSO mode, the Koffset = 5 or 8. At step19 “The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms.”, which will cause the Koffset value as “zero” and cause the failure of UL sync from UE to sent the message at step 21. |
| Summary of change | - At step 19 for NGSO mode, the differential Koffset should be corrected to not value “5”, but as value “1” |
| TTCN module | NBIOT_MAC_NTN.ttcn |

Before Change:

function f_TC_22_3_1_13_NBIOT() runs on NBIOT_PTC {//NB-IoT / NTN / UE specific TA report / UE specific Koffset //@sic R5-241563, R5-234494 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var SubFrameTiming_Type v_TimingStart; var NB_SYSTEM_IND v_NB_SYSTEM_IND; var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) DL_CCCH_Message_NB v_RRCConnectionSetup; var integer v_Koffset; …… //@siclog "Step 18A-18J" siclog@ //Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1))/500.0); //@sic R5s240435 sic@ added delay to ensure RLC ACK is sent before moving to transparent mode //switch to L2 test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@siclog "Step 19" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, -, cs_MAC_PDU_DifferentialKoffset(int2bit(5,6))); //@sic R5s240246, ch 2.1.9 sic@ v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); v_Koffset := f_NBIOT_CellInfo_Get_Koffset(nbiot_Cell1) - 5; //@sic R5-241556 sic@ f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_Update_NTNConfig(nbiot_Cell1, v_TimingInfo, -, cs_NB_NTN_Config(cs_NB_NTN_TimingOffset (v_Koffset)))); //@sic R5s240246, ch 2.1.9, R5-241556 sic@ f_Delay(2.0); //@sic R5s240246, ch 2.1.10, R5s240435 sic@ added delay and remove activation time, as the sequence of events have to happen with immediate activation time for Step 20-21 to be sent immediately after restoring SRBs to normal mode //leave L2 test mode f_NBIOT_L2_CPMode_RestoreSRBs(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); //@sic R5s240246, ch 2.1.10, R5s240235 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //@sic R5s240246 sic@ //@siclog "Steps 20-21" siclog@ //The SS sends UECapabilityEnquiry-NB message to the UE //Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms received at step 19? f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@sic R5s240435 sic@ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); }

After Change:

function f_TC_22_3_1_13_NBIOT() runs on NBIOT_PTC {//NB-IoT / NTN / UE specific TA report / UE specific Koffset //@sic R5-241563, R5-234494 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var SubFrameTiming_Type v_TimingStart; var NB_SYSTEM_IND v_NB_SYSTEM_IND; var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) DL_CCCH_Message_NB v_RRCConnectionSetup; var integer v_Koffset; var integer v_Differential_Koffset; …… //@siclog "Step 18A-18J" siclog@ //Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1))/500.0); //@sic R5s240435 sic@ added delay to ensure RLC ACK is sent before moving to transparent mode //switch to L2 test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@siclog "Step 19" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. if(f_NBIOT_MobileInfo_GetNTNSatelliteType()==ntn_GSO) //WA#WI 1449845 { v_Differential_Koffset := 5; } if(f_NBIOT_MobileInfo_GetNTNSatelliteType()==ntn_NGSO) //WA#WI 1449845 { v_Differential_Koffset := 1; } f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, -, cs_MAC_PDU_DifferentialKoffset(int2bit(v_Differential_Koffset,6))); //@sic R5s240246, ch 2.1.9 sic@ WA#WI 1449845 v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); v_Koffset := f_NBIOT_CellInfo_Get_Koffset(nbiot_Cell1) - v_Differential_Koffset; //@sic R5-241556 sic@ //WA#WI 1449845 f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_Update_NTNConfig(nbiot_Cell1, v_TimingInfo, -, cs_NB_NTN_Config(cs_NB_NTN_TimingOffset (v_Koffset)))); //@sic R5s240246, ch 2.1.9, R5-241556 sic@ f_Delay(2.0); //@sic R5s240246, ch 2.1.10, R5s240435 sic@ added delay and remove activation time, as the sequence of events have to happen with immediate activation time for Step 20-21 to be sent immediately after restoring SRBs to normal mode //leave L2 test mode f_NBIOT_L2_CPMode_RestoreSRBs(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); //@sic R5s240246, ch 2.1.10, R5s240235 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //@sic R5s240246 sic@ //@siclog "Steps 20-21" siclog@ //The SS sends UECapabilityEnquiry-NB message to the UE //Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms received at step 19? f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@sic R5s240435 sic@ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); }