3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250375

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0986 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to tsc_ECallTestService |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As stated in RFC 3966 If the local-number format is used, it MUST be qualified by the 'phone-context' parameter. The conformant UE has to use number ”123456” (as given by EF_FDNURI on USIM) in RequestURI for the eCall, however given the RFC 3966 it must be appended with ‘phone-context’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the tsc_ECallTestService so that string with ‘phone-context’ and px_IMS_HomeDomainName is newly appended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail a PSAP callback test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.10.NR5GC,11.11.NR5GC, 11.12.NR5GC, 11.13.NR5GC and 8.1.4.1.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact: 	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Constant name | tsc_ECallTestService |
| --- | --- |
| Reason for change | As stated in RFC 3966 If the local-number format is used, it MUST be qualified by the 'phone-context' parameter. The conformant UE has to use number ”123456” (as given by EF_FDNURI on USIM) in RequestURI for the eCall, however given the RFC 3966 it must be appended with ‘phone-context’. |
| Summary of change | Changed the tsc_ECallTestService so that string with ‘phone-context’ and px_IMS_HomeDomainName is newly appended. |
| TTCN module | IMS_eCall_Functions.ttcn |

Before Change:

const charstring tsc_ECallTestService := "123456";

After Change:

const charstring tsc_ECallTestService := "123456" & ";phone-context=" & px_IMS_HomeDomainName;