3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250377

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0988 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS CEN test case 21.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 21.24 to the IMS_EUTRA test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 21.24 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1 Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Qualcomm Snapdragon Auto 5G Modem-RF	3

5	References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 21.24 which is part of the IMS_EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:		Juan Garcia Martin

Juan.garciamartin@rohde-schwarz.com

## 1.1 Verification Test Summary

Test Case: 	21.24

Test Group:                            \IMS\21\IMS_eCall_Testcases.ttcn

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	Rohde & Schwarz CMW500

UE used:	Qualcomm Snapdragon Auto 5G Modem-RF UE

Verification Status:	PASS

# Corrections required

No changes were required.

# Branches executed

The test case was executed on on EUTRA band 1 using ciphering algorithm eea1 and integrity algorithm eia1 on Qualcomm Snapdragon Auto 5G Modem-RF UE.

# Execution Log Files

## Qualcomm Snapdragon Auto 5G Modem-RF

The Qualcomm Snapdragon Auto 5G Modem-RF UE passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_21_24.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s250378: Supporting information for Addition of IMS eCall over LTE CEN test case 21.24 |
| --- | --- |