3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250440

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0992 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_ModifyPDUSession_ConfigureCallMedia |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, within the for loop the value of I is not incremented, which results in this loop running infinitely for a testcase where lengthof(p_MediaType) = 2. This needs to be fixed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the for loop implementation, please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_NR5GC_ModifyPDUSession_ConfigureCallMedia |
| --- | --- |
| Reason for change | In the current TTCN implementation, within the for loop the value of I is not incremented, which results in this loop running infinitely for a testcase where lengthof(p_MediaType) = 2. This needs to be fixed |
| Summary of change | Updated the for loop implementation, please see below. |
| TTCN module | NR5GC_IMSProcedures.ttcn |

Before Change:

<<SKIPPED CODE>> v_DRBList := f_NR_GetNextDRBIds(p_GSM_MobilityInfo, false, lengthof(p_MediaType)); // @sic R5s210877 sic@ for (i := 0; i < lengthof(p_MediaType); i := +1) { f_NR5GC_ModifyPDUSession_BuildMediaParams(v_QoS_Rule[i], v_QFI[i], v_QoSFlow[i], v_MappedEPSContext[i], p_MediaType[i], p_GSM_MobilityInfo); // build components of Reconfiguration message v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {v_QFI[i]})), v_DRBList[i], -, -, cs_38508_PDCP_Config ( -, ms80, omit)); // UM as per 38.508-1 Table 4.6.3-99: PDCP-Config @sic R5-213445 sic@ v_SS_Config[i] := f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_DRBList[i], -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {v_QFI[i]})))); v_RLCConfigList[i] := cs_38508_RLC_BearerConfig_DRB(v_DRBList[i], cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB); v_QosFlowId := cs_IP_QosFlowId(oct2int(v_SessionId), v_QFI[i]); if (match(p_MediaType[i], (Speech, Emergency))) { f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); } else { f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M2, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> v_DRBList := f_NR_GetNextDRBIds(p_GSM_MobilityInfo, false, lengthof(p_MediaType)); // @sic R5s210877 sic@ for (i := 0; i < lengthof(p_MediaType); i := i+1) { //WA#WI1454486 f_NR5GC_ModifyPDUSession_BuildMediaParams(v_QoS_Rule[i], v_QFI[i], v_QoSFlow[i], v_MappedEPSContext[i], p_MediaType[i], p_GSM_MobilityInfo); // build components of Reconfiguration message v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {v_QFI[i]})), v_DRBList[i], -, -, cs_38508_PDCP_Config ( -, ms80, omit)); // UM as per 38.508-1 Table 4.6.3-99: PDCP-Config @sic R5-213445 sic@ v_SS_Config[i] := f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_DRBList[i], -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {v_QFI[i]})))); v_RLCConfigList[i] := cs_38508_RLC_BearerConfig_DRB(v_DRBList[i], cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB); v_QosFlowId := cs_IP_QosFlowId(oct2int(v_SessionId), v_QFI[i]); if (match(p_MediaType[i], (Speech, Emergency))) { f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); } else { f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M2, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); } } <<SKIPPED CODE>>