3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250057

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4883 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcases 22.3.1.3 and 22.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For NB-IoT NTN, Koffset needs to be taken into account to ensure the RLC STATUS PDU is received by UE and then SS transmits MAC PDU.This change was covered in R5s240706 for Steps 7A/8 of test case 22.3.1.4 but is required in several places for test case 22.3.1.3 and 22.3.1.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0) After several steps in both the test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.3 and 22.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the f_TC_22_3_1_3_NBIOT	2

2.2	Change to the f_TC_22_3_1_4_NBIOT	11

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_3_NBIOT

| Function name | f_TC_22_3_1_3_NBIOT |
| --- | --- |
| Reason for change | For NB-IoT NTN, Koffset needs to be taken into account to ensure the RLC STATUS PDU is received by UE and then SS transmits MAC PDU.This change was covered in R5s240706 for Steps 7A/8 of test case 22.3.1.4 but is required in several places for test case 22.3.1.3 and 22.3.1.4 |
| Summary of change | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0) - After Step 6 - After Step 12 - After Step 16 - After Step 20A - After Step 23 |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted, but different implementation is proposed. Since the issue generally appears in a relation with MAC PDU transmissions from SS it is proposed for the sake of consistency and the maintenance simplification to update the function f_NBIOT_MAC_CPMode_SendRlcStatus with the corresponding delay for the NTN execution |

Before Change:

function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC { … … … //@siclog "Step 6" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ //@siclog "Step 7" siclog@ f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 8, 10" siclog@ v_TransRetransmissionList := { cs_NB_UL_TransRetransmission_NewTransmission, // step 8 cs_NB_UL_TransRetransmission_ReTransmission(1) // step 10 }; v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); /* @sic R5-181306: Iru := 2, Imcs := 5 => TBS=224 sic@ @sic R5-182584: Iru := 2, Imcs := 4 => TBS=208 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); // MAC PDU as expected for step 9 and 11 v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //@siclog "Step 9" siclog@ v_IncrementVRR := 0; /* AM_VRR shall not be incremented for this step as there is an adaptive HARQ restransmission at step 11 */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX, v_IncrementVRR); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Step 11" siclog@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 11"); //@siclog "Step 12" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 13" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSduList := { crs_PDCP_SDU_15B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList, tsc_Poll); //@siclog "Step 14" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 15" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_RlcAmdPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_RlcSduList); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 16" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 17" siclog@ // Void //@siclog "Step 18" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 19" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 20" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 20A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ //@siclog "Step 21" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_11B; // RLC SDU of 11 bytes results in AM RLC PDU of 13 bytes v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 22" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-2415646, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 23" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); // @sic R5s170236 change 8: noBSR sic@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Step 23A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ //@siclog "Step 24" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 25" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 26" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := (f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, twoBytePadding, anyBSR, v_MAC_SDUList_RX), f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX)); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 26"); //@siclog "Step 27" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); }

After Change:

| function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC { … … … //@siclog "Step 6" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //WA#WI=1381782 Delay shall be after Step 7A due to immediate activation time if(f_NBIOT_MobileInfo_IsNTN()) { f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ //@siclog "Step 7" siclog@ f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 8, 10" siclog@ v_TransRetransmissionList := { cs_NB_UL_TransRetransmission_NewTransmission, // step 8 cs_NB_UL_TransRetransmission_ReTransmission(1) // step 10 }; v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); /* @sic R5-181306: Iru := 2, Imcs := 5 => TBS=224 sic@ @sic R5-182584: Iru := 2, Imcs := 4 => TBS=208 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); // MAC PDU as expected for step 9 and 11 v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //@siclog "Step 9" siclog@ v_IncrementVRR := 0; /* AM_VRR shall not be incremented for this step as there is an adaptive HARQ restransmission at step 11 */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX, v_IncrementVRR); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Step 11" siclog@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 11"); //@siclog "Step 12" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 13" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSduList := { crs_PDCP_SDU_15B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList, tsc_Poll); //@siclog "Step 14" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 15" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_RlcAmdPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_RlcSduList); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 16" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 17" siclog@ // Void //@siclog "Step 18" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 19" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 20" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 20A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 21" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_11B; // RLC SDU of 11 bytes results in AM RLC PDU of 13 bytes v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 22" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-2415646, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 23" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); // @sic R5s170236 change 8: noBSR sic@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Step 23A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 24" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 25" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 26" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := (f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, twoBytePadding, anyBSR, v_MAC_SDUList_RX), f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX)); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 26"); //@siclog "Step 27" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } |  |
| --- | --- |

MCC alternative implemetation:

| function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) RLC_SDU_Type v_RlcSdu; var template (value) RLC_SDUList_Type v_RlcSduList; var EUTRA_ASN1_C_RNTI_Type v_C_RNTI; var NB_L2_DATA_IND v_NB_L2_DATA_IND; var TimingInfo_Type v_Timestamp; var SubFrameTiming_Type v_SearchSpaceForULTransmission; var template (value) TimingInfo_Type v_TimingInfo; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList; var SubFrameTiming_Type v_TimingStart; var template (present) MAC_PDU_Type v_MAC_PDU_RX; var template (present) MAC_SDUList_Type v_MAC_SDUList_RX; var template (value) NB_DciUlInfo_Type v_ULGrant; var integer v_IncrementVRR; var integer v_NpdcchBundleOffset; var boolean v_IsNTN; f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode); v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@ //------------------------------------ f_NBIOT_TestBody_Set(true, layer2); v_C_RNTI := f_NBIOT_CellInfo_GetC_RNTI(v_CellId); v_IsNTN := f_NBIOT_MobileInfo_IsNTN(); /*@sic R5s250057 sic@*/ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170236 change 2: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // NOTE: steps 1 - 6 correspond to LTE test case 7.1.4.1 //@siclog "Step 1" siclog@ v_RlcSdu := crs_PDCP_SDU_16B; // same as in TC_7_1_4_1 step 2 v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 2 - 3" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 0.5)); f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, v_TimingInfo, cds_NB_NewC_RNTI_ConfigReq(tsc_C_RNTI_Def2)); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(v_NpdcchBundleOffset))); //@sic R5s240266, R5-241564 sic@ v_ULGrant := cs_NB_DciInfo_CcchDcchDtchUL(0, 10); /* @sic R5s170970: to avoid timing issues grants shall take just one RU => 144 bits sic@ */ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(v_ULGrant, 10)), v_TimingInfo); /* 10 times grant of 144 bits */ //@siclog "Step 4" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 10.5 * int2float(v_NpdcchBundleOffset))); //@sic R5s240266, R5-241564 sic@ f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, v_TimingInfo, cds_NB_NewC_RNTI_ConfigReq(v_C_RNTI)); v_SearchSpaceForULTransmission := f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(11 * v_NpdcchBundleOffset)); //@sic R5s240266, R5-241564 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SearchSpaceForULTransmission); f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant), v_TimingInfo); // one default grant if(v_IsNTN){//@sic R5s240442, R5s25057 sic@ v_SearchSpaceForULTransmission := f_NBIOT_IncrementSearchSpace(v_SearchSpaceForULTransmission, int2float(v_NpdcchBundleOffset) - 1.0); } //@siclog "Step 5" siclog@ v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); v_NB_L2_DATA_IND := f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); v_Timestamp := v_NB_L2_DATA_IND.Common.TimingInfo; if (not f_NBIOT_TimeStamp_CheckSearchSpace(v_SearchSpaceForULTransmission, v_Timestamp.SubFrame)) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3 - MAC PDU not received within search space corresponding to UL grant of step 4"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } //@siclog "Step 6" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /*@sic R5s250057 sic@*/ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ //@siclog "Step 7" siclog@ f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 8, 10" siclog@ v_TransRetransmissionList := { cs_NB_UL_TransRetransmission_NewTransmission, // step 8 cs_NB_UL_TransRetransmission_ReTransmission(1) // step 10 }; v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); /* @sic R5-181306: Iru := 2, Imcs := 5 => TBS=224 sic@ @sic R5-182584: Iru := 2, Imcs := 4 => TBS=208 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); // MAC PDU as expected for step 9 and 11 v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //@siclog "Step 9" siclog@ v_IncrementVRR := 0; /* AM_VRR shall not be incremented for this step as there is an adaptive HARQ restransmission at step 11 */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX, v_IncrementVRR); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Step 11" siclog@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 11"); //@siclog "Step 12" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /*@sic R5s250057 sic@*/ //@siclog "Step 13" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSduList := { crs_PDCP_SDU_15B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList, tsc_Poll); //@siclog "Step 14" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3*v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 15" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_RlcAmdPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_RlcSduList); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 16" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /*@sic R5s250057 sic@*/ //@siclog "Step 17" siclog@ // Void //@siclog "Step 18" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 19" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 2, 3, v_TimingInfo); // UL grant of 176 bits (ITBS=3, IRU=2) //@siclog "Step 20" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 20A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /* @sic R5-172834/R5s170236 change, R5s250057 6 sic@ */ //@siclog "Step 21" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_11B; // RLC SDU of 11 bytes results in AM RLC PDU of 13 bytes v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 22" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-2415646, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 23" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); // @sic R5s170236 change 8: noBSR sic@ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Step 23A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /* @sic R5-172834/R5s170236, change 6, R5s250027 sic@ */ //@siclog "Step 24" siclog@ v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); // @sic R5-181306: scheduled transmission sic@ v_RlcSdu := crs_PDCP_SDU_8B; v_RlcSduList := { v_RlcSdu }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 25" siclog@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, int2float(3 * v_NpdcchBundleOffset))); // @sic R5s240266, R5-241564, R5-181306: scheduled transmission sic@ f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 4, 0, v_TimingInfo); // UL grant of 120 bits (ITBS=0, IRU=4) @sic R5s170236 change 7: IRU=4 sic@ //@siclog "Step 26" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := (f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, twoBytePadding, anyBSR, v_MAC_SDUList_RX), f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, anyBSR, v_MAC_SDUList_RX)); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 26"); //@siclog "Step 27" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172834/R5s170236 change 6 sic@ */ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } |  |
| --- | --- |

## Change to the f_TC_22_3_1_4_NBIOT

| Function name | f_TC_22_3_1_4_NBIOT () |
| --- | --- |
| Reason for change | For NB-IoT NTN, Koffset needs to be taken into account to ensure the RLC STATUS PDU is received by UE and then SS transmits MAC PDU.This change was covered in R5s240706 for Steps 7A/8 of test case 22.3.1.4 but is required in several places for test case 22.3.1.3 and 22.3.1.4 |
| Summary of change | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0) - After Step 10C - After Step 14 - After Step 18 - After Step 21A |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted, but different implementation is proposed. Since the issue generally appears in a relation with MAC PDU transmissions from SS it is proposed for the sake of consistency and the maintenance simplification to update the function f_NBIOT_MAC_CPMode_SendRlcStatus with the corresponding delay for the NTN execution, see alternative implementation. |

Before Change:

function fl_TC_22_3_1_4_RetxBSR_Timer_NpdcchPeriods(RetxBSR_Timer_NB_r13 p_RetxBSR_Timer) return integer { … … … //@siclog "Step 10C" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 11" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_10B, crs_PDCP_SDU_10B, crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 12" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170821 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 9, v_TimingInfo); /* UL grant of 296 bits (ITBS=9, IRU=1) -> 37 bytes */ //@siclog "Step 13" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 3 bytes for 2 LIs, 3 * 10 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); /* 1 byte padding, 1 byte SDU sub-header, 35 bytes MAC SDU */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 15" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170238 change 2.5: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_8B, crs_PDCP_SDU_7B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 16" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-172835/R5s170238 change 2.5: three NPDCCH cycles to give the UE time for processing the data sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 8, v_TimingInfo); /* UL grant of 256 bits (ITBS=8, IRU=1) @sic R5s170238 change 2.6: ITBS=8 sic@ */ //@siclog "Step 17" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 2 bytes for 1 LI, 8 + 7 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, zeroBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); //@siclog "Step 19" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5s170821 sic@ */ v_RlcSduList := { crs_PDCP_SDU_12B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 20" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170785: scheduled grant rather than "now" sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 0, 9, v_TimingInfo); /* UL grant of 136 bits (ITBS=9, IRU=0) */ //@siclog "Step 21" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, zeroBSR, v_MAC_SDUList_RX); /* @sic R5s170238 change 2.8 - MMC160 implementation: zeroBSR instead of anyBSR sic@ */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@siclog "Step 21A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172835/R5s170238 change 2.9 sic@ */ //@siclog "Step 21B": leave test mode and release RRC connection siclog@ f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, disable); f_NBIOT_L2_CPMode_RestoreSRBs(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start); f_NBIOT_RRC_ConnectionRelease(v_CellId); /* @sic R5-176882, R5s170821: RRC Connecntion Release sic@ */ //@siclog "Step 21C" siclog@ /* 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed: Get UE back to connected mode but with changed BSR timer values */ v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer_Step21C; v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Step21C; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_SS_ConfigRachProcedureMsg4(v_CellId, v_RrcConnSetup); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(v_CellId); /* @sic R5-176882, R5s170821: 'Generic Test Procedure acc. to 8.1.5A.2.3 (check Idle state) sic@ */ // return to test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(v_CellId, TransparentMode_RLC_MAC); /* @sic R5s180148 sic@ */ //@siclog "Step 22" siclog@ v_RlcSduList := { crs_PDCP_SDU_14B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, -, v_RlcSduList); //@siclog "Step 23 - 24" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); v_BSR_Index_Min := 1; fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); for (i := 0; i < 2; i := i + 1) { //@siclog "Step 25" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); //@sic R5s240593 sic@ v_BSR_Index_Min := 1; v_NoOfNpdcchPeriods := fl_TC_22_3_1_4_PeriodicBSR_Timer_NpdcchPeriods(v_PeriodicBSR_Timer_Step21C) + v_NpdcchBundleOffset; // @sic R5s240266, R5-241564 sic@ "+1" (Npdcch period bundle) to be sure that PeriodicBSR-Timer has been expired v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, int2float(v_NoOfNpdcchPeriods)); //@siclog "Step 26 - 27" siclog@ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 27 - " & int2str(i)); } f_NBIOT_TestBody_Set(false); //------------------------------------ // give default grant to the UE to allow transmission of pending data f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170238 change 2.11: single grant sic@ */ v_MAC_PDU_RX := ?; // any MAC PDU containing pending data f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); }

After Change:

| function fl_TC_22_3_1_4_RetxBSR_Timer_NpdcchPeriods(RetxBSR_Timer_NB_r13 p_RetxBSR_Timer) return integer { … … … //@siclog "Step 10C" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-176882, R5s170821 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 11" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_10B, crs_PDCP_SDU_10B, crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 12" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170821 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 9, v_TimingInfo); /* UL grant of 296 bits (ITBS=9, IRU=1) -> 37 bytes */ //@siclog "Step 13" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 3 bytes for 2 LIs, 3 * 10 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); /* 1 byte padding, 1 byte SDU sub-header, 35 bytes MAC SDU */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 15" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170238 change 2.5: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_8B, crs_PDCP_SDU_7B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 16" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-172835/R5s170238 change 2.5: three NPDCCH cycles to give the UE time for processing the data sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 8, v_TimingInfo); /* UL grant of 256 bits (ITBS=8, IRU=1) @sic R5s170238 change 2.6: ITBS=8 sic@ */ //@siclog "Step 17" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 2 bytes for 1 LI, 8 + 7 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, zeroBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 19" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5s170821 sic@ */ v_RlcSduList := { crs_PDCP_SDU_12B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 20" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170785: scheduled grant rather than "now" sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 0, 9, v_TimingInfo); /* UL grant of 136 bits (ITBS=9, IRU=0) */ //@siclog "Step 21" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, zeroBSR, v_MAC_SDUList_RX); /* @sic R5s170238 change 2.8 - MMC160 implementation: zeroBSR instead of anyBSR sic@ */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@siclog "Step 21A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-172835/R5s170238 change 2.9 sic@ */ if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 21B": leave test mode and release RRC connection siclog@ f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, disable); f_NBIOT_L2_CPMode_RestoreSRBs(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start); f_NBIOT_RRC_ConnectionRelease(v_CellId); /* @sic R5-176882, R5s170821: RRC Connecntion Release sic@ */ //@siclog "Step 21C" siclog@ /* 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed: Get UE back to connected mode but with changed BSR timer values */ v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer_Step21C; v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Step21C; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_SS_ConfigRachProcedureMsg4(v_CellId, v_RrcConnSetup); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(v_CellId); /* @sic R5-176882, R5s170821: 'Generic Test Procedure acc. to 8.1.5A.2.3 (check Idle state) sic@ */ // return to test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(v_CellId, TransparentMode_RLC_MAC); /* @sic R5s180148 sic@ */ //@siclog "Step 22" siclog@ v_RlcSduList := { crs_PDCP_SDU_14B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, -, v_RlcSduList); //@siclog "Step 23 - 24" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); v_BSR_Index_Min := 1; fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); for (i := 0; i < 2; i := i + 1) { //@siclog "Step 25" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); //@sic R5s240593 sic@ v_BSR_Index_Min := 1; v_NoOfNpdcchPeriods := fl_TC_22_3_1_4_PeriodicBSR_Timer_NpdcchPeriods(v_PeriodicBSR_Timer_Step21C) + v_NpdcchBundleOffset; // @sic R5s240266, R5-241564 sic@ "+1" (Npdcch period bundle) to be sure that PeriodicBSR-Timer has been expired v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, int2float(v_NoOfNpdcchPeriods)); //@siclog "Step 26 - 27" siclog@ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 27 - " & int2str(i)); } f_NBIOT_TestBody_Set(false); //------------------------------------ // give default grant to the UE to allow transmission of pending data f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170238 change 2.11: single grant sic@ */ v_MAC_PDU_RX := ?; // any MAC PDU containing pending data f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } |  |
| --- | --- |

MCC alternative implementation:

| function f_TC_22_3_1_4_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var RetxBSR_Timer_NB_r13 v_RetxBSR_Timer_Preamble := pp16; /* @sic R5-176882, R5s170821 change 7: pp4 -> pp16 sic@ */ var PeriodicBSR_Timer_NB_r13 v_PeriodicBSR_Timer := pp64; /* @sic R5-176882, R5s170821 change 7: pp4 -> pp64 sic@ */ var RetxBSR_Timer_NB_r13 v_RetxBSR_Timer_Step21C := pp64; /* @sic R5-176882, R5s170821 change 7 sic@ */ var PeriodicBSR_Timer_NB_r13 v_PeriodicBSR_Timer_Step21C := pp16; /* @sic R5-176882, R5s170821 change 7 sic@ */ var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var SubFrameTiming_Type v_SubFrameTiming; var template (value) TimingInfo_Type v_TimingInfo; var template (value) RLC_SDUList_Type v_RlcSduList; var template (value) RLC_SDUList_Type v_RlcSduList_Step5C; var template (value) RLC_SDUList_Type v_RlcSduList_Step7; var template (present) MAC_SDUList_Type v_MAC_SDUList_RX; var template (present) MAC_PDU_Type v_MAC_PDU_RX; var integer v_NoOfNpdcchPeriods; var integer v_BSR_Index_Min; var template (value) NB_RarList_Type v_NB_RarList; var template (value) NB_RachProcedureList_Type v_RachProcedureList; var float v_RetxBSRTimerValueMin; var integer i; var integer v_NpdcchBundleOffset; var float v_SchedulingOffset; var boolean v_IsNTN; v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer; v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Preamble; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup); v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@ v_IsNTN := f_NBIOT_MobileInfo_IsNTN(); if(v_IsNTN) {//@sic R5s240285, R5s240266, R5-241564, R5250057 sic@ v_SchedulingOffset := int2float(v_NpdcchBundleOffset); } else { v_SchedulingOffset := int2float(3 * v_NpdcchBundleOffset); } f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, enable); /* @sic R5-176882, R5s170821 sic@ */ //------------------------------------ f_NBIOT_TestBody_Set(true, layer2); //@siclog "Step 1 (void)" siclog@ //@siclog "Step 2" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170238 change 2.1: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_10B, crs_PDCP_SDU_10B }; v_RlcSduList_Step5C := { substr(v_RlcSduList[0], 0, 3) }; v_RlcSduList_Step7 := { substr(v_RlcSduList[0], 3, 7), v_RlcSduList[1] }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); v_BSR_Index_Min := 6; //@siclog "Step 3 - 4" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-172835/R5s170238 change 2.1: three NPDCCH cycles to give the UE time for processing the data sic@ */ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // schedule RAR and Msg4 at the SS v_NB_RarList := { cs_NB_RandomAccessResponseParameters_Def(cs_RapIdCtrl_Auto, cs_TempC_RNTI_Explicit(tsc_C_RNTI_Def2)) }; /* @sic R5-176882 change 10: different values for CRNTI and T-CRNTI sic@ */ v_RachProcedureList := { cs_NB_RachProcedure(cs_NB_RandomAccessResponseConfig_Common(v_NB_RarList), cs_NB_ContentionResolutionCtrl_CRNTI(1, 6)) }; /* (1, 6) -> 176 bits */ f_NBIOT_SS_CommonCellConfig(v_CellId, cas_NB_RachProcedure_Config_REQ(v_CellId, cs_TimingInfo_Now, -, -, v_RachProcedureList)); /* @sic R5-176882, R5s170821: RACH procedure sic@ */ //@siclog "Step 5" siclog@ v_NoOfNpdcchPeriods := fl_TC_22_3_1_4_RetxBSR_Timer_NpdcchPeriods(v_RetxBSR_Timer_Preamble); v_RetxBSRTimerValueMin := f_NBIOT_SetTimerToleranceMin(rrcTimer, int2float(v_NoOfNpdcchPeriods * f_NBIOT_SearchSpace_GetPdcchPeriod()) / 1000.0); f_Delay(v_RetxBSRTimerValueMin); //Wait for retxBSR-Timer expiry on UE side //@siclog "Step 5A" siclog@ SYSIND.receive(car_NB_RachPreamble_IND(v_CellId)); /* @sic R5-176882, R5s170821: RACH procedure sic@ */ f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 5A"); //@siclog "Step 5B" siclog@ // RAR scheduled at the SS //@siclog "Step 5C" siclog@ v_BSR_Index_Min := 4; //10-3+10=17 bytes -> Index=4 fl_TC_22_3_1_4_Step5c_ReceiveAndCheckMsg3MacPdu(v_RLC_CountsInfoListByRef, v_CellId, v_RlcSduList_Step5C, v_BSR_Index_Min); /* @sic R5-176882, R5s170821: RACH procedure sic@ */ //@siclog "Step 6" siclog@ // Msg4 scheduled at the SS: C-RNTI based contention resolution with UL grant of 176 //@siclog "Step 7" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList_Step7) }; /* @sic R5-176882, R5s170821: RACH procedure sic@ */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, noBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); //@siclog "Step 7A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /*//@sic R5s240593, R5s240706, R5s250057 sic@*/ /* @sic R5-176882, R5s170821: RLC STATUS PDU sic@ */ //@siclog "Step 8" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5-176882, R5s170821: timing sic@ */ v_RlcSduList := { crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 9 - 10" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* 3 NPDCCH cycles after sending of the additional RLC SDU at step 8 @sic R5-172835/R5s170238 change 2.3: 2 cycles instead of 1 sic@ @sic R5-176882, R5s170821: 3 cycles instead of 2 sic@ * @sic R5-241564 3 bundles for TN, 1 bundle for NTN sic@ */ v_BSR_Index_Min := 1; /* @sic R5-176882, R5s170821: 1 instead of 8 sic@ */ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 10A" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); //@sic R5-241564 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 0, 7, v_TimingInfo); /* @sic R5-176882, R5s170821: sic@ UL grant of 104 bits (ITBS=7, IRU=0) -> 13 bytes */ //@siclog "Step 10B" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 10 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, noBSR, v_MAC_SDUList_RX); /* 1 byte SDU sub-header, 12 bytes MAC SDU */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); /* @sic R5-176882, R5s170821: sic@ */ //@siclog "Step 10C" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /* @sic R5-176882, R5s170821, R5s250057 sic@ */ //@siclog "Step 11" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_10B, crs_PDCP_SDU_10B, crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); /* @sic R5-176882, R5s170821 sic@ */ //@siclog "Step 12" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170821 sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 9, v_TimingInfo); /* UL grant of 296 bits (ITBS=9, IRU=1) -> 37 bytes */ //@siclog "Step 13" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 3 bytes for 2 LIs, 3 * 10 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, singleBytePadding, noBSR, v_MAC_SDUList_RX); /* 1 byte padding, 1 byte SDU sub-header, 35 bytes MAC SDU */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /*@sic R5s250057 sic@*/ //@siclog "Step 15" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); /* @sic R5s170238 change 2.5: 200ms offset sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); v_RlcSduList := { crs_PDCP_SDU_8B, crs_PDCP_SDU_7B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 16" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-172835/R5s170238 change 2.5: three NPDCCH cycles to give the UE time for processing the data sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 1, 8, v_TimingInfo); /* UL grant of 256 bits (ITBS=8, IRU=1) @sic R5s170238 change 2.6: ITBS=8 sic@ */ //@siclog "Step 17" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; /* 2 bytes RLC AM header fixed part, 2 bytes for 1 LI, 8 + 7 bytes data */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, normalPadding, zeroBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /*@sic R5s250057 sic@*/ //@siclog "Step 19" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5s170821 sic@ */ v_RlcSduList := { crs_PDCP_SDU_12B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 20" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* @sic R5-241564, R5-176882, R5s170785: scheduled grant rather than "now" sic@ */ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); f_NBIOT_ULGrantTransmission_ExplicitGrant(v_CellId, 0, 9, v_TimingInfo); /* UL grant of 136 bits (ITBS=9, IRU=0) */ //@siclog "Step 21" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList) }; v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, zeroBSR, v_MAC_SDUList_RX); /* @sic R5s170238 change 2.8 - MMC160 implementation: zeroBSR instead of anyBSR sic@ */ f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); //@siclog "Step 21A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId, -, -, v_IsNTN); /* @sic R5-172835/R5s170238 change 2.9, R5s250057 sic@ */ //@siclog "Step 21B": leave test mode and release RRC connection siclog@ f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, disable); f_NBIOT_L2_CPMode_RestoreSRBs(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start); f_NBIOT_RRC_ConnectionRelease(v_CellId); /* @sic R5-176882, R5s170821: RRC Connecntion Release sic@ */ //@siclog "Step 21C" siclog@ /* 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed: Get UE back to connected mode but with changed BSR timer values */ v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer_Step21C; v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Step21C; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_SS_ConfigRachProcedureMsg4(v_CellId, v_RrcConnSetup); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(v_CellId); /* @sic R5-176882, R5s170821: 'Generic Test Procedure acc. to 8.1.5A.2.3 (check Idle state) sic@ */ // return to test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(v_CellId, TransparentMode_RLC_MAC); /* @sic R5s180148 sic@ */ //@siclog "Step 22" siclog@ v_RlcSduList := { crs_PDCP_SDU_14B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, -, v_RlcSduList); //@siclog "Step 23 - 24" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); v_BSR_Index_Min := 1; fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); for (i := 0; i < 2; i := i + 1) { //@siclog "Step 25" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId); //@sic R5s240593 sic@ v_BSR_Index_Min := 1; v_NoOfNpdcchPeriods := fl_TC_22_3_1_4_PeriodicBSR_Timer_NpdcchPeriods(v_PeriodicBSR_Timer_Step21C) + v_NpdcchBundleOffset; // @sic R5s240266, R5-241564 sic@ "+1" (Npdcch period bundle) to be sure that PeriodicBSR-Timer has been expired v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, int2float(v_NoOfNpdcchPeriods)); //@siclog "Step 26 - 27" siclog@ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 27 - " & int2str(i)); } f_NBIOT_TestBody_Set(false); //------------------------------------ // give default grant to the UE to allow transmission of pending data f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170238 change 2.11: single grant sic@ */ v_MAC_PDU_RX := ?; // any MAC PDU containing pending data f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_L2_Postamble_State2B_NB(v_CellId, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } |  |
| --- | --- |

## MCC160 proposed implementation

Before Change:

function f_NBIOT_MAC_CPMode_SendRlcStatus(NB_RLC_CountsInfoList_Type p_RLC_CountsInfoList, NBIOT_CellId_Type p_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) integer p_DL_TBS := omit) runs on NBIOT_PTC { /* @sic R5s170234: p_DL_TBS sic@ */ var integer v_AckSN := p_RLC_CountsInfoList.AM_VRR; var octetstring v_EncodedRlcPdu := f_RLC_Status_1PDU_Encvalue(v_AckSN); var MAC_SDUList_Type v_MAC_SDUList := { v_EncodedRlcPdu }; f_NBIOT_MAC_CPMode_SendMacPdu(p_CellId, p_TimingInfo, f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList, p_DL_TBS)); }

After Change:

| function f_NBIOT_MAC_CPMode_SendRlcStatus(NB_RLC_CountsInfoList_Type p_RLC_CountsInfoList, NBIOT_CellId_Type p_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) integer p_DL_TBS := omit, boolean p_IsNTN := false) runs on NBIOT_PTC { /* @sic R5s170234: p_DL_TBS sic@ * @sic R5s250057: p_IsNTN sic@*/ var integer v_AckSN := p_RLC_CountsInfoList.AM_VRR; var octetstring v_EncodedRlcPdu := f_RLC_Status_1PDU_Encvalue(v_AckSN); var MAC_SDUList_Type v_MAC_SDUList := { v_EncodedRlcPdu }; f_NBIOT_MAC_CPMode_SendMacPdu(p_CellId, p_TimingInfo, f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList, p_DL_TBS)); if(p_IsNTN) { /*@sic R5s250057 sic@*/ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(p_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } } |  |
| --- | --- |