3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250098

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3642 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 10.1.2.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-03-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 10.1.2.3 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 10.1.2.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 10.1.2.3	4

2.1	Change 1	4

2.2	Change 2	9

2.3	Change 3	12

2.4	Change 4	14

2.5	Change 5	16

2.6	Change 6	18

2.7	Change 7	20

2.8	Change 8	20

2.9	Change 9	22

3	Branches executed	23

4	Execution Log Files	23

4.1	Qualcomm	23

5	References	24

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 10.1.2.3 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	10.1.2.3.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# Corrections required to test case 10.1.2.3

## Change 1

| Function name | fl_TC_10_1_2_3_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation : - The variable v_PDN_Address2 is initialized based on PDN_5. This would also mean that in the testcase body, PDN_5 would then need to be started. The impact of this is that a totally new set of IP addresses will be deployed, which is unnecessary. - Function f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); is called after extracting the v_PDUSessionToModify. After this call, it is also needed to close the TCP connection on the IMS PTC. This is missing - The current TTCN implementation for build up of v_QosFlowID changes the QFI from 25. This is not specified in Prose, and this will also result in change of SDAP configuration which is not needed for any of the TPs. This change of QFI is not needed. - The PDN_1 has been stopped and a PDN with updated s the P-CSCF addresses needs to be started before transmitting the PDU Session Modification REQ. As of now, PDN_5 is being started, but as pointed out in (1) above, there is no need to use PDN_5. |
| Summary of change | - Instead of using PDN_5 to initialize v_PDN_Address2, it is proposed to reuse the PDN_1 addresses, and only update the v_PDN_Address2. PCSCF_IPAddressIPv4 and v_PDN_Address2. PCSCF_IPAddressIPv6 but assigning them values from px_IPv4_Address_Other and px_IPv6_Address_Other respectively. These PICS are already used in IMS TCs 6.2 and 6.3. - Sent a coordination message to IMS PTC to tell the IMS PTC to close the TCP connection. And then waited for a coordination message from IMS PTC to get a confirm that TCP connection is closed. - Changed the QFI from 52 while build up of v_QosFlowId - Called function f_IP_Handling_StartPDN(IP, PDN_1a, cs_DrbInfo_NR(v_NGC_NASCellA, v_QosFlowId)); before transmitting the PDU Session Modification REQ. (Please check change XX for more information on why to use PDN_1a instead of PDN_5) |
| TTCN module | GSM_NR5GC.ttcn |
| MCC160 Comment | 1. Rejected. Not neeeded 2.Accepted in principle, but sent the new PCSCF addresses instead 3.Alternative solution implemented – removed the variable 4.Not needed |

Before Change:

function fl_TC_10_1_2_3_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); //@sic R5-244198 sic@ var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var PDUSessionInfoList_Type v_PDUSessionInfoList; var GSM_MobilityInfo_Type v_PDUSessionToModify; var template (value) NAS_ExtdProtocolConfigOptions_Type v_ExtdPCO; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO_IE := omit; var template (value) QosFlow_Identification_Type v_QosFlowId; var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); var PDN_AddressInfo_Type v_PDN_Address3 := f_PDN_AddressInfo_Get(PDN_6); var octetstring v_Contents; var integer i; var integer v_Count := 0; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; //@siclog "Steps 1" siclog@ //The SS transmits a PDU SESSION MODIFICATION COMMAND message containing new P-CSCF addresses. v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionInfo := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN); v_PDUSessionToModify := valueof(v_PDUSessionInfo); // Remove PDN_1 from the routing table then add PDN_5 f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); // Build up QoSFlowId for PDN_5 v_QosFlowId := cs_IP_QosFlowId(oct2int(v_PDUSessionToModify.SessionId), 5); f_IP_Handling_StartPDN(IP, PDN_5, cs_DrbInfo_NR(v_NGC_NASCellA, v_QosFlowId)); for (i := 0; i < lengthof(v_PDUSessionToModify.PCO.pco); i := i +1) { if (v_PDUSessionToModify.PCO.pco[i].protocolID == '0001'O) { // IPv6 v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count] := cs_ProtocolContainer('0001'O, v_Contents); v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('0001'O, v_Contents); v_Count := v_Count + 2; } else if(v_PDUSessionToModify.PCO.pco[i].protocolID == '000C'O) { // IPv4 v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count] := cs_ProtocolContainer('000C'O, v_Contents); v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('000C'O, v_Contents); v_Count := v_Count + 2; } } // Convert into a sending template in TLV format v_ExtdPCO_IE := f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ExtdPCO); v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionToModify.SessionId, -, -, -, -, -, -, -, -, v_ExtdPCO_IE); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 2" siclog@ //The UE transmits a PDU SESSION MODIFICATION COMPLETE message. SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionToModify.SessionId )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } //@siclog "Steps 3-10" siclog@ //Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? //(Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registration done"); //@siclog f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS registration done");"Steps 11" siclog@ //SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA); }

After Change:

function fl_TC_10_1_2_3_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); //@sic R5-244198 sic@ var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var PDUSessionInfoList_Type v_PDUSessionInfoList; var GSM_MobilityInfo_Type v_PDUSessionToModify; var template (value) NAS_ExtdProtocolConfigOptions_Type v_ExtdPCO; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO_IE := omit; var template (value) QosFlow_Identification_Type v_QosFlowId; //WA#10_1_2_3 WA#WI1371153 COMMENTED OUT var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_1); //WA#10_1_2_3 WA#WI1371153 var PDN_AddressInfo_Type v_PDN_Address3 := f_PDN_AddressInfo_Get(PDN_6); var octetstring v_Contents; var integer i; var integer v_Count := 0; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; //@siclog "Steps 1" siclog@ //The SS transmits a PDU SESSION MODIFICATION COMMAND message containing new P-CSCF addresses. v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionInfo := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN); v_PDUSessionToModify := valueof(v_PDUSessionInfo); // Remove PDN_1 from the routing table then add PDN_5 f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //WA#10_1_2_3 WA#WI1371153 Send coord message to IMS PTC to close TCP connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Closed TCP connection"); //WA#10_1_2_3 WA#WI1371153 // Build up QoSFlowId for PDN_5 v_QosFlowId := cs_IP_QosFlowId(oct2int(v_PDUSessionToModify.SessionId), 2); //WA#10_1_2_3 WA#WI1371153 5->2 This is needed to avoid sdap reconfguration v_PDN_Address2.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; //WA#10_1_2_3 WA#WI1371153 v_PDN_Address2.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; //WA#10_1_2_3 WA#WI1371153 f_IP_Handling_StartPDN(IP, PDN_1a, cs_DrbInfo_NR(v_NGC_NASCellA, v_QosFlowId)); //WA#10_1_2_3 WA#WI1371153 PDN_5 --> PDN_1a for (i := 0; i < lengthof(v_PDUSessionToModify.PCO.pco); i := i +1) { if (v_PDUSessionToModify.PCO.pco[i].protocolID == '0001'O) { // IPv6 v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count] := cs_ProtocolContainer('0001'O, v_Contents); v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv6); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('0001'O, v_Contents); v_Count := v_Count + 2; } else if(v_PDUSessionToModify.PCO.pco[i].protocolID == '000C'O) { // IPv4 v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address2.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count] := cs_ProtocolContainer('000C'O, v_Contents); v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_Address3.PCSCF_IPAddressIPv4); v_ExtdPCO[v_Count+1] := cs_ProtocolContainer('000C'O, v_Contents); v_Count := v_Count + 2; } } // Convert into a sending template in TLV format v_ExtdPCO_IE := f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ExtdPCO); v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionToModify.SessionId, -, -, -, -, -, -, -, -, v_ExtdPCO_IE); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 2" siclog@ //The UE transmits a PDU SESSION MODIFICATION COMPLETE message. SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionToModify.SessionId )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } //@siclog "Steps 3-10" siclog@ //Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? //(Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registration done"); //@siclog f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS registration done");"Steps 11" siclog@ //SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA); }

## Change 2

| Function name | f_TC_10_1_2_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In the current TTCN implementation : - The variable v_PDN_Address2 is initialized based on PDN_5. This would also mean that in the testcase body, PDN_5 would then need to be started. The impact of this is that a totally new set of IP addresses will be deployed, which is unnecessary. - There is no call to initialize PDN_1 on IMS PTC. This needs to be implemented. Also after the initial IMS registration, on the NR PTC the PDN_1 is stopped and after that coordination message is sent by NR PTC for closing of TCP connection. This also needs to be accommodated. Please see Change 1 above, point 2. - The current TTCN implementation calls function f_IMS_Register_NonGIBA to handle the IMS re-registration after P-CSCF restoration. It is proposed to call function f_IMS_Register instead, which handles the GIBA and nonGIBA cases both. |
| Summary of change | - Instead of using PDN_5 to initialize v_PDN_Address2, it is proposed to reuse the PDN_1 addresses, and only update the v_PDN_Address2. PCSCF_IPAddressIPv4 and v_PDN_Address2. PCSCF_IPAddressIPv6 but assigning them values from px_IPv4_Address_Other and px_IPv6_Address_Other respectively. These PICS are already used in IMS TCs 6.2 and 6.3. - Waited for a coordination message from NR PTC to close TCP connection. Then called the funciton f_IMS_TCP_CloseHandling, and then called function f_IMS_IPCAN_SendCoOrdMsgto inform NR PTC that TCP connection is closed. - Instead of calling function f_IMS_Register_NonGIBA, called function f_IMS_Register. |
| TTCN module | GSM_NR5GC_IMS.ttcn |
| MCC160 Comment | 1. Alternative solution implemented – restarted the PCSCF with the addresses sent from the NR PTC 2.1. Accepted 2.2.Alternative solution implemented – called f_IMS_PTC_Reset 3.Rejected. The prose refers to 34.229-5 A.2, which does not support GIBA |

Before Change:

function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; var IP_AddrInfo_Type v_NW_Address; var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); //v_IMS_DATA_REQ := f_IMS_Register(p_IpcanType, p_IMS_DATA_REQ, p_SigCompIsActive, p_RegisterExpiration); // Now check that that the NW address of the REGISTER's RoutingInfo contains the newly allocated P-CSCF address v_NW_Address := v_IMS_DATA_REQ.RoutingInfo.NW_Address; /* at least one address check must succeed ... */ if ((ischosen(v_NW_Address.V4) and not (f_IPAddresses_Compare(v_NW_Address.V4.Addr, v_PDN_Address2.PCSCF_IPAddressIPv4))) or (ischosen(v_NW_Address.V6) and not (f_IPAddresses_Compare(v_NW_Address.V6.Addr, v_PDN_Address2.PCSCF_IPAddressIPv6)))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "P-CSCF address check failed"); } f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true); // Inform NR: IMS registration done f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registration done")); }

After Change:

import from IMS_Procedures_Common all; //WA#10_1_2_3 WA#WI1371153 import from Parameters all; //WA#10_1_2_3 WA#WI1371153 . . . function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; var IP_AddrInfo_Type v_NW_Address; //WA#10_1_2_3 WA#WI1371153 COMMENTED OUT var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_5); var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_1); //WA#10_1_2_3 WA#WI1371153 f_IMS_PTC_Init(PDN_1);//WA#10_1_2_3 WA#WI1371153 f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //WA#10_1_2_3 WA#WI1371153 f_IMS_TCP_CloseHandling(waitAndClose); //WA#10_1_2_3 WA#WI1371153 f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Closed TCP connection")); //WA#10_1_2_3 WA#WI1371153 //WA#10_1_2_3 WA#WI1371153 COMMENTED OUT v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); v_IMS_DATA_REQ := f_IMS_Register(); //WA#10_1_2_3 WA#WI1371153 v_PDN_Address2.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; //WA#10_1_2_3 WA#WI1371153 v_PDN_Address2.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; //WA#10_1_2_3 WA#WI1371153 //v_IMS_DATA_REQ := f_IMS_Register(p_IpcanType, p_IMS_DATA_REQ, p_SigCompIsActive, p_RegisterExpiration); // Now check that that the NW address of the REGISTER's RoutingInfo contains the newly allocated P-CSCF address v_NW_Address := v_IMS_DATA_REQ.RoutingInfo.NW_Address; /* at least one address check must succeed ... */ if ((ischosen(v_NW_Address.V4) and not (f_IPAddresses_Compare(v_NW_Address.V4.Addr, v_PDN_Address2.PCSCF_IPAddressIPv4))) or (ischosen(v_NW_Address.V6) and not (f_IPAddresses_Compare(v_NW_Address.V6.Addr, v_PDN_Address2.PCSCF_IPAddressIPv6)))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "P-CSCF address check failed"); } f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true); // Inform NR: IMS registration done f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registration done")); }

## Change 3

| Function name | type enumerated PDN_Index_Type |
| --- | --- |
| Reason for change | Please refer to Change 1, point 1 which states that “The variable v_PDN_Address2 is initialized based on PDN_5. This would also mean that in the testcase body, PDN_5 would then need to be started. The impact of this is that a totally new set of IP addresses will be deployed, which is unnecessary.” In addition to this when PDN_5 is started, the function a_ApplicationCtrl only starts the IP address servers for PDN_5, not the IMS servers. This causes issues to receive/send IMS messages. Also since tsc_NoOfPDNs is set to 3, there is a runtime error when PDN_5 is used. |
| Summary of change | In order to simplify the IP address handling, it is proposed that a new PDN Index Type , PDN_1a is created, which has same index as PDN_1, but for which the PCSCF_IPAddressIPV4 and PCSCF_IPAddressIPV6 would be initialized differently from PDN_1. This would be used when the P-CSCF address of the first PDN is being changed (like in 10.1.2.3 and 10.1.3.3). This new PDN_1a will then be started instead of PDN_1 in testcase 10.1.2.3, after assigning the new IP addresses for PCSCF_IPAddressIPV4 and PCSCF_IPAddressIPV6. Please refer to change 1 point 1. |
| TTCN module | CommonDefs.ttcn |
| MCC160 Comment | Alternative solution implemented – removed PDN_2b Also updated tsc_NoOfPDNs := 6 |

Before Change:

type enumerated PDN_Index_Type { /* 'index name' associated to a PDN: The major purpose is to associate a PDN specific set of IP addresses to the given PDN (e.g. UE addresses, P-CSCF address etc.); in general there are one or two PDNs configured at the same time and - from TTCN point of view - the IMS PDN is considered to be the first one; a second PDN may be configured in case of emergency call or e.g. for XCAP signalling; the 'internet PDN' (according to TS 36.508 clause 4.5.2) is considered as (optional) second PDN during initial registration and gets released after initial registration; in case of WLAN a separate group of index names is used to distinguish the different configuration of the emulated IP network */ PDN_1 (0), /* "default" PDN being kept connected to during a test case (in case of LTE in general the IMS PDN) */ PDN_2 (1), /* second PDN: during initial registration (TS 36.508 clause 4.5.2) for LTE and "multiple PDN' this is the internet PDN; after initial registration it is used if needed according to the test purpose (e.g. emergency call) */ PDN_2a, /* used for the special case when the UE IP address of the second PDN changes in a test case */ PDN_2b, /* used for the special case when the P-CSCF address of the second PDN changes in a test case */ PDN_Internet, /* mainly used as alias for PDN2 during initial registration */ ePDG_IMS1, /* WLAN: PDN for 'normal' IMS */ ePDG_IMS2, /* WLAN: PDN for emergency IMS (in general) */ ePDG_XCAP, /* WLAN: PDN for XCAP in case of XCAP server being part of 3GPP-network NOTE: In contrast to LTE for WLAN there is a different IP architecture to be consider by TTCN for XCAP and IMS emergency */ ePDG_Internet, /* place-holder for WLAN-offload scenarios */ nrPDN_Internet, /* mainly used as alias during initial registration in ENDC @sic R5s191074 sic@ */ PDN_3 (2), /* used in ENDC and 5GC for the third PDN @sic R5s191074 sic@ */ PDN_4 (3), /* used in 5GC for the URLLC PDN @sic R5-221467 sic@ */ PDN_5 (4), /* used in 5GC for the MIoT PDN @sic R5-221467 sic@ */ PDN_6 (5) /* used in 5GC for the V2X PDN @sic R5-221467 sic@ */ };

After Change:

type enumerated PDN_Index_Type { /* 'index name' associated to a PDN: The major purpose is to associate a PDN specific set of IP addresses to the given PDN (e.g. UE addresses, P-CSCF address etc.); in general there are one or two PDNs configured at the same time and - from TTCN point of view - the IMS PDN is considered to be the first one; a second PDN may be configured in case of emergency call or e.g. for XCAP signalling; the 'internet PDN' (according to TS 36.508 clause 4.5.2) is considered as (optional) second PDN during initial registration and gets released after initial registration; in case of WLAN a separate group of index names is used to distinguish the different configuration of the emulated IP network */ PDN_1 (0), /* "default" PDN being kept connected to during a test case (in case of LTE in general the IMS PDN) */ PDN_1a, /* used for the special case when the UE IP address of the First PDN changes in a test case WA#10_1_2_3 WA#WI1371153 */ PDN_2 (1), /* second PDN: during initial registration (TS 36.508 clause 4.5.2) for LTE and "multiple PDN' this is the internet PDN; after initial registration it is used if needed according to the test purpose (e.g. emergency call) */ PDN_2a, /* used for the special case when the UE IP address of the second PDN changes in a test case */ PDN_2b, /* used for the special case when the P-CSCF address of the second PDN changes in a test case */ PDN_Internet, /* mainly used as alias for PDN2 during initial registration */ ePDG_IMS1, /* WLAN: PDN for 'normal' IMS */ ePDG_IMS2, /* WLAN: PDN for emergency IMS (in general) */ ePDG_XCAP, /* WLAN: PDN for XCAP in case of XCAP server being part of 3GPP-network NOTE: In contrast to LTE for WLAN there is a different IP architecture to be consider by TTCN for XCAP and IMS emergency */ ePDG_Internet, /* place-holder for WLAN-offload scenarios */ nrPDN_Internet, /* mainly used as alias during initial registration in ENDC @sic R5s191074 sic@ */ PDN_3 (2), /* used in ENDC and 5GC for the third PDN @sic R5s191074 sic@ */ PDN_4 (3), /* used in 5GC for the URLLC PDN @sic R5-221467 sic@ */ PDN_5 (4), /* used in 5GC for the MIoT PDN @sic R5-221467 sic@ */ PDN_6 (5) /* used in 5GC for the V2X PDN @sic R5-221467 sic@ */ };

## Change 4

| Function name | a_ApplicationCtrl |
| --- | --- |
| Reason for change | Please refer to Change 1 Point 1, Change 3 above. The current implementation needs to be extended to accommodate PDN_1a. |
| Summary of change | Updated the TTCN to accommodate PDN_1a. |
| TTCN module | IP_PTC_Main.ttcn |
| MCC160 Comment | Alternative solution implemented – removed PDN_2b |

Before Change:

<<SKIPPED CODE>> if (ischosen(v_IP_ConfigReq.StartPDN)) { v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId; v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo; f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); select (v_PdnIndex) { case (PDN_1, PDN_2, PDN_2a, PDN_2b) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); fl_IP_ImsServer_Start(v_PdnIndex); } case (PDN_Internet, nrPDN_Internet, PDN_3, PDN_4, PDN_5, PDN_6) { // @sic R5s191074, R5-221467 sic@ fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); } case (ePDG_IMS1, ePDG_IMS2) { fl_IP_ImsServer_Start(v_PdnIndex); } case (ePDG_XCAP) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly); } case (ePDG_Internet) { // nothing to be done } } } else if (ischosen(v_IP_ConfigReq.StopPDN)) { <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (ischosen(v_IP_ConfigReq.StartPDN)) { v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId; v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo; f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); select (v_PdnIndex) { case (PDN_1, PDN_2, PDN_2a, PDN_2b, PDN_1a) { //WA#10_1_2_3 WA#WI1371153 fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); fl_IP_ImsServer_Start(v_PdnIndex); } case (PDN_Internet, nrPDN_Internet, PDN_3, PDN_4, PDN_5, PDN_6) { // @sic R5s191074, R5-221467 sic@ fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); } case (ePDG_IMS1, ePDG_IMS2) { fl_IP_ImsServer_Start(v_PdnIndex); } case (ePDG_XCAP) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly); } case (ePDG_Internet) { // nothing to be done } } } else if (ischosen(v_IP_ConfigReq.StopPDN)) { <<SKIPPED CODE>>

## Change 5

| Function name | f_PDN_PdnIndex2Integer |
| --- | --- |
| Reason for change | Please refer to Change 1 Point 1, Change 3 above. The current implementation needs to be extended to accommodate PDN_1a. |
| Summary of change | Updated the TTCN to accommodate PDN_1a. |
| TTCN module | CommonIP.ttcn |
| MCC160 Comment | Alternative solution implemented – removed PDN_2b |

Before Change:

function f_PDN_PdnIndex2Integer(PDN_Index_Type p_PdnIndex) return integer { /* PDN_2a to support change of the UE's IP address for IMS test case 19.5.1 */ /* PDN_2b to support change of the P-CSCF address for IMS test case 11.4.13 */ /* @sic R5s150031: fl_IP_ServerIndex -> f_PDN_PdnIndex2Integer sic@ */ var integer v_Index; select (p_PdnIndex) { case (PDN_1, ePDG_IMS1) { v_Index := 0; } case (PDN_2, PDN_2a, PDN_2b, PDN_Internet, nrPDN_Internet, ePDG_IMS2, ePDG_XCAP, ePDG_Internet) { // @sic R5s191074, R5-241531 sic@ v_Index := 1; } case (PDN_3) { // @sic R5s191074 sic@ v_Index := 2; } case (PDN_4) { // @sic R5-221467 sic@ v_Index := 3; } case (PDN_5) { // @sic R5-221467 sic@ v_Index := 4; } case (PDN_6) { // @sic R5-221467 sic@ v_Index := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid PDN index"); } } return v_Index; }

After Change:

function f_PDN_PdnIndex2Integer(PDN_Index_Type p_PdnIndex) return integer { /* PDN_2a to support change of the UE's IP address for IMS test case 19.5.1 */ /* PDN_2b to support change of the P-CSCF address for IMS test case 11.4.13 */ /* @sic R5s150031: fl_IP_ServerIndex -> f_PDN_PdnIndex2Integer sic@ */ var integer v_Index; select (p_PdnIndex) { case (PDN_1, ePDG_IMS1, PDN_1a /*WA#10_1_2_3 WA#WI1371153*/) { v_Index := 0; } case (PDN_2, PDN_2a, PDN_2b, PDN_Internet, nrPDN_Internet, ePDG_IMS2, ePDG_XCAP, ePDG_Internet) { // @sic R5s191074, R5-241531 sic@ v_Index := 1; } case (PDN_3) { // @sic R5s191074 sic@ v_Index := 2; } case (PDN_4) { // @sic R5-221467 sic@ v_Index := 3; } case (PDN_5) { // @sic R5-221467 sic@ v_Index := 4; } case (PDN_6) { // @sic R5-221467 sic@ v_Index := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid PDN index"); } } return v_Index; }

## Change 6

| Function name | f_PDN_AddressInfo_Get |
| --- | --- |
| Reason for change | Please refer to Change 1 Point 1, Change 3 above. The current implementation needs to be extended to accommodate PDN_1a. |
| Summary of change | Updated the TTCN to accommodate PDN_1a. |
| TTCN module | CommonIP.ttcn |
| MCC160 Comment | Alternative solution implemented – removed PDN_2b |

Before Change:

function f_PDN_AddressInfo_Get(PDN_Index_Type p_PdnIndex) return PDN_AddressInfo_Type { /* PDN_2a to support change of the IP address for IMS test case 19.5.1 */ /* PDN_2b to support change of the P-CSCF address for IMS test case 11.4.13 */ var integer v_Index := f_PDN_PdnIndex2Integer(p_PdnIndex); var PDN_AddressInfo_Type v_PDN_AddressInfo := tsc_PDN_AddressInfo[v_Index]; if (p_PdnIndex == PDN_2a) { v_PDN_AddressInfo.UE_IPAddressIPv4 := px_IPv4_Address2a_UE; /* @sic implementation of IMS TC_19_5_1 sic@ */ v_PDN_AddressInfo.UE_IPAddressIPv6 := px_IPv6_Address2a_UE; /* @sic implementation of IMS TC_19_5_1 sic@ */ } if (p_PdnIndex == PDN_2b) { if (f_IPAddresses_Compare(v_PDN_AddressInfo.PCSCF_IPAddressIPv4, px_IPv4_Address3_NW) or f_IPAddresses_Compare(v_PDN_AddressInfo.PCSCF_IPAddressIPv6, px_IPv6_Address3_NW)) { FatalError(__FILE__, __LINE__, "P-CSCF addresses for PDN_2 and PDN_2b shall be different"); } v_PDN_AddressInfo.PCSCF_IPAddressIPv4 := px_IPv4_Address3_NW; /* @sic R5-241531: implementation of IMS TC_11_4_13 sic@ */ v_PDN_AddressInfo.PCSCF_IPAddressIPv6 := px_IPv6_Address3_NW; /* @sic R5-241531: implementation of IMS TC_11_4_13 sic@ */ } return v_PDN_AddressInfo; }

After Change:

function f_PDN_AddressInfo_Get(PDN_Index_Type p_PdnIndex) return PDN_AddressInfo_Type { /* PDN_2a to support change of the IP address for IMS test case 19.5.1 */ /* PDN_2b to support change of the P-CSCF address for IMS test case 11.4.13 */ var integer v_Index := f_PDN_PdnIndex2Integer(p_PdnIndex); var PDN_AddressInfo_Type v_PDN_AddressInfo := tsc_PDN_AddressInfo[v_Index]; if (p_PdnIndex == PDN_2a) { v_PDN_AddressInfo.UE_IPAddressIPv4 := px_IPv4_Address2a_UE; /* @sic implementation of IMS TC_19_5_1 sic@ */ v_PDN_AddressInfo.UE_IPAddressIPv6 := px_IPv6_Address2a_UE; /* @sic implementation of IMS TC_19_5_1 sic@ */ } if (p_PdnIndex == PDN_2b) { if (f_IPAddresses_Compare(v_PDN_AddressInfo.PCSCF_IPAddressIPv4, px_IPv4_Address3_NW) or f_IPAddresses_Compare(v_PDN_AddressInfo.PCSCF_IPAddressIPv6, px_IPv6_Address3_NW)) { FatalError(__FILE__, __LINE__, "P-CSCF addresses for PDN_2 and PDN_2b shall be different"); } v_PDN_AddressInfo.PCSCF_IPAddressIPv4 := px_IPv4_Address3_NW; /* @sic R5-241531: implementation of IMS TC_11_4_13 sic@ */ v_PDN_AddressInfo.PCSCF_IPAddressIPv6 := px_IPv6_Address3_NW; /* @sic R5-241531: implementation of IMS TC_11_4_13 sic@ */ } if (p_PdnIndex == PDN_1a) { //WA#10_1_2_3 WA#WI1371153 v_PDN_AddressInfo.PCSCF_IPAddressIPv4 := px_IPv4_Address_Other; v_PDN_AddressInfo.PCSCF_IPAddressIPv6 := px_IPv6_Address_Other; } return v_PDN_AddressInfo; }

## Change 7

| Function name | module NR_Parameters |
| --- | --- |
| Reason for change | As a result of change 6 above, the PIXITS px_IPv4_Address_Other and px_IPv6_Address_Other were needed to be moved to module Parameters. |
| Summary of change | Moved the PIXITS px_IPv4_Address_Other and px_IPv6_Address_Other to module Parameters |
| TTCN module | NR_Parameters.ttcn |
| MCC160 Comment | Not needed |

Before Change:

<<SKIPPED CODE>> modulepar charstring px_IPv4_Address_Other; // @desc Other IPv4 Address // (see NOTE 4) modulepar charstring px_IPv6_Address_Other; // @desc Other IPv6 Address // (see NOTE 4) <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //WA#10_1_2_3 WA#WI1371153 moved to Parameters modulepar charstring px_IPv4_Address_Other; // @desc Other IPv4 Address // (see NOTE 4) //WA#10_1_2_3 WA#WI1371153 moved to Parameters modulepar charstring px_IPv6_Address_Other; // @desc Other IPv6 Address // (see NOTE 4) <<SKIPPED CODE>>

## Change 8

| Function name | module Parameters |
| --- | --- |
| Reason for change | As a result of change 6 above, the PIXITS px_IPv4_Address_Other and px_IPv6_Address_Other were needed to be moved to module Parameters. |
| Summary of change | Moved the PIXITS px_IPv4_Address_Other and px_IPv6_Address_Other to module Parameters |
| TTCN module | Parameters.ttcn |
| MCC160 Comment | Not needed |

Before Change:

<<SKIPPED CODE>> modulepar charstring px_IPv6_Address_HomeAgent; // @desc IPv6 Home Address Agent modulepar integer px_MFBI_UTRAN_FrequencyBand; // @desc A supported UTRAN MFBI frequency band. // Reference to other Spec: 34.123-3, Table B.1.20 <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> modulepar charstring px_IPv6_Address_HomeAgent; // @desc IPv6 Home Address Agent modulepar charstring px_IPv4_Address_Other; // @desc Other IPv4 Address // (see NOTE 4) //WA#10_1_2_3 WA#WI1371153 moved from NR_Parameters modulepar charstring px_IPv6_Address_Other; // @desc Other IPv6 Address // (see NOTE 4) //WA#10_1_2_3 WA#WI1371153 moved from NR_Parameters modulepar integer px_MFBI_UTRAN_FrequencyBand; // @desc A supported UTRAN MFBI frequency band. // Reference to other Spec: 34.123-3, Table B.1.20 <<SKIPPED CODE>>

## Change 9

| Function name | module IMS_NR5GC_RegistrationTestcases |
| --- | --- |
| Reason for change | As a result of change 7 and 8 above, the module Parameters needs to be imported in module IMS_NR5GC_RegistrationTestcases to avoid compilation errors. |
| Summary of change | Imported module Parameters into module IMS_NR5GC_RegistrationTestcases |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment | Not needed |

Before Change:

module IMS_NR5GC_RegistrationTestcases { import from CommonDefs all; import from UpperTesterFunctions all; import from LibSip_SIPTypesAndValues all; import from IMS_CommonDefs all; import from IMS_ASP_TypeDefs all; import from IMS_ASP_Templates all; import from IMS_Component all; import from IMS_IP_Config all; import from IMS_SIP_Templates all; import from IMS_CommonFunctions all; import from IMS_Procedures_Common all; import from IMS_Procedures_Registration all; import from IMS_Procedures_RegistrationCommon all; import from IMS_XML_Templates all; import from IMS_CC_CommonFunctions all; import from IMS_PTC_CoordMsg all; import from IMS_CommonParameters all; import from IMS_CC_Authentication all; import from NR_Parameters all; import from EUTRA_NR_Parameters all; <<SKIPPED CODE>>

After Change:

module IMS_NR5GC_RegistrationTestcases { import from CommonDefs all; import from UpperTesterFunctions all; import from LibSip_SIPTypesAndValues all; import from IMS_CommonDefs all; import from IMS_ASP_TypeDefs all; import from IMS_ASP_Templates all; import from IMS_Component all; import from IMS_IP_Config all; import from IMS_SIP_Templates all; import from IMS_CommonFunctions all; import from IMS_Procedures_Common all; import from IMS_Procedures_Registration all; import from IMS_Procedures_RegistrationCommon all; import from IMS_XML_Templates all; import from IMS_CC_CommonFunctions all; import from IMS_PTC_CoordMsg all; import from IMS_CommonParameters all; import from IMS_CC_Authentication all; import from NR_Parameters all; import from EUTRA_NR_Parameters all; import from Parameters all; //WA#WI1371153 <<SKIPPED CODE>>

  - Additional Change

| Function name | f_TC_10_1_2_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | A new initial IMS registration happens between steps 2 and 3-10, this needs to be addressed, as IMS information and parameters. |
| Summary of change | Added function f_IMS_GlobalRelease() after step 2, to reset IMS information stored in IMS global variables. |
| TTCN module | NR5GC/10_1/GSM_NR5GC_IMS.ttcn |
| MCC160 Comment | Alternative solution implemented, see below |

Before Change:

function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; var IP_AddrInfo_Type v_NW_Address; var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_1); f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); f_IMS_TCP_CloseHandling(waitAndClose); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Closed TCP connection")); v_IMS_DATA_REQ := f_IMS_Register(); <<SKIPPED CODE>>

After Change:

function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; var IP_AddrInfo_Type v_NW_Address; var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(PDN_1); f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); f_IMS_TCP_CloseHandling(waitAndClose); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Closed TCP connection")); f_IMS_GlobalRelease(); v_IMS_DATA_REQ := f_IMS_Register(); <<SKIPPED CODE>>

#### MCC160 Implementation

const integer tsc_NoOfPDNs := 6;    /* @sic R5s191074 sic@ */

const integer tsc_NoOfIMSPDNs := 2; /* @sic R5s201177 sic@ */

const integer tsc_Index_IMS1 := 0;

const integer tsc_Index_IMS2 := 1;

type enumerated PDN_Index_Type {   /* 'index name' associated to a PDN:

The major purpose is to associate a PDN specific set of IP addresses to the given PDN (e.g. UE addresses, P-CSCF address etc.);

in general there are one or two PDNs configured at the same time and - from TTCN point of view - the IMS PDN is considered to be the first one;

a second PDN may be configured in case of emergency call or e.g. for XCAP signalling;

the 'internet PDN' (according to TS 36.508 clause 4.5.2) is considered as (optional) second PDN during initial registration and gets released after initial registration;

in case of WLAN a separate group of index names is used to distinguish the different configuration of the emulated IP network */

PDN_1 (0),       /* "default" PDN being kept connected to during a test case (in case of LTE in general the IMS PDN) */

PDN_2 (1),       /* second PDN: during initial registration (TS 36.508 clause 4.5.2) for LTE and "multiple PDN' this is the internet PDN;

after initial registration it is used if needed according to the test purpose (e.g. emergency call) */

PDN_2a,          /* used for the special case when the UE IP address of the second PDN changes in a test case */

PDN_Internet,    /* mainly used as alias for PDN2 during initial registration */

ePDG_IMS1,       /* WLAN: PDN for 'normal' IMS */

ePDG_IMS2,       /* WLAN: PDN for emergency IMS (in general) */

ePDG_XCAP,       /* WLAN: PDN for XCAP in case of XCAP server being part of 3GPP-network

NOTE: In contrast to LTE for WLAN there is a different IP architecture to be consider by TTCN for XCAP and IMS emergency */

ePDG_Internet,   /* place-holder for WLAN-offload scenarios */

nrPDN_Internet,  /* mainly used as alias during initial registration in ENDC  @sic R5s191074 sic@ */

PDN_3 (2),       /* used in ENDC and 5GC for the third PDN  @sic R5s191074 sic@ */

PDN_4 (3),       /* used in 5GC for the URLLC PDN  @sic R5-221467 sic@ */

PDN_5 (4),       /* used in 5GC for the MIoT PDN  @sic R5-221467 sic@ */

PDN_6 (5)        /* used in 5GC for the V2X PDN  @sic R5-221467 sic@ */

};

function f_PDN_PdnIndex2Integer(PDN_Index_Type p_PdnIndex) return integer

{ /* PDN_2a to support change of the UE's IP address for IMS test case 19.5.1 */

/* PDN_2b to support change of the P-CSCF address for IMS test case 11.4.13 */

/* @sic R5s150031: fl_IP_ServerIndex -> f_PDN_PdnIndex2Integer sic@ */

var integer v_Index;

select (p_PdnIndex) {

case (PDN_1, ePDG_IMS1) {

v_Index := 0;

}

case (PDN_2, PDN_2a, PDN_Internet, nrPDN_Internet, ePDG_IMS2, ePDG_XCAP, ePDG_Internet) { // @sic R5s191074, R5-241531 sic@

v_Index := 1;

}

case (PDN_3) { // @sic R5s191074 sic@

v_Index := 2;

}

case (PDN_4) { // @sic R5-221467 sic@

v_Index := 3;

}

case (PDN_5) { // @sic R5-221467 sic@

v_Index := 4;

}

case (PDN_6) { // @sic R5-221467 sic@

v_Index := 5;

}

case else {

FatalError(__FILE__, __LINE__, "Invalid PDN index");

}

}

return v_Index;

}

function f_PDN_AddressInfo_Get(PDN_Index_Type p_PdnIndex) return PDN_AddressInfo_Type

{ /* PDN_2a to support change of the IP address for IMS test case 19.5.1 */

/* PDN_2b to support change of the P-CSCF address for IMS test case 11.4.13 */

var integer v_Index := f_PDN_PdnIndex2Integer(p_PdnIndex);

var PDN_AddressInfo_Type v_PDN_AddressInfo := tsc_PDN_AddressInfo[v_Index];

if (p_PdnIndex == PDN_2a) {

v_PDN_AddressInfo.UE_IPAddressIPv4 := px_IPv4_Address2a_UE;   /* @sic implementation of IMS TC_19_5_1 sic@ */

v_PDN_AddressInfo.UE_IPAddressIPv6 := px_IPv6_Address2a_UE;   /* @sic implementation of IMS TC_19_5_1 sic@ */

}

return v_PDN_AddressInfo;

}

altstep a_ApplicationCtrl(IP_PTC_CTRL_PORT p_CtrlPort) runs on IP_PTC

{

var IP_ConfigReq_Type v_IP_ConfigReq;

var PDN_Index_Type v_PdnIndex;

var IP_DrbInfo_Type v_DrbInfo;

var template (omit) IP_DrbInfo_Type v_CurrentDrbInfo;

var integer i;

[] p_CtrlPort.receive(cr_IP_CtrlAnyReq) -> value v_IP_ConfigReq {

if (ischosen(v_IP_ConfigReq.StartPDN)) {

v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId;

v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo;

f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo);

select (v_PdnIndex) {

case (PDN_1, PDN_2, PDN_2a) {

fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo);

fl_IP_ImsServer_Start(v_PdnIndex);

}

case (PDN_Internet, nrPDN_Internet, PDN_3, PDN_4, PDN_5, PDN_6) { // @sic R5s191074, R5-221467 sic@

fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo);

}

case (ePDG_IMS1, ePDG_IMS2) {

fl_IP_ImsServer_Start(v_PdnIndex);

}

case (ePDG_XCAP) {

fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly);

}

case (ePDG_Internet) {

// nothing to be done

}

}

...

function f_GetNewPCSCFaddr(PDN_Index_Type p_NewPdnIndex,

PDN_Index_Type p_ExistingPdnIndex := PDN_1) return CharStringList_Type

{

var CharStringList_Type v_AddrList := {};

var PDN_AddressInfo_Type v_PDN_Address1 := f_PDN_AddressInfo_Get(p_ExistingPdnIndex);

var PDN_AddressInfo_Type v_PDN_Address2 := f_PDN_AddressInfo_Get(p_NewPdnIndex);

v_AddrList[0] := v_PDN_Address2.PCSCF_IPAddressIPv4;

v_AddrList[1] := v_PDN_Address2.PCSCF_IPAddressIPv6;

if (f_IPAddresses_Compare(v_PDN_Address1.PCSCF_IPAddressIPv4, v_AddrList[0]) or

f_IPAddresses_Compare(v_PDN_Address1.PCSCF_IPAddressIPv6, v_AddrList[1]) ) {

return {};

}

return v_AddrList;

}

function f_NR5GC_GetPCO_NewPCSCFaddr(IMS_IPCAN_CO_ORD_PORT p_ImsPort,

ExtdProtocolConfigOptions p_PcoFromUE,

PDN_Index_Type p_PDN_SendAddrtoIMS,

template (omit) PDN_Index_Type p_ThirdPDNaddr,

PDN_Index_Type p_ExistingPdnIndex := PDN_1) runs on NR5GC_PTC return template (omit) ExtdProtocolConfigOptions

{

var CharStringList_Type v_AddrList_2 := {};

var CharStringList_Type v_AddrList_3 := {};

var template (value) NAS_ExtdProtocolConfigOptions_Type v_ExtdPCO;

var boolean v_ThirdAddr := false;

var octetstring v_Contents;

var integer i;

var integer v_Count := 0;

v_AddrList_2 := f_GetNewPCSCFaddr(p_PDN_SendAddrtoIMS, p_ExistingPdnIndex);

if (lengthof(v_AddrList_2) == 0) {

return omit;

}

f_IMS_IPCAN_SendCoOrdMsg(p_ImsPort, cms_IPCAN_IMS_Data(v_AddrList_2[0])); // IPv4

f_IMS_IPCAN_SendCoOrdMsg(p_ImsPort, cms_IPCAN_IMS_Data(v_AddrList_2[1])); // IPv6

if (isvalue(p_ThirdPDNaddr)) {

v_ThirdAddr := true;

v_AddrList_3 := f_GetNewPCSCFaddr(valueof(p_ThirdPDNaddr), p_ExistingPdnIndex);

if (lengthof(v_AddrList_3) == 0) {

return omit;

}

// Also check addresses 2 & 3 are different

if (f_IPAddresses_Compare(v_AddrList_2[0], v_AddrList_3[0]) or

f_IPAddresses_Compare(v_AddrList_2[1], v_AddrList_3[1]) ) {

return omit;

}

}

for (i := 0; i < lengthof(p_PcoFromUE.pco); i := i +1) {

if (p_PcoFromUE.pco[i].protocolID == '0001'O) { // IPv6

v_Contents := f_Convert_IPv6Addr2OctString(v_AddrList_2[1]);

v_ExtdPCO[v_Count] := cs_ProtocolContainer('0001'O, v_Contents);

v_Count := v_Count + 1;

if (v_ThirdAddr) {

v_Contents := f_Convert_IPv6Addr2OctString(v_AddrList_3[1]);

v_ExtdPCO[v_Count] := cs_ProtocolContainer('0001'O, v_Contents);

v_Count := v_Count + 1;

}

} else if(p_PcoFromUE.pco[i].protocolID == '000C'O) { // IPv4

v_Contents := f_Convert_IPv4Addr2OctString(v_AddrList_2[0]);

v_ExtdPCO[v_Count] := cs_ProtocolContainer('000C'O, v_Contents);

if (v_ThirdAddr) {

v_Contents := f_Convert_IPv4Addr2OctString(v_AddrList_3[0]);

v_ExtdPCO[v_Count] := cs_ProtocolContainer('000C'O, v_Contents);

v_Count := v_Count + 1;

}

}

}

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Address changed"); // @sic R5s250098 sic@

// Convert into a sending template in TLV format

return f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ExtdPCO);

}

import from NR5GC_IMSProcedures all;

...

function fl_TC_10_1_2_3_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); //@sic R5-244198 sic@

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo;

var PDUSessionInfoList_Type v_PDUSessionInfoList;

var GSM_MobilityInfo_Type v_PDUSessionToModify;

var template (omit) NAS_ExtdProtocolConfigOptions v_ExtdPCO;

var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2;

//@siclog "Steps 1" siclog@

//The SS transmits a PDU SESSION MODIFICATION COMMAND message containing new P-CSCF addresses.

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

v_PDUSessionInfo := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN);

v_PDUSessionToModify := valueof(v_PDUSessionInfo);

v_ExtdPCO := f_NR5GC_GetPCO_NewPCSCFaddr(v_PDUSessionToModify.PCO, PDN_5, PDN_6);

if (not isvalue(v_ExtdPCO)) {

FatalError (__FILE__, __LINE__, "PCSCF addresses shall be different");

}

v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionToModify.SessionId, -, -, -, -, -, -, -, -, v_ExtdPCO);

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

//@siclog "Step 2" siclog@

//The UE transmits a PDU SESSION MODIFICATION COMPLETE message.

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionToModify.SessionId )) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2");

}

//@siclog "Steps 3-10" siclog@

//Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses?

//(Steps 1-8 of Annex A.2 in TS 34.229-5 [41])

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registration done");

//@siclog  f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS registration done");"Steps 11" siclog@

//SS transmits an RRCRelease message.

f_NR_RRCRelease(v_NGC_NASCellA);

}

function f_IMS_NewPCSCFaddr(IMS_IPCAN_CO_ORD_PORT p_Port,

PDN_Index_Type p_ExistingPdnIndex := PDN_1) runs on IMS_PTC

{

var charstring v_IPv4_Address2;

var charstring v_IPv6_Address2;

v_IPv4_Address2 := f_IMS_IPCAN_WaitForData(p_Port);

v_IPv6_Address2 := f_IMS_IPCAN_WaitForData(p_Port);

f_IMS_PTC_Reset();

f_IMS_RestartPCSCF(p_ExistingPdnIndex, v_IPv4_Address2, v_IPv6_Address2);

f_IMS_IPCAN_SendCoOrdMsg(p_Port, cms_IMS_IPCAN_Trigger("Address changed"));

}

function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC

{ //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration

var IMS_DATA_REQ v_IMS_DATA_REQ;

var IP_AddrInfo_Type v_NW_Address;

f_IMS_PTC_Init();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_Registration();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_NewPCSCFaddr(IPCAN);

v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true);

f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// Inform NR: IMS registration done

f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registration done"));

}

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA3 ciphering algorithms.

# Execution Log Files

## Qualcomm

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_10_1_2_3_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s250099: Supporting information for addition of NR5GC testcase 10.1.2.3 in FR1 |
| --- | --- |