3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250100

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3643 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 7.1.2.3.9a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 7.1.2.3.9a to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.2.3.9a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.9a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required to test case 7.1.2.3.9a	5

2.1	Change 1	5

2.2	Change 2	6

2.3	Change 3	7

2.4	Change 4	8

2.5	Change 5	10

2.6	Change 6	11

2.7	Change 7	12

2.8	Change 8	12

2.9	Change 9	13

2.10	Change 10	14

2.11	Change 11	15

2.12	Change 12	16

2.13	Change 13	19

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.2.3.9a which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.2.3.9a.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

# Corrections required to testcase 7.1.2.3.9a

## Change 1

| Function name | f_NR5GC_CellInfo_SetSIB19 |
| --- | --- |
| Reason for change | According to TS 38.508-1 Table 4.4.3.1.2-1, The NR-28 combination currently lists only SIB1 and SIB19. However, the way in which the TTCN architecture is done, this results in a RUNTIME exception in the function f_NR_GetSIB1_SiSchedulingInfo. Furthermore, according to TS 38.331 section 5.2.2.1, it is stated as below: “The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED. The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB19 (if UE is accessing NR via NTN access). The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20 and SIB21, regardless of the RRC state the UE is in. The UE shall ensure having a valid version of the posSIB requested by upper layers.” Hence, for the NR-28 combination, SIB2 is added. |
| Summary of change | For NR-28 combination, add SIB2 in the system information list, correction the index of SIB19 for NR-28 combination to overcome the RUNTIME exception. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Not needed. NR-29 combination will be used instead NR-28 combination. Agreed as per prose CR R5-252350. function f_NR5GC_CellInfo_SetSIB19 will be modified accordingly as shown below. As per R5-253084. |

Before Change:

function f_NR5GC_CellInfo_SetSIB19(NR_CellId_Type p_NR_CellId, template (value) SIB19_r17 p_Sib19) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (f_NR5GC_IsSib19InSysinfoCombination(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700 := valueof(p_Sib19); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } };

After Change:

function f_NR5GC_CellInfo_SetSIB19(NR_CellId_Type p_NR_CellId, template (value) SIB19_r17 p_Sib19) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (f_NR5GC_IsSib19InSysinfoCombination(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700 := valueof(p_Sib19); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } };

MCC160 Implementation:

function f_NR5GC_CellInfo_SetSIB19(NR_CellId_Type p_NR_CellId, template (value) SIB19_r17 p_Sib19) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB19_SI_Index := f_NR_GetSIB19_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB19_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19 := valueof(p_Sib19); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

## Change 2

| Function name | f_NR_GetSIB19_SI_Index |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 in the system information list, correction the index of SIB19 for NR-28 combination. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Not needed. NR-29 combination will be used instead NR-28 combination. Agreed as per prose CR R5-252350. function f_NR_GetSIB19_SI_Index will be modified accordingly as shown below. As per R5-253084. |

Before Change:

function f_NR_GetSIB19_SI_Index (NR_SysinfoCombination_Type p_NR_SysinfoCombination) return integer { var integer v_SIB19_SI_Index; select (p_NR_SysinfoCombination) { case (NR_28) { v_SIB19_SI_Index := 0; } case (NR_29) { v_SIB19_SI_Index := 1; } case (NR_32) { v_SIB19_SI_Index := 2; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } return v_SIB19_SI_Index; }

After Change:

function f_NR_GetSIB19_SI_Index (NR_SysinfoCombination_Type p_NR_SysinfoCombination) return integer { var integer v_SIB19_SI_Index; select (p_NR_SysinfoCombination) { case (NR_28) { v_SIB19_SI_Index := 1; } case (NR_29) { v_SIB19_SI_Index := 1; } case (NR_32) { v_SIB19_SI_Index := 2; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } return v_SIB19_SI_Index; }

MCC160 Implementation:

function f_NR_GetSIB19_SI_Index (NR_SysinfoCombination_Type p_NR_SysinfoCombination) return integer { var integer v_SIB19_SI_Index; select (p_NR_SysinfoCombination) { case (NR_28) { v_SIB19_SI_Index := 0; } case (NR_29) { v_SIB19_SI_Index := 1; } case (NR_32) { v_SIB19_SI_Index := 2; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } return v_SIB19_SI_Index; }

## Change 3

| Function name | f_NR_CellInfo_GetSIB19 |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 in the system information list, correction the index of SIB19 for NR-28 combination. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Not needed. See changes 1 and 2 |

Before Change:

function f_NR_CellInfo_GetSIB19(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return SIB19_r17 { <<SKIPPED CODE>> select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_28) { v_SIB19 := v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700; } <<SKIPPED CODE>> }

After Change:

function f_NR_CellInfo_GetSIB19(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return SIB19_r17 { <<SKIPPED CODE>> select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_28) { v_SIB19 := v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700; } <<SKIPPED CODE>> }

## Change 4

| Function name | f_NR_InitSystemInformation |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | Add SIB2 message in SIB1 system list for the NR-28 combination. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment | Not needed. See changes 1 and 2 |

Before Change:

function f_NR_InitSystemInformation(NR_SysinfoCombination_Type p_NR_SysinfoCombination, NR_FrequencyBand_Type p_NR_FrequencyBand, SubcarrierSpacing p_SubcarrierSpacing, template (value) NR_NAS_Parameters_Type p_NAS_Parameters, template (value) NR_DownlinkBWP_List_Type p_DL_BWPs, template (omit) NR_UplinkBWP_List_Type p_UL_BWPs, integer p_DeltaNRFs, SSB_Index p_SSB_Index, NR_CellId_Type p_NR_CellId ) runs on NR_BASE_PTC return template (value) NR_CellSysInfo_Type { <<SKIPPED CODE>> case(NR_28){ //@sic R5-232436 sic@ //SIB1 with cellBarredNTN-r17 = notBarred v_SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.cellBarredNTN_r17 := notBarred; //SI1: SIB19 v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB19(f_NR_InitSIB19()))); v_SI_List := {v_BCCH_SI1}; } <<SKIPPED CODE>> }

After Change:

function f_NR_InitSystemInformation(NR_SysinfoCombination_Type p_NR_SysinfoCombination, NR_FrequencyBand_Type p_NR_FrequencyBand, SubcarrierSpacing p_SubcarrierSpacing, template (value) NR_NAS_Parameters_Type p_NAS_Parameters, template (value) NR_DownlinkBWP_List_Type p_DL_BWPs, template (omit) NR_UplinkBWP_List_Type p_UL_BWPs, integer p_DeltaNRFs, SSB_Index p_SSB_Index, NR_CellId_Type p_NR_CellId ) runs on NR_BASE_PTC return template (value) NR_CellSysInfo_Type { <<SKIPPED CODE>> case(NR_28){ //@sic R5-232436 sic@ //SIB1 with cellBarredNTN-r17 = notBarred v_SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.cellBarredNTN_r17 := notBarred; //SI1: SIB2 v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB2(v_SIB2))); //SI2: SIB19 v_BCCH_SI2 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB19(f_NR_InitSIB19()))); v_SI_List := {v_BCCH_SI1, v_BCCH_SI2}; } <<SKIPPED CODE>> }

## Change 5

| Function name | fl_NR_InitSI_SchedulingInfo_v1700 |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 in the SIB1 scheduling list, so correction to the index for SIB19. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment | Not needed. See changes 1 and 2 |

Before Change:

function fl_NR_InitSI_SchedulingInfo_v1700(NR_SysinfoCombination_Type p_NR_SysinfoCombination) return template (omit) SI_SchedulingInfo_v1700 { <<SKIPPED CODE>> case(NR_28) { v_SchedulingInfoList2 := { cs_SchedulingInfo2_r17(1, tsc_SIB19_Periodicity, {cs_SIB_TypeInfo_Type1(sibType19)})}; v_SI_SchedulingInfo_v1700 := cs_SI_SchedulingInfo_v1700_Def(v_SchedulingInfoList2); } <<SKIPPED CODE>> }

After Change:

function fl_NR_InitSI_SchedulingInfo_v1700(NR_SysinfoCombination_Type p_NR_SysinfoCombination) return template (omit) SI_SchedulingInfo_v1700 { <<SKIPPED CODE>> case(NR_28) { v_SchedulingInfoList2 := { cs_SchedulingInfo2_r17(2, tsc_SIB19_Periodicity, {cs_SIB_TypeInfo_Type1(sibType19)})}; //WA# v_SI_SchedulingInfo_v1700 := cs_SI_SchedulingInfo_v1700_Def(v_SchedulingInfoList2); } <<SKIPPED CODE>> }

## Change 6

| Function name | fl_NR_InitSI_SchedulingInfo |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | For NR-28 combination, add SIB2 message into SIB1 schedulinginfolist. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment | Not needed. See changes 1 and 2 |

Before Change:

function fl_NR_InitSI_SchedulingInfo(NR_SysinfoCombination_Type p_NR_SysinfoCombination, FreqBandIndicatorNR p_FreqBandIndicatorNR) return template (omit) SI_SchedulingInfo { <<SKIPPED CODE>> case(NR_28) { // SIB19 scheduled in si-SchedulingInfo-v1700 - done inside fl_NR_InitSI_SchedulingInfo_v1700 } <<SKIPPED CODE>> }

After Change:

function fl_NR_InitSI_SchedulingInfo(NR_SysinfoCombination_Type p_NR_SysinfoCombination, FreqBandIndicatorNR p_FreqBandIndicatorNR) return template (omit) SI_SchedulingInfo { <<SKIPPED CODE>> case(NR_28) { // SIB19 scheduled in si-SchedulingInfo-v1700 - done inside fl_NR_InitSI_SchedulingInfo_v1700 v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB2_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType2)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } <<SKIPPED CODE>> }

## Change 7

| Function name | cs_NTN_SemistaticTA_GSO_Cell1 |
| --- | --- |
| Reason for change | According to the chapter 7.5.2.2 in 38.523-3, the NtnUeSpecificTA of GSO NTN Cell1 should be 59771544, but the current TTCN configure it with wrong value. |
| Summary of change | Follow the chapter 7.5.2.2 in 38.523-3, change the NtnUeSpecificTA of GSO NTN Cell1 from 7476932 to 59771544 |
| TTCN module | NR_NTN_Configuration.ttcn |
| MCC160 Comment | Accepted. As per R5w250105r1. |

Before Change:

//@sic R5-243597 sic@ template (value) NR_NTN_SemistaticTA_Type cs_NTN_SemistaticTA_GSO_Cell1 := cs_NTN_SemistaticTA_Def(65896378,7476932); /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

After Change:

//@sic R5-243597 sic@ template (value) NR_NTN_SemistaticTA_Type cs_NTN_SemistaticTA_GSO_Cell1 := cs_NTN_SemistaticTA_Def(65896378,59771544); //WI=1299522/* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

## Change 8

| Function name | cs_38508_UE_TimersAndConstants |
| --- | --- |
| Reason for change | For the current TTCN implementation, the value of t300 is ms1000, it is not suitable for the NR NTN GSO scenario, it does not take the NR NTN RTT(550ms) into account after the UE sends RRCSetupRequest and wait for RRCSetup message. |
| Summary of change | Enlarge the t300 from ms1000 to ms2000. A Prose CR will be raised for this. |
| TTCN module | NR_SysInfo_Templates.ttcn |
| MCC160 Comment | Accepted in principle per prose CR agreement R5-253086. MCC160 implementation see below. |

Before Change:

template (value) UE_TimersAndConstants cs_38508_UE_TimersAndConstants := { // Acc to 38.508-1 Table 4.6.3-143AE: UE-TimersAndConstants /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ t300 := ms1000, t301 := ms1000, t310 := ms1000, n310 := n1, t311 := ms30000, n311 := n1, t319 := ms1000 };

After Change:

template (value) UE_TimersAndConstants cs_38508_UE_TimersAndConstants := { // Acc to 38.508-1 Table 4.6.3-143AE: UE-TimersAndConstants /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ t300 := ms2000, t301 := ms1000, t310 := ms1000, n310 := n1, t311 := ms30000, n311 := n1, t319 := ms1000 };

MCC160 Implementation:

template (value) UE_TimersAndConstants cds_38508_UE_TimersAndConstants_NTN :=

{ // UE-TimersAndConstants for TC 7.1.2.3.9a

t300 := ms2000

};

template (value) SIB1 cs_38508_SIB1_Def(template (value) PLMN_IdentityInfo          p_PLMN_IdentityInfo,

template (omit)  SI_SchedulingInfo          p_SI_SchedulingInfo,

template (value) DownlinkConfigCommonSIB    p_DownlinkConfigCommonSIB,

template (omit)  UplinkConfigCommonSIB      p_UplinkConfigCommonSIB,

template (omit)  TDD_UL_DL_ConfigCommon     p_TDD_UL_DL_ConfigCommon,

template (value) ServingCellConfigCommonSIB.ssb_PositionsInBurst    p_Ssb_PositionsInBurst,

Q_RxLevMin p_Q_RxLevMin,

template (omit) Q_RxLevMin p_Q_RxLevMinSUL := omit,

template (omit) Q_QualMin  p_Q_QualMin := omit,

template (omit) SIB1.eCallOverIMS_Support p_ECallOverIMS_Support := omit,

template (omit) NPN_IdentityInfoList_r16 p_NPN_IdentityInfoList := omit,

template (omit) SIB1.cellAccessRelatedInfo.cellReservedForOtherUse p_CellReservedForOtherUse := omit,

template (omit) SIB1_v1610_IEs p_SIB1_v1610 := omit,

template (omit) ServingCellConfigCommonSIB.channelAccessMode_r16 p_ChannelAccessMode_r16 := omit,

template (value) UE_TimersAndConstants p_UE_TimersAndConstants := cs_38508_UE_TimersAndConstants

) :=

{ //Acc to 38.508-1 Table 4.6.1-28: SIB1

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

// @sic R5-216327: added p_ECallOverIMS_Support R5-227518: added p_SIB1_v1610 sic@

cellSelectionInfo         := cs_38508_SIB1_CellSelectionInfo(p_Q_RxLevMin, p_Q_RxLevMinSUL, p_Q_QualMin),

cellAccessRelatedInfo     := cs_38508_CellAccessRelatedInfo(p_PLMN_IdentityInfo, p_NPN_IdentityInfoList, p_CellReservedForOtherUse),

connEstFailureControl     := cs_38508_ConnEstFailureControl,

si_SchedulingInfo         := p_SI_SchedulingInfo,

servingCellConfigCommon   := cs_38508_ServingCellConfigCommonSIB(p_DownlinkConfigCommonSIB,

p_UplinkConfigCommonSIB,

p_TDD_UL_DL_ConfigCommon,

p_Ssb_PositionsInBurst,

p_ChannelAccessMode_r16),

ims_EmergencySupport      := true_,  //@sic R5-206304 sic@

eCallOverIMS_Support      := p_ECallOverIMS_Support,

ue_TimersAndConstants     := p_UE_TimersAndConstants,

uac_BarringInfo           := omit,

useFullResumeID           := omit,

lateNonCriticalExtension  := omit,

nonCriticalExtension      := p_SIB1_v1610

};

function fl_NR_InitSIB1(NR_SysinfoCombination_Type                 p_NR_SysinfoCombination,

NR_FrequencyBand_Type                      p_NR_FrequencyBand,

SubcarrierSpacing                          p_SubcarrierSpacing,

template (value) NR_NAS_Parameters_Type    p_NAS_Parameters,

template (omit) NR_DownlinkBWP_List_Type   p_DL_BWPs,

template (omit) NR_UplinkBWP_List_Type     p_UL_BWPs,

integer                                    p_DeltaNRFs,

SSB_Index                                  p_SSB_Index,

NR_CellId_Type                             p_NR_CellId

) runs on NR_BASE_PTC return template (value) BCCH_DL_SCH_Message

{ //@sic R5-187745 sic@

var template (value) BCCH_DL_SCH_Message v_SIB1;

var template (omit) SI_SchedulingInfo v_SI_SchedulingInfo := fl_NR_InitSI_SchedulingInfo(p_NR_SysinfoCombination, p_NR_FrequencyBand.FrequencyBandIndicator);

var template (omit) SI_SchedulingInfo_v1700 v_SI_SchedulingInfo_v1700 := fl_NR_InitSI_SchedulingInfo_v1700(p_NR_SysinfoCombination); //@sic R5-233379 sic@

var template (omit) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon := fl_NR_InitTDD_UL_DL_ConfigCommon( p_NR_FrequencyBand.FrequencyBandIndicator, p_SubcarrierSpacing);

var template (value) DownlinkConfigCommonSIB v_DownlinkConfigCommonSIB;

var template (omit) UplinkConfigCommonSIB v_UplinkConfigCommonSIB := omit;

var boolean v_IsFR1 := f_NR_IsBandFR1(p_NR_FrequencyBand.FrequencyBandIndicator);

var P_Max v_P_Max := f_NR_InitP_Max();

var template (value) FrequencyInfoDL_SIB v_FrequencyInfoDL_SIB;

var template (value) FrequencyInfoUL_SIB v_FrequencyInfoUL;

var NR_DownlinkBWP_Type v_DL_BWP := f_NR_RetrieveDownlinkBWP(valueof(p_DL_BWPs), tsc_NR_BWP_Id);

var template (omit) NR_UplinkBWP_Type v_UL_BWP := omit;

var template (value) BWP_DownlinkCommon v_BWP_DownlinkCommon := f_NR_RetrieveBWP_DownlinkCommon(v_DL_BWP, p_NR_FrequencyBand.Coreset0_Index, true);

var template (value) ServingCellConfigCommonSIB.ssb_PositionsInBurst v_Ssb_PositionsInBurst;

var Q_RxLevMin v_Q_RxLevMin;       //@sic R5-197243 sic@

var template (omit) Q_QualMin  v_Q_QualMin := omit;       //@sic R5-197243 sic@

var template (omit) SIB1.eCallOverIMS_Support v_ECallOverIMS_Support := omit; //@sic R5-216327 sic@

var template (omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList := f_NR_InitNPN_IdentityInfoList(p_NR_CellId, p_NAS_Parameters);

var template (omit) SIB1.cellAccessRelatedInfo.cellReservedForOtherUse v_CellReservedForOtherUse := f_NR_InitCellReservedForOtherUse(); //@sic R5-225299 sic@

var template (omit) SIB1_v1610_IEs v_SIB1_v1610 := omit;                             //@sic R5-227518 sic@

var template (omit) RedCap_ConfigCommonSIB_r17 v_RedCap_ConfigCommonSIB_r17 := omit; //@sic R5-227518 sic@

var template (omit) ServingCellConfigCommonSIB.channelAccessMode_r16 v_ChannelAccessMode_r16 := omit;

var NTN_Satellite_Type v_SatelliteType := f_NR_MobileInfo_GetNTNSatelliteType();  //@sic R5s250100 sic@

var template (value) UE_TimersAndConstants v_UE_TimersAndConstants := cs_38508_UE_TimersAndConstants; //@sic R5s250100 sic@

if (v_IsFR1) {

v_Ssb_PositionsInBurst := cs_NR_SIB_Ssb_PositionInBurst_FR1(f_NR_ConvertSSB_IndexToBit(p_SSB_Index,8));

v_Q_RxLevMin := -55;

} else {

v_Ssb_PositionsInBurst := cs_NR_SIB_Ssb_PositionInBurst_FR2(f_NR_ConvertSSB_IndexToBit(p_SSB_Index,8));

// FR2 q-RxLevMin = ROUND((-110+Delta(NRfs))/2) @sic R5-197243 sic@

v_Q_RxLevMin := f_NR_CalculateQ_RxLevMin(-110.0, p_DeltaNRFs);

}

if (f_GetTestcaseAttrib_Qbased_Rsrq(testcasename())) {

v_Q_QualMin := -20;

}

v_FrequencyInfoDL_SIB := cs_38508_FrequencyInfoDL_SIB(cs_38508_NR_MultiBandInfo( p_NR_FrequencyBand.FrequencyBandIndicator),  // Table 4.6.5-0H: MultiFrequencyBandListNR-SIB

p_NR_FrequencyBand.Dl_OffsetToPointA,

p_NR_FrequencyBand.FrequencyInfoDL.scs_SpecificCarrierList);

//@sic R5-240531, R5w240005 sic@ - SharedSpectrum condition

if (f_NR_IsBandSharedSpectrum(p_NR_FrequencyBand.FrequencyBandIndicator)) {

if (pc_ul_Semi_StaticChAccess_r16) {

v_ChannelAccessMode_r16 := cs_38508_ChannelAccessMode_SemiStatic_SIB; // @sic R5-242772 sic@

}

else if (pc_ul_DynamicChAccess_r16){

v_ChannelAccessMode_r16 := cs_38508_ChannelAccessMode_Dynamic_SIB;  // @sic R5-242772 sic@

}

else {

FatalError(__FILE__, __LINE__, "neither semiStatic nor dynamic shared spectrum channel access are supported");

}

}

v_DownlinkConfigCommonSIB := cs_38508_DownlinkConfigCommonSIB(v_FrequencyInfoDL_SIB,

v_BWP_DownlinkCommon,

cs_38508_BCCH_Config,

cs_38508_PCCH_Config

);

if (f_NR_IsBandFDD( p_NR_FrequencyBand.FrequencyBandIndicator)) {  //@sic R5-194805 sic@

v_FrequencyInfoUL := cs_38508_FrequencyInfoUL_SIB({cs_38508_NR_MultiBandInfo( p_NR_FrequencyBand.FrequencyBandIndicator)},

p_NR_FrequencyBand.FrequencyInfoUL.absoluteFrequencyPointA,

p_NR_FrequencyBand.FrequencyInfoUL.scs_SpecificCarrierList,

v_P_Max);

} else {//@sic R5-194805 sic@

v_FrequencyInfoUL := cs_38508_FrequencyInfoUL_SIB(omit,

omit,

p_NR_FrequencyBand.FrequencyInfoUL.scs_SpecificCarrierList,

v_P_Max);

}

if (isvalue(p_UL_BWPs)) {

v_UL_BWP := f_NR_RetrieveUplinkBWP(valueof(p_UL_BWPs), tsc_NR_BWP_Id);

v_UplinkConfigCommonSIB := cs_38508_UplinkConfigCommonSIB(v_FrequencyInfoUL, f_NR_RetrieveBWP_UplinkCommon(valueof(v_UL_BWP)));

}

if (f_GetTestcaseAttrib_ECallOverIMS(testcasename())) {  //@sic R5-216327 sic@

v_ECallOverIMS_Support := true_;

}

if (pc_supportOfRedCap_r17 or pc_supportOfERedCap_r18) {  //@sic R5-227518 R5-247239 sic@

if (pc_halfDuplexFDD_TypeA_RedCap_r17) { //@sic R5-233406 sic@

v_RedCap_ConfigCommonSIB_r17 := cs_RedCap_ConfigCommonSIB(true_);

}

v_SIB1_v1610 := cs_SIB1_v1610_IEs_Def( omit, cs_SIB1_v1630_IEs_Def(cs_SIB1_v1700_IEs_Def(v_RedCap_ConfigCommonSIB_r17, allowed, v_SI_SchedulingInfo_v1700)));

} else {

if (isvalue(v_SI_SchedulingInfo_v1700)) {

v_SIB1_v1610 := cs_SIB1_v1610_IEs_Def(omit, cs_SIB1_v1630_IEs_Def(cs_SIB1_v1700_IEs_Def(omit, omit, v_SI_SchedulingInfo_v1700)));

}

}

if (pc_supportOfERedCap_r18) {  //@sic R5-245624 sic@

if (isvalue(v_SIB1_v1610)) {

v_SIB1_v1610.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := cs_SIB1_v1740_IEs_Def( -, cs_SIB1_v1800_IEs_Def( omit, omit, allowed));

} else {

v_SIB1_v1610 := cs_SIB1_v1610_IEs_Def(omit, cs_SIB1_v1630_IEs_Def(cs_SIB1_v1700_IEs_Def(-, -, -, -, -, cs_SIB1_v1740_IEs_Def( -, cs_SIB1_v1800_IEs_Def( omit, omit, allowed)))));

}

}

select (v_SatelliteType) {

case (ntn_GSO) {v_UE_TimersAndConstants := cds_38508_UE_TimersAndConstants_NTN};

}

v_SIB1 := cs_NR_BCCH_DL_SCH_SIB1(cs_38508_SIB1_Def(cs_NR_PLMN_IdentityInfo(p_NAS_Parameters.PLMN_Identity,

p_NAS_Parameters.TrackingAreaCode,

p_NAS_Parameters.Ranac,

p_NAS_Parameters.CellIdentity),

v_SI_SchedulingInfo,

v_DownlinkConfigCommonSIB,

v_UplinkConfigCommonSIB,

v_TDD_UL_DL_ConfigCommon,

v_Ssb_PositionsInBurst,

v_Q_RxLevMin,

omit,

v_Q_QualMin,

v_ECallOverIMS_Support,

v_NPN_IdentityInfoList,

v_CellReservedForOtherUse,

v_SIB1_v1610,

v_ChannelAccessMode_r16,

v_UE_TimersAndConstants

));

return v_SIB1;

}

## Change 9

| Function name | cs_38508_UL_RLC_SRB () |
| --- | --- |
| Reason for change | For the current TTCN implementation,configure the t_PollRetransmit to ms45 for SRB of UL RLC AM, it is not suitable for the NR NTN GSO scenario, it does not take the NR NTN RTT(550ms) into account. |
| Summary of change | Enlarge the t_PollRetransmit from ms45 to ms2000. Please see change 12, point 2. A Prose CR will be raised for this. |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment | Accepted in principle conditional to prose CR agreement R5-253216 on 38.523-3. MCC160 implementation see below. |

Before Change:

template (value) UL_AM_RLC cs_38508_UL_RLC_SRB := { //As per 38.331 cl 9.2.1 /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ sn_FieldLength := size12, t_PollRetransmit := ms45, pollPDU := infinity_, pollByte := infinity_, maxRetxThreshold := t8 };

After Change:

template (value) UL_AM_RLC cs_38508_UL_RLC_SRB := { //As per 38.331 cl 9.2.1 /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ sn_FieldLength := size12, t_PollRetransmit := ms2000, pollPDU := infinity_, pollByte := infinity_, maxRetxThreshold := t8 };

MCC160 Implementation:

/*

* @desc      Function returning T_PollRetransmit as according to TS 38.331 cl 9.2.1 or TS 38.523-3 for NTN GSO FFS

*/

function f_NR_GetDefaultT_PollRetransmit ()runs on NR_BASE_PTC return T_PollRetransmit

{ //@sic R5s250100 sic@

var NTN_Satellite_Type v_SatelliteType := f_NR_MobileInfo_GetNTNSatelliteType();

var T_PollRetransmit v_T_PollRetransmit := ms45;    //Default value as per 38.331 cl 9.2.1 for SRB1/2/3

if (v_SatelliteType == ntn_GSO) { v_T_PollRetransmit := ms2000 }

return v_T_PollRetransmit

}

function f_NR_SS_ConfigureSRBs(NR_CellId_Type p_NR_CellId,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now) runs on NR_BASE_PTC

{

var T_PollRetransmit v_T_PollRetransmit := f_NR_GetDefaultT_PollRetransmit();         //@sic R5s250100 sic@

var template (value) NR_RadioBearerList_Type v_SrbList := {

cs_NR_SS_SRB0_Config,

cs_NR_SS_SRB1_Config(v_T_PollRetransmit),

cs_NR_SS_SRB2_Config(v_T_PollRetransmit)

};

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SrbList, p_TimingInfo);

}

function f_NR_SS_SRBs_DRBs_Config(NR_CellId_Type  p_NR_CellId,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (omit) NR_RadioBearerList_Type p_DrbConfigList,

template (omit) boolean p_CnfFlag := omit,

boolean p_FollowOnFlag := tsc_NoFollowOnFlag

) runs on NR_BASE_PTC

{ //@sic R5s220176: added p_FollowOnFlag sic@

var template (value) NR_RadioBearerList_Type v_DrbConfigList;

var T_PollRetransmit v_T_PollRetransmit := f_NR_GetDefaultT_PollRetransmit();         //@sic R5s250100 sic@

var template (value) NR_RadioBearerList_Type v_SrbDrbList := {

cs_NR_SS_SRB1_Config(v_T_PollRetransmit),

cs_NR_SS_SRB2_Config(v_T_PollRetransmit)

};

var integer i := lengthof(v_SrbDrbList);

var integer k;

if (ispresent(p_DrbConfigList)) {

v_DrbConfigList := valueof(p_DrbConfigList);

for (k:=0; k < lengthof(v_DrbConfigList); k:= k+1) {

v_SrbDrbList[i+k] := v_DrbConfigList[k];

}

}

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SrbDrbList, p_TimingInfo, p_CnfFlag, p_FollowOnFlag);

}

template (value) UL_AM_RLC cs_38508_UL_RLC_SRB(T_PollRetransmit p_T_PollRetransmit := ms45) :=

{ //As per 38.331 cl 9.2.1

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

//@sic R5s250100 added p_T_PollRetransmit sic@

sn_FieldLength    := size12,

t_PollRetransmit  := p_T_PollRetransmit,

pollPDU           := infinity_,

pollByte          := infinity_,

maxRetxThreshold  := t8

};

template (value) NR_RlcBearerConfig_Type cs_NR_RlcBearerConfig_SRB1SRB2SRB3(NR_LogicalChannelId_Type p_LogicalChannelId,

integer p_LogicalChannelPriority,

template (omit) NR_RLC_TestModeConfig_Type p_TestModeConfig := cs_NR_RLC_TestModeConfig_None, //@sic R5s210283 Ch. 2 sic@

T_PollRetransmit p_T_PollRetransmit := ms45

) :=

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

//@sic R5s250100: added p_T_PollRetransmit sic@

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(cs_38508_UL_RLC_SRB(p_T_PollRetransmit), cs_38508_DL_RLC_SRB, p_TestModeConfig),

p_LogicalChannelId,

cs_NR_SS_MAC_Configuration(-, p_LogicalChannelPriority));

template (value) NR_RadioBearer_Type cs_NR_SS_SRB1_Config (T_PollRetransmit p_T_PollRetransmit := ms45):=

//Acc to 38.331 cl 9.2.1

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

//@sic R5s250100: added p_T_PollRetransmit sic@

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1,

cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB),

cs_NR_RlcBearerConfig_SRB1SRB2SRB3(tsc_NR_SRB1_LogicalChannelId, tsc_NR_SRB1_LogicalChannelPriority, -, p_T_PollRetransmit),

cs_SDAP_Configuration_None);

template (value) NR_RadioBearer_Type cs_NR_SS_SRB2_Config(T_PollRetransmit p_T_PollRetransmit := ms45) :=

//Acc to 38.331 cl 9.2.1

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

//@sic R5s250100: added p_T_PollRetransmit sic@

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB2,

cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB),

cs_NR_RlcBearerConfig_SRB1SRB2SRB3(tsc_NR_SRB2_LogicalChannelId, tsc_NR_SRB2_LogicalChannelPriority, -, p_T_PollRetransmit),

cs_SDAP_Configuration_None);

## Change 10

| Function name | cs_NR_PeriodicityAndOffsetSl640 () |
| --- | --- |
| Reason for change | For the current TTCN implementation,there is no suitable PeriodicityAndOffset settings for NR NTN testing. |
| Summary of change | For NR NTN GSO test, add a new template, cs_NR_PeriodicityAndOffsetSl640(). Please see change 11 below. A Prose CR will be raised for this. |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment | Accepted in principle per prose CR agreement R5-253086. |

New Add:

template (value) SchedulingRequestResourceConfig.periodicityAndOffset cs_NR_PeriodicityAndOffsetSl640 := { sl640 := 639 }; //WI=1398470

## Change 11

| Function name | f_NR_ReturnPeriodicityAndOffset () |
| --- | --- |
| Reason for change | For NR NTN GSO scenario,the value of PeriodicityAndOffset for kHz15 need to take the NR NTN RTT(550ms) into account. |
| Summary of change | For NR NTN GSO test, enlarge the PeriodicityAndOffset from sl10 to sl640. |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment | Accepted in principle per prose CR agreement R5-253086. |

Before Change:

function f_NR_ReturnPeriodicityAndOffset(SubcarrierSpacing p_SubcarrierSpacing) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset { var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset; //As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig select (p_SubcarrierSpacing) { case(kHz15) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl10; } case(kHz30) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20; } case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80; } case else { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } return v_PeriodicityAndOffset; }

After Change:

function f_NR_ReturnPeriodicityAndOffset(SubcarrierSpacing p_SubcarrierSpacing) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset { var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset; //As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig select (p_SubcarrierSpacing) { case(kHz15) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl640; } case(kHz30) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20; } case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80; } case else { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } return v_PeriodicityAndOffset; }

MCC160 Implementation:

template (value) SchedulingRequestResourceConfig.periodicityAndOffset cs_NR_PeriodicityAndOffsetSl640 := { sl640 := 639 }; /* @sic R5-250100 sic@ */

function f_NR_ReturnPeriodicityAndOffset(SubcarrierSpacing p_SubcarrierSpacing,

NTN_Satellite_Type p_SatelliteType := ntn_None

) return template (value) SchedulingRequestResourceConfig.periodicityAndOffset

{ //@sic R5s250100 added p_SatelliteType sic@

var template (value) SchedulingRequestResourceConfig.periodicityAndOffset v_PeriodicityAndOffset;

//As per 38.508-1 Table 4.6.3-157: SchedulingRequestResourceConfig

select (p_SubcarrierSpacing) {

case(kHz15)  {

if (p_SatelliteType == ntn_GSO) { //@sic R5s250100 sic@

v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl640;

} else {

v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl10;

}

}

case(kHz30)  { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl20; }

case(kHz120) { v_PeriodicityAndOffset := cs_NR_PeriodicityAndOffsetSl80; }

case else    { FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); }

}

return v_PeriodicityAndOffset;

}

template (value) SchedulingRequestResourceConfig cs_38508_SchedulingRequestResourceConfig(SubcarrierSpacing p_SubcarrierSpacing,

NTN_Satellite_Type p_SatelliteType := ntn_None ) :=

//As per 38.508-1 Table 4.6.3-118: SchedulingRequestResourceConfig

// @sic R5s250100: added p_SatelliteType sic@

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

cs_NR_SchedulingRequestResourceConfigDef(tsc_NR_SchedulingRequestResourceId, //Table 4.6.3-158: SchedulingRequestResourceId

0,

f_NR_ReturnPeriodicityAndOffset(p_SubcarrierSpacing, p_SatelliteType),

f_NR_InitSchedulingRequestResource());  //@sic R5-235279 sic@

//-----------------------------------------------------------------------------------------------------------------------

template (value) PUCCH_Config cs_38508_PUCCH_Config(SubcarrierSpacing p_SubcarrierSpacing,

PRB_Id p_PRB_Id := 0,

PRB_Id p_PRB_SecondHopPRBId := 0,

SSB_Index p_SSB_Index,

template (omit) PUCCH_Config.dl_DataToUL_ACK p_DL_DataToUL_ACK,

template (omit) PUCCH_Config.ul_AccessConfigListDCI_1_1_r16 p_Ul_AccessConfigListDCI_1_1 := omit, //@sic R5-225544 sic@

NTN_Satellite_Type p_SatelliteType := ntn_None

) :=

{ // As per 38.508-1 Table 4.6.3-112: PUCCH-Config

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

//@sic R5-203287 R5-214381: added p_DL_DataToUL_ACK R5s250100: added p_SatelliteType sic@

resourceSetToAddModList       := {cs_38508_PUCCH_ResourceSet0, cs_38508_PUCCH_ResourceSet1}, //@sic R5-198958 sic@

resourceSetToReleaseList      := omit,

resourceToAddModList          := {cs_38508_PUCCH_ResourceDef(0,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,0)),

cs_38508_PUCCH_ResourceDef(1,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,2)),

cs_38508_PUCCH_ResourceDef(2,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,4)),

cs_38508_PUCCH_ResourceDef(3,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,6)),

cs_38508_PUCCH_ResourceDef(4,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,8)),

cs_38508_PUCCH_ResourceDef(5,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,10)),

cs_38508_PUCCH_ResourceDef(6,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat0(0,2,12)),

cs_38508_PUCCH_ResourceDef(7,  p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat1(0,14,0,0)),

cs_38508_PUCCH_ResourceDef(8,  p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,0)),

cs_38508_PUCCH_ResourceDef(9,  p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,2)),

cs_38508_PUCCH_ResourceDef(10, p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,4)),

cs_38508_PUCCH_ResourceDef(11, p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,6)),

cs_38508_PUCCH_ResourceDef(12, p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,8)),

cs_38508_PUCCH_ResourceDef(13, p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,10)),

cs_38508_PUCCH_ResourceDef(14, p_PRB_Id, p_PRB_SecondHopPRBId-6, cs_38508_ResourceFormat2(6,2,12)),

cs_38508_PUCCH_ResourceDef(15, p_PRB_Id, p_PRB_SecondHopPRBId-1, cs_38508_ResourceFormat3(1,14,0))

},

resourceToReleaseList         := omit,

format1                       := { setup :=  cs_38508_PUCCH_FormatConfigFormat1 }, //@sic R5-200996 sic@

format2                       := { setup :=  cs_38508_PUCCH_FormatConfigFormat2 }, //@sic R5-200996 sic@

format3                       := { setup :=  cs_38508_PUCCH_FormatConfigFormat3 }, //@sic R5-200996 sic@

format4                       := omit,

schedulingRequestResourceToAddModList  := { cs_38508_SchedulingRequestResourceConfig(p_SubcarrierSpacing, p_SatelliteType)},

schedulingRequestResourceToReleaseList := omit,

multi_CSI_PUCCH_ResourceList           := omit,

dl_DataToUL_ACK                        := p_DL_DataToUL_ACK,

spatialRelationInfoToAddModList        := omit,

spatialRelationInfoToReleaseList       := omit,

pucch_PowerControl                     := cs_38508_PUCCH_PowerControl(p_SSB_Index),

resourceToAddModListExt_v1610          := omit,              /* @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

dl_DataToUL_ACK_r16                    := omit,              /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

ul_AccessConfigListDCI_1_1_r16         := p_Ul_AccessConfigListDCI_1_1,  /* @sic R5s201387 BASELINE MOVING 2020, R5-245544 sic@ */

subslotLengthForPUCCH_r16              := omit,              /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

dl_DataToUL_ACK_DCI_1_2_r16            := omit,              /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

numberOfBitsForPUCCH_ResourceIndicatorDCI_1_2_r16 := omit,   /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

dmrs_UplinkTransformPrecodingPUCCH_r16            := omit,   /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

spatialRelationInfoToAddModListSizeExt_v1610      := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

spatialRelationInfoToReleaseListSizeExt_v1610     := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

spatialRelationInfoToAddModListExt_v1610          := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

spatialRelationInfoToReleaseListExt_v1610         := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

resourceGroupToAddModList_r16                     := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

resourceGroupToReleaseList_r16                    := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

sps_PUCCH_AN_List_r16                             := omit,   /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

schedulingRequestResourceToAddModListExt_v1610    := omit,   /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */

format0_r17                                       := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

format2Ext_r17                                    := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

format3Ext_r17                                    := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

format4Ext_r17                                    := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

ul_AccessConfigListDCI_1_2_r17                    := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

mappingPattern_r17                                := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

powerControlSetInfoToAddModList_r17               := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

powerControlSetInfoToReleaseList_r17              := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

secondTPCFieldDCI_1_1_r17                         := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

secondTPCFieldDCI_1_2_r17                         := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

dl_DataToUL_ACK_r17                               := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

dl_DataToUL_ACK_DCI_1_2_r17                       := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

ul_AccessConfigListDCI_1_1_r17                    := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

schedulingRequestResourceToAddModListExt_v1700    := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

dmrs_BundlingPUCCH_Config_r17                     := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

dl_DataToUL_ACK_v1700                             := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

dl_DataToUL_ACK_MulticastDCI_Format4_1_r17        := omit,   /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

sps_PUCCH_AN_ListMulticast_r17                    := omit    /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

};

template (value) NR_UplinkBWP_Type cs_NR_UplinkBWP_Initial(SubcarrierSpacing               p_SubcarrierSpacing,

BWP.locationAndBandwidth        p_LocationAndBandwidth,

NR_Prach_RootSequenceIndex_Type p_Prach_RootSequenceIndex,

RACH_ConfigCommon.ssb_perRACH_OccasionAndCB_PreamblesPerSSB p_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB,

template (value) RACH_ConfigGeneric p_RACH_ConfigGeneric,

template (value) PUSCH_TimeDomainResourceAllocationList p_PUSCH_TimeDomainResourceAllocationList,

template (value) PUSCH_Config   p_PUSCH_Config,

template (omit) BWP_UplinkDedicated.srs_Config  p_SRS_Config,

SSB_Index p_SSB_Index,

PRB_Id p_PRB_SecondHopPRBId,

template (omit) PUCCH_Config.dl_DataToUL_ACK p_DL_DataToUL_ACK,

template (omit) BWP_UplinkDedicated.lbt_FailureRecoveryConfig_r16 p_LBT_FailureRecoveryConfig_r16 := omit,

template (omit) PUCCH_Config.ul_AccessConfigListDCI_1_1_r16 p_Ul_AccessConfigListDCI_1_1, //@sic R5-245544 sic@

NTN_Satellite_Type p_SatelliteType := ntn_None

) :=

{ /* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

//@sic R5-202820: added p_PRB_SecondHopPRBId R5-214381: added p_DL_DataToUL_ACK R5s250100: added p_SatelliteType sic@

Id := tsc_NR_BWP_Id,

Common := {

R15 := cs_38508_BWP_UplinkCommon(cs_38508_BWP (p_SubcarrierSpacing, p_LocationAndBandwidth),

cs_38508_RACH_ConfigCommon(p_Prach_RootSequenceIndex,

p_SubcarrierSpacing,

p_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB,

p_RACH_ConfigGeneric),

cs_38508_PUSCH_ConfigCommon(p_PUSCH_TimeDomainResourceAllocationList),

cs_38508_PUCCH_ConfigCommon)

},

Dedicated := {

R15 := cs_38508_BWP_UplinkDedicated(cs_38508_PUCCH_Config(p_SubcarrierSpacing, 0, p_PRB_SecondHopPRBId, p_SSB_Index, p_DL_DataToUL_ACK, p_Ul_AccessConfigListDCI_1_1, p_SatelliteType),//@sic R5-245544 sic@

p_PUSCH_Config,

p_SRS_Config,    /* @sic R5s190852 change 1: p_SRS_Config sic@ */

p_LBT_FailureRecoveryConfig_r16)

}

};

function f_NR_InitUL_BWPs(SubcarrierSpacing p_SubcarrierSpacing,

FreqBandIndicatorNR p_FrequencyBandIndicator,

NR_Prach_RootSequenceIndex_Type p_Prach_RootSequenceIndex,

SSB_Index p_SSB_Index,

integer p_ChannelBandwidth,

PRB_Id p_PRB_SecondHopPRBId,

NTN_Satellite_Type p_SatelliteType := ntn_None

) return template (value) NR_UplinkBWP_List_Type

{

//@sic R5-202820: added p_PRB_SecondHopPRBId  R5s250100: added p_SatelliteType sic@

var template (value) NR_UplinkBWP_List_Type v_UL_BWPs;

var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition;

var PUSCH_TimeDomainResourceAllocation.k2 v_K2forMsg3;

var template (value) PUSCH_TimeDomainResourceAllocationList v_PUSCH_TimeDomainResourceAllocationList;

var template (value) PUSCH_Config v_PUSCH_Config;

var template (omit) BWP_UplinkDedicated.srs_Config  v_SRS_Config := omit;  //@sic R5s200057 sic@

var BWP.locationAndBandwidth v_LocationAndBandwidth:= f_NR_InitLocationAndBandwidth(p_FrequencyBandIndicator, p_SubcarrierSpacing, p_ChannelBandwidth); //@sic R5s190134 change 4.3 sic@

var RACH_ConfigCommon.ssb_perRACH_OccasionAndCB_PreamblesPerSSB v_SSB_perRACH_Occasion;

var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric;

var integer v_C_SRS := 0;   //@sic R5-198860 sic@

var RACH_ConfigGeneric.ra_ResponseWindow v_RA_ResponseWindow := sl20; //@sic R5-199423 sic@

var template (omit) PUCCH_Config.dl_DataToUL_ACK v_DL_DataToUL_ACK := cs_38508_DL_DataToUL_ACK; //@sic As per 38.508-1 Table 4.6.3-112: PUCCH-Config R5-214381 sic@

var template (omit) BWP_UplinkDedicated.lbt_FailureRecoveryConfig_r16 v_LBT_FailureRecoveryConfig_r16 := omit;

var template (omit) PUCCH_Config.ul_AccessConfigListDCI_1_1_r16 v_Ul_AccessConfigListDCI_1_1_r16 := omit; //@sic R5-245544 sic@

var template (omit) PUSCH_Config.ul_AccessConfigListDCI_0_1_r16 v_Ul_AccessConfigListDCI_0_1_r16 := omit; //@sic R5-245544 sic@

//@sic R5-240531, R5w240005, R5-245544 sic@ - SharedSpectrum conditions

if (f_NR_IsBandSharedSpectrum(p_FrequencyBandIndicator)) {

v_LBT_FailureRecoveryConfig_r16 := cs_38508_LBT_FailureRecoveryConfigSetup; //in Table 4.6.3-15: BWP-UplinkDedicated

v_Ul_AccessConfigListDCI_1_1_r16 := cs_38508_Ul_AccessConfigListDCI_1_1; //in Table 4.6.3-112: PUCCH-Config

v_Ul_AccessConfigListDCI_0_1_r16 := cs_38508_Ul_AccessConfigListDCI_0_1; //in Table 4.6.3-118: PUSCH-Config

}

if (f_NR_IsBandFR1(p_FrequencyBandIndicator)) {

// FR1

select (p_SubcarrierSpacing) {   /* @sic R5-190723 sic@ */

case (kHz15) {v_K2forMsg3 := 2;}

case (kHz30) {v_K2forMsg3 := 6;}

}

v_DMRS_UL_AdditionalPosition := pos1;

v_SSB_perRACH_Occasion := { one := n8 }; //Table 4.6.3-128: RACH-ConfigCommon

//Set ra-ResponseWindow as per 38.508-1 Table 4.6.3-130: RACH-ConfigGeneric

if ( p_SubcarrierSpacing == kHz15) { v_RA_ResponseWindow := sl10; }

v_RACH_ConfigGeneric := cs_38508_RACH_ConfigGeneric_FR1(v_RA_ResponseWindow);

if (f_NR_IsBandFDD(p_FrequencyBandIndicator) and pc_halfDuplexFDD_TypeA_RedCap_r17) { //@sic R5-233406 sic@

v_RACH_ConfigGeneric := cs_38508_RACH_ConfigGeneric_FR1_HDUE(v_RA_ResponseWindow);

}

if (not (f_GetTestcaseAttrib_ShortDCI(testcasename()))) { //@sic R5s190892 R5s190895 sic@

//set c-SRS acc to Table 4.6.3-182: SRS-Config

if ( p_ChannelBandwidth == 100 ) { v_C_SRS := 63; }   //@sic R5-198860 sic@

v_SRS_Config := cs_NR_Srs_Config_Setup(cs_38508_SRS_Config_FR1(v_C_SRS, p_SSB_Index));

v_PUSCH_Config := cs_38508_PUSCH_Config_FR1(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), p_SSB_Index, v_Ul_AccessConfigListDCI_0_1_r16); //@sic R5-245544 sic@

} else { //TC using Short_DCI x_0

v_PUSCH_Config := cs_38508_PUSCH_Config_ShortDCI_FR1(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), p_SSB_Index);

}

} else {

// FR2

v_K2forMsg3 := 3;   /* @sic R5-190723 sic@ */

v_DMRS_UL_AdditionalPosition := pos0;

v_SSB_perRACH_Occasion := { one := n4 };  //Table 4.6.3-128: RACH-ConfigCommon @sic R5-187672 sic@

v_RACH_ConfigGeneric := cs_38508_RACH_ConfigGeneric_FR2;

if (not (f_GetTestcaseAttrib_ShortDCI(testcasename()))) { //@sic R5s190892 R5s190895 sic@

//set c-SRS acc to Table 4.6.3-182: SRS-Config

if ( p_ChannelBandwidth == 100 ) { v_C_SRS := 17; }  //@sic R5-198860 sic@

v_SRS_Config := cs_NR_Srs_Config_Setup(cs_38508_SRS_Config_FR2(v_C_SRS, p_SSB_Index));

v_PUSCH_Config := cs_38508_PUSCH_Config_FR2(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), p_SSB_Index);

} else { //TC using Short_DCI x_0

v_PUSCH_Config := cs_38508_PUSCH_Config_ShortDCI_FR2(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), p_SSB_Index);

}

}

if (f_GetTestcaseAttrib_ShortDCI(testcasename())){

//  As per 38.508-1 Table 4.6.3-112: PUCCH-Config condition Short_DCI

v_DL_DataToUL_ACK := omit;

}

v_PUSCH_TimeDomainResourceAllocationList := cs_38508_PUSCH_TimeDomainResourceAllocationList(v_K2forMsg3);   /* @sic R5-190723 sic@ */

v_UL_BWPs := {

cs_NR_UplinkBWP_Initial(p_SubcarrierSpacing, v_LocationAndBandwidth, p_Prach_RootSequenceIndex, v_SSB_perRACH_Occasion, v_RACH_ConfigGeneric, v_PUSCH_TimeDomainResourceAllocationList, v_PUSCH_Config, v_SRS_Config, p_SSB_Index, p_PRB_SecondHopPRBId, v_DL_DataToUL_ACK, v_LBT_FailureRecoveryConfig_r16, v_Ul_AccessConfigListDCI_1_1_r16, p_SatelliteType)    /* @sic R5s190852 change 2: v_SRS_Config, R5-245544 sic@ */

};

return v_UL_BWPs;

}

function f_NR_CellInfo_Init(NR_CellId_Type                p_NR_CellId,

template (value) NR_Frequencyf1tof4_Type p_NR_Frequencyf1tof4,

FreqBandIndicatorNR           p_BandIndicatorNR,

NR_SysinfoCombination_Type    p_NR_SysinfoCombination,

DeltaValues_Type              p_DeltaValues ) runs on NR_BASE_PTC

return NR_CellInfo_Type

{

var template (value) NR_CellInfo_Type v_NR_CellInfo;

var template (value) NR_NAS_Parameters_Type v_NAS_Parameters := fl_NR_InitNAS_forRRC_Cells(p_NR_CellId);

var NR_CellIdentities_Type  v_NR_CellIdentities := fl_NR_InitCellIdentities(p_NR_CellId);

var PhysCellId v_PhysCellId := v_NR_CellIdentities.PhysicalCellId;

var NR_FrequencyBand_Type v_NR_FrequencyBand := fl_NR_InitCellFrequency(p_NR_CellId, p_NR_Frequencyf1tof4, p_BandIndicatorNR);

var SubcarrierSpacing v_SSB_SubcarrierSpacing := v_NR_FrequencyBand.SSB_SubcarrierSpacing;

var SubcarrierSpacing v_Carrier_SubcarrierSpacing := v_NR_FrequencyBand.FrequencyInfoDL.scs_SpecificCarrierList[0].subcarrierSpacing;

var template (value) NR_DuplexMode_Type v_NR_DuplexMode := f_NR_InitDuplexMode(p_BandIndicatorNR, v_SSB_SubcarrierSpacing);

var CellTimingInfo_Type v_CellTimingInfo := f_NR_InitCellTiming(p_NR_CellId, p_BandIndicatorNR);

var SSB_Index v_SSB_Index := fl_NR_InitSSB_Index(p_NR_CellId);

var template (value) NR_DownlinkBWP_List_Type v_DL_BWPs := f_NR_InitDL_BWPs(v_PhysCellId, v_NR_FrequencyBand, v_SSB_Index);

var template (value) NR_CellSysInfo_Type v_NR_CellSysInfo;

var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex := f_NR_InitPrach_RootSequenceIndex(p_NR_CellId);

var template (omit) NR_UplinkBWP_List_Type v_UL_BWPs;

var NR_AbsoluteCellPower_Type v_MaxReferencePower;

var template (value) NR_SSB_BurstConfig_Type v_SSB_Burstconfig := f_NR_ConvertSSB_PositionsInBurst(v_SSB_Index, p_BandIndicatorNR, v_SSB_SubcarrierSpacing);

var template (value) NR_ServingCellConfig_Type v_NR_ServingCellConfig := f_NR_ServingCellConfig_Init();

var template (value) SSB_MTC v_SSB_MTC := f_NR_InitSSB_MTC(p_BandIndicatorNR);

var integer v_DeltaNRFs := fl_NR_InitCellDeltaValue(p_NR_CellId, p_DeltaValues);

var PRB_Id v_PRB_SecondHopPRBId;

var template (value) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig := cs_38508_RadioLinkMonitoringConfigDef(v_SSB_Index); //@sic R5-212409 sic@

var NR_TestEnvironment_Type v_NR_TestEnvironment := f_NR_MobileInfo_GetTestEnvironment(); //@sic R5s220477 sic@

var NTN_Satellite_Type v_SatelliteType := f_NR_MobileInfo_GetNTNSatelliteType(); //@sic R5-242320 sic@

var template (omit) NR_NTN_TimingAdvanceConfig_Type v_NtnTA := omit; //@sic R5-242320 sic@

//Initialise Maximum power reference as per 38.508-1 clause 6.2.2

if (v_NR_TestEnvironment == Conducted) {    //@sic R5s220477 sic@

v_MaxReferencePower := tsc_NR_ServingCellSSS_EPRE_FR1;

} else {

v_MaxReferencePower := tsc_NR_ServingCellSSS_EPRE_FR2;

}

if (not fl_IsUL_FrequencyAvailable(v_NR_FrequencyBand)) { //SDL or no UL @sic R5-226041 sic@

v_UL_BWPs := omit;

} else {

//Acc to 38.508-1 Table 4.6.3-109: PRB-Id

//Set to value of the number of RBs - 1 where number of RBs is found in Table 4.3.1.0D-1 or Table 4.3.1.0D-2 for the bandwidth and SCS configured

v_PRB_SecondHopPRBId := v_NR_FrequencyBand.FrequencyInfoUL.scs_SpecificCarrierList[0].carrierBandwidth;  //@sic R5-203287 sic@

v_UL_BWPs := f_NR_InitUL_BWPs(v_Carrier_SubcarrierSpacing, p_BandIndicatorNR, v_Prach_RootSequenceIndex, v_SSB_Index, v_NR_FrequencyBand.ChannelBandwidth, v_PRB_SecondHopPRBId, v_SatelliteType);

}

//Initialise NTN TA if needed

if (v_SatelliteType == ntn_GSO)

{//@sic R5-242320 sic@

v_NtnTA := cs_NTN_TimingAdvanceConfig(cs_NTN_SemistaticTA_GSO_Cell1);

}

else if (v_SatelliteType == ntn_NGSO)

{//@sic R5-243597 sic@

v_NtnTA := cs_NTN_TimingAdvanceConfig(cs_NTN_SemistaticTA_NGSO_Cell1);

}

v_NR_CellSysInfo := f_NR_InitSystemInformation(p_NR_SysinfoCombination,

v_NR_FrequencyBand,

v_Carrier_SubcarrierSpacing,

v_NAS_Parameters,

v_DL_BWPs,

v_UL_BWPs,

v_DeltaNRFs,

v_SSB_Index,

p_NR_CellId);

v_NR_CellInfo := cs_NR_CellInfoDef(v_NR_CellIdentities,

v_NR_FrequencyBand,

v_NR_CellSysInfo,

v_CellTimingInfo,

v_NR_DuplexMode,

v_Carrier_SubcarrierSpacing,

v_SSB_Burstconfig,

v_DL_BWPs,

v_UL_BWPs,

omit,                   //@sic R5s180640 change 1.4.2 sic@

v_MaxReferencePower,

v_NAS_Parameters,

v_NR_ServingCellConfig,

v_SSB_MTC,

v_NR_FrequencyBand.OffsetCarrierCORESET0,

v_RadioLinkMonitoringConfig,   //@sic R5-212409 sic@

v_NtnTA,    //@sic R5-242320 sic@

v_NR_FrequencyBand.ChannelBandwidth  //@sic R5-250309 sic@

);

return valueof(v_NR_CellInfo);

}

## Change 12

| Function name | f_TC_7_1_2_3_9a_NR5GC() |
| --- | --- |
| Reason for change | - The procedure of UE preconfiguration is already invokded by f_NR5GC_Preamble (). - For NR NTN GSO scenario, the value of t-PollRetransmit should take the NR NTN RTT(550ms) into account. |
| Summary of change | - Delete redundant UE preconfiguration procedure. - Configure the t-PollRetransmit = ms2000, For this, a Prose CR will be raised. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Rejected |

Before Change:

function f_TC_7_1_2_3_9a_NR5GC() runs on NR5GC_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DRB_Identity v_DRBId; var UL_t_PollRetransmit_Type v_T_PollRetransmit; f_NR5GC_Init(NR_28); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // preconfigure the UE location f_PreconfigureUE_ForNTN(UT); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // @sic R5s190312 ch. 4.3 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Establishing UL AM RLC parameters v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, f_NR_38508_SetSNFieldLengthAM(), v_T_PollRetransmit, // t_PollRetransmit -, -, -, f_NR_38508_SetSNFieldLengthAM(), ms200,// t_Reassembly -,// t_StatusProhibit -, ms2200);// t_ReassemblyExt_r17 f_NR5GC_RRC_ConnectedState3N_RLC_AM (v_DRBId, v_RLC_BearerConfigList[0], f_NR_38508_SetSNFieldLengthAM(), -, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId)); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); // @sic R5s190906 ch. 1 sic@ if (pc_supportOfRedCap_r17 == true) { v_RLC_Rec.AM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.2; v_RLC_Rec.t_ReassemblyExt_r17 := 2.2; f_NR_TestBody_Set(true); f_TC_7_1_2_3_9a_NR_TestBody (v_RLC_Rec, v_DRBId); f_NR_TestBody_Set(false); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s190312 ch. 4.3 sic@ // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); @sic R5s230130 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_7_1_2_3_9a_NR5GC() runs on NR5GC_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var DRB_Identity v_DRBId; var UL_t_PollRetransmit_Type v_T_PollRetransmit; f_NR5GC_Init(NR_28); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // preconfigure the UE location //f_PreconfigureUE_ForNTN(UT); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // @sic R5s190312 ch. 4.3 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Establishing UL AM RLC parameters v_T_PollRetransmit := ms2000; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, f_NR_38508_SetSNFieldLengthAM(), v_T_PollRetransmit, // t_PollRetransmit -, -, -, f_NR_38508_SetSNFieldLengthAM(), ms200,// t_Reassembly -,// t_StatusProhibit -, ms2200);// t_ReassemblyExt_r17 f_NR5GC_RRC_ConnectedState3N_RLC_AM (v_DRBId, v_RLC_BearerConfigList[0], f_NR_38508_SetSNFieldLengthAM(), -, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId)); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); // @sic R5s190906 ch. 1 sic@ if (pc_supportOfRedCap_r17 == true) { v_RLC_Rec.AM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.2; v_RLC_Rec.t_ReassemblyExt_r17 := 2.2; f_NR_TestBody_Set(true); f_TC_7_1_2_3_9a_NR_TestBody (v_RLC_Rec, v_DRBId); f_NR_TestBody_Set(false); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s190312 ch. 4.3 sic@ // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); @sic R5s230130 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 13

| Function name | f_TC_7_1_2_3_9a_NR_TestBody() |
| --- | --- |
| Reason for change | - The default value of ACK SN is 0, but according to the step2 and step 4 of Table 7.1.2.3.9a.3.2-1, the ACK SN of receiving message should be 1 - At step4, there are some duplicate TTCN code with step 2. - For different UEs, depend on its capability, some may need more time to send RLC STATUS PDU back after the t-ReassemblyExt-r17 expired. - t_WatchDog is not stopped after receiving the RLC STATUS PDU. |
| Summary of change | - Set the expected value of ACK SN to 1. - At step4, delete duplicate TTCN code. - Enlarge the timer t_WatchDog from 1s to 2s to compansate different UE capability. - Stop t_WatchDog after receiving the RLC STATUS PDU. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted 2. Accepted. Same code, can be saved. Same prose text. 3. Accepted in principle per prose CR agreement R5-251943. 4. Accepted. |

Before Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 2" siclog@ // Check: For 1 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(1.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-1.0); // 1 sec of step 2 has already elapsed ... // @siclog "Step 4" siclog@ // Check: For 1 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(1.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

After Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @siclog "Step 2" siclog@ // Check: For 1 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(1.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-1.0); // 1 sec of step 2 has already elapsed ... // @siclog "Step 4" siclog@ // Check: For 1 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // COMMENTED OUT v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; // COMMENTED OUT v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); // COMMENTED OUT v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

MCC160 Implementation:
implementing R5-251943

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @siclog "Step 2" siclog@ // Check: For 2 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-1.0); // 1 sec of step 2 has already elapsed ... // @siclog "Step 4" siclog@ // Check: For 2 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // COMMENTED OUT v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; // COMMENTED OUT v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); // COMMENTED OUT v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n55” using NIA1 Integrity and NEA0 ciphering algorithms on R&S 5G Conformance Test platform.

# Execution Log Files

## Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_7_1_2_3_9a_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250101: Supporting information for addition of NR-NTN testcase 7.1.2.3.9a |
| --- | --- |