3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250107

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3646 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 7.1.2.2.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 7.1.2.2.5a to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.2.2.5a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.5a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes mentioned in R5s250100 are needed in addition to those mentioned |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 7.1.2.2.5a	4

2.1	Change 1	4

2.2	Change 2	7

3	Branches executed	12

4	Execution Log Files	12

4.1	Unisoc UMS9632	12

5	References	12

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.2.2.5a which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.2.2.5a.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

# Corrections required to testcase 7.1.2.2.5a

Changes mentioned in R5s250100 are needed in addition to the below mentioned changes.

## Change 1

| Function name | f_TC_7_1_2_2_5a_NR5GC() |
| --- | --- |
| Reason for change | - The procedure of UE preconfiguration is already invokded by f_NR5GC_Preamble (). - According to the Table 7.1.2.2.5a.3.1-1 of 38.523-1, the value of t-Reassembly should be ms200. |
| Summary of change | - Delete redundant UE preconfiguration procedure. - Change the value of t-Reassembly from 0.02 to 0.2. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Accepted Changes 1-11 of R5s250100 apply ! |

Before Change:

function f_TC_7_1_2_2_5a_NR5GC() runs on NR5GC_PTC { // NR NTN / UM RLC / Receive window operation and t-Reassembly expiry var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var SN_FieldLengthUM v_DL_Len; var SN_FieldLengthUM v_UL_Len; var T_Reassembly v_T_Reassembly; var T_ReassemblyExt_r17 v_T_ReassemblyExt_r17; var DRB_Identity v_DRBId; f_NR5GC_Init(NR_28); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // preconfigure the UE location f_PreconfigureUE_ForNTN(UT); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); if (pc_um_WithShortSN) { v_DL_Len := size6; v_UL_Len := size6; v_RLC_Rec.UM_SN_Size := 6; } else { v_DL_Len := size12; v_UL_Len := size12; v_RLC_Rec.UM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.02; v_RLC_Rec.t_ReassemblyExt_r17 := 1.1; v_T_Reassembly := ms200; v_T_ReassemblyExt_r17 := ms1100; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM (v_DRBId, v_DL_Len, v_UL_Len, v_T_Reassembly, -, -, v_T_ReassemblyExt_r17); f_NR5GC_RRC_ConnectedState3N_RLC_UM (v_DRBId, v_RLC_BearerConfigList[0], v_DL_Len); // @sic R5s190548, R5-197250 sic@ f_NR_TestBody_Set(true); f_TC_7_1_2_2_5a_NR_TestBody(v_RLC_Rec, v_DRBId); f_NR_TestBody_Set(false); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s190312 ch. 4.3 sic@ // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); @sic R5s230130 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_7_1_2_2_5a_NR5GC() runs on NR5GC_PTC { // NR NTN / UM RLC / Receive window operation and t-Reassembly expiry var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var SN_FieldLengthUM v_DL_Len; var SN_FieldLengthUM v_UL_Len; var T_Reassembly v_T_Reassembly; var T_ReassemblyExt_r17 v_T_ReassemblyExt_r17; var DRB_Identity v_DRBId; f_NR5GC_Init(NR_28); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // preconfigure the UE location //f_PreconfigureUE_ForNTN(UT); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); if (pc_um_WithShortSN) { v_DL_Len := size6; v_UL_Len := size6; v_RLC_Rec.UM_SN_Size := 6; } else { v_DL_Len := size12; v_UL_Len := size12; v_RLC_Rec.UM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.2; v_RLC_Rec.t_ReassemblyExt_r17 := 1.1; v_T_Reassembly := ms200; v_T_ReassemblyExt_r17 := ms1100; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM (v_DRBId, v_DL_Len, v_UL_Len, v_T_Reassembly, -, -, v_T_ReassemblyExt_r17); f_NR5GC_RRC_ConnectedState3N_RLC_UM (v_DRBId, v_RLC_BearerConfigList[0], v_DL_Len); // @sic R5s190548, R5-197250 sic@ f_NR_TestBody_Set(true); f_TC_7_1_2_2_5a_NR_TestBody(v_RLC_Rec, v_DRBId); f_NR_TestBody_Set(false); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s190312 ch. 4.3 sic@ // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); @sic R5s230130 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 2

| Function name | f_TC_7_1_2_2_5a_NR_TestBody() |
| --- | --- |
| Reason for change | - According to the Table 7.1.2.2.5a.3.2-1 of 38.523-1, at step 2, wait for t-Reassembly to ensure t-Reassembly expires. the current TTCN code does not take the NR NTN GSO RTT(550ms) into account. For NR NTN GSO, the DL data at step1 need more time to transmit it. - According to the Table 7.1.2.2.5a.3.2-1 of 38.523-1, step1 and step3, The SN for UMD PDU is 0. - According to the Table 7.1.2.2.5a.3.2-1 of 38.523-1, at step 4, it depends on UE’s capability, some UEs need more time to loop back the RLC SDU#1. - According to the Table 7.1.2.2.5a.3.2-1 of 38.523-1, at step 6, wait for t-ReassemblyExt-r17 to ensure t-ReassemblyExt-r17 expires. the current TTCN code does not take the NR NTN GSO RTT(550ms) into account. For NR NTN GSO, the DL data at step5 need more time to transmit it. - According to the Table 7.1.2.2.5a.3.2-1 of 38.523-1, step 5 and step7, The SN for UMD PDU is 1. - According to the Table 7.1.2.2.5a.3.2-1 of 38.523-1, at step 8, it depends on UE’s capability, some UEs need more time to loop back the RLC SDU#2. |
| Summary of change | - At step2, include the value of NR NTN GSO RTT in the value of delay. - Change the SN from 2 to 0. - At step4, enlarge the t_WatchDog from 1.0 to 2.0. - Stop the t_WatchDog timer. - At step6, include the value of NR NTN GSO RTT in the value of delay. - Change the SN from 0 to 1. - At step8. enlarge the t_WatchDog from 1.0 to 2.0 - Stop the t_WatchDog timer. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted 2. Accepted 3. Accepted 4. Accepted 5. Accepted 6. Accepted 7. Accepted 8. Accepted |

Before Change:

function f_TC_7_1_2_2_5a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / UM RLC / Receive window operation and t-Reassembly expiry var integer v_RLC_Len1stSeg := 7; // Length of 1st segment var integer v_RLC_Len2ndSeg := 5; // Length of 2nd segment var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_DataField_Type v_NR_RLC_Data; timer t_WatchDog; v_NR_RLC_DataList := fl_NR_GenerateRLC_UM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 2); // @siclog "Step 1" siclog@ //The SS transmits UMD PDU#1 containing first segment of RLC SDU#1, SN=0. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); // @siclog "Step 2" siclog@ //Wait for t-Reassembly to ensure that t-Reassembly for the UMD PDU#1 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_Reassembly)); // @siclog "Step 3" siclog@ //The SS transmits UMD PDU#2 containing last segment of RLC SDU#1, SN=0. p_RLC_Rec.UM_TX_Next := 0; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 4" siclog@ //Check: For 1 sec after step 3, does the UE transmit RLC SDU#1? v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, substr(v_NR_RLC_DataList[tsc_RLC_SDU1], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU1]) - 3))); t_WatchDog.start(1.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} [] t_WatchDog.timeout {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4");} } // @siclog "Step 5" siclog@ //The SS transmits UMD PDU#3 containing first segment of RLC SDU#2, SN=1. p_RLC_Rec.UM_TX_Next := 1; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); // @siclog "Step 6" siclog@ //Wait for t-ReassemblyExt-r17 to ensure that t-ReassemblyExt-r17 for the UMD PDU#3 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)); // @siclog "Step 7" siclog@ //The SS transmits UMD PDU#4 containing last segment of RLC SDU#2, SN=1. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 8" siclog@ //Check: For 1 sec after step 7, does the UE transmit RLC SDU#2? v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, substr(v_NR_RLC_DataList[tsc_RLC_SDU2], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU2]) - 3))); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8");} [] t_WatchDog.timeout {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");} } }

After Change:

function f_TC_7_1_2_2_5a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / UM RLC / Receive window operation and t-Reassembly expiry var integer v_RLC_Len1stSeg := 7; // Length of 1st segment var integer v_RLC_Len2ndSeg := 5; // Length of 2nd segment var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_DataField_Type v_NR_RLC_Data; timer t_WatchDog; var integer v_Koffset; var float v_Tolerance; v_NR_RLC_DataList := fl_NR_GenerateRLC_UM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 2); // @siclog "Step 1" siclog@ //The SS transmits UMD PDU#1 containing first segment of RLC SDU#1, SN=0. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); v_Koffset := f_NR_CellInfo_GetKoffset (nr_Cell1); v_Tolerance := int2float(v_Koffset)/1000.0 + p_RLC_Rec.t_Reassembly; // @siclog "Step 2" siclog@ //Wait for t-Reassembly to ensure that t-Reassembly for the UMD PDU#1 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, v_Tolerance)); // @siclog "Step 3" siclog@ //The SS transmits UMD PDU#2 containing last segment of RLC SDU#1, SN=0. p_RLC_Rec.UM_TX_Next := 0; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 4" siclog@ //Check: For 1 sec after step 3, does the UE transmit RLC SDU#1? v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, substr(v_NR_RLC_DataList[tsc_RLC_SDU1], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU1]) - 3))); t_WatchDog.start(2.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_WatchDog.stop; } [] t_WatchDog.timeout {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4");} } // @siclog "Step 5" siclog@ //The SS transmits UMD PDU#3 containing first segment of RLC SDU#2, SN=1. p_RLC_Rec.UM_TX_Next := 1; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); v_Tolerance := int2float(v_Koffset)/1000.0 + p_RLC_Rec.t_ReassemblyExt_r17; // @siclog "Step 6" siclog@ //Wait for t-ReassemblyExt-r17 to ensure that t-ReassemblyExt-r17 for the UMD PDU#3 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, v_Tolerance)); // @siclog "Step 7" siclog@ //The SS transmits UMD PDU#4 containing last segment of RLC SDU#2, SN=1. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 8" siclog@ //Check: For 1 sec after step 7, does the UE transmit RLC SDU#2? v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, substr(v_NR_RLC_DataList[tsc_RLC_SDU2], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU2]) - 3))); t_WatchDog.start(2.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); t_WatchDog.stop; } [] t_WatchDog.timeout {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");} } }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n55” using NIA1 Integrity and NEA0 ciphering algorithms on R&S 5G Conformance Test platform.

# Execution Log Files

## Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_7_1_2_2_5a_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250108: Supporting information for addition of NR-NTN testcase 7.1.2.2.5a |
| --- | --- |