3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250109

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3647 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.3.1.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per 38.33.1cl 5.3.13.4, Quote [TS 38.331, clause 5.3.13.4] The UE shall: … 1> release the suspendConfig except the ran-NotificationAreaInfo; .... 1> if the RRCResume message includes the measConfig: 2> perform the measurement configuration procedure as specified in 5.5.2; 1> resume measurements if suspended; … … - submit the RRCResumeComplete message to lower layers for transmission; … Unquote - At step 4a4, SS transmits an RRCResume message without including MeasConfig . - But we need to note that ,at Step 4a1, SS transmits an RRCRelease message with suspendConfig -- so all the measurements are suspended at this point in time - on receiving resume message at step 4a4 , UE could be quick in sending measurement report before sending resume complete based on above specification snippet . - similar condition can occur in step 4a11 and 4a12 , where UE can be quick in sending measurement report on receiving resume message - Please note that , we have SS Configurations for reconfiguring SS to set DcchDtch on USS with DCI format 1_1 after receiving the resume complete message - UE could be quick in sending MeasurementReport while SS is doing the local end configurations after receiving the resume complete and TTCN waits for CNF for these message as well. It is proposed to add this default handler activation between resume and resume complete function so that these measurement reports can be consumed |  |  |  |  |  |  |  |  |
| Summary of change: |  | In between the sequence for resume and resume complete , default handler is added to handle the measurement reports sent by UE at steps 4a4 and 4a11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.23.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_3_1_23_TestBody() |
| --- | --- |
| Reason for change | [TS 38.331, clause 5.3.13.4] The UE shall: … 1> release the suspendConfig except the ran-NotificationAreaInfo; .... 1> if the RRCResume message includes the measConfig: 2> perform the measurement configuration procedure as specified in 5.5.2; 1> resume measurements if suspended; … … - submit the RRCResumeComplete message to lower layers for transmission; - At step 4a4, SS transmits an RRCResume message without including MeasConfig . - But we need to note that ,at Step 4a1, SS transmits an RRCRelease message with suspendConfig -- so all the measurements are suspended at this point in time - on receiving resume message at step 4a4 , UE could be quick in sending measurement report before sending resume complete based on above specification snippet . - similar condition can occur in step 4a11 and 4a12 , where UE can be quick in sending measurement report on receiving resume message - Please note that , we have SS Configurations for reconfiguring SS to set DcchDtch on USS with DCI format 1_1 after receiving the resume complete message - UE could be quick in sending MeasurementReport while SS is doing the local end configurations after receiving the resume complete and TTCN waits for CNF for these message as well. - It is proposed to add this default handler activation between resume and resume complete function so that these measurement reports can be consumed |
| Summary of change | In between the sequence for resume and resume complete , default handler is added to handle the measurement reports sent by UE |
| TTCN module | ttcn\develop\NR5GC\8_1_3\RRC_Measurement_NR5GC.ttcn |
| MCC160 Comment | Accepted (in principle), but implemented differently (please see MCC160 proposal) |

Before Change:

function f_TC_8_1_3_1_23_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR2 := -82; //@sic R5-234560 sic@ var NR_AbsoluteCellPower_Type v_CellPower_NeighbourCell_FR1_FR2 := -91; //@sic R5-234560 sic@ var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE := omit; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var float v_TimerValue_5s := 5.0; var default v_MyDefaultVar := null; //@sic R5s201450 sic@ //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.3.1.23.3.2-1 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * */ //Power levels of NR cell 1 and cell 2 FR2 acc. Table 8.1.3.1.23.3.2-2 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -82 ¦ -91 * */ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingCell_FR1, v_CellPower_ServingCell_FR2), //@sic R5-234560 sic@ cs_NR_CellPower(nr_Cell2, v_CellPower_NeighbourCell_FR1_FR2, v_CellPower_NeighbourCell_FR1_FR2)}; //@sic R5-234560 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR periodic measurement reporting. //The UE transmits an RRCReconfigrationComplete message v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 2A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-200803 sic@ //@siclog "Step 3" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); //EXCEPTION: Steps 4a1-4a14 describe behaviour that depends on UE configuration; //the "lower case letter" identifies a step sequence that takes place if inactiveState is configured if(pc_inactiveState){ //@siclog "Step 4a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ f_NR_RRC_InactiveDef(nr_Cell1); //@sic R5s201450 sic@ deactivate(v_MyDefaultVar); //@sic R5s201450 sic@ //@siclog "Step 4a2" siclog@ //The SS waits for 5 sec transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 4a3" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@siclog "Step 4a4" siclog@ //The SS transmits an RRCResume message without including MeasConfig. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 4a5" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 4a6" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-201356 sic@ //@siclog "Step 4a7" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 4a7"); v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s210271 sic@ //@siclog "Step 4a8" siclog@ //The SS transmits an RRCRelease message with suspendConfig f_NR_RRC_InactiveDef(nr_Cell1); deactivate(v_MyDefaultVar); //@sic R5s210271 sic@ //@siclog "Step 4a9" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); //@sic R5-204441 sic@ f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@sic R5-204441 sic@ //@siclog "Step 4a10" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@sic R5-204441 sic@ //@siclog "Step 4a11" siclog@ //The SS transmits an RRCResume message including MeasConfig. v_MeasIdConfigList := {cs_NR_MeasId_Config_id2_obj1_conf1}; //@sic R5-204441 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE, -, -, {tsc_NR_MeasId1}); //@sic R5s201450, R5-204441 sic@ f_NR_SendRRCResume_Def(nr_Cell1, -, -, omit, omit, v_MeasConfig); //@sic R5-204441 sic@ //@siclog "Step 4a12" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@sic R5-204441 sic@ //@siclog "Step 4a13" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-204441 sic@ //@siclog "Step 4a14" siclog@ //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId2, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450, R5-204441 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 4a14"); //@sic R5-204441 sic@ } //@sic R5-213113 sic@ //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@sic R5-204441 sic@ } // f_TC_8_1_3_1_23_TestBody

After Change:

function f_TC_8_1_3_1_23_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR2 := -82; //@sic R5-234560 sic@ var NR_AbsoluteCellPower_Type v_CellPower_NeighbourCell_FR1_FR2 := -91; //@sic R5-234560 sic@ var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE := omit; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var float v_TimerValue_5s := 5.0; var default v_MyDefaultVar := null; //@sic R5s201450 sic@ //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.3.1.23.3.2-1 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * */ //Power levels of NR cell 1 and cell 2 FR2 acc. Table 8.1.3.1.23.3.2-2 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -82 ¦ -91 * */ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingCell_FR1, v_CellPower_ServingCell_FR2), //@sic R5-234560 sic@ cs_NR_CellPower(nr_Cell2, v_CellPower_NeighbourCell_FR1_FR2, v_CellPower_NeighbourCell_FR1_FR2)}; //@sic R5-234560 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR periodic measurement reporting. //The UE transmits an RRCReconfigrationComplete message v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 2A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-200803 sic@ //@siclog "Step 3" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); //EXCEPTION: Steps 4a1-4a14 describe behaviour that depends on UE configuration; //the "lower case letter" identifies a step sequence that takes place if inactiveState is configured if(pc_inactiveState){ //@siclog "Step 4a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ f_NR_RRC_InactiveDef(nr_Cell1); //@sic R5s201450 sic@ deactivate(v_MyDefaultVar); //@sic R5s201450 sic@ //@siclog "Step 4a2" siclog@ //The SS waits for 5 sec transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 4a3" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@siclog "Step 4a4" siclog@ //The SS transmits an RRCResume message without including MeasConfig. f_NR_SendRRCResume_Def(nr_Cell1); v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ //WA#WI=1391727 //@siclog "Step 4a5" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); deactivate(v_MyDefaultVar); //@sic R5s201450 sic@ //WA#WI=1391727 //@siclog "Step 4a6" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-201356 sic@ //@siclog "Step 4a7" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 4a7"); v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@siclog "Step 4a8" siclog@ //The SS transmits an RRCRelease message with suspendConfig f_NR_RRC_InactiveDef(nr_Cell1); deactivate(v_MyDefaultVar); //@sic R5s210271 sic@ //@siclog "Step 4a9" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); //@sic R5-204441 sic@ f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@sic R5-204441 sic@ //@siclog "Step 4a10" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@sic R5-204441 sic@ //@siclog "Step 4a11" siclog@ //The SS transmits an RRCResume message including MeasConfig. v_MeasIdConfigList := {cs_NR_MeasId_Config_id2_obj1_conf1}; //@sic R5-204441 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE, -, -, {tsc_NR_MeasId1}); //@sic R5s201450, R5-204441 sic@ f_NR_SendRRCResume_Def(nr_Cell1, -, -, omit, omit, v_MeasConfig); //@sic R5-204441 sic@ v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ //WA#WI=1391727 //@siclog "Step 4a12" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@sic R5-204441 sic@ //WA#WI=1392197 deactivate(v_MyDefaultVar);//WA#WI=1392197 //@siclog "Step 4a13" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-204441 sic@ //@siclog "Step 4a14" siclog@ //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId2, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450, R5-204441 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 4a14"); //@sic R5-204441 sic@ } //@sic R5-213113 sic@ //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@sic R5-204441 sic@ } // f_TC_8_1_3_1_23_TestBody

MCC160 proposal:

function f_TC_8_1_3_1_23_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR2 := -82; //@sic R5-234560 sic@ var NR_AbsoluteCellPower_Type v_CellPower_NeighbourCell_FR1_FR2 := -91; //@sic R5-234560 sic@ var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE := omit; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var float v_TimerValue_5s := 5.0; var default v_MyDefaultVar := null; //@sic R5s201450 sic@ //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.3.1.23.3.2-1 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * */ //Power levels of NR cell 1 and cell 2 FR2 acc. Table 8.1.3.1.23.3.2-2 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -82 ¦ -91 * */ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingCell_FR1, v_CellPower_ServingCell_FR2), //@sic R5-234560 sic@ cs_NR_CellPower(nr_Cell2, v_CellPower_NeighbourCell_FR1_FR2, v_CellPower_NeighbourCell_FR1_FR2)}; //@sic R5-234560 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR periodic measurement reporting. //The UE transmits an RRCReconfigrationComplete message v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 2A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-200803 sic@ //@siclog "Step 3" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); //EXCEPTION: Steps 4a1-4a14 describe behaviour that depends on UE configuration; //the "lower case letter" identifies a step sequence that takes place if inactiveState is configured if(pc_inactiveState){ //@siclog "Step 4a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ f_NR_RRC_InactiveDef(nr_Cell1); //@sic R5s201450 sic@ deactivate(v_MyDefaultVar); //@sic R5s201450 sic@ //@siclog "Step 4a2" siclog@ //The SS waits for 5 sec transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 4a3" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@siclog "Step 4a4" siclog@ //The SS transmits an RRCResume message without including MeasConfig. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 4a5" siclog@ //The UE transmits an RRCResumeComplete message. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete)); //The UE transmits an RRCResumeComplete message //f_NR_RRCResumeComplete_Def(nr_Cell1); v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s250109 sic@ // Reconfigure SS - set DcchDtch on USS with DCI format 1_1 //@sic R5s240461 sic@ f_NR_SS_ReconfigSStoUSS_DCI1_1(nr_Cell1); //@sic R5s250109 sic@ // Reconfigure RACH procedure as for initial access. @sic R5s240440 sic@ f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@sic R5s250109 sic@ deactivate(v_MyDefaultVar); //@sic R5s250109 sic@ //@siclog "Step 4a6" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-201356 sic@ //@siclog "Step 4a7" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 4a7"); v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@siclog "Step 4a8" siclog@ //The SS transmits an RRCRelease message with suspendConfig f_NR_RRC_InactiveDef(nr_Cell1); deactivate(v_MyDefaultVar); //@sic R5s210271 sic@ //@siclog "Step 4a9" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); //@sic R5-204441 sic@ f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@sic R5-204441 sic@ //@siclog "Step 4a10" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@sic R5-204441 sic@ //@siclog "Step 4a11" siclog@ //The SS transmits an RRCResume message including MeasConfig. v_MeasIdConfigList := {cs_NR_MeasId_Config_id2_obj1_conf1}; //@sic R5-204441 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE, -, -, {tsc_NR_MeasId1}); //@sic R5s201450, R5-204441 sic@ f_NR_SendRRCResume_Def(nr_Cell1, -, -, omit, omit, v_MeasConfig); //@sic R5-204441 sic@ //The UE transmits an RRCResumeComplete message //f_NR_RRCResumeComplete_Def(nr_Cell1); //@sic R5-204441 sic@ //@siclog "Step 4a12" siclog@ //The UE transmits an RRCResumeComplete message SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete)); //@sic R5s250109 sic@ v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s250109 sic@ // Reconfigure SS - set DcchDtch on USS with DCI format 1_1 //@sic R5s240461 sic@ f_NR_SS_ReconfigSStoUSS_DCI1_1(nr_Cell1); //@sic R5s250109 sic@ // Reconfigure RACH procedure as for initial access. @sic R5s240440 sic@ f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@sic R5s250109 sic@ deactivate(v_MyDefaultVar); //@sic R5s250109 sic@ //@siclog "Step 4a13" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-204441 sic@ //@siclog "Step 4a14" siclog@ //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId2, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450, R5-204441 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 4a14"); //@sic R5-204441 sic@ } //@sic R5-213113 sic@ //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@sic R5-204441 sic@ } // f_TC_8_1_3_1_23_TestBody