3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250162

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3664 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.3.2b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following changes are needed for ENDC, for the case when pc_configuredUL_GrantType1 as "TRUE” as was also done for NR5GC - The UE should be allowed to act upon the transmitted CG-PUSCH Type 1 for the configured DRBs by setting the IE configuredGrantType1Allowed to TRUE in the ul_SpecificParameters in the peer signalling message as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - The parameter p_T1 should be 40 according to Table 7.1.1.3.2b.3.2-3 and the parameter p_T2 should be set to 0 so that normal DCI based cyclic UL grants are not transmitted as the UE has to use the CG-PUSCH Type 1 grants to loop back the data - Further, the Co-ordination message containing the RRC Reconfiguration message at Step 15 to release UL configured grant type 1 needs to be handled at EUTRA PTC and the subsequent “complete” co-ordination message should be sent back to NR PTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - configuredGrantType1 is set to TRUE for DRB1 and DRB3 as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - The parameter p_T1 is set to 40 and p_T2 is set to 0. - Co-ordination messages to receive “Reconfig” message at EUTRA PTC and send “Complete” message to NR PTC are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail this test case in ENDC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2b.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_2b_ENDC_NR |
| --- | --- |
| Reason for change | The following changes are needed for ENDC, for the case when pc_configuredUL_GrantType1 as "TRUE” as was also done for NR5GC - The UE should be allowed to act upon the transmitted CG-PUSCH Type 1 for the configured DRBs by setting the IE configuredGrantType1Allowed to TRUE in the ul_SpecificParameters in the peer signalling message as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - The parameter p_T1 should be 40 according to Table 7.1.1.3.2b.3.2-3 and the parameter p_T2 should be set to 0 so that normal DCI based cyclic UL grants are not transmitted as the UE has to use the CG-PUSCH Type 1 grants to loop back the data |
| Summary of change | - configuredGrantType1 is set to TRUE for DRB1 and DRB3 as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - The parameter p_T1 is set to 40 and p_T2 is set to 0. |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC TF160 Comments | Accepted |

Before Change:

function f_TC_7_1_1_3_2b_ENDC_NR() runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := f_NR_CellInfo_GetUplinkBWP_LocationAndBandwidth(nr_Cell1); //@sic R5-243537 sic@ v_BwpSize := f_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := f_NR_RIV_Calculate(v_BwpSize, 23, 0); <<SKIPPED CODE>> v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s220053 sic@ omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); <<SKIPPED CODE>> //@sic steps 11-14 sic@ //@sic R5s220053 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,360,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B),true); //T1 =0 to C-RNTI grant. <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_2b_ENDC_NR() runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ //WA# v_RLC_BearerConfigList[0]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,-,-,-,true_)) ; v_RLC_BearerConfigList[1]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,-,-,-,-)) ; v_RLC_BearerConfigList[2]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,-,-,-, true_)) ; v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := f_NR_CellInfo_GetUplinkBWP_LocationAndBandwidth(nr_Cell1); //@sic R5-243537 sic@ v_BwpSize := f_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := f_NR_RIV_Calculate(v_BwpSize, 23, 0); <<SKIPPED CODE>> v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s220053 sic@ v_RLC_BearerConfigList, //WA# omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); <<SKIPPED CODE>> //@sic steps 11-14 sic@ //@sic R5s220053 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,40,0,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B),true); //T1 =0 to C-RNTI grant. WA# <<SKIPPED CODE>>

## Change 2

| Function name | f_TC_7_1_1_3_2b_ENDC_EUTRA |
| --- | --- |
| Reason for change | Further, the Co-ordination message containing the RRC Reconfiguration message at Step 15 to release UL configured grant type 1 needs to be handled at EUTRA PTC and the subsequent “complete” co-ordination message should be sent back to NR PTC |
| Summary of change | Co-ordination messages to receive “Reconfig” message at EUTRA PTC and send “Complete” message to NR PTC are added |
| TTCN module | MAC_ENDC_EUTRA.ttcn |
| MCC TF160 Comments | Accepted |

Before Change:

function f_TC_7_1_1_3_2b_ENDC_EUTRA() runs on EUTRA_5GS_PTC { <<SKIPPED CODE>> //@siclog steps 9-10 siclog@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig); }; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_2b_ENDC_EUTRA() runs on EUTRA_5GS_PTC { <<SKIPPED CODE>> //@siclog steps 9-10 siclog@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig); //WA# v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig); }; <<SKIPPED CODE>>