3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250168

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3665 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 9.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 9.4.1.1 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.4.1.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes mentioned in R5s250100 are needed in addition to those mentioned |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 9.4.1.1	4

2.1	Change 1	4

2.2	Change 2	5

3	Branches executed	9

4	Execution Log Files	9

4.1	Unisoc UMS9632	9

5	References	9

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of NR5GC test case 9.4.1.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	9.4.1.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

- Corrections required to test case 9.4.1.1

  - Change 1

| Function name | f_TC_9_4_1_1_NR5GC |
| --- | --- |
| Reason for change | The procedure of UE preconfiguration is already invokded by f_NR5GC_Preamble (). |
| Summary of change | Delete redundant UE preconfiguration procedure. |
| TTCN module | MobilityManagement_NTN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_9_4_1_1_NR5GC() runs on NR5GC_PTC { //NR NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" //@sic R5-243556 sic@ //@sic R5-244411 sic@ //@sic R5-244665 sic@ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellG := f_NR5GC_MapNASCell(ngc_CellG, NG_CellsOnDifferentPLMN); f_NR5GC_Init_NAS(NR_29, NG_CellsOnDifferentPLMN, -, {v_NGC_NASCellG}); // The pre-configured UE location is defined in TS 38.508-1 Clause 4.5C f_NR_PreconfigureNTN_UE(); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellG); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); fl_TC_9_4_1_1_Body(); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

After Change:

function f_TC_9_4_1_1_NR5GC() runs on NR5GC_PTC { //NR NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" //@sic R5-243556 sic@ //@sic R5-244411 sic@ //@sic R5-244665 sic@ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellG := f_NR5GC_MapNASCell(ngc_CellG, NG_CellsOnDifferentPLMN); f_NR5GC_Init_NAS(NR_29, NG_CellsOnDifferentPLMN, -, {v_NGC_NASCellG}); // The pre-configured UE location is defined in TS 38.508-1 Clause 4.5C //f_NR_PreconfigureNTN_UE(); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellG); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); fl_TC_9_4_1_1_Body(); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

  - Change 2

| Function name | fl_TC_9_4_1_1_Body |
| --- | --- |
| Reason for change | 1. Before step 15, UE already does registration on ngc_CellA, so at step 15, NW should send Registration Reject with SecurityHeaderType = tsc_SHT_IntegrityProtected_Ciphered. 2. At step 15, UE receives Registration Reject, so UE clears all security information including ngKSI.so at step 19, UE sends REGISTRATION REQUEST with SecurityHeaderType = tsc_SHT_NoSecurityProtection. |
| Summary of change | 1. At step 15, change the SecurityHeaderType from tsc_SHT_NoSecurityProtection to tsc_SHT_IntegrityProtected_Ciphered. 2. At step 19, change the SecurityHeaderType from tsc_SHT_IntegrityProtected to tsc_SHT_NoSecurityProtection. |
| TTCN module | MobilityManagement_NTN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_9_4_1_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellG := f_NR5GC_MapNASCell(ngc_CellG, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellG); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); //var PLMN_Identity v_ASN_PLMN_CellA := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); //var NAS_PlmnId v_PLMN_CellA := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); //var template (value) NG_NAS_DL_Message_Type v_MsgToSend; timer t_Timer := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 30.0); //@siclog "Step 1" siclog@ //Set NGC Cell G to serving cell power and NGC Cell A to Non-suitable cell power f_NR_SetCellPower (v_NGC_NASCellG, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Steps 3-14" siclog@ //Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellG, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellG, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_Secure, NOT_SUPPORTED, 1); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellG, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_Cause_PLMN_NotAllowedToOperateAtPresentUELocation), -, -, -, -, -, -, -, -, cs_GprsTimer3('3A'O, tsc_Gprs3TimerUnit_30sec, '00001'B))))); //@siclog "Step 16" siclog@ f_NR_RRCRelease(v_NGC_NASCellG); //@siclog "Step 17" siclog@ t_Timer.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellG, cr_38508_RRCSetupRequest(?, ?))) // assuming this is the RRC Connection to transmit the REGISTRATION REQUEST { t_Timer.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } [] t_Timer.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); } } //@siclog "Step 18" siclog@ //Set NGC Cell A to serving cell power and NGC Cell G to Non-suitable cell power f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellG, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 19 - 35a1" siclog@ // @sic R5-245564 sic@ t_Timer.start(30.0); alt{ [] t_Timer.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 19"); } [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest))) { // assuming this is the RRC Connection to transmit the REGISTRATION REQUEST t_Timer.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, rrcConnectionRelease, Initial_Secure, -, '00'O, -, '0000'B); //@siclog "Step 36" siclog@ f_UT_DeleteNTN_PLMNsNotAllowed(UT); } //End of fl_TC_9_4_1_1_Body

After Change:

function fl_TC_9_4_1_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellG := f_NR5GC_MapNASCell(ngc_CellG, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellG); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); //var PLMN_Identity v_ASN_PLMN_CellA := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); //var NAS_PlmnId v_PLMN_CellA := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); //var template (value) NG_NAS_DL_Message_Type v_MsgToSend; timer t_Timer := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 30.0); //@siclog "Step 1" siclog@ //Set NGC Cell G to serving cell power and NGC Cell A to Non-suitable cell power f_NR_SetCellPower (v_NGC_NASCellG, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Steps 3-14" siclog@ //Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellG, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellG, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_Secure, NOT_SUPPORTED, 1); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellG, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_Cause_PLMN_NotAllowedToOperateAtPresentUELocation), //WI = 1405876 -, -, -, -, -, -, -, -, cs_GprsTimer3('3A'O, tsc_Gprs3TimerUnit_30sec, '00001'B))))); //@siclog "Step 16" siclog@ f_NR_RRCRelease(v_NGC_NASCellG); //@siclog "Step 17" siclog@ t_Timer.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellG, cr_38508_RRCSetupRequest(?, ?))) // assuming this is the RRC Connection to transmit the REGISTRATION REQUEST { t_Timer.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } [] t_Timer.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); } } //@siclog "Step 18" siclog@ //Set NGC Cell A to serving cell power and NGC Cell G to Non-suitable cell power f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellG, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 19 - 35a1" siclog@ // @sic R5-245564 sic@ t_Timer.start(30.0); alt{ [] t_Timer.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 19"); } [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest))) { // assuming this is the RRC Connection to transmit the REGISTRATION REQUEST t_Timer.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); //WI = 1405876 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { //WI = 1405876 f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, rrcConnectionRelease, Initial_NoSecurity, -, '00'O, -, '0000'B); //WI = 1405876 //@siclog "Step 36" siclog@ f_UT_DeleteNTN_PLMNsNotAllowed(UT); } //End of fl_TC_9_4_1_1_Body

- Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA1 Integrity and NEA1 ciphering algorithms in GSO scenario on R&S 5G Conformance Test platform.

- Execution Log Files

  - Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_9_4_1_1_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s250169: Supporting information for addition of NR-NTN testcase 9.4.1.1 |
| --- | --- |