3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250170

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3666 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN testcase 9.4.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 9.4.1.3 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.4.1.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes mentioned in R5s250100 are needed in addition to those mentioned |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	1

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 9.4.1.3	4

2.1	Change 1	4

2.2	Change 2	5

3	Branches executed	8

4	Execution Log Files	8

4.1	Unisoc UMS9632	8

5	References	8

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of NR5GC test case 9.4.1.3 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	9.4.1.3.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Unisoc UMS9632 UE

Verification Status:	PASS

- Corrections required to test case 9.4.1.3

  - Change 1

| Function name | f_TC_9_4_1_3_NR5GC |
| --- | --- |
| Reason for change | The procedure of UE preconfiguration is already invokded by f_NR5GC_Preamble (). |
| Summary of change | Delete redundant UE preconfiguration procedure. |
| TTCN module | MobilityManagement_NTN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_9_4_1_3_NR5GC() runs on NR5GC_PTC { //NR NTN / TAI(s) not allowed / forbidden TAI(s) in the Registration Accept message //@sic R5-243558 sic@ //@sic R5-244665 sic@ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_29, -, -, {v_NGC_NASCellB}); // The pre-configured UE location is defined in TS 38.508-1 Clause 4.5C f_NR_PreconfigureNTN_UE(); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_4_1_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Postamble f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A); } // end of f_TC_9_4_1_3_NR5GC

After Change:

function f_TC_9_4_1_3_NR5GC() runs on NR5GC_PTC { //NR NTN / TAI(s) not allowed / forbidden TAI(s) in the Registration Accept message //@sic R5-243558 sic@ //@sic R5-244665 sic@ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_29, -, -, {v_NGC_NASCellB}); // The pre-configured UE location is defined in TS 38.508-1 Clause 4.5C //f_NR_PreconfigureNTN_UE(); //WI = 1406299 //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_4_1_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Postamble f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A); } // end of f_TC_9_4_1_3_NR5GC

  - Change 2

| Function name | fl_TC_9_4_1_3_NR5GC_TestBody |
| --- | --- |
| Reason for change | - Before step 15, UE already does registration on ngc_CellA, so at step 15, NW should send Registration Reject with SecurityHeaderType = tsc_SHT_IntegrityProtected_Ciphered. - From Step 19, UE need do registration on ngc_CellB. - At step 15, UE receives Registration Reject, so UE clears all security information including ngKSI.so at step 19 UE sends REGISTRATION REQUEST with SecurityHeaderType = tsc_SHT_NoSecurityProtection. |
| Summary of change | - At step 15, change the SecurityHeaderType from tsc_SHT_NoSecurityProtection to tsc_SHT_IntegrityProtected_Ciphered. - Change cell id from v_NGC_NASCellA to v_NGC_NASCellB. - At step 19, change the SecurityHeaderType from tsc_SHT_IntegrityProtected to tsc_SHT_NoSecurityProtection. |
| TTCN module | MobilityManagement_NTN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_9_4_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; // main behaviour // @siclog "Step 1" siclog@ // The SS configures:- NGC Cell A as the "Serving cell ".- NGC Cell B as "Non-suitable "Off" cell". f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 2" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT); // @siclog "Step 3-14" siclog@ // Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF, Initial_Secure); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #12 "Tracking area not allowed". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_Cause_TA_NotAllowed))))); // @siclog "Step 16" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(v_NGC_NASCellA); // @siclog "Step 17" siclog@ // Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 30 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 30.0); // @siclog "Step 18" siclog@ // The SS reconfigures:- NGC cell B as the "Serving cell".- NGC cell A as a "Non-suitable "Off" cell". f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 19" siclog@ // Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell B? SRB.check(receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellB, cr_38508_RRCSetupRequest))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19: UE triggers RRC Connection establishment to transmit the REGISTRATION REQUEST"); f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19: REGISTRATION RREQUEST Message is received"); // @siclog "Steps 20-35a1" siclog@ // Steps 5 to 20a1 from procedure in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellB, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); } // end of fl_TC_9_4_1_3_NR5GC_TestBody }//End of Module

After Change:

function fl_TC_9_4_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; // main behaviour // @siclog "Step 1" siclog@ // The SS configures:- NGC Cell A as the "Serving cell ".- NGC Cell B as "Non-suitable "Off" cell". f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 2" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT); // @siclog "Step 3-14" siclog@ // Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF, Initial_Secure); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #12 "Tracking area not allowed". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_Cause_TA_NotAllowed))))); //WI = 1406299 // @siclog "Step 16" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(v_NGC_NASCellA); // @siclog "Step 17" siclog@ // Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 30 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 30.0); // @siclog "Step 18" siclog@ // The SS reconfigures:- NGC cell B as the "Serving cell".- NGC cell A as a "Non-suitable "Off" cell". f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 19" siclog@ // Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell B? SRB.check(receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellB, cr_38508_RRCSetupRequest))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19: UE triggers RRC Connection establishment to transmit the REGISTRATION REQUEST"); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); //WI = 1406299 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19: REGISTRATION RREQUEST Message is received"); // @siclog "Steps 20-35a1" siclog@ // Steps 5 to 20a1 from procedure in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellB, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); } // end of fl_TC_9_4_1_3_NR5GC_TestBody }//End of Module

- Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA1 Integrity and NEA1 ciphering algorithms in GSO scenario on R&S 5G Conformance Test platform.

- Execution Log Files

  - Unisoc UMS9632

The Unisoc UMS9632 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_9_4_1_3_Unisoc.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s250171: Supporting information for addition of NR-NTN testcase 9.4.1.3 |
| --- | --- |