3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250175

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3668 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Idle Mode test case 6.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 6.4.2.1 is failing while executing in manual mode due to parallel handling of AT Command and cell power level changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a UT command for Conformation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | The test case 6.4.2.1 is failing while executing in manual mode due to parallel handling of AT Command and cell power level changes. |
| Summary of change | Adding a UT command for Conformation. |
| TTCN module | Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 R5-212084 R5s210581 R5s220736 R5-233462 sic@ var integer v_T1_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; /***************** Skipping the code *************************************/ // @siclog "Step 5" siclog@ // The UE is switched off. f_UT_SwitchOffUE(UT, false); // @siclog "Steps 6/6A - Void" siclog@ // @siclog "Step 6B" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_Msg4Based( f_NR_CellInfo_GetPhysicalParameters(nr_Cell1), cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup))); // @siclog "Step 6C" siclog@ // The UE is switched on. f_NR5GC_SwitchOnAndResetIMS(); /***************** Skipping the code *************************************/ }

After Change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 R5-212084 R5s210581 R5s220736 R5-233462 sic@ var integer v_T1_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; /***************** Skipping the code *************************************/ // @siclog "Step 5" siclog@ // The UE is switched off. f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); // @siclog "Steps 6/6A - Void" siclog@ // @siclog "Step 6B" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_Msg4Based( f_NR_CellInfo_GetPhysicalParameters(nr_Cell1), cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup))); // @siclog "Step 6C" siclog@ // The UE is switched on. f_NR5GC_SwitchOnAndResetIMS(); /***************** Skipping the code *************************************/ }