3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250186

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3677 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC MAC testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In the Function f_TC_7_1_1_1_2_NR_TestBody1 , During Step 2 of the MAC PDU transmission, the PDU will undergo RLC retransmissions. This is due to the configuration of the TTCN which instructs the SS to ignore Scheduling Requests and refrain from allocating any uplink grants. Successful PRACH attempts are expected to occur only at Step 17, and this is when the UE is anticipated to transmit an RLC acknowledgment (ACK) for this MAC PDU, and also effectively looping back the PDU. In the context of FR1 FDD, a PRACH configuration index of 119 is used. This results in PRACH preamble transmissions occurring in a radio frame that meets the condition nSFN mod 8 = 1. It is important to note that the RLC ACK is expected to be sent by the UE using the PRACH procedure, which is triggered by the UE for radio frames that satisfy the condition nSFN mod 8 = 1 (i.e., every 8 System Frame Numbers (SFNs)). The current TTCN configuration sets the v_T_PollRetransmit timer to a default value of 80 ms for DRB. This leads to the RLC ACK being received outside the v_T_PollRetransmit window. To prevent unexpected RLC retransmissions of this MAC PDU, it is recommended to adjust the v_T_PollRetransmit timer to 120 milliseconds for this scenario. So that we ensure , RLC ACK is received well within the v_T_PollRetransmit timer expiry - At Step 17A, a reconfiguration message is sent, prompting UE to initiate the PRACH procedure to complete the reconfiguration process. The v_T_PollRetransmit timer is set to 45 milliseconds for Signaling Radio Bearers (SRBs). Based on the PRACH configuration index used for FR1 FDD, the UE will transmit PRACH for every radio frame that meets the condition nSFN mod 8 = 1 (i.e., every 8 System Frame Numbers (SFNs)). Since the RLC acknowledgment (ACK) is received outside the 45-millisecond v_T_PollRetransmit window, the network will continue retransmitting the reconfiguration message. Upon receiving the C-RNTI uplink grant on completing the PRACH procedure, further in response to the RLC retransmission of the reconfiguration message, the UE will attempt to send a scheduling request to transmit the RLC ACK. If SR retransmissions expire, the UE will proceed with the PRACH procedure. However, the current TTCN configuration enables PRACH and SR indications within the function f_TC_7_1_1_1_2_NR_TestBody2, also stopping UL grant configurations.This may cause TTCN to detect unexpected system indications due to ongoing RLC retransmissions of the reconfiguration message. To address this instability, it is recommended to introduce a slight delay between the function calls f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext) and f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId). This delay will allow RLC retransmissions to settle down before entering the function f_TC_7_1_1_1_2_NR_TestBody2, thereby enabling PRACH and SR indications more reliably. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to have DRB v_T_PollRetransmit to 120 milliseconds - It is proposed to introduce a slight delay between the function calls f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext) and f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId), allowing RLC retransmissions to stabilize before entering the function f_TC_7_1_1_1_2_NR_TestBody2 and enabling PRACH and SR indications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testacse will remain unstable and unfairly fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_1_2_NR5GC |
| --- | --- |
| Reason for change | - In the Function f_TC_7_1_1_1_2_NR_TestBody1 , During Step 2 of the MAC PDU transmission, the PDU will undergo RLC retransmissions. This is due to the configuration of the TTCN which instructs the SS to ignore Scheduling Requests and refrain from allocating any uplink grants. Successful PRACH attempts are expected to occur only at Step 17, and this is when the UE is anticipated to transmit an RLC acknowledgment (ACK) for this MAC PDU, and also effectively looping back the PDU. In the context of FR1 FDD, a PRACH configuration index of 119 is used. This results in PRACH preamble transmissions occurring in a radio frame that meets the condition nSFN mod 8 = 1. It is important to note that the RLC ACK is expected to be sent by the UE using the PRACH procedure, which is triggered by the UE for radio frames that satisfy the condition nSFN mod 8 = 1 (i.e., every 8 System Frame Numbers (SFNs)). The current TTCN configuration sets the v_T_PollRetransmit timer to a default value of 80 ms for DRB. This leads to the RLC ACK being received outside the v_T_PollRetransmit window. To prevent unexpected RLC retransmissions of this MAC PDU, it is recommended to adjust the v_T_PollRetransmit timer to 120 milliseconds for this scenario. So that we ensure , RLC ACK is received well within the v_T_PollRetransmit timer expiry - At Step 17A, a reconfiguration message is sent, prompting UE to initiate the PRACH procedure to complete the reconfiguration process. The v_T_PollRetransmit timer is set to 45 milliseconds for Signaling Radio Bearers (SRBs). Based on the PRACH configuration index used for FR1 FDD, the UE will transmit PRACH for every radio frame that meets the condition nSFN mod 8 = 1 (i.e., every 8 System Frame Numbers (SFNs)). Since the RLC acknowledgment (ACK) is received outside the 45-millisecond v_T_PollRetransmit window, the network will continue retransmitting the reconfiguration message. Upon receiving the C-RNTI uplink grant on completing the PRACH procedure, further in response to the RLC retransmission of the reconfiguration message, the UE will attempt to send a scheduling request to transmit the RLC ACK. If SR retransmissions expire, the UE will proceed with the PRACH procedure. However, the current TTCN configuration enables PRACH and SR indications within the function f_TC_7_1_1_1_2_NR_TestBody2, also stopping UL grant configurations.This may cause TTCN to detect unexpected system indications due to ongoing RLC retransmissions of the reconfiguration message. To address this instability, it is recommended to introduce a slight delay between the function calls f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext) and f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId). This delay will allow RLC retransmissions to settle down before entering the function f_TC_7_1_1_1_2_NR_TestBody2, thereby enabling PRACH and SR indications more reliably. |
| Summary of change | - It is proposed to have DRB v_T_PollRetransmit to 120 milliseconds - It is proposed to introduce a slight delay between the function calls f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext) and f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId), allowing RLC retransmissions to stabilize before entering the function f_TC_7_1_1_1_2_NR_TestBody2 and enabling PRACH and SR indications. |
| TTCN module | ttcn\develop\NR5GC\7_1_1\MAC_NR5GC.ttcn |
| MCC TF160 comments | - Implemented as per R5-252244 - Accepted |

Before Change:

function f_TC_7_1_1_1_2_NR5GC () runs on NR5GC_PTC { // Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself var DRB_Identity v_DRBId ; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric ; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) RACH_ConfigCommon.ssb_perRACH_OccasionAndCB_PreamblesPerSSB v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //@sic R5s190842 sic@ var NR_DRBInfoList_Type v_ExistingDRBs; var integer i :=0; //@sic R5s201454 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_NR5GC_Init(NR_1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tag_Config.tag_ToAddModList[0].timeAlignmentTimer := ms750; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //@sic R5s190842 sic@ v_ExistingDRBs:= f_NR5GC_MobileInfo_GetDRBInfoList(); for(i :=0;i<lengthof(v_ExistingDRBs);i :=i+1) { v_ExistingDRBs[i].SS_Config.Config.AddOrReconfigure.Sdap:={None :=true}; } f_NR5GC_MobileInfo_SetDRBInfoList(v_ExistingDRBs); //Bring UE to State 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_TDD; } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { //@sic R5-233408 sic@ v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_HDFDD; } else //FR1 FDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_FDD; } } else { // FR2 v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR2; } v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s200043 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190842 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; //@sic R5s240378 sic@ //v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdcch_ConfigCommon.setup.searchSpaceSIB1 := omit; //@sic Step 0AA //Update the same in Sys info //@sic R5s200883 sic@ f_NR_CellInfo_SetUplinkConfigCommon(nr_Cell1, v_RACH_ConfigGeneric, 42, v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB, cs_GroupBConfigured_Def, sf48); f_NR_CellInfo_SetUplinkBWP_List(nr_Cell1, v_NR_UplinkBWP_List); f_NR_SS_ConfigureSysinfo(nr_Cell1, - , f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); // Configure SS to handle CBRA with UL Grant as a result of Reconfiguration with Sync //@sic R5s220673 R5-223419 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE //@sic R5s190842 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); //@sic R5s200549 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_2_NR_TestBody1(v_DRBId); //@sic Step 17A-17B //@sic R5s201454 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_1_2_NR5GC () runs on NR5GC_PTC { // Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself var DRB_Identity v_DRBId ; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric ; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) RACH_ConfigCommon.ssb_perRACH_OccasionAndCB_PreamblesPerSSB v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //@sic R5s190842 sic@ var NR_DRBInfoList_Type v_ExistingDRBs; var integer i :=0; //@sic R5s201454 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var UInt_Type v_LCID1; //WA#WI=1405342 var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //WA#WI=1405342 var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; //WA#WI=1405342 var T_PollRetransmit v_T_PollRetransmit := ms120; //WA#WI=1405342 //Init Cell parameters f_NR5GC_Init(NR_1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tag_Config.tag_ToAddModList[0].timeAlignmentTimer := ms750; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //@sic R5s190842 sic@ v_ExistingDRBs:= f_NR5GC_MobileInfo_GetDRBInfoList(); for(i :=0;i<lengthof(v_ExistingDRBs);i :=i+1) { v_ExistingDRBs[i].SS_Config.Config.AddOrReconfigure.Sdap:={None :=true}; } f_NR5GC_MobileInfo_SetDRBInfoList(v_ExistingDRBs); v_LCID1 :=v_DRBId+3; //WA#WI=1405342 v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB_LCID( v_LCID1, v_DRBId, cs_38508_RLC_Config_AM_DRB(cs_38508_UL_AM_RLC(v_T_PollRetransmit), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB) ; //WA#WI=1405342 v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None,-,v_T_PollRetransmit)}; //WA#WI=1405342 //Bring UE to State 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, -, -, //p_IpPduDelayTime v_RLC_BearerConfigList, //WA#WI=1405342 -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList //WA#WI=1405342 ); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_TDD; } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { //@sic R5-233408 sic@ v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_HDFDD; } else //FR1 FDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_FDD; } } else { // FR2 v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR2; } v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s200043 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190842 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; //@sic R5s240378 sic@ //v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdcch_ConfigCommon.setup.searchSpaceSIB1 := omit; //@sic Step 0AA //Update the same in Sys info //@sic R5s200883 sic@ f_NR_CellInfo_SetUplinkConfigCommon(nr_Cell1, v_RACH_ConfigGeneric, 42, v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB, cs_GroupBConfigured_Def, sf48); f_NR_CellInfo_SetUplinkBWP_List(nr_Cell1, v_NR_UplinkBWP_List); f_NR_SS_ConfigureSysinfo(nr_Cell1, - , f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); // Configure SS to handle CBRA with UL Grant as a result of Reconfiguration with Sync //@sic R5s220673 R5-223419 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE //@sic R5s190842 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); //@sic R5s200549 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_2_NR_TestBody1(v_DRBId); //@sic Step 17A-17B //@sic R5s201454 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_Delay(0.1); //WA#WI=1405342 added small delay to allow the reconfiguration retx to finish f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };