3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250195

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4909 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to CR R5s240293 implemented currently in TTCN, the subframe 4 is used to configure UL grant for pc_ue_CategoryDL_M1 branch while subframe 8 is used for LTE branch. Which is unreasonable because according to LTE TDD Configuartion 1, subframe 8 is UL that can’t allocate UL grant which will cause LTE TDD bands test failure. Besides, according to 36523-3 cl 7.26, subframe 8 is usually used to locate UL grant for BL/CE UE. So the subframe for UL grant need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change subframe 8 for pc_ue_CategoryDL_M1 branch and subframe 4 for LTE branch. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformat UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | Correction to function f_TC_7_1_6_2_EUTRA |
| --- | --- |
| Reason for change | According to CR R5s240293 implemented currently in TTCN, the subframe 4 is used to configure UL grant for pc_ue_CategoryDL_M1 branch while subframe 8 is used for LTE branch. Which is unreasonable because according to LTE TDD Configuartion 1, subframe 8 is UL that can’t allocate UL grant which will cause LTE TDD bands test failure. Besides, according to 36523-3 cl 7.26, subframe 8 is usually used to locate UL grant for BL/CE UE. So the subframe for UL grant need to be corrected. |
| Summary of change | Change subframe 8 for pc_ue_CategoryDL_M1 branch and subframe 4 for LTE branch. |
| TTCN module | \LTE\7_1\MAC_716.ttcn |
| MCC160 Comments | Accepted |

Before change

| function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception */ //@sic R5s110173 R5-113679 sic@ const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@ const integer tsc_OnDurationTimer := 20; const integer tsc_DRXRetranTimer := 6; const integer tsc_SubFrameX := 6; var integer v_SubFrameY; //@sic R5s240293 sic@ // var integer v_SubFrameZ; var integer v_SubFrameGrant; var integer v_NormalSF := 0; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var SubFrameTiming_Type v_Timing2 := v_Timing; var SubFrameTiming_Type v_Timing3 := v_Timing; var SubFrameTiming_Type v_Timing4 := v_Timing; var SubFrameTiming_Type v_Timing5 := v_Timing; var integer v_HARQRTTTimer := 0; var octetstring v_EncodedRlcPdu; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; /////////////////////////////////////////////////////////////////////////////////// // PREAMBLE ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_Init(c1); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1); f_EUTRA_CellConfig_Def (eutra_Cell1); f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA); f_EUTRA_AS_CipheringAlgorithm_Set(eea0); // set null AS ciphering algorithm //f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1); // creates cell and performs registration //@sic R5-104796 R5-106675DRX support on SS mandated sic@ //f_EUTRA_GenericRbEst_Common_716( eutra_Cell1, 0, 0, omit, cs_MAC_MainConfig_Explicit_RBC_DrxL(0), omit, omit, omit, 5); //@sic R5-112665 sic@ // Period is set as 4 = 80% of 6ms the Inactivity Timer //@sic R5s100466 change of period for TA from 16 to 4 sic@ //f_EUTRA_CloseUE_TestLoopModeA( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) ); //f_Delay(0.66); //@sic R5s100466 add timer for TA MCE to be sent once sic@ //f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def); // Go to state 4 to return no data in UL // step to configure SS RLC/PDCP and MAC in No header manipulation mode on SS Side for DRB //f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // to configure SS to report reception of HARQ ACK or NACK //f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable ); //@sic R5-104796 R5-106675 DRX support on SS mandated sic@ f_EUTRA_LoopBackActivation_State4_Def( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) ); /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); //@sic R5s240293 sic@ if(pc_ue_CategoryDL_M1){ v_SubFrameY := 1; // v_SubFrameZ := 2; v_SubFrameGrant := 4; } else { v_SubFrameY := 3; // v_SubFrameZ := 4; v_SubFrameGrant := 8; } |  |
| --- | --- |

After change

| function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception */ //@sic R5s110173 R5-113679 sic@ const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@ const integer tsc_OnDurationTimer := 20; const integer tsc_DRXRetranTimer := 6; const integer tsc_SubFrameX := 6; var integer v_SubFrameY; //@sic R5s240293 sic@ // var integer v_SubFrameZ; var integer v_SubFrameGrant; var integer v_NormalSF := 0; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var SubFrameTiming_Type v_Timing2 := v_Timing; var SubFrameTiming_Type v_Timing3 := v_Timing; var SubFrameTiming_Type v_Timing4 := v_Timing; var SubFrameTiming_Type v_Timing5 := v_Timing; var integer v_HARQRTTTimer := 0; var octetstring v_EncodedRlcPdu; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; timer t_Watchdog := 5.0; /////////////////////////////////////////////////////////////////////////////////// // PREAMBLE ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_Init(c1); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1); f_EUTRA_CellConfig_Def (eutra_Cell1); f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA); f_EUTRA_AS_CipheringAlgorithm_Set(eea0); // set null AS ciphering algorithm //f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1); // creates cell and performs registration //@sic R5-104796 R5-106675DRX support on SS mandated sic@ //f_EUTRA_GenericRbEst_Common_716( eutra_Cell1, 0, 0, omit, cs_MAC_MainConfig_Explicit_RBC_DrxL(0), omit, omit, omit, 5); //@sic R5-112665 sic@ // Period is set as 4 = 80% of 6ms the Inactivity Timer //@sic R5s100466 change of period for TA from 16 to 4 sic@ //f_EUTRA_CloseUE_TestLoopModeA( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) ); //f_Delay(0.66); //@sic R5s100466 add timer for TA MCE to be sent once sic@ //f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def); // Go to state 4 to return no data in UL // step to configure SS RLC/PDCP and MAC in No header manipulation mode on SS Side for DRB //f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // to configure SS to report reception of HARQ ACK or NACK //f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable ); //@sic R5-104796 R5-106675 DRX support on SS mandated sic@ f_EUTRA_LoopBackActivation_State4_Def( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) ); /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); //@sic R5s240293 sic@ if(pc_ue_CategoryDL_M1){ v_SubFrameY := 1; // v_SubFrameZ := 2; v_SubFrameGrant := 8; } else { v_SubFrameY := 3; // v_SubFrameZ := 4; v_SubFrameGrant := 4; } |  |
| --- | --- |